3GPP TSG RAN Meeting #110	RP-253284

Baltimore, USA, December 8-11, 2025

Agenda Item:	8.2.1.4

Source:	Huawei, HiSilicon, VODAFONE

Title:	Consideration on support of Massive Communication IoT

Document for:	Discussion and Decision

# Introduction

In newly updated TR 38.914 [1], the minimum requirements for 6G IoT are as follows:

The 6GR and 6G RAN architecture shall support the following minimum requirements for Massive Communication (IoT):

- 6G Massive Communication (IoT) shall be supported for FR1.

  - 6GR should have a common/scalable design that supports the above usage scenario in addition to eMBB

    - Prioritize 6GR design for eMBB

  - The above usage scenario should not overlap with Ambient IoT and NB-IoT

- [PHY or MAC] [minimum] peak data rate is [TBD] Mbps in DL and [TBD] Mbps in UL for lowest-tier device.

Editor note:	“6G should support coexistence with NB-IoT (all deployment modes) and eMTC via semi-static configuration” is moved to 5.2 (migration and architecture)

In this contribution, further considerations and views on massive IoT communication are discussed. The text proposal to the design target for 6G IoT is also provided.

# 6GR IoT

## Background

Both 5G NR and LTE support a wide range of IoT technologies, such as NR Rel-17/18 RedCap, as well as LTE Cat-1, Cat-1 bis, Cat-4, NB-IoT, and Cat-M. These IoT device categories differ in bandwidth capability, peak data rate, antenna configuration, coverage requirements, cost, battery life, and other factors, thereby meeting different application scenarios. Figure 1 shows the landscape of diverse IoT types of 5G and prior.

Figure 1. The landscape of diverse IoT types of 5G and prior

As mentioned in TR 38.914 [1], 6G targets a common/scalable design that can support IoT scenario in addition to eMBB, while minimizing operators’ cost and implementation complexity. Therefore, the design target for IoT devices needs to be discussed and decided.

## Consideration on design target for IoT devices

According to agreements from RAN#109, the candidate IoT devices to be supported mainly refer to the RedCap-like devices and eMTC-like devices. The characteristics of these two IoT devices from 4G/5G are summarized as below:

- 5G (e)RedCap has similar design targets as LTE Cat 4/1(bis), which typically has 20 MHz bandwidth, and addresses IoT applications such as the wearables, monitoring cameras, and smart grid sensors. These applications usually do not require coverage extension.

- eMTC has a smaller bandwidth of 1.4 MHz and provides up to 10dB coverage extension, which addresses IoT applications such as smart metering and smart home scenarios in the indoor environments.

It can be seen that the major design constraints for IoT devices are the smallest maximum channel bandwidth, and extended coverage. These two aspects have a fundamental impact on initial access design as well. Additionally, the number of antennas is also influential to the coverage performance and complexity of IoT devices. Therefore, a detailed analysis of these aspects is provided below.

### Smallest Maximum Bandwidth for IoT Devices

NR RedCap has been introduced in Rel-17, with 20 MHz bandwidth design. eRedCap has been introduced in Rel-18 to reduce the baseband bandwidth to 5 MHz while keeping 20 MHz for RF bandwidth. During the (e)RedCap study, evaluations of complexity reduction regarding 100 MHz, 20 MHz and 5 MHz was summarized in Table 1 below from TR 38.865 [2].

Note that the reference Rel-15 NR UE has the following features:

- FDD: 100 MHz, 2 Rx, 2 layers, DL 256QAM, FD-FDD

- TDD: 100 MHz, 4 Rx, 4 layers, DL 256QAM

The main UE bandwidth reduction options considered for FR1 are as follows:

- Option BW1: Both RF and BB bandwidths are 5 MHz for UL and DL.

- Option BW2: 5 MHz BB bandwidth for all signals and channels with 20 MHz RF bandwidth for UL and DL.

- Option BW3: 5 MHz BB bandwidth only for PDSCH (for both unicast and broadcast) and PUSCH with 20 MHz RF bandwidth for UL and DL. The other physical channels and signals are still allowed to use a BWP up to the 20 MHz maximum UE RF+BB bandwidth.

Table 1: Average relative UE complexity (RF+BB) achieved by BW reduction

| Options | Rel-15 reference | Rel-17 RedCap | BW1 | BW2 | BW3 |
| --- | --- | --- | --- | --- | --- |
| FD-FDD 1Rx | ## 100% | ## 44.74% | ## 39.44% | ## 40.65% | ## 41.15% |
| TDD 1Rx | ## 100% | ## 31.41% | ## 27.88% | ## 28.88% | ## 29.01% |
| HD-FDD 1Rx | ## 100% | ## 39.07% | ## 33.58% | ## 34.41% | ## 35.60% |
| FD-FDD 2Rx | ## 100% | ## 65.24% | ## 55.90% | ## 57.76% | ## 59.55% |
| TDD 2Rx | ## 100% | ## 43.99% | ## 38.08% | ## 40.12% | ## 40.61% |
| HD-FDD 2Rx | ## 100% | ## 58.99% | ## 50.26% | ## 51.79% | ## 53.57% |

It can be seen from Table 1 that when the bandwidth is reduced from 100 MHz to 20 MHz, the complexity can be reduced by about 35%~70% depending on the specific duplex and antenna options, while further reduction of bandwidth from 20 MHz to 5 MHz only gives about ~10% more relative complexity reduction: e.g., in option HD-FDD 1Rx, ~61% complexity reduction can be achieved from 100 MHz to 20 MHz, but only ~66% complexity reduction from 100 MHz to 5 MHz.

On the other hand, compared with 20 MHz, UEs with 5 MHz bandwidth need to support more repetitions to reach the same coverage target. For example, as analyzed in TR 38.865 [2], for SIB1 reception, transmitting over 48RBs (17.28 MHz with 30kHz SCS) versus being limited to 11RBs (3.96 MHz with 30kHz SCS) shows a performance gap of about 8~9dB. To achieve the same coverage, the latter requires 8~10 times more repetitions as shown in Figure 2, which impacts overall system efficiency, especially energy efficiency from both network side and UE side. Similarly, the bandwidth of SSB also affects the coverage. Based on our analysis, the transmission of 5MHz SSB needs to be repeated ~6 times to achieve the same coverage as the 20MHz SSB. In summary to support 5MHz instead of 20MHz as the smallest maximum channel bandwidth does not bring significant complexity reduction for IoT devices, but on the other hand degrades the system performance, e.g. coverage loss.

Figure 2. The relation between coverage and SIB1 bandwidth according to TR 38.865

Regarding bandwidth even smaller than 5 MHz, e.g., 3 MHz, 1.4 MHz, similar analysis as the above applies. The relative complexity reduction will become even smaller compared with 5 MHz, while the additional repetitions required for coverage would further degrades system performance, especially the energy efficiency from both network and UE sides. Therefore, supporting IoT devices with bandwidth capability below 20 MHz is not considered as a good option.

In [3], statistics on the global shipment volume of IoT modules are presented as Figure 3 below. It can be seen that LTE Cat-1 bis has occupied a significant ratio of the market and is expected to maintain its position for a long time, while Rel-17 RedCap is expected to experience rapid growth in the coming years, gradually increasing its market share. Both LTE Cat-1 bis and Rel-17 RedCap has a bandwidth capability of 20 MHz.

Figure 3. Statistics on the global shipment volume of IoT modules

In conclusion, considering trade-off among device complexity, system performance and market demands, it is proposed that IoT devices in 6G support a smallest maximum channel bandwidth of 20 MHz. In this case, the network can have more efficient resource allocation to accommodate different spectrum resources: on bands where there are spectrum resources larger than 20MHz, the network can configure at least 20MHz for initial access for better performance, and there is no problem that IoT devices can access to the network; on bands where spectrum resources are smaller than 20MHz, e.g., 5MHz, these IoT devices are still capable of accessing to the network. Hence the initial access can avoid fragmented design as there is no need to support various kinds of small bandwidth for IoT devices.

Proposal 1: The lowest-tier devices for 6G, i.e. IoT devices, support 20 MHz as the smallest maximum bandwidth for both RF and baseband.

Coverage Target of IoT devices

For IoT devices, the need to support coverage extension depends on the application requirements and the actual deployment scenarios. In some cases, applications require coverage extension due to indoor deployment.

Some applications require moderate coverage extension, e.g., up to 154 dB MCL, which was supported by eMTC. The number of repetitions for a 5~10dB coverage extension will be 8~32 times on an FDD spectrum band, and it may require more repetitions on TDD bands, therefore the impact on system performance needs to be analyzed carefully before determining the exact target. In SSB and SIB1 transmission, IoT devices can enhance the coverage through multi-period combining reception, without the need for network to provide repetitions. However, this will result in an increase to the access latency of IoT devices and require larger modification period for systems information. For example, SSB and SIB1 transmissions can achieve 154 dB coverage extension by combining 16 receptions. On one hand, the access latency for IoT devices increases by 16 times; on the other hand, the system needs to ensure that 16 consecutive system messages remain unchanged, which may also increase the delay for eMBB devices to get the updated system information. Alternatively, the network can make repetitions for SSB and SIB1 transmission, but the system overhead and access latency will also be increased for eMBB devices. Therefore, it is worth studying a reasonable coverage extension target (e.g. 5~10dB) considering both the market needs and performance impact for eMBB services.

Another aspect to consider for coverage target is the deployment bands. Coverage extension for IoT devices is not desirable to apply to all bands, considering the large path-loss differences across different bands. For example, for sub-3 GHz FDD spectrum, the propagation loss is relatively small, and existing macro cells operating on these bands can already provide relatively good coverage performance. However, for bands around 7 GHz, due to additional propagation loss, there may already be a coverage gap considering co-site deployment with mid-Band (~2.6GHz, ~3.5GHz). Support of IoT devices in such bands could make the coverage gap even larger. Thus, whether IoT devices need to be deployed around 7 GHz needs further study. In any case, it is desirable to consider some common techniques that applies to all device types for coverage improvement to avoid solution fragmentation.

Proposal 2: The exact coverage extension target (e.g., 5~10dB) for IoT devices needs to be studied to address market demands, with performance impact on eMBB services (e.g. access latency, system overhead) to be evaluated and minimized.

Proposal 3: Coverage extension for 6G IoT devices should prioritize sub-3 GHz spectrum, and the potential applicability to TDD bands, e.g. around 7 GHz, requires further study.

Antennas of IoT devices

The demand for peak rate, capacity, and spatial multiplexing gain in IoT scenarios is much lower than in eMBB and more advanced device type(s), so a reduction in the number of antennas can significantly reduce cost. Meanwhile, the number of antennas should also consider the performance of 6G IoT devices.

- The RF cost savings and BB cost savings from reducing the number of UE Rx branches was summarized in Table 2 below from TR 38.875 [4]. It can be seen from Table 2 that when the number of UE Rx branches is reduced from 4Rx to 1/2Rx, the cost can be reduced by over 30%.

Table 2: Estimated relative device cost for reduced number of UE Rx branches

| Reduced number of UE Rx branches | 4Rx | 2Rx | 1Rx |
| --- | --- | --- | --- |
| ## RF: Power amplifier | ## 25.0% | ## 25.0% | ## 25.0% |
| ## RF: Filters | ## 15% | ## 7.6% | ## 3.9% |
| ## RF: Transceiver (including LNAs, mixer, and local oscillator) | ## 55% | ## 30.4% | ## 17.8% |
| ## RF: Duplexer / Switch | ## 5% | ## 4.9% | ## 4.9% |
| # RF: Total relative cost | # 100% | # 67.9% | # 51.6% |
| ## BB: ADC / DAC | ## 9% | ## 5.2% | ## 3.4% |
| ## BB: FFT/IFFT | ## 4% | ## 2.2% | ## 1.3% |
| ## BB: Post-FFT data buffering | ## 10% | ## 5.3% | ## 3.0% |
| ## BB: Receiver processing block | ## 29% | ## 15.7% | ## 9.0% |
| ## BB: LDPC decoding | ## 9% | ## 8.7% | ## 8.6% |
| ## BB: HARQ buffer | ## 12% | ## 11.6% | ## 11.4% |
| ## BB: DL control processing & decoder | ## 4% | ## 4.0% | ## 3.9% |
| ## BB: Synchronization / cell search block | ## 9% | ## 4.8% | ## 2.7% |
| ## BB: UL processing block | ## 5% | ## 5.0% | ## 5.0% |
| ## BB: MIMO specific processing blocks | ## 9% | ## 7.9% | ## 7.3% |
| # BB: Total relative cost | # 100% | # 70.4% | # 55.7% |
| RF+BB: Total relative cost | 100% | 69.4% | 54.0% |

- On the other hand, degradation of downlink performance is expected when reducing the number of Rx branches, which is illustrated in Table 3 from TR 38.875 [4]. It can be seen that when the number of Rx branches is reduced from 4Rx to 2Rx, the coverage loss can be 5~6.6dB. A further reduction to 1Rx increases the loss to 9.1 ~ 11.1dB relative to the 4Rx reference UE. Thus, the potential impact on eMBB services needs to be carefully evaluated, and proper access control mechanism should be studied as well to avoid impact on network performance.

Table 3: Coverage loss between 4Rx RedCap UE and 2Rx/1Rx RedCap UE at 2.6Ghz

|  | PDCCH CSS | PDCCH USS | PDSCH | Msg2 | Msg4 | PBCH |
| --- | --- | --- | --- | --- | --- | --- |
| ## 4Rx vs 2Rx | ## 5.8dB | ## 5.8dB | ## 6.2dB | ## 6.6dB | ## 6.2dB | ## 5dB |
| ## 4Rx vs 1Rx | ## 9.2dB | ## 9.8dB | ## 9.6dB | ## 11.1dB | ## 9.7dB | ## 9.1dB |

Proposal 4: Both 1Rx and 2Rx can be considered for 6G IoT devices, with the proper access control mechanism studied to avoid impact on network performance.

# Conclusion

According to the above discussions, we have the following proposals:

Proposal 1: The lowest-tier devices for 6G, i.e. IoT devices, support 20 MHz as the smallest maximum bandwidth for both RF and baseband.

Proposal 2: The exact coverage extension target (e.g., 5~10dB) for IoT devices needs to be studied to address market demands, with performance impact on eMBB services (e.g. access latency, system overhead) to be evaluated and minimized.

Proposal 3: Coverage extension for 6G IoT devices should prioritize sub-3 GHz spectrum, and the potential applicability to TDD bands, e.g. around 7GHz, requires further study.

Proposal 4: Both 1Rx and 2Rx can be considered for 6G IoT devices, with the proper access control mechanism studied to avoid impact on network performance.

# References

TR 38.914, Study on 6G Scenarios and Requirements (Release 20)

TR 38.865, Study on further NR RedCap UE complexity reduction (Release 18)

https://iotbusinessnews.com/download/white-papers/TSR-2024-2025-Cellular-Broadband-Device-Module-Market-Summary.pdf

TR 38.875, Study on support of reduced capability NR devices (Release 17)