3GPP TSG RAN Meeting #110														RP-253338

Baltimore, USA, December 8-11, 2025

Agenda Item:			8.2.1.0

Source:					Ofinno

Title:							Discussion on 6G deployment scenarios

Document for:		Decision

# Introduction

One of the objectives of the RAN level study item is to identify and define high level description of practical deployment scenarios envisage for 6G [1]:

This study item aims to

- investigate a candidate set of items for minimum TPRs based on the Recommendation ITU-R M.2160, and, where applicable, the associated target values and key assumptions for the identified minimum TPRs.

  - The outcome is expected to be shared by LS with ITU-R WP5D, as suitable, and used as a baseline for the subsequent study 6G in RAN

- Identify typical and practical deployment scenarios defined by attributes such as carrier frequency, inter-site distance, user density, maximum mobility speed, and other relevant factors.

- Develop 3GPP requirements for 6G Radio for improvement of existing services and for new services.

- Determine the applicability of legacy services to 6G Radio, and define radio requirements for these, as appropriate.

The Non-Terrestrial Network (NTN) is included as one of the identified deployment scenarios in the SI TR 38.914 [2]. However, attributes and the high-level description for the NTN deployment are currently missing in the SI TR 38.914 [2]. In this contribution we provide our views on the attributes and their respective values for the NTN deployment scenario.

# NTN deployment scenario attributes

In the following sections, we provide our views on the attributes and the high-level description of the NTN deployment scenarios in TR 38.914.

## Carrier frequency for NTN

The 5G NTN currently supports L-band, S-band, Ku-band, and Ka-band. Therefore, at least these bands should be included in the attributes for the NTN deployment scenarios. For the 6G Study Item (SI) in RAN1, the following frequencies have been agreed for the NTN evaluations:

- L-band (i.e., 1.5GHz)

- S-band (i.e., 2 GHz)

- Ku-band (i.e., 14 GHz for UL and 11 GHz for DL)

- Ka-band (i.e., 30 GHz for UL and 20GHz for DL)

The above carrier frequencies can be used for the 6G NTN deployment scenarios in TR 38.914.

Proposal 1: For the 6G NTN deployment scenario, consider carrier frequencies around 1.5 GHz (L-band), 2 GHz (S-band), 14 GHz (UL) / 11 GHz (DL) in Ku-band, and 30 GHz (UL) / 20 GHz (DL) in Ka-band.

## General Simulation Parameters

The 5G NTN link- and system-level studies have mainly focused on LEO and GEO satellite orbits [3][4]. For LEO studies, the satellite altitudes of 600 km and 1200 km have been considered[3][4]. Due to ever growing interest in NTN deployment, all three main satellite orbits (i.e., LEO , MEO, and GEO) should be considered for 6GR NTN deployment scenarios. Furthermore, LEO evaluations should include very low Earth orbit (VLEO) altitude around 300 km in addition to 600 km and 1200 km. The satellite orbit for MEO can range from 7000 km to 25000 km [3].

In 5G NTN, only transparent payload on the satellite was supported in Rel-17 and Rel-18. The regenerative payload (i.e., gNB functionalities on satellite) was introduced in Rel-19. For 6G NTN, NTN architectures supporting transparent payload and regenerative payload should be specified from Day 1. Therefore, both payload types (i.e., transparent payload on satellite and regenerative payload on satellite) should be considered for 6GR NTN scenarios.

Around 1.5 GHz (i.e., L-band) carrier frequency, several 5G NTN bands (i.e., n250, n251, n253, n254, and n255) are defined according to clause 5.2.2 of TS 38.101-5. The maximum passband among these bands can slightly exceeding 30 MHz (e.g., n255) in both UL and DL. Therefore, the maximum system bandwidth for evaluation around 1.5 GHz can be 2*30 MHz. However, 2*30 MHz may not likely represent typical L-band deployment, which has a 20 MHz of maximum channel bandwidth according to TS 38.101-5. Therefore, a system bandwidth for 6GR NTN deployment scenarios can be either 2*20 MHz or 2*30 MHz around 1.5 GHz carrier frequency.

Around 2 GHz (i.e., S-band) carrier frequency, two 5G NTN bands (i.e., n252 and n256) are defined according to clause 5.2.2 of TS 38.101-5. The maximum passband within these bands is 30 MHz in UL and 50 MHz in DL (e.g., in band n256). Therefore, the maximum system bandwidth for evaluation around 2 GHz can also be 2*30 MHz.

Around 14 GHz (UL) / 11 GHz (DL) carrier frequencies in Ku-band, two FR1 bands (i.e., n247 and n248) and two FR2 bands (n509 and n508) are defined according to clause 5.2.2 and clause 5.2.3 of TS 38.101-5, respectively. The maximum supported bandwidths in FR1 and FR2 are 2*100 MHz and 2*400 MHz respectively. However, for 6GR NTN evaluations in this frequency range, smaller bandwidths, which may be used in typical deployment, should be also considered , e.g., 2*100 MHz, 2*200 MHz, and 2*400 MHz. The system bandwidth for 6GR NTN deployment scenarios can be 2*200 MHz around 14 GHz (UL) / 11 GHz (DL) carrier frequencies.

Around 30 GHz (UL) /20 GHz (DL) carrier frequencies in Ka-band, two FR2 bands (n511 and n512) are defined according to clause 5.2.3 of TS 38.101-5. The maximum supported bandwidth in 30/20 GHz is 2*400 MHz. Similar to the Ku-band case, smaller bandwidths, which reflect typical deployment, e.g., 2*100 MHz, 2*200 MHz, and 2*400 MHz, should be also considered for 6GR NTN evaluations. The system bandwidth 2*200 MHz is appropriate for deployment scenarios around 30/20 GHz (UL/DL) carrier frequencies.

Regarding duplexing, at least FDD should be assumed in all four carrier frequency ranges. For NTN IoT, HD-FDD is typically used to reduce device cost and complexity. Therefore, both FDD and HD-FDD duplexing modes should be included in 6GR NTN deployment scenarios.

The users served by the satellite are typically outdoor. The users can be either fixed or mobile with mobility spanning a wide range of speeds.

The summary of proposed system parameters for 6GR NTN deployment scenarios is provided in table 1.

Proposal 2: Consider LEO, MEO, and GEO satellite orbits for the 6G NTN deployment scenario.

Proposal 3: Consider satellite altitudes of 300 km, 600 km, and 12000 km for LEO, for MEO for the 6G NTN deployment scenario.

Proposal 4: Consider both transparent payload type and regenerative payload type the 6G NTN deployment scenario.

Proposal 5: Consider following system bandwidths for the 6G NTN deployment scenario:

- around 1.5 GHz and 2 GHz: Up to 2*20 MHz or 2*30 MHz (DL+UL)

- around 2 GHz and 2 GHz: Up to 2*30 MHz (DL+UL)

- around 11/14 GHz (DL/UL): Up to 2*200 MHz (DL+UL)

- around 20/30 GHz (DL/UL): Up to 2*200 MHz (DL+UL)

Proposal 6: Both FDD and TDD duplexing modes are used included in the 6G NTN deployment scenario.

Table 1: Attributes for NTN deployment scenario

| Attributes | Values or assumptions |
| --- | --- |
| Carrier Frequency | Around 1.5 GHz Around 2 GHz Around 11 GHz (DL) and 14 GHz (UL) Around 20 GHz (DL) and 30 GHz (UL) |
| System bandwidth | Around 1.5 GHz: 2*20 MHz or 2*30 MHz (DL+UL) Around 2 GHz: 2*30 MHz (DL+UL) Around 12/14 GHz: 2*200 MHz (DL+UL) Around 20/30 GHz: 2*200 MHz (DL+UL) |
| Satellite orbit altitude | LEO (300 km, 600 km, and 1200 km); MEO (7000-25000 km); GEO (35786 km) |
| Satellite architecture/payload types | Transparent (bent pipe) payload Regenerative payload |
| Duplexing | FDD, HD-FDD |
| User distribution | 100% outdoor |
| User mobility | Fixed, mobile |
| Service profile | NOTE: Whether to use full buffer traffic or non-full-buffer traffic depends on the evaluation methodology adopted for each KPI. |

# Summary

In this contribution we have identified necessary parameters and attributes related to NTN deployment scenarios in 6G under RAN level SI on 6G. It is proposed to adapt the attributes and their values provided in Table 1 below for 6G NTN deployment scenarios in TR 38.914:

Table 1: Attributes for NTN deployment scenario

| Attributes | Values or assumptions |
| --- | --- |
| Carrier Frequency | Around 1.5 GHz Around 2 GHz Around 11 GHz (DL) and 14 GHz (UL) Around 20 GHz (DL) and 30 GHz (UL) |
| System bandwidth | Around 1.5 GHz: 2*20 MHz or 2*30 MHz (DL+UL) Around 2 GHz: 2*30 MHz (DL+UL) Around 12/14 GHz: 2*200 MHz (DL+UL) Around 20/30 GHz: 2*200 MHz (DL+UL) |
| Satellite orbit altitude | LEO (300 km, 600 km, and 1200 km); MEO (7000-25000 km); GEO (35786 km) |
| Satellite architecture/payload types | Transparent (bent pipe) payload Regenerative payload |
| Duplexing | FDD, HD-FDD |
| User distribution | 100% outdoor |
| User mobility | Fixed, mobile |
| Service profile | NOTE: Whether to use full buffer traffic or non-full-buffer traffic depends on the evaluation methodology adopted for each KPI. |

# References

- RP-251395, “Study on 6G Scenarios and requirements”, SID, RAN#108.

- TR 38.914, “Study on 6G Scenarios and requirements”, v0.2.0.

- TR 38.811, “Study on New Radio (NR) to support non-terrestrial networks”.

- TR 38.821, “Solutions for NR to support non-terrestrial networks (NTN)”.