3GPP TSG-RAN Meeting #110			RP-253366

Baltimore, US, December 08-11, 2025

Source:	NTT DOCOMO, INC., Telstra, Tejas Networks, Boost Mobile, Verizon Wireless, Qualcomm Inc, FiberCop, KT Corp, Jio Platforms, T-Mobile USA, IIT-Hyderabad, WiSiG Networks, NEC, Fujitsu, AT&T, SONY, Deutsche Telekom, LG Electronics, CEWIT, Mavenir, CMCC

Title:	Discussion on requirements for architecture and migration for 6G

Agenda item:	8.2.1.2

Document for:	Discussion

- Introduction

In the last RAN plenary meeting, the following requirements for architecture and migration were agreed and captured in TR 38.914[2]. This contribution further discusses on requirements for architecture and migration for 6G.

======

The RAN design for the 6G Radio Access Technologies shall be designed to fulfil the following requirements:

-	The 6G RAN architecture shall support standalone RAN architecture.

-	The 6G RAN shall support Multi-RAT Spectrum Sharing between 6GR and NR.

-	The 6G RAN architecture shall support inter-RAT mobility between the 6GR and NR.

-	The 6G RAN architecture shall support connectivity through multiple TRPs, either collocated or non-collocated.

-	The 6G RAT shall support Spectrum Aggregation (e.g. Carrier Aggregation) for both uplink and downlink, and for both co-located and non-co-located TRPs.

-	3GPP defined interfaces for 6G RAN shall be open for multi-vendor interoperability.

-	The 6G RAN architecture shall allow for control plane and user plane separation.

-	The 6G RAN architecture shall support sharing of the RAN between multiple operators.

-	The 6G RAN architecture shall allow for the operation of network slicing.

-	The 6G RAN architecture shall be designed considering both terrestrial network and non-terrestrial network.

-	The 6G RAN architecture shall support enhanced service awareness in RAN.

-	The design of the 6G RAN shall allow enhanced resilience compared to NR if/where applicable.

-	The design of the 6G RAN shall enable lower CAPEX/OPEX with respect to current networks.

-	The 6G RAN architecture shall allow non-public networks.

=======

- Discussion

RAN cloudification

RAN logical nodes can be realized by means of Physical Network Functions (PNF), Network Function Virtualization (NFV) and Cloud Native Network Function (CNF).

PNF implementation means custom dedicated hardware will be used and is very expensive, not scalable, lack of flexibility and very expensive to add new services.

NFV allows RAN to be implemented in COTS (Commercial Off The Shelf) hardware or in cloud infrastructure. This approach decouples network functions from proprietary hardware, allowing for more flexibility, faster service deployment, and reduced costs through the use of virtualization and standard servers.

But still NFV has its own limitations when compared to Cloud Native Network Function (CNF) design and deployment in modern day cloud infrastructure.

We think one of the foundational requirements for 6G RAN is the ability to support Cloud Native Network Function (CNF) design to leverage cloud native technologies as an evolution from legacy Network Function Virtualization (NFV) within a cloud environment. From the perspective of MNOs, this capability is not merely a technical enhancement, it is a strategic enabler that allows MNOs to leverage cloud native technologies because  CNF offers many advantages compared to traditional NFV, in terms of better scalability, resource efficiency and deployment speed, resiliency and fault tolerance, better adaptability and flexibility, faster development/innovation cycle, cloud native observability and high cost efficiency.

Cloud Native definitions are specified in both ITU and CNCF (Cloud Native Computing Foundation), which is given below for quick reference.

ITU Cloud Native Definition from section 7.1 [3]:

Cloud native is a methodology of software on cloud computing, which takes full advantage of capabilities of cloud computing during the process of software design, development, deployment, running and management. This methodology is supported by principles and typical technologies, which are described in clause 7.2 and clause 7.3 in detail.

NOTE 1 – In ETSI [b-NFV-IFA 029], cloud native is defined as a software design principle with certain properties and a set of non-functional characteristics.

Being "cloud native" means an application grows on cloud computing and its lifecycle takes place within cloud computing as much as possible, following certain principles and using typical technologies. Cloud computing provides all about the application needs including cloud infrastructure capability, platform capability, security, maintenance, etc. except for application business logic code.

The goal of cloud native is to speed up application building, delivering, managing and optimization, making an application lightweight, flexible and automatic, as well as simplifying the work of the application developer and maintainer.

Cloud Native Computing Foundation (CNCF), has following official definition for what “Cloud Native” means

toc/DEFINITION.md at main · cncf/toc · GitHub [4]

Cloud native practices empower organizations to develop, build, and deploy workloads in computing environments (public, private, hybrid cloud) to meet their organizational needs at scale in a programmatic and repeatable manner. It is characterized by loosely coupled systems that interoperate in a manner that is secure, resilient, manageable, sustainable, and observable.

Cloud native technologies and architectures typically consist of some combination of containers, service meshes, multi-tenancy, microservices, immutable infrastructure, serverless, and declarative APIs — this list is non-exhaustive.

Observation 1: Cloud Native Network Function (CNF) offers many advantages compared to traditional NFV, in terms of better scalability, resource efficiency and deployment speed, resiliency and fault tolerance, better adaptability and flexibility, faster development/innovation cycle and high cost efficiency.

Observation 2: Cloud Native Network Function (CNF) for 6G RAN design and deployment allows MNOs to leverage latest cloud native technologies and allows MNOs to operate networks more efficiently in cloud infrastructure, agility for new service addition and modernize networks to better compete in next 15-20 years.

Adopting CNF design for 6G RAN significantly reduces CAPEX when compared with NFV. Operators can scale network functions up or down based on real-time demand, optimizing utilization and reducing OPEX.

Moreover, CNF facilitates rapid deployment and innovation by leveraging rapidly evolving cloud native technologies. In traditional RAN setups, introducing new services or upgrading network functions often involves lengthy procurement and integration cycles. With CNF, operators can deploy new features as software updates in cloud environment, accelerating time-to-market and enhancing competitiveness. This agility is particularly vital in the 6G era than in previous generations, where use cases such as immersive XR, digital twins, AI native 6G RAN and autonomous systems demand continuous evolution of network capabilities.

Another key advantage is geographical flexibility and centralized management. CNF enables centralized orchestration of distributed RAN functions, allowing operators to manage diverse deployments—from dense urban areas to remote rural zones. This not only simplifies operations but also improves fault tolerance and service continuity through automated failover and recovery mechanisms.

From a sustainability standpoint, CNF supports energy-efficient operations. By dynamically allocating compute resources and consolidating workloads, operators can reduce energy consumption and carbon footprint. This aligns with global environmental goals and regulatory pressures, making CNF not only a technical necessity but also a corporate responsibility.

Finally, CNF fosters vendor diversity and ecosystem growth. Operators are no longer locked into monolithic solutions from a single vendor. Instead, they can integrate best-of-breed components from multiple suppliers, encouraging innovation and reducing costs through competitive dynamics.

Thus, in 6G future era, it is paramount importance for MNOs to deploy 6G RAN as CNF in cloud infrastructure but to provide some implementation flexibility, we can also consider NFV in cloud infrastructure in addition to CNF.

Thus, we propose

Proposal 1: The 6G RAN architecture and interfaces design shall allow deployments using Cloud-Native Network Functions (CNFs) in cloud-native manner, as well as Network Function Virtualization (NFV) within a cloud environment.

Proposal 2: 6G TR 38.914 to adopt ITU and Cloud Native Computing Foundation (CNCF) provided official cloud native definitions as described in this document.

New services (e.g. AIML, sensing)

One of the most transformative aspects of 6G is its ability to support new service domains, particularly those involving AI/ML and integrated sensing capabilities.

From an operator’s standpoint, the ability to handle new services (e.g. AI/ML and sensing) at scale is essential for unlocking new revenue streams and delivering differentiated services. AI/ML is critical for enabling higher UE performance, autonomous network optimization, predictive maintenance. Similarly, sensing data—such as location, motion, and environmental context—can support advanced use cases like illegal objects (e.g. unauthorized UAV) intrusion detection.

To realize these capabilities, the RAN must be designed to natively support data collection, preprocessing, and transport mechanisms that are optimized for these new data types. Unlike traditional traffic, AI/ML and sensing data often require transport with context-aware prioritization. Operators also benefit from network-wide visibility and control when data flows are standardized and efficiently managed.

Proposal 3: The design of the 6G RAN architecture shall allow the collection and transport of new service data (e.g. AI data, sensing data) to fulfil new service requirements.

# 3	Conclusion

In this contribution, we have the following key observations and proposals .

Observation 1: Cloud Native Network Function (CNF) offers many advantages compared to traditional NFV, in terms of better scalability, resource efficiency and deployment speed, resiliency and fault tolerance, better adaptability and flexibility, faster development/innovation cycle and high cost efficiency.

Observation 2: Cloud Native Network Function (CNF) for 6G RAN design and deployment allows MNOs to leverage latest cloud native technologies and allows MNOs to operate networks more efficiently in cloud infrastructure, agility for new service addition and modernize networks to better compete in next 15-20 years.

Proposal 1: The 6G RAN architecture and interfaces design shall allow deployments using Cloud-Native Network Functions (CNFs) in cloud-native manner, as well as Network Function Virtualization (NFV) within a cloud environment.

Proposal 2: 6G TR 38.914 to adopt ITU and Cloud Native Computing Foundation (CNCF) provided official cloud native definitions as described in this document.

Proposal 3: The design of the 6G RAN architecture shall allow the collection and transport of new service data (e.g. AI data, sensing data) to fulfil new service requirements.

# 4	References

[1]	3GPP TR 38.914 V0.1.1, Study on 6G Scenarios and Requirements, 2025-09.

[2]	RP-252880, “Summary of Discussion on 6G Requirements for architecture and migration”  NTT DOCOMO

[3] 	Recommendation ITU-T Y.3532 (05/2023) - Cloud computing - Functional requirements of Platform as a Service for cloud native applications

[4] 	toc/DEFINITION.md at main · cncf/toc · GitHub

[5] 	R3-258274: [pCR to draft TR 38.760-3] 6G RAN Requirements for cloud native support

# 5	Annex (Text Proposal for TR 38.914)

Start of TP

## 5.2	Requirements for architecture and migration

Editor note: 6G RAN architecture, 5G-6G migration

The RAN design for the 6G Radio Access Technologies shall be designed to fulfil the following requirements:

-	The 6G RAN architecture shall support standalone RAN architecture.

-	The 6G RAN shall support Multi-RAT Spectrum Sharing between 6GR and NR.

-	The 6G RAN architecture shall support inter-RAT mobility between the 6GR and NR.

-	The 6G RAN architecture shall support connectivity through multiple TRPs, either collocated or non-collocated.

-	The 6G RAT shall support Spectrum Aggregation (e.g. Carrier Aggregation) for both uplink and downlink, and for both co-located and non-co-located TRPs.

-	3GPP defined interfaces for 6G RAN shall be open for multi-vendor interoperability.

-	The 6G RAN architecture shall allow for control plane and user plane separation.

-	The 6G RAN architecture shall support sharing of the RAN between multiple operators.

-	The 6G RAN architecture shall allow for the operation of network slicing.

-	The 6G RAN architecture shall be designed considering both terrestrial network and non-terrestrial network.

-	The 6G RAN architecture shall support enhanced service awareness in RAN.

-	The design of the 6G RAN shall allow enhanced resilience compared to NR if/where applicable.

-	The design of the 6G RAN shall enable lower CAPEX/OPEX with respect to current networks.

-	The 6G RAN architecture shall allow non-public networks.

-	The 6G RAN architecture and interfaces shall allow deployments using Cloud Native Network Function (CNF) and Network Function Virtualization (NFV) within a cloud environment.

-	The design of the 6G RAN architecture shall allow the collection and transport of new service data (e.g. AIML data, sensing data) to fulfil new service requirements.

End of TP