3GPP TSG RAN Meeting #110	RP-253457

Baltimore, US, December 08-11, 2025

Status Report to TSG

Agenda item:			9.3.4.5

| WI / SI Name | NR demodulation performance: Phase 6 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: No | Performance part: Yes |  | Testing part: No |
| Acronym | NR_demod_Ph6-Perf |  |  |  |  |
| Unique ID | 1092069 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-252957 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: NA | Core part: NA | Performance part: 03/2027 | Testing part: NA |  |
| Overall Completion level | Study Item: NA | Core part: NA | Performance Part: 22% | Testing part: NA |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN WG4 |
| --- | --- | --- |
| Rapporteur | Name | Jingzhou Wu, Ryan Husbands |
|  | Company | China Telecom, BT |
|  | Email | wujingzhou@chinatelecom.cn, ryan.husbands@bt.com |

1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

2.1	RAN1

2.1.1	Agreements

2.1.2	Remaining Open issues

2.2	RAN2

2.2.1	Agreements

2.2.2	Remaining Open issues

2.3	RAN3

2.3.1	Agreements

2.3.2	Remaining Open issues

2.4	RAN4

2.4.1	Agreements

RAN4#116bis:

- The Work plan for NR demodulation performance: Phase 6 is approved in R4-2514854

- The Topic summary for [116bis][227] NR_demod_Ph6_Ph5 is provided in R4-2513533

- The Ad-hoc minutes for NR_demod_Ph6_Ph5 is approved in R4-2514822

- The WF on NR_demod_Ph6_Ph5 is approved in R4-2514823 with the following agreements:

Topic #1: UE requirements for FR1 SU-MIMO with spatial channel model Angle scaling for rCDL - Agreement: - Update the rCDL channel model based on the latest angular scaling procedure as in TR 38.901 v19.1.0 and further discuss the details. - Agreement on the details: - Use the ‘s’ values for tabulated desired AS values rather than the desired AS values we used in 38.753 to save us the effort of obtaining ‘s’ and aligning among companies. - Obtain ‘s’ from the tabulated example AS desired values and use the following desired AS: ASD – 25; ASA – 60; ZSD – 5, ZSA – 15 as baseline to configure rCDL channel model - Additional rCDL Channel Models based on different AS are not precluded Frequency generalization for rCDL channel modeling - Agreement: - Define the requirements based on the same channel model for all different FR1 frequencies for this WI - The maximum Doppler fD should be fixed as X Hz - As baseline for Rel-20 demod requirement, X =10 - FFS if other fD can be considered - On the note to Maximum Doppler: - The ratio between the UE speed v, i.e., magnitude of the velocity vector, and the wavelength is normalized to be X Hz (maximum Doppler). Duplex mode and SCS - Agreement: - Cover TDD 30kHz SCS with 40MHz CHBW and FFS whether to also cover FDD 15kHz SCS with 10MHz CHBW Test scope for PDSCH demodulation requirements with SCM - Agreement: - Cover PDSCH mapping type A demodulation requirements with 8T8R 8 layer case and the 4T4R 4 layer case. Precoding, MCS and test metric - Agreement and candidate options: - For 4T4R 4L test, it is agreed to use the test metric: SNR @ 70% max TP. - For 8T8R 8L test, it is agreed to use test metric: Define single SNR and further discuss the SNR is based on: - Option a: Based on per CW thput with Different MCS per CW - Option b: Based on total thput with different MCS per CW - Option c: Based on total thput with same MCS per CW - Further discuss: How to define the SNR for option a. How to select the MCS for different CW. Topic #2: UE requirements for 6Rx UE with interference Test scope - Agreements: - Focus PDSCH requirements on Mapping Type A only. - Do not cover PDSCH absolute physical layer throughput requirements Receiver assumption - Agreement: - Only MMSE-IRC is considered Duplex mode and SCS - Agreement: - Align with the basic 6Rx requirement: Only to define requirements for FR1 TDD with 30kHz SCS/40MHz 7D1S2U with S: 6D+4G+4U PDSCH requirements with inter cell interference Test scenarios and interference profile - Agreement: - Cover both HomNet and HetNet scenarios - Interference modelling: Reuse the same interference modeling defined for Rel-17 2/4Rx and Rel-19 8Rx: - INR based inter cell interference modeling with the following interference level: - INR1 = 7.77dB and INR2 = 2.29dB for HomNet (if covered) - INR1 = 7.58dB for HetNet (if covered) - Random rank with 70% and 30% probability for rank 1 and rank 2 for the interference cell(s) - Random 16QAM modulated symbols for the interference cell(s) - Frequency shift to the target cell: 300Hz for interference cell 1, -100Hz for interference cell 2 - SSB configuration: First SSB in Slot #0 for all interference cells (Same as serving cell). Rank for the serving cell - Agreement: - For initial evaluation purpose, cover rank 1,2 and 4 And make further selection based on simulation. Antenna configuration - Agreement: For initial simulation purpose, 2T6R for rank 1/2 and 4T6R for rank 4 Other parameters - Agreement: - Reuse the same configuration for Rel-17 2/4Rx as the starting point. PDSCH requirements with intra cell inter user interference MU-MIMO Beamforming modeling - Agreement: - Reuse MU-MIMO Beamforming in TS 38.101-4 Annex B.4.2 Signal power assumptions and SNR assumptions - Agreement: - Average target UE signal power is equal to RankTargetUE/RankTotal and average interference UE signal power is equal to RankInterfUE/RankTotal - SNR = (STargetUE+ SInterfUE)/N Antenna configuration and rank for target + co-scheduled UE - Agreement: - For initial evaluation purpose, cover Rank 1+1 (with 2T6R) and Rank 2+2 (with 4T6R). - Select rank 2+2 if simulation results show feasibility Other test parameters and simulation assumptions - Agreement: - 1 Co-scheduled UE number - Precoding for the target UE: Random Single Panel Type I precoding - 2PRBs for PRB bundling size and precoding granularity - PDCCH and PDSCH allocation in time domain - Use symbols #0 and #1 of each slot for PDCCH - PDSCH mapping type A, Start symbol 2, Duration 12 - PDSCH allocation in frequency domain - Full PRB.

RAN4#117:

- The Topic summary for [117][231] NR_demod_Ph6 is provided in R4-2520856

- The Ad-hoc minutes for NR_demod_Ph6 is approved in R4-2522661

- The WF on NR_demod_Ph6 is approved in R4-2522662 with the following agreements:

Topic #1: UE requirements for FR1 SU-MIMO with spatial channel model #### Cluster simplification and Delay scaling after cluster truncation Agreement: Use 12 clusters, choosing the 12 strongest clusters; the process for cluster selection should include sub-cluster combining, delay scaling, power normalisation, i.e. removing spatial filtering (AAV+AEP) and RMS delay spread rescaling from the 38.753 process (step 2, 6 and 7). Omit the post-truncation delay scaling step, instead having one initial RMS delay scaling, then choosing the 12 strongest clusters. #### Angle scaling for rCDL – updated rCDL channel model Way Forward Companies to update table based off the agreements at RAN4#117, applicable ‘s’ values in 38.901 may be reused, if AS used are not captured in 38.901 ‘s’ values should be derived and captured in the specification. For information This results in the following parameter table(s) for rCDL-C2, to be confirmed in the next meeting: Cluster #; Cluster delay [ns]; Power in [dB]; AOD in [°]; AOA in [°]; ZOD in [°]; ZOA in [°]. 1; 0; -7.4; -38.4; -117.2; 95.4; 101.1. 2; 76; -1.2; -22.8; 123.5; 98.0; 70.9. 3; 80; -5.5; -34.5; -140.0; 101.6; 66.9. 4; 232; 0; -7.8; 166.7; 99.1; 77.1. 5; 240; -10.4; 39.6; 68.0; 110.0; 61.7. 6; 290; -10.1; -50.0; 77.5; 92.0; 54.7. 7; 300; -13.7; 44.2; -71.8; 111.7; 69.5. 8; 340; -14.1; -71.2; 60.7; 88.7; 48.2. 9; 448; -8.1; -44.0; 78.9; 107.3; 106.9. 10; 478; -9.8; -49.9; -89.5; 108.2; 47.4. 11; 792; -11.7; -59.1; 90.4; 87.8; 49.2. 12; 990; -16.2; 58.9; 46.8; 108.2; 126.6. Per-Cluster Parameters (“c-values”). Parameter; cASD in [°]; cASA in [°]; cZSD in [°]; cZSA in [°]; XPR in [dB]. Value; 2; 15; 3; 7; 7. Scale factor values for CDL-C2 model (“s-factors” in 38.901 V19.1 section 7.7.5.1). CDL Type; Desired AOD Spread（°）; Scale Factor (AOD); Desired AOA Spread（°）; Scale Factor (AOA); Desired ZOA Spread（°）; Scale Factor (ZOA); Desired ZOD Spread（°）; Scale Factor (ZOD). CDL-C2; 25; 0.6513; 60; 0.8585; 15; 1.9504; 5; 1.8219. #### UE antenna modelling Agreement Use UE antenna placement and polarisation assumptions listed in Table 6.1-2 of TR 38.753 as baseline For PDSCH, use UE antenna radiation assumptions listed in Table 6.1-2 of TR 38.753 as baseline, i.e. omni #### Further rCDL channel alignment Way Forward: Interested companies may measure and compare channel statistics such as Spatial Domain Power Density (SDPD), Time Coherence (TC), and Frequency Coherence (FC), noting that it cannot be captured as an output of the work item. Therefore, no online time should be allocated to this channel statistic comparison. Alignment for the results will be on absolute performance criteria only. #### Frequency generalization for rCDL channel modeling Agreement: Regarding maximum doppler, focus on X=10 as baseline, other values can be considered in future meetings only if: 1) consensus can be reached 2) alignment on the simulation results can be reached Test scope and test parameters for SCM SU-MIMO requirements #### Duplex mode and SCS Agreement: Cover both TDD 30kHz SCS with 40MHz CHBW and FDD 15kHz SCS with 10MHz CHBW Note: under the condition that the simulation results can be aligned for both cases #### Precoding for PDSCH demodulation requirements with SCM Agreement: Random Single Panel Type I and randomized precoder selection for every PRB bundle and updated per slot with equal probability of each applicable i1/i2 combination or codebook #### MCS and test metric for 8T8R 8L test case Agreement: - As starting point for simulation alignment: - Use different MCS per CW - As starting point, assign MCS levels (13, [TBD]) for the two codeword. - Collect both per CW Tput and total Tput - Note: Use single SNR for the two CWs Further discuss the performance metric based on the simulation results #### PMI bias Agreement: RAN4 shall investigate the PMI bias for PMI tests, options could be but are not limited to: - Beam Steering - Base Station Bearing Angle Rotation - UE Azimuth Rotation - Mean Cluster Angle Rotation - RAN1 spatially consistent time evolution #### Whether to define eType II PMI reporting requirements based on SCM Way Forward: Address PMI biasing and further simulation result alignment is required before requirement definition Topic #2: UE requirements for 6Rx UE with interference #### Coverage of UE types Agreement: Define only a single requirement for both handheld and FWA devices, i.e., for each test, the same test parameters will be used for both UE device types. #### Antenna correlation Agreement: For each test, no need to differentiate handheld and FWA with different parameters. FFS the Antenna correlation level for simulation #### Propagation condition Agreement: For initial simulation purpose - For 2 interfering cells (HomNet): - Option 1: TDLA30-10 - Option 2: TDLC300-100 (prioritized if companies can only provide simulation for one propagation channel) - For 1 interfering cell (HetNet): - TDLA30-10 #### Rank for the serving cell Agreement: For intial simulation purpose, Rank 1 and 2 for HomNet, and rank 2 and 4 for HetNet #### Whether to introduce CQI reporting requirements with inter-cell interference Agreement: Following the WID, define CQI reporting requirements with inter-cell interference. #### Rank for the serving cell for CQI reporting requirements with inter-cell interference Agreement: Rank 2. Interference modelling and test parameters for CQI reporting requirements with inter-cell interference Agreement: Use the configuration provided in below table as starting point for simulation: Parameter; Value. SCS/CBW; 30kHz/40MHz, 7D1S2U(S=6D+4G+4U). SINR; TBD based on simulation results. Propagation conditions; TDLA30-5 for the serving cell AWGN for the interference cell. Antenna and MIMO correlation for serving cell; 2T6R ULA correlation channel: FFS Option 1: α = 0 and β = [0.07179] Option 2: α = 0 and β = 0 Option 3: the same assumption as PDSCH Option 1 is the first priority for initial simulation, and interested companies are also encouraged to simulate other options.. Antenna configuration for the interference cell:; 1T6R. Interference power level; INR = 10.04dB for the serving cell.. Measurement channel; As specified in Table A.4-2. Other parameters; For other parameters re-use the same configuration as used in Rel-17 requirements.. Test metric for CQI reporting requirements with inter-cell interference Agreement: Reuse the same test metric for Rel-17 2Rx, 4Rx and 8Rx a) the ratio of the throughput obtained when transmitting the transport format indicated by each reported wideband CQI index subject to an interference source with specified INR and that obtained when transmitting the transport format indicated by each reported wideband CQI index subject to a white Gaussian noise source shall be ≥  b) when transmitting the transport format indicated by each reported wideband CQI index subject to an interference source with specified INR, the average BLER for the indicated transport formats shall be greater than or equal to X%..

    - Remaining Open issues

- Specify optional performance requirements under a single spatial channel model for SU-MIMO in FR1

- Define performance requirements for 6Rx UE (both handheld and FWA) with interference

3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

3.1	SAx/CTs

3.1.1	Agreements with cross-TSG impacts

3.1.2	Remaining Open issues with cross-TSG impacts

4.	References

R4-2513141	Discussion on UE requirements for FR1 SU-MIMO with spatial channel model	China Telecom

R4-2513142	Discussion on UE requirements for 6Rx UE with interference	China Telecom

R4-2513428	Discussion on 6Rx requirements with interference	Huawei,HiSilicon

R4-2513430	Discussion on demodulation and CSI requirements for SCM	Huawei,HiSilicon

R4-2513533	Topic summary for [116bis][227] NR_demod_Ph6_Ph5	Moderator (China Telecom)

R4-2513559	Views on PDSCH demodulation performance requirements for 6Rx devices with interference	Qualcomm India Pvt Ltd

R4-2513598	On Demodulation Performance Requirements for 6Rx UEs with Inter-Cell and Intra-Cell Inter-User Interference	Apple

R4-2513604	Discussion on Requirements with Spatial Channel Model	Apple

R4-2513763	Discussion on performance requirements for FR1 SU-MIMO with spatial channel model	MediaTek inc.

R4-2513827	Performance requirements using SU SCM	Nokia

R4-2513874	Discussion on Rel-20 performance requirements for FR1 SU-MIMO with spatial channel model	Samsung

R4-2513875	Discussion on Rel-20 performance requirements for 6Rx UE with interference	Samsung

R4-2513924	Work Plan for Rel-20 Demodulation, views on SCM activity	Nokia

R4-2513954	On demodulation requirements for 6Rx reception with interference mitigation	Ericsson

R4-2513955	On general aspects for performance enhancement phase 6	Ericsson

R4-2514130	On Performance and CSI reporting requirements for 6Rx UE with interference	Nokia

R4-2514137	Discussion on SCM for FR1 SU-MIMO performance requirements	Ericsson

R4-2514148	Discussion on demodulation performance requirements for 6Rx UE interference	ZTE Corporation, Sanechips

R4-2514150	Discussion on demodulation performance requirements for FR1 SU-MIMO with spatial channel model	ZTE Corporation, Sanechips

R4-2514292	Views on UE Performance requirements for FR1 SU-MIMO with SCM	Qualcomm Incorporated

R4-2514412	Discussion on performance requirements for SU-MIMO cases using Spatial Channel Model	BT plc

R4-2514822	Ad-hoc minutes for NR_demod_Ph6_Ph5	China Telecom

R4-2514823	WF on NR_demod_Ph6_Ph5	China Telecom

R4-2514854	Work plan for NR demodulation performance: Phase 6	China Telecom, BT

R4-2520052	Discussion on UE requirements for FR1 SU-MIMO with spatial channel model	China Telecom

R4-2520053	Discussion on UE requirements for 6Rx UE with interference	China Telecom

R4-2520231	Discussion on performance requirements for FR1 SU-MIMO with spatial channel model	MediaTek inc.

R4-2520232	Discussion on performance requirements for 6Rx UE with interference	MediaTek inc.

R4-2520250	On Performance and CSI reporting requirements for 6Rx UE with interference	Nokia

R4-2520532	Performance requirements using SU SCM	Nokia

R4-2520637	Discussion on Requirements with Spatial Channel Model	Apple

R4-2520648	On UE Demodulation Performance Requirements for 6Rx with Interference in Rel-20	Apple

R4-2520856	Topic summary for [117][231] NR_demod_Ph6	Moderator (China Telecom)

R4-2521042	Discussion on Rel-20 performance requirements for FR1 SU-MIMO with spatial channel model	Samsung

R4-2521043	Discussion on Rel-20 performance requirements for 6Rx UE with interference	Samsung

R4-2521121	Views on PDSCH demodulation performance requirements for 6Rx devices with interference	Qualcomm India Pvt Ltd

R4-2521130	Discussion on the demodulation and CSI reporting requirement for 6Rx with interference	Ericsson

R4-2521190	Discussion on demodulation performance requirements for 6Rx UE interference	ZTE Corporation, Sanechips

R4-2521191	Simulation results on performance requirements for 6Rx UE with interference	ZTE Corporation, Sanechips

R4-2521192	Discussion on demodulation performance requirements for FR1 SU-MIMO with spatial channel model	ZTE Corporation, Sanechips

R4-2521377	Discussions on spatial channel model for demodulation requirements	Huawei, HiSilicon

R4-2521378	Discussion on 6Rx requirements with interference	Huawei, HiSilicon

R4-2521771	Discussion on Rel-20 demodulation requirements with SCM	Ericsson

R4-2521772	Simulation results on Rel-20 demodulation requirements with SCM	Ericsson

R4-2522059	Views on UE Performance requirements for FR1 SU-MIMO with SCM	Qualcomm Incorporated

R4-2522197	Discussion on NR SU-MIMO performance requirements using SCM	BT plc

R4-2522661	Ad-hoc minutes for NR_demod_Ph6	Nokia, China Telecom

R4-2522662	WF on NR_demod_Ph6	Nokia, China Telecom