# 3GPP TSG RAN Meeting #110	RP-253462

# Baltimore , US, 08-11, December 2025

Status Report to TSG

Agenda item:			9.6.4.1

| WI / SI Name | NR FR1 7 MHz Channel Bandwidth |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: Yes |  | Testing part: No |
| Acronym | NR_FR1_7MHz_BW |  |  |  |  |
| Unique ID | 1060083 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-251453 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: n/a | Core part: 09/2025 | Performance part: 12/2025(-3 months) | Testing part: n/a |  |
| Overall Completion level | Study Item: n/a | Core part: 100% | Performance Part: 100% (+20%) | Testing part: n/a |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN4 |
| --- | --- | --- |
| Rapporteur (primary) | Name | Lei GAO, Bill Shvodian |
|  | Company | China Telecom, T-Mobile USA |
|  | Email | gaol8@chinatelecom.cn, bill.shvodian@t-mobile.com |

1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

2.1	RAN1

N/A.

2.1.1	Agreements

2.1.2	Remaining Open issues

2.2	RAN2

N/A.

2.2.1	Agreements

2.2.2	Remaining Open issues

2.3	RAN3

N/A.

2.3.1	Agreements

2.3.2	Remaining Open issues

2.4	RAN4

2.4.1	Agreements

In RAN4#114, the following agreements were made.

Scope

Agreement:

- The 7 MHz channel bandwidth is only applicable to FDD bands that also support 5 MHz channel bandwidth.

NB-IoT support

Agreement:

- In-band NB-IoT can be supported in 7MHz channel BW

System parameters – Release independence

Agreement:

- 35 RBs will be supported.

Release independence

Agreement:

- Support of 7 MHz channel BW could be Release independent from Rel-15

  - FFS on if the maximum initial BWP size shall then be restricted to 5 MHz.

  - Support of 7MHz is optional in Rel-19 and in the previous releases

Channel spacing, channel raster and sync raster

Agreement:

- Introducing 7 MHz channel BW has no impact on channel spacing, channel raster nor sync raster.

UE RF

NS_100

Agreement:

- There is no impact on A-MPR requirements for NS_100.

NS_12, NS_13, NS_14, NS_15

Agreement:

- For n26

  - NS_12 and NS_15 shall be supported for 7 MHz channel BW.

  - To be further discussed if NS_13 and NS_14 shall also be supported.

SEM

Agreement:

- Same SEM requirement for 7 MHz and 10 MHz channel BW.

MPR

Agreement:

- No MPR impact when introducing 7 MHz channel bandwidth.

Minimum Output power, Transmit Off power, Occupied bandwidth, ACLR, general spurious

Agreement:

- The requirement for 7 MHz is the same requirement than the requirement for 3-20 MHz. 7 MHz shall be added to:

  - Table 6.3.1-1 (minimum output power).

  - Table 6.3.2-1(Transmit Off power).

  - Table 6.5.1-1 (occupied bandwidth).

  - Table 6.5.2.4.1-1 (ACLR).

  - Table 6.5.3.1-1 (general spurious).

Maximum input level

Agreement:

- The requirement for 7 MHz is the same requirement than the requirement for 3-20 MHz. 7 MHz shall be added to Table7.4-1.

ACS, In-band blocking, Out of band blocking, Narrow Band blocking, Intermodulation, Spurious response

Agreement:

- The requirement for 7 MHz is the same requirement than the requirement for 5 and 10 MHz. 7 MHz shall be added to:

  - Tables 7.5-1, 7.5-3 and 7.5-4 (ACS).

  - Table 7.6.2-1 (In-band blocking).

  - Table 7.6.3-1 (Out of band blocking).

  - Table 7.6.4-1 and evaluate Pw (Narrow band blocking).

  - Table 7.8.2-1(Intermodulation) and Table 7.7-1(Spurious response).

BS RF

BS type

Agreement:

- Conducted requirements shall be covered within this WI

Open issue:

- FFS if BS OTA requirements should also be covered within this WI.

Multi-carrier and CA

Agreement:

- Muti-carrier (including MSR) and CA related requirements are considered for new added 7MHz WID but no new BCS is specified within this WI.

IAB and NCR

Agreement:

- 7 MHz should not be supported by IAB

- 7 MHz should not be supported by NCR

Work effort split

According to the Work Plan, only Draft CRs for BS and UE specifications for generic requirements (TS 38.1010-1, TS 38.104, TS 37.104 and [TS 37.105]) should be provided for the RAN4#114bis meeting.

|  | Companies |
| --- | --- |
| TS 38.104 | Ericsson |
| TS 38.141-1 | Nokia |
| [TS 38.141-2] | Huawei |
| TS 38.101-1 | CR responsible: Qualcomm Sub-parts split (draft CRs only): System parameters: Nokia Tx: Huawei Rx and Annexes (if needed): Qualcomm |
| TS 38.101-4 | Qualcomm |
| TS 38.133 | ZTE |
| TS 38.307 | China Telecom |
| TS 37.104 | ZTE |
| TS 37.141 | ZTE |
| TS 38.106 (*) | Ericsson |
| TS 38.115-1 (*) | CATT |
| [TS 38.115-2] (*) | CATT |
| TS 38.174 (*) | Nokia |
| TS 38.176-1 (*) | Nokia |
| [TS 38.176-2] (*) | Huawei |
| TS 38.113 | ZTE |
| TS 37.113 | Ericsson |
| [TS 37.105] | Huawei |
| [TS 37.145-1] | Huawei |
| [TS 37.145-2] | Huawei |
| (*) The NCR and IAB specs would need to be updated to clarify 7 MHz channel BW is not supported. Note: TS in [] are pending on agreement on OTA BS requirements to be in the scope or not. |  |

In RAN4#114-bis, the following agreements were made.

General

Initial BWP

Agreement:

- There is no restriction on the initial BWP size.

UE RF

NS_12, NS_13, NS_14 and NS_15 – A-MPR and frequency carrier Fc

Agreement:

- Evaluate A-MPR for:

  - NS_12 considering 7 MHz deployed within 814-824 MHz

  - NS_13 considering 7 MHz deployed in 817-824 MHz (Fc = 820.5 MHz).

  - NS_14 considering 7 MHz deployed within 824-849 MHz

  - NS_15 considering 7 MHz deployed within 814-845 MHz or within 841.5-849 MHz.

REFSENS for n5

Agreement:

- 7 MHz channel BW REFSENS for n5: -96.5 dBm value (25 RBs uplink configuration).

REFSENS for n26

Agreement:

- 7 MHz channel BW REFSENS for n26: -96.0 dBm value (25 RBs uplink configuration).

Remaining open issue:

- A-MPR requirement

BS RF

OTA requirements

Agreement:

- Specify OTA requirements for 7 MHz channel BW.

FRCs

Agreement:

- Reuse existing FRC(s) for 7MHz BS testing.

Output power dynamics

Agreement:

- Output power dynamics

| BS channel | Total power dynamic range (dB) |  |  |
| --- | --- | --- | --- |
| bandwidth (MHz) | 15 kHz SCS | 30 kHz SCS | 60 kHz SCS |
| 7 | 15.4 | N/A | N/A |

Output power dynamics – NB-IoT in-band

Agreement:

- Output power dynamics – NB-IoT in-band

| BS channel bandwidth (MHz) | NB-IoT RB frequency position | NB-IoT RB power dynamic range (dB) |
| --- | --- | --- |
| 5, 7, 10 | Any | +6 |

ACLR

Agreement:

- For 7 MHz channel BW, the ACLR limit in non-contiguous spectrum or multiple bands and the CACLR limit shall be the same ones as for 5 MHz channel BW.

REFSENS

Agreement:

- Specify REFSENS for 7 MHz channel BW according to the following table:

| BS channel bandwidth (MHz) | Sub-carrier spacing (kHz) | Reference measurement channel (Note 5) | Reference sensitivity power level, PREFSENS (dBm) |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | WA | MR | LA |
| 7 | 15 | G-FR1-A1-1 (Note 1) | -101.7 | -96.7 | -93.7 |

Dynamic range

Agreement:

- Specify Dynamic Range for 7MHz channel BW according to the following table:

| BS channel bandwidth (MHz) | Sub-carrier spacing (kHz) | Reference measurement channel | Wanted signal mean power (dBm) |  |  | Interfering signal mean power (dBm) / BWConfig |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | WA | MR | LA | WA | MR | LA |
| 7 | 15 | G-FR1-A2-1 (Note 1) | -70.7 | -65.7 | -62.7 | -81.0 | -76.0 | -73.0 |

Dynamic range for NB-IoT operation in NR in-band

Agreement:

- Specify Dynamic Range for NB-IoT operation in NR in-band and for 7MHz channel BW according to the following table:

| BS channel bandwidth (MHz) | Reference measurement channel | Wanted signal mean power (dBm) | Interfering signal mean power (dBm) / BWConfig | Type of interfering signal |
| --- | --- | --- | --- | --- |
| 7 | FRC A15-1 in Annex A.15 in TS 36.104 [13] | -99.7 | -81 | AWGN |
| 7 | FRC A15-2 in Annex A.15 in TS 36.104 [13] | -105.6 | -81 | AWGN |

ACS

Agreement:

- Specify ACS for 7MHz channel BW according to the following tables:

| BS channel bandwidth of the lowest/highest carrier received (MHz) | Wanted signal mean power (dBm) | Interfering signal mean power (dBm) |
| --- | --- | --- |
| 3 | PREFSENS + 8 dB | Wide Area BS: -52 Medium Range BS: -47 Local Area BS: -44 |
| 5, 7, 10, 15, 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 (Note 1) | PREFSENS + 6 dB |  |

Table 7.4.1.2-2: Base Station ACS interferer frequency offset values

| BS channel bandwidth of the lowest/highest carrier received (MHz) | Interfering signal centre frequency offset from the lower/upper Base Station RF Bandwidth edge or sub-block edge inside a sub-block gap (MHz) | Type of interfering signal |
| --- | --- | --- |
| 7, 10 | ±2.5075 | 5 MHz DFT-s-OFDM NR signal 15 kHz SCS, 25 RBs |

In-band general blocking

Agreement:

- Specify In-band general blocking for 7MHz channel BW according to the following table:

| BS channel bandwidth of the lowest/highest carrier received (MHz) | Wanted signal mean power (dBm) (Note 2) | Interfering signal mean power (dBm) | Interfering signal centre frequency minimum offset from the lower/upper Base Station RF Bandwidth edge or sub-block edge inside a sub-block gap (MHz) | Type of interfering signal |
| --- | --- | --- | --- | --- |
| 5, 7, 10, 15, 20 | PREFSENS + x dB | Wide Area BS: -43 Medium Range BS: -38 Local Area BS: -35 | ±7.5 | 5 MHz DFT-s-OFDM NR signal 15 kHz SCS, 25 RBs |

Narrowband blocking - limit

Agreement:

- Specify narrowband blocking limit for 7MHz channel BW according to the following tables:

Table 7.4.2.2-2: Base Station narrowband blocking requirement

| BS channel bandwidth of the lowest/highest carrier received (MHz) | Wanted signal mean power (dBm) | Interfering signal mean power (dBm) |
| --- | --- | --- |
| 3, 5, 7, 10, 15, 20, 25, 30, 35, 40, 45, 50, 60, 70, 80,90, 100 (Note 1) | PREFSENS + 6 dB | Wide Area BS: -49 Medium Range BS: -44 Local Area BS: -41 |

Table 7.4.2.2-2a: Base Station narrowband blocking requirement for NB-IoT operation in NR in-band

| BS channel bandwidth (MHz) | Wanted signal mean power (dBm) | Interfering signal mean power (dBm) |
| --- | --- | --- |
| 3, 5, 7, 10, 15, 20, 25, 30, 35, 40, 45, 50 | PREFSENS + x dB (Note 2) | Wide Area: -49 Medium Range: -44 Local Area: -41 |

Narrowband blocking – interferer

Agreement:

- Specify narrowband blocking interferer for 7MHz channel BW according to the following table:

Table 7.4.2.2-3: Base Station narrowband blocking interferer frequency offsets

| BS channel bandwidth of the lowest/highest carrier received (MHz) | Interfering RB centre frequency offset to the lower/upper Base Station RF Bandwidth edge or sub-block edge inside a sub-block gap (kHz) (Note 2) | Type of interfering signal |
| --- | --- | --- |
| 7, 10 | ±(355+m*180), m=0, 1, 2, 3, 4, 9, 14, 19, 24 | 5 MHz DFT-s-OFDM NR signal, 15 kHz SCS, 1 RB |

Intermodulation interferer

Agreement:

- Specify intermodulation interferer for 7MHz channel BW according to the following table:

Table 7.7.2-2: Interfering signals for intermodulation requirement

| BS channel bandwidth of the lowest/highest carrier received (MHz) | Interfering signal centre frequency offset from the lower/upper Base Station RF Bandwidth edge (MHz) | Type of interfering signal (Note 3) |
| --- | --- | --- |
| 7 | ±7.45 | CW |
|  | ±17.5 | 5 MHz DFT-s-OFDM NR signal (Note 1) |

Narrowband intermodulation interferer

Agreement:

- Specify narrowband intermodulation interferer for 7MHz channel BW according to the following table:

| BS channel bandwidth of the lowest/highest carrier received (MHz) | Interfering RB centre frequency offset from the lower/upper Base Station RF Bandwidth edge or sub-block edge inside a sub-block gap (kHz) (Note 3) | Type of interfering signal |
| --- | --- | --- |
| 7 | ±355 | CW |
|  | ±1240 | 5 MHz DFT-s-OFDM NR signal, 1 RB (Note 1) |

EVM

Agreement:

- Specify EVM window length for 7MHz channel BW according to the following table:

| Channel bandwidth (MHz) | FFT size | CP length for symbols 1-6 and 8-13 in FFT samples | EVM window length W | Ratio of W to total CP length for symbols 1-6 and 8-13 (Note) (%) |
| --- | --- | --- | --- | --- |
| 7, 10 | 1024 | 72 | 28 | 40 |

Remaining Open issues:

- ICS requirement

In RAN4#115, the following agreements were made.

UE A-MPR

NS_12: A-MPR regions

Agreement:

- The following NS_12 A-MPR regions definition was agreed. The corresponding A-MPR values are kept [] for further checking.

| Channel BW | Carrier Frequency, Fc, MHz | RBStart*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- | --- |
| 7MHz | 817.5 ≤ Fc ≤ 820.5 | ≤1.8 | ≤0.72 | [A2] |
|  |  | ≤1.8 | >0.72 | [A1] |
|  |  | >1.8 | >1.44 | [A1] |

| Modulation/Waveform | A1 | A2 |
| --- | --- | --- |
|  | Outer/Inner | Outer/Inner |
| DFT-s-OFDM PI/2 BPSK | ≤ 5 | ≤ 4 |
| DFT-s-OFDM QPSK | ≤ 5 | ≤ 4.5 |
| DFT-s-OFDM 16 QAM | ≤ 5.5 | ≤ 4.5 |
| DFT-s-OFDM 64 QAM | ≤ 5.5 | ≤ 5 |
| DFT-s-OFDM 256 QAM | ≤ 9.5 | ≤ 5 |
| CP-OFDM QPSK | ≤ 7 | ≤ 6 |
| CP-OFDM 16 QAM | ≤ 7 | ≤ 6 |
| CP-OFDM 64 QAM | ≤ 7 | ≤ 6 |
| CP-OFDM 256 QAM | ≤ 9.5 |  |

NS_13: A-MPR regions

Agreement:

- The following NS_13 A-MPR regions definition was agreed. The corresponding A-MPR values are kept [] for further checking.

| Channel BW | Carrier Frequency, Fc, MHz | RBStart*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- | --- |
| 7MHz |  | ≤1.8 | <1.44 | [A1] |
|  | 820.5 = Fc | ≤1.8 | ≥1.44 | [A2] |
|  |  | >1.8 | ≥1.8 | [A7] |

| Modulation/Waveform | A1 | A2 | A7 |
| --- | --- | --- | --- |
|  | Outer/Inner | Outer/Inner | Outer/Inner |
| DFT-s-OFDM PI/2 BPSK | ≤ 3.5 | ≤ 4.5 | ≤ 3.5 |
| DFT-s-OFDM QPSK | ≤ 3.5 | ≤ 4.5 | ≤ 3.5 |
| DFT-s-OFDM 16 QAM | ≤ 3.5 | ≤ 5 | ≤ 3.5 |
| DFT-s-OFDM 64 QAM | ≤ 4.5 | ≤ 5 | ≤ 3.5 |
| DFT-s-OFDM 256 QAM | ≤ 8 | ≤ 6 | ≤ 3.5 |
| CP-OFDM QPSK | ≤ 5 | ≤ 6.5 | ≤ 4.5 |
| CP-OFDM 16 QAM | ≤ 5 | ≤ 6.5 | ≤ 4.5 |
| CP-OFDM 64 QAM | ≤ 6 | ≤ 6.5 | ≤ 4.5 |
| CP-OFDM 256 QAM | ≤ 8 | ≤ 8 | ≤ 4.5 |

NS_14: A-MPR

Agreement:

- No A-MPR is needed for NS_14 and 7 MHz channel bandwidth.

NS_15: A-MPR regions

Agreement:

- The following NS_14 A-MPR regions definition was agreed. The corresponding A-MPR values are kept [] for further checking.

| Channel BW | Carrier Frequency, Fc, MHz | RBend*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- | --- |
| 7 MHz | 841.5 < Fc ≤ 845.5 | ≥ 4.5 | > 0 | [A1 or A7] |
|  |  | < 4.5, ≥ 2.16 | ≥ 1.8 | [A2] |
|  |  | ≤ 1.44 | ≤ 0.36 | [A3] |
|  | 838 < Fc ≤ 841.5 | > 0 | > 2.7 | [A2] |
|  |  | ≥ 5.94 | ≤ 2.7 | [A3 or A8] |

| Modulation/Waveform | A1 | A2 | A3 | A7 | A8 |
| --- | --- | --- | --- | --- | --- |
|  | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Edge/Outer |
| DFT-s-OFDM PI/2 BPSK | ≤ 9 | ≤ 5 | ≤ 4 | ≤ 7 | ≤ 2 |
| DFT-s-OFDM QPSK | ≤ 9 | ≤ 5 | ≤ 4 | ≤ 7 | ≤ 2 |
| DFT-s-OFDM 16 QAM | ≤ 9 | ≤ 5 | ≤ 4 | ≤ 7 | ≤ 2 |
| DFT-s-OFDM 64 QAM | ≤ 9 | ≤ 5 | ≤ 4 | ≤ 7 | ≤ 2 |
| DFT-s-OFDM 256 QAM | ≤ 9 | ≤ 5 | ≤ 9 | ≤ 7 | ≤ 2.5 |
| CP-OFDM QPSK | ≤ 10.5 | ≤ 6.5 | ≤ 4 | ≤ 8.5 | ≤ 3 |
| CP-OFDM 16 QAM | ≤ 10.5 | ≤ 6.5 | ≤ 4 | ≤ 8.5 | ≤ 3 |
| CP-OFDM 64 QAM | ≤ 10.5 | ≤ 6.5 | ≤ 4 | ≤ 8.5 | ≤ 3 |
| CP-OFDM 256 QAM | ≤ 10.5 | ≤ 6.5 | ≤ 9 | ≤ 8.5 | ≤ 3 |

A-MPR regions for PC2

Agreement:

- The following PC2 NS_12 A-MPR regions definition was agreed as starting point, meaning the exact borders might be further tuned in next meeting but the general shape of each region should be unchanged.

- The corresponding A-MPR values are kept [] for further checking.

| Channel BW | Carrier Frequency, Fc, MHz | RBStart*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- | --- |
| 7MHz | 817.5 ≤ Fc < 820.5 | ≤1.8 | ≤0.72 | [A3] |
|  |  | ≤1.8 | >0.72 | [A1] |
|  |  | >1.8 | >1.44 | [A1] |

Table 3: A-MPR for NS_12 (PC2), from [3]

| Modulation/Waveform | A1 | A2 | A3 |
| --- | --- | --- | --- |
|  | Outer/Inner | Outer/Inner | Outer/Inner |
| DFT-s-OFDM PI/2 BPSK | ≤ 7.5 | 3 | ≤ 5.5 |
| DFT-s-OFDM QPSK | ≤ 7.5 | 3 | ≤ 6 |
| DFT-s-OFDM 16 QAM | ≤ 7.5 | 3.5 | ≤ 6 |
| DFT-s-OFDM 64 QAM | ≤ 7.5 | 4 | ≤ 6.5 |
| DFT-s-OFDM 256 QAM | ≤ 9.5 | 5.0 | ≤ 6.5 |
| CP-OFDM QPSK | ≤ 8.5 | 4.5 | ≤ 7.5 |
| CP-OFDM 16 QAM | ≤ 8.5 | 4.5 | ≤ 7.5 |
| CP-OFDM 64 QAM | ≤ 8.5 | 4.5 | ≤ 7.5 |
| CP-OFDM 256 QAM | ≤ 9.5 | 6.5 | ≤ 7.5 |

NS_13: A-MPR regions for PC2

Agreement:

- The following PC2 NS_13 A-MPR regions definition was agreed as starting point, meaning the exact borders might be further tuned in next meeting but the general shape of each region should be unchanged.

- The corresponding A-MPR values are kept [] for further checking.

| Channel BW | Carrier Frequency, Fc, MHz | RBStart*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- | --- |
| 7MHz |  | ≤1.8 | <1.44 | [A1] |
|  | 820.5 ≤ Fc | ≤1.8 | ≥1.44 | [A2] |
|  |  | >1.8 | ≥1.8 | [A1] |

Table 6: A-MPR for NS_13 (Power Class 2), from [3]

| Modulation/Waveform | A1 | A2 | A3 | A4 |
| --- | --- | --- | --- | --- |
|  | Outer/Inner | Outer/Inner | Outer | Outer |
| DFT-s-OFDM PI/2 BPSK | ≤ 5 | ≤ 6.5 | ≤ 4 | ≤ 3 |
| DFT-s-OFDM QPSK | ≤ 5 | ≤ 6.5 | ≤ 4 | ≤ 3 |
| DFT-s-OFDM 16 QAM | ≤ 5 | ≤ 6.5 | ≤ 4 | ≤ 3 |
| DFT-s-OFDM 64 QAM | ≤ 5 | ≤ 6.5 | ≤ 4 | ≤ 3 |
| DFT-s-OFDM 256 QAM | ≤ 5 | ≤ 7 |  |  |
| CP-OFDM QPSK | ≤ 7 | ≤ 7.5 | ≤ 5.5 | ≤ 4.5 |
| CP-OFDM 16 QAM | ≤ 7 | ≤ 7.5 | ≤ 5.5 | ≤ 4.5 |
| CP-OFDM 64 QAM | ≤ 7 | ≤ 7.5 | ≤ 5.5 | ≤ 4.5 |
| CP-OFDM 256 QAM | ≤ 8 | ≤ 8 |  |  |

NS_14: A-MPR for PC2

Agreement:

- No A-MPR is needed for NS_14 PC2 and 7 MHz channel bandwidth.

NS_15: A-MPR regions for PC2

Agreement:

- The following PC2 NS_15 A-MPR regions definition was agreed as starting point, meaning the exact borders might be further tuned in next meeting but the general shape of each region should be unchanged.

- The corresponding A-MPR values are kept [] for further checking.

| Channel BW | Carrier Frequency, Fc, MHz | RBend*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- | --- |
| 7MHz | 841.5 < Fc ≤ 845.5 | ≥4.68 | >0 | [A1] |
|  |  | <4.68, ≥2.52 | ≥2.16 | [A2] |
|  |  | <4.68, ≥2.52 | <2.16, ≥1.44 | [A2] |
|  |  | <1.8 | ≤0.36 | [A3] |
|  | 837 < Fc ≤ 841.5 | ≥0 | ≥2.7 | [A2] |
|  |  | ≥5.94 | <2.7 | [A3] |

Table 10: A-MPR for NS_15 (Power Class 2), from [3]

| Modulation/Waveform | A1 | A2 | A3 | A4 | A5 | A6 |
| --- | --- | --- | --- | --- | --- | --- |
|  | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner |
| DFT-s-OFDM PI/2 BPSK | ≤ 12 | ≤ 6.5 | ≤ 7 | ≤ 10.5 | ≤ 3 | ≤ 4 |
| DFT-s-OFDM QPSK | ≤ 12 | ≤ 6.5 | ≤ 7 | ≤ 10.5 | ≤ 3 | ≤ 4 |
| DFT-s-OFDM 16 QAM | ≤ 12 | ≤ 6.5 | ≤ 7 | ≤ 10.5 | ≤ 3 | ≤ 4.5 |
| DFT-s-OFDM 64 QAM | ≤ 12 | ≤ 6.5 | ≤ 7 | ≤ 10.5 | ≤ 3 | ≤ 5 |
| DFT-s-OFDM 256 QAM | ≤ 12 | ≤ 6.5 | ≤ 9 | ≤ 13.5 | ≤ 4.5 | ≤ 5 |
| CP-OFDM QPSK | ≤ 13.5 | ≤ 8 | ≤ 7 | ≤ 12 | ≤ 4.5 | ≤ 6 |
| CP-OFDM 16 QAM | ≤ 13.5 | ≤ 8 | ≤ 7 | ≤ 12 | ≤ 4.5 | ≤ 6 |
| CP-OFDM 64 QAM | ≤ 13.5 | ≤ 8 | ≤ 7 | ≤ 12 | ≤ 4.5 | ≤ 6 |
| CP-OFDM 256 QAM | ≤ 13.5 | ≤ 8 | ≤ 9 | ≤ 13.5 | ≤ 6.5 | ≤ 7 |

BS RF

Narrowband blocking – interferer offset

Agreement:

- The narrowband blocking interferer offset for 7MHz channel BW is 400kHz.

ICS

Agreement:

- The size of the interferer for ICS and 7MHz channel BW is15 RBs

RRM performance requirements:

Agreement:

- No need to define new RRM performance requirements to support 7MHz CBW.

- No RRM specification impacts for this WI.

UE Demodulation performance requirements

PDSCH requirements:

Agreement:

- Don’t define 7MHz for PDSCH requirements for single carrier

- Define normal PDSCH type A CA requirements for 7MHz

- Not to consider HST-SFN and HST-DPS scenario.

- Interested companies are welcome to bring results for 7MHz.

- FFS how to define requirements:

  - Option 1: Define new requirements for 7MHz CC based on the simulations

  - Option 2: Reuse existing 5MHz/10MHz requirements

Simulation assumptions for 7MHz CC for CA requirements

Agreement:

- Duplex mode: Only FDD

- Bandwidth/SCS: 7MHz/15kHz

- Number of RB for CORESET:30

- Number of RB: 35PRBs

- MCS: 13

- Rank: Rank2

- MCS: 16QAM,0.48

- Propagation conditions: TDLA30-10

- Antenna configuration: 2T2R, 2T4R ULA Low

- Other parameters: Reuse Table 5.2.1 in TS 38.101-4

SDR requirements

Agreement:

- Define SDR requirements for 7MHz bandwidth

  - For Table 5.5A-1 in TS 38.101-4, add the following in TS38.101-4 for Number of PDCCH candidates and aggregation levels: “2/AL2 for 15kHz / 7MHz”.

  - For Table 5.5A-4 in TS 38.101-4, set the number of CORESET PRBs to ‘30’.

CSI requirements

Agreement:

- Don’t define CQI requirements for 7MHz for single carrier

- Define CQI requirements for 7MHz for CA

- Set CQI subbandSize to ‘8’ for 7MHz CBW in TS 38.101-4 Table 6.2A.3.1.1-2.

PDCCH and PBCH requirements

Agreement:

- Don’t define PDCCH and PBCH requirements for 7MHz

Release independence

Proposal

- Option 1: RAN4 to follow the discussion on release independent for support of 7MHz in RF session, once it is agreed, it should also apply to demodulation and CSI requirements.

BS demodulation requirements

Agreement:

- Don’t introduce BS demodulation requirements for 7MHz

- Don’t update applicability rules for BS supporting different bandwidth.

CR split

Companies are welcome to bring the CRs according to following CR splitting table next meeting.

| CR/simulation summary | Content | Company |
| --- | --- | --- |
| PDSCH requirements with CA for 7MHz | Capture the requirements table for 7MHz for both 2Rx and 4Rx. Capture the number of RB for CORESET in common parameter table | Nokia |
| FRC for 7MHz | Capture the 7MHz FRC | Apple |
| CQI and SDR requirements for 7MHz | Update the test setup for CQI and SDR requirements for 7MHz | CTC |
| Simulation summary for 7MHz | Spread sheet for simulation summary | ZTE |
| Big CR on introducing demodulation and CSI requirements for 7MHz | Big CR | Huawei |

In RAN4#116, the following agreements were made

System parameters – Release independence

Agreement:

- In single carrier, 7 MHz can’t be the lowest channel BW in a band.

- In CA, 7MHz can be min/max CBW of one band in the combination supporting BCS5.

- In MSD, 7MHz cannot be a minimum channel BW in CA.

Agreement:

- Remove the texts “Outer/Inner”, “Outer” and “Edge/Inner” from the NS_12, NS_13, NS_14 and NS_15 A-MPR tables

- RAN4 can further discuss clarification on the general statement on RB allocation for the aforementioned NS values.

UE A-MPR

NS_12: A-MPR regions

Agreement:

- The following NS_12 A-MPR regions definition was agreed.

| Channel BW | Carrier Frequency, Fc, MHz | RBStart*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- | --- |
| 7MHz | 817.5 ≤ Fc ≤ 820.5 | ≤1.8 | ≤0.72 | A3 |
|  |  | ≤1.8 | >0.72 | A4 |
|  |  | >1.8 | >1.44 | A5 |

| Modulation/Waveform | A1 | A2 | A3 | A4 | A5 |
| --- | --- | --- | --- | --- | --- |
|  | Outer/Inner | Outer/Inner |  |  |  |
| DFT-s-OFDM PI/2 BPSK | ≤ 5 | ≤ 4 | ≤ 3.5 | ≤ 4.5 | ≤ 4 |
| DFT-s-OFDM QPSK | ≤ 5 | ≤ 4.5 | ≤ 4 | ≤ 4.5 | ≤ 4 |
| DFT-s-OFDM 16 QAM | ≤ 5.5 | ≤ 4.5 | ≤ 4 | ≤ 5 | ≤ 4.5 |
| DFT-s-OFDM 64 QAM | ≤ 5.5 | ≤ 5 | ≤ 4.5 | ≤ 5 | ≤ 5 |
| DFT-s-OFDM 256 QAM | ≤ 9.5 | ≤ 5 | ≤ 5 | ≤ 5.5 | ≤ 5.5 |
| CP-OFDM QPSK | ≤ 7 | ≤ 6 | ≤ 5.5 | ≤ 6.5 | ≤ 6.5 |
| CP-OFDM 16 QAM | ≤ 7 | ≤ 6 | ≤ 5.5 | ≤ 6.5 | ≤ 6.5 |
| CP-OFDM 64 QAM | ≤ 7 | ≤ 6 | ≤ 6 | ≤ 7 | ≤ 6.5 |
| CP-OFDM 256 QAM | ≤ 9.5 |  |  | ≤ 7 | ≤ 6.5 |

NS_13: A-MPR regions

Agreement:

- The following NS_13 A-MPR regions definition was agreed.

| Channel BW | Carrier Frequency, Fc, MHz | RBStart*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- | --- |
| 7MHz |  | ≤1.8 | <1.44 | A5 |
|  | 820.5 = Fc | ≤1.8 | ≥1.44 | A6 |
|  |  | >1.8 | ≥1.8 | A7 |

| Modulation/Waveform | A1 | A2 | A3 | A4 | A5 | A6 | A7 |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Outer/Inner | Outer/Inner | Outer | Outer/Inner |  |  |  |
| DFT-s-OFDM PI/2 BPSK | ≤ 3.5 | ≤ 4.5 | ≤ 3 | ≤ 3 | ≤ 3.5 | ≤ 4 | ≤ 3 |
| DFT-s-OFDM QPSK | ≤ 3.5 | ≤ 4.5 | ≤ 3 | ≤ 4 | ≤ 3.5 | ≤ 4 | ≤ 3 |
| DFT-s-OFDM 16 QAM | ≤ 3.5 | ≤ 5 | ≤ 3 | ≤ 4 | ≤ 3.5 | ≤ 4.5 | ≤ 3.5 |
| DFT-s-OFDM 64 QAM | ≤ 4.5 | ≤ 5 | ≤ 3 | ≤ 4 | ≤ 4.5 | ≤ 4.5 | ≤ 4 |
| DFT-s-OFDM 256 QAM | ≤ 8 | ≤ 6 |  |  | ≤ 5 | ≤ 6 | ≤ 4.5 |
| CP-OFDM QPSK | ≤ 5 | ≤ 6.5 | ≤ 4.5 | ≤ 5.5 | ≤ 5.5 | ≤ 6 | ≤ 4.5 |
| CP-OFDM 16 QAM | ≤ 5 | ≤ 6.5 | ≤ 4.5 | ≤ 5.5 | ≤ 5.5 | ≤ 6 | ≤ 4.5 |
| CP-OFDM 64 QAM | ≤ 6 | ≤ 6.5 | ≤ 4.5 | ≤ 5.5 | ≤ 6 | ≤ 6 | ≤ 5 |
| CP-OFDM 256 QAM | ≤ 8 | ≤ 8 |  |  | ≤ 8 | ≤ 8 | ≤ 6.5 |

NS_15: A-MPR regions

Agreement:

- The following NS_15 A-MPR regions definition was agreed.

| Channel BW | Carrier Frequency, Fc, MHz | RBend*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- | --- |
| 7 MHz | 841.5 < Fc ≤ 845.5 | ≥ 4.5 | > 0 | A7 |
|  |  | < 4.5, ≥ 2.16 | ≥ 1.8 | A2 |
|  |  | ≤ 1.44 | ≤ 0.36 | A8 |
|  | 838 < Fc ≤ 841.5 | > 0 | > 2.7 | A9 |
|  |  | ≥ 5.94 | ≤ 2.7 | A10 |

| Modulation/Waveform | A1 | A2 | A3 | A4 | A5 | A6 | A7 | A8 | A9 | A10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Edge/Inner |  |  |  |  |
| DFT-s-OFDM PI/2 BPSK | ≤ 9 | ≤ 5 | ≤ 4 | ≤ 9 | ≤ 5 | 1 | ≤ 8.5 | ≤ 5 | ≤ 4 | ≤ 2 |
| DFT-s-OFDM QPSK | ≤ 9 | ≤ 5 | ≤ 4 | ≤ 9 | ≤ 5.5 | 1.5 | ≤ 10 | ≤ 6.5 | ≤ 4.5 | ≤ 2 |
| DFT-s-OFDM 16 QAM | ≤ 9 | ≤ 5 | ≤ 4 | ≤ 9 | ≤ 5.5 | 2 | ≤ 10 | ≤ 6.5 | ≤ 4.5 | ≤ 2.5 |
| DFT-s-OFDM 64 QAM | ≤ 9 | ≤ 5 | ≤ 4 | ≤ 9 | ≤ 5.5 |  | ≤ 10 | ≤ 6.5 | ≤ 4.5 | ≤ 2.5 |
| DFT-s-OFDM 256 QAM | ≤ 9 | ≤ 5 | ≤ 9 | ≤ 13.5 | ≤ 6 |  | ≤ 10 | ≤ 9 | ≤ 5 | ≤ 3.5 |
| CP-OFDM QPSK | ≤ 10.5 | ≤ 6.5 | ≤ 4 | ≤ 10.5 | ≤ 7 | 3 | ≤ 11 | ≤ 6.5 | ≤ 6 | ≤ 3 |
| CP-OFDM 16 QAM | ≤ 10.5 | ≤ 6.5 | ≤ 4 | ≤ 10.5 | ≤ 7 | 3 | ≤ 11 | ≤ 6.5 | ≤ 6 | ≤ 3 |
| CP-OFDM 64 QAM | ≤ 10.5 | ≤ 6.5 | ≤ 4 | ≤ 10.5 | ≤ 7 |  | ≤ 11 | ≤ 6.5 | ≤ 6 | ≤ 4 |
| CP-OFDM 256 QAM | ≤ 10.5 | ≤ 6.5 | ≤ 9 | ≤ 13.5 | ≤ 7.5 |  | ≤ 11 | ≤ 9.5 | ≤ 6.5 | ≤ 5 |

A-MPR regions for PC2

Agreement:

- The following PC2 NS_12 A-MPR regions definition was agreed.

| Channel BW | Carrier Frequency, Fc, MHz | RBStart*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- | --- |
| 7MHz | 817.5 ≤ Fc < 820.5 | ≤1.8 | ≤0.72 | A4 |
|  |  | ≤1.8 | >0.72 | A5 |
|  |  | >1.8 | >1.44 | A6 |

Table 3: A-MPR for NS_12 (PC2)

| Modulation/Waveform | A1 | A2 | A3 | A4 | A5 | A6 |
| --- | --- | --- | --- | --- | --- | --- |
|  | Outer/Inner | Outer/Inner | Outer/Inner |  |  |  |
| DFT-s-OFDM PI/2 BPSK | ≤ 7.5 | 3 | ≤ 5.5 | ≤ 4 | ≤ 6 | ≤ 5 |
| DFT-s-OFDM QPSK | ≤ 7.5 | 3 | ≤ 6 | ≤ 4.5 | ≤ 6 | ≤ 5 |
| DFT-s-OFDM 16 QAM | ≤ 7.5 | 3.5 | ≤ 6 | ≤ 4.5 | ≤ 6.5 | ≤ 5 |
| DFT-s-OFDM 64 QAM | ≤ 7.5 | 4 | ≤ 6.5 | ≤ 5 | ≤ 6.5 | ≤ 5 |
| DFT-s-OFDM 256 QAM | ≤ 9.5 | 5.0 | ≤ 6.5 | ≤ 5.5 | ≤ 7.5 | ≤ 6.5 |
| CP-OFDM QPSK | ≤ 8.5 | 4.5 | ≤ 7.5 | ≤ 6 | ≤ 7.5 | ≤ 6.5 |
| CP-OFDM 16 QAM | ≤ 8.5 | 4.5 | ≤ 7.5 | ≤ 6 | ≤ 7.5 | ≤ 6.5 |
| CP-OFDM 64 QAM | ≤ 8.5 | 4.5 | ≤ 7.5 | ≤ 6 | ≤ 7.5 | ≤ 6.5 |
| CP-OFDM 256 QAM | ≤ 9.5 | 6.5 | ≤ 7.5 | ≤ 7 | ≤ 8 | ≤ 7.5 |

NS_13: A-MPR regions for PC2

Agreement:

- The following PC2 NS_13 A-MPR regions definition was agreed.

| Channel BW | Carrier Frequency, Fc, MHz | RBStart*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- | --- |
| 7MHz |  | ≤1.8 | <1.44 | A5 |
|  | 820.5 ≤ Fc | ≤1.8 | ≥1.44 | A6 |
|  |  | >1.8 | ≥1.8 | A7 |

Table 6: A-MPR for NS_13 (Power Class 2)

| Modulation/Waveform | A1 | A2 | A3 | A4 | A5 | A6 | A7 |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Outer/Inner | Outer/Inner | Outer | Outer |  |  |  |
| DFT-s-OFDM PI/2 BPSK | ≤ 5 | ≤ 6.5 | ≤ 4 | ≤ 3 | ≤ 4 | ≤ 5.5 | ≤ 4 |
| DFT-s-OFDM QPSK | ≤ 5 | ≤ 6.5 | ≤ 4 | ≤ 3 | ≤ 4 | ≤ 5.5 | ≤ 4 |
| DFT-s-OFDM 16 QAM | ≤ 5 | ≤ 6.5 | ≤ 4 | ≤ 3 | ≤ 4.5 | ≤ 5.5 | ≤ 4 |
| DFT-s-OFDM 64 QAM | ≤ 5 | ≤ 6.5 | ≤ 4 | ≤ 3 | ≤ 4.5 | ≤ 6 | ≤ 4 |
| DFT-s-OFDM 256 QAM | ≤ 5 | ≤ 7 |  |  | ≤ 5.5 | ≤ 6 | ≤ 6 |
| CP-OFDM QPSK | ≤ 7 | ≤ 7.5 | ≤ 5.5 | ≤ 4.5 | ≤ 6 | ≤ 6.5 | ≤ 6 |
| CP-OFDM 16 QAM | ≤ 7 | ≤ 7.5 | ≤ 5.5 | ≤ 4.5 | ≤ 6 | ≤ 7 | ≤ 6 |
| CP-OFDM 64 QAM | ≤ 7 | ≤ 7.5 | ≤ 5.5 | ≤ 4.5 | ≤ 6.5 | ≤ 7 | ≤ 6 |
| CP-OFDM 256 QAM | ≤ 8 | ≤ 8 |  |  | ≤ 8 | ≤ 8 | ≤ 8 |

NS_15: A-MPR regions for PC2

Agreement:

- The following PC2 NS_15 A-MPR regions definition was agreed.

| Channel BW | Carrier Frequency, Fc, MHz | RBend*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- | --- |
| 7MHz | 841.5 < Fc ≤ 845.5 | ≥4.68 | >0 | A8 |
|  |  | <4.68, ≥2.52 | ≥2.16 | A9 |
|  |  | <4.68, ≥2.52 | <2.16, ≥1.44 | A10 |
|  |  | <1.8 | ≤0.36 | A11 |
|  | 837 < Fc ≤ 841.5 | ≥0 | ≥2.7 | A12 |
|  |  | ≥5.94 | <2.7 | A13 |

Table 10: A-MPR for NS_15 (Power Class 2)

| Modulation/Waveform | A1 | A2 | A3 | A4 | A5 | A6 | A7 | A8 | A9 | A10 | A11 | A12 | A13 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner |  |  |  |  |  |  |
| DFT-s-OFDM PI/2 BPSK | ≤ 12 | ≤ 6.5 | ≤ 7 | ≤ 10.5 | ≤ 3 | ≤ 0 | ≤ 3.5 | 11 | 6 | 4.5 | 7.5 | 5.5 | 3.5 |
| DFT-s-OFDM QPSK | ≤ 12 | ≤ 6.5 | ≤ 7 | ≤ 10.5 | ≤ 3 | ≤ 4 | ≤ 4 | 11.5 | 6.5 | 5 | 8.5 | 5.5 | 4 |
| DFT-s-OFDM 16 QAM | ≤ 12 | ≤ 6.5 | ≤ 7 | ≤ 10.5 | ≤ 3 | ≤ 4.5 | ≤ 4.5 | 11.5 | 6.5 | 5 | 8.5 | 5.5 | 4 |
| DFT-s-OFDM 64 QAM | ≤ 12 | ≤ 6.5 | ≤ 7 | ≤ 10.5 | ≤ 3 | ≤ 5 | ≤ 5 | 11.5 | 6.5 | 5 | 8.5 | 5.5 | 4 |
| DFT-s-OFDM 256 QAM | ≤ 12 | ≤ 6.5 | ≤ 9 | ≤ 13.5 | ≤ 4.5 | ≤ 5 | ≤ 5 | 12.5 | 6.5 | 5 | 9 | 5.5 | 5.5 |
| CP-OFDM QPSK | ≤ 13.5 | ≤ 8 | ≤ 7 | ≤ 12 | ≤ 4.5 | ≤ 6 | ≤ 6 | 13 | 8 | 6 | 10.5 | 7 | 5 |
| CP-OFDM 16 QAM | ≤ 13.5 | ≤ 8 | ≤ 7 | ≤ 12 | ≤ 4.5 | ≤ 6 | ≤ 6 | 13 | 8.5 | 6 | 10.5 | 7 | 5 |
| CP-OFDM 64 QAM | ≤ 13.5 | ≤ 8 | ≤ 7 | ≤ 12 | ≤ 4.5 | ≤ 6 | ≤ 6 | 13 | 8.5 | 6 | 10.5 | 7 | 5 |
| CP-OFDM 256 QAM | ≤ 13.5 | ≤ 8 | ≤ 9 | ≤ 13.5 | ≤ 6.5 | ≤ 7 | ≤ 7 | 13.5 | 8.5 | 7 | 10.5 | 7.5 | 7 |

UE demodulation and CSI requirements

How to define requirements

Agreement:

- RAN4 to define new requirements for 7MHz for CA with new simulation results.

Release independency

Agreement:

- 7MHz performance requirements are released independent from Rel-15.

CR split

| CR/simulation summary | Content | Company |
| --- | --- | --- |
| PDSCH requirements with CA for 7MHz | Capture the requirements table for 7MHz for both 2Rx and 4Rx. Capture the number of RB for CORESET in common parameter table | Nokia |
| FRC for 7MHz | Capture the 7MHz FRC | Apple |
| CQI and SDR requirements for 7MHz | Update the test setup for CQI and SDR requirements for 7MHz | CTC |
| Simulation summary for 7MHz | Spread sheet for simulation summary | ZTE |
| Introduction of release independency for 7MHz performance requirements | Introduce release independency for 7MHz requirements in TS 38.307 | Ericsson |
| Big CR on introducing demodulation and CSI requirements for 7MHz | Big CR | Huawei |

In RAN4#117, the following agreements were made

- The bigCR was agreed.

2.5	RAN5

N/A.

2.5.1	Agreements

2.5.2	Remaining Open issues

2.5.3	Remaining Open issues with cross-WG dependencies

2.6	RAN6

N/A.

2.6.1	Agreements

2.6.2	Remaining Open issues

3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

n/a

3.1	SAx/CTs

3.1.1	Agreements with cross-TSG impacts

3.1.2	Remaining Open issues with cross-TSG impacts

NOTE: This section should also flag any critical dependencies that need TSG attention.

4.	References

NOTE:	This can be e.g. a list of all related Tdocs in the affected WGs since last TSG, references to LSs, produced TRs/TSs, the work/study item description or status reports of previous TSGs.

4.1	List of RAN WG1 contributions

4.2	List of RAN WG2 contributions

4.3	List of RAN WG3 contributions

4.4	List of RAN WG4 contributions

4.4.1	RAN4#114

R4-2500103	On BS specification impact due to introduction of 7MHz channel bandwidth	Nokia

R4-2500310	On the introduction of 7MHz CBW	Apple

R4-2500408	Introduction of 7MHz channel bandwidth: System parameters and RF requirements	Huawei, HiSilicon

R4-2500420	Inputs to 7MHz CBW	Skyworks Solutions Inc.

R4-2500496	7 MHz Channel Bandwidth for Bands n26 and n5	Qualcomm Incorporated

R4-2500557	Discussion on system parameters and UE RF requirements to introduce 7MHz for n5 and n26	ZTE Corporation, Sanechips

R4-2500558	Discussion on BS RF core requirements for supporting 7 MHz NR FR1 channel bandwidth	ZTE Corporation, Sanechips

R4-2500559	Draft CR to TS38.104 Introduction of 7 MHz NR FR1 channel bandwidth for NR bands n26 and n5	ZTE Corporation, Sanechips

R4-2500560	Draft CR to TS38.141-1 Introduction of 7 MHz NR FR1 channel bandwidth for NR bands n26 and n5	ZTE Corporation, Sanechips

R4-2500561	Draft CR to TS38.141-2 Introduction of 7 MHz NR FR1 channel bandwidth for NR bands n26 and n5	ZTE Corporation, Sanechips

R4-2500679	Topic summary for [114][127] NR_FR1_7MHz_BW	Moderator(Ericsson)

R4-2500695	Work plan on 7MHz Channel Bandwidth for n26 and n5	China Telecom,T-Mobile USA

R4-2501059	Discussion on release independence for 7MHz channel bandwidth	CATT

R4-2501060	Discussion on spectrum utilization and UE RF requirements for 7MHz channel bandwidth	CATT

R4-2501061	Discussion on BS RF requirements for 7MHz channel bandwidth	CATT

R4-2501361	7 MHz channel BW: system parameters and UE RF impacts	Ericsson

R4-2501363	7 MHz channel BW: BS RF impacts	Ericsson

R4-2501479	Discussion on UE RF requirements and system parameters for 7MHz 	MediaTek (Wuhan) Inc.

R4-2501824	Discussion on system parameter and UE RF aspects of 7 MHz channel bandwidth	Nokia

R4-2502133	BS specifications missing in the WID	Huawei, HiSilicon

R4-2502134	BS RF impact for 7 MHz Channel Bandwidth for n26 and n5	Huawei, HiSilicon

R4-2502199	Release Independence of the 7 MHz channel bandwidth	T-Mobile USA, China Telecom, Ericsson

R4-2502200	Spectrum Utilization for the 7 MHz channel bandwidth	T-Mobile USA

R4-2502870	WF on requirements for 7MHz channel bandwidth	Ericsson

R4-2503017	LS on Signalling for 7 MHz Channel Bandwidth	T-Mobile USA

4.4.2	RAN4#114-bis

R4-2503192	On the introduction of 7MHz CBW	Apple

R4-2503232	n26 7 MHz A-MPR simulations and proposal	Nokia

R4-2503234	On BS specification impact due to introduction of 7MHz channel bandwidth	Nokia

R4-2503262	Discussion on initial BWP size restriction to 5 MHz for 7MHz Channel Bandwidth	China Telecom

R4-2503276	draftCR for Tx RF requirements for introduction of 7MHz	Huawei Technologies France

R4-2503473	Clarification on the usage of 7MHz channel bandwidth	CATT

R4-2503474	Further discussion on UE RF requirements for 7MHz channel bandwidth	CATT

R4-2503475	Further discussion on BS RF requirements for 7MHz channel bandwidth	CATT

R4-2503642	Discussion on A-MPR of 7MHz for band n26	ZTE Corporation, Sanechips

R4-2503643	Discussion on BS RF requirements for supporting 7 MHz channel bandwidth	ZTE Corporation, Sanechips

R4-2503950	7 MHz channel BW: UE RF and system parameters	Ericsson

R4-2503951	7 MHz channel BW: BS RF impacts	Ericsson

R4-2504261	Discussion on UE RF requirements for 7MHz CBW	MediaTek (Wuhan) Inc.

R4-2504484	Band n26 A-MPR Analysis for 7 MHz Channel Bandwidth	Qualcomm Incorporated

R4-2504588	OTA requirements consideration for network-nodes specifications for 7MHz channel introduction	Huawei, HiSilicon

R4-2504680	Topic summary for [114bis][122] NR_FR1_7MHz_BW	Moderator(Ericsson)

R4-2505098	WF on UE and BS RF requirements for 7MHz channel bandwidth	Ericsson

R4-2505136	Introduction of 7 MHz channel bandwidth for n26 and n5	Nokia

R4-2505137	Draft CR to TS37.104: Introduction of 7 MHz NR FR1 channel bandwidth	ZTE Corporation, Sanechips

R4-2505138	Draft running CR to TS 38.104 - Introduction of 7 MHz channel BW	Ericsson

R4-2505233	Draft CR to TS 37.105: 7MHz channel bandwidth introduction	Huawei, HiSilicon

4.4.3	RAN4#115

R4-2505387	Discussion on demodulation aspects for NR FR1 7 MHz Channel Bandwidth	China Telecom

R4-2505428	On the introduction of 7MHz CBW	Apple

R4-2505496	Views on NR FR1 7MHz CBW for UE Demod	Qualcomm Incorporated

R4-2505557	Topic summary for [115][208] NR_FR1_7MHz_BW	Moderator (China Telecom)

R4-2505719	A-MPR requirements for 7MHz channel bandwidth	Huawei Technologies France

R4-2505803	On BS RF requirements due to introduction of 7MHz channel bandwidth	Nokia

R4-2505900	On UE Demodulation and CSI Reporting Requirements for 7MHz	Apple

R4-2505972	View on demodulation requirement for 7MHz	Samsung

R4-2506062	Further discussion on UE RF requirements for 7MHz channel bandwidth	CATT

R4-2506063	Further discussion on BS RF requirements for 7MHz channel bandwidth	CATT

R4-2506149	7 MHz channel BW: UE RF and system parameters	Ericsson

R4-2506150	7 MHz channel BW: BS RF impacts	Ericsson

R4-2506151	Running CR to TS 38.104 - Introduction of 7 MHz channel BW	Ericsson

R4-2506153	CR to TS 37113 - Introduction of 7 MHz channel BW	Ericsson

R4-2506318	Discussion on A-MPR of 7MHz for band n26	ZTE Corporation, Sanechips

R4-2506323	Discussion on BS RF requirements for supporting 7 MHz NR FR1 channel bandwidth	ZTE Corporation, Sanechips

R4-2506325	Draft CR to TS38.113: Introduction of 7 MHz NR FR1 channel bandwidth	ZTE Corporation, Sanechips

R4-2506330	Discussion on RRM performance requirements for NR FR1 7 MHz Channel Bandwidth	China Telecom

R4-2506444	Discussion on UE_BS demodulation performance requirements for 7 MHz CBW	Nokia

R4-2506518	n26 7 MHz A-MPR simulations and proposals	Nokia

R4-2506634	Discussion on Rel-19 NR FR1 7 MHz Channel Bandwidth performance requirements	Ericsson

R4-2506675	Discussion on UE demodulation and CSI performance requirements for 7MHz CHBW	Samsung

R4-2506780	Discussion on performance requirements for 7MHz	Huawei, HiSilicon

R4-2506952	Discussion on UE and BS demodulation and CSI performance requirements for 7MHz channel bandwidth	ZTE Corporation, Sanechips

R4-2507031	Discussion on UE and BS performance requirements for 7MHz	Huawei, HiSilicon

R4-2507032	Work plan on UE and BS performance requirements for 7MHz	Huawei, HiSilicon

R4-2507157	RRM impact of 7 MHz channel bandwidth	Ericsson

R4-2507437	Discussion on BS RF in-channel selectivity requirement for 7 MHz channel bandwidth	Huawei, HiSilicon

R4-2507566	Topic summary for [115][127] NR_FR1_7MHz_BW	Moderator (Ericsson)

R4-2507651	Band n26 A-MPR Analysis for 7 MHz Channel Bandwidth	Qualcomm Incorporated

R4-2507652	7 MHz Channel Bandwidth for n5 and n26	Qualcomm Incorporated

R4-2507653	PC2 UE : Band n26 A-MPR Analysis for 7 MHz Channel Bandwidth	Qualcomm Incorporated

R4-2507867	Topic summary for [115][335] NR_7MHz_demod	Moderator (Huawei)

R4-2507880	RRM requirements for 7MHz Channel BW	Qualcomm Incorporated

R4-2507918	draftCR to TS 38.115-1 on 7MHz channel bandwidth	CATT

R4-2507920	CR to TS 38106 - 7 MHz channel BW clarification	Ericsson

R4-2507922	Draft CR to TS37.141: Introduction of 7 MHz NR FR1 channel bandwidth	ZTE Corporation, Sanechips

R4-2507923	Draft CR to TS 37.105: 7MHz channel bandwidth introduction	Huawei, HiSilicon

R4-2507924	Draft CR to TS 37.145-1: 7MHz channel bandwidth introduction	Huawei, HiSilicon

R4-2507925	Draft CR to TS 37.145-2: 7MHz channel bandwidth introduction	Huawei, HiSilicon

R4-2507926	Draft CR to TS 38.141-2: 7MHz channel bandwidth introduction	Huawei, HiSilicon

R4-2507927	Draft CR to TS 38.176-2: restriction of 7MHz channel bandwidth introduction	Huawei, HiSilicon

R4-2507954	WF on NR_FR1_7MHz_BW	Ericsson

R4-2508088	Reply LS to RAN2 on Signalling for 7 MHz Channel Bandwidth	T-Mobile USA

R4-2508260	WF on RRM requirements for NR_FR1_7MHz_BW	China Telecom

R4-2508722	Offline meeting minutes for [115][335] NR_7MHz_demod	Huawei

R4-2508757	Way Forward for [115][335] NR_7MHz_demod	Huawei

4.4.4	RAN4#116

R4-2509094	Topic summary for [116][127] NR_FR1_7MHz_BW	Moderator (Ericsson)

R4-2509143	Draft CR on CQI and SDR requirements for 7MHz	China Telecom

R4-2509144	Discussion and simulation results for UE demodulation requirements for 7MHz	China Telecom

R4-2509389	CR to TS 38.115-1 on 7MHz channel bandwidth	CATT

R4-2509390	draftCR to TS 38.104 on ACLR and blocking requirements for 7MHz channel bandwidth	CATT

R4-2509476	On UE Demodulation and CSI Reporting Requirements for 7MHz	Apple

R4-2509477	Draft CR on FRC for 7MHz CBW	Apple

R4-2509544	On the introduction of 7MHz CBW	Apple

R4-2509578	Big CR on 7 MHz Channel Bandwidth for bands n5 and n26	Qualcomm Incorporated

R4-2509579	7 MHz Channel Bandwidth A-MPR Analysis for band n26	Qualcomm Incorporated

R4-2509857	Views on NR FR1 7MHz CBW for UE Demod	Qualcomm Incorporated

R4-2509942	n26 7 MHz A-MPR proposals	Nokia

R4-2510405	CR to TS37.104 Introduction of 7 MHz NR FR1 channel bandwidth	ZTE Corporation, Sanechips

R4-2510406	CR to TS37.141 Introduction of 7 MHz NR FR1 channel bandwidth	ZTE Corporation, Sanechips

R4-2510407	CR to TS38.113 Introduction of 7 MHz NR FR1 channel bandwidth	ZTE Corporation, Sanechips

R4-2510427	Discussion on UE_BS demodulation performance requirements for 7 MHz CBW	Nokia

R4-2510436	Discussion on UE demodulation and CSI performance requirements for 7MHz CHBW	Samsung

R4-2510464	PC2 A-MPR requirements for 7MHz channel bandwidth	Huawei, HiSilicon

R4-2510510	7 MHz channel BW: UE A-MPR results	Ericsson

R4-2510512	CR to TS 38106 -  7 MHz channel BW clarification	Ericsson

R4-2510513	CR to TS 37113 - Introduction of 7 MHz channel BW	Ericsson

R4-2510832	UE demodulation requirements for 7MHz CBW	Ericsson

R4-2510892	Discussions on demodulation requirements for 7 MHz	Huawei,HiSilicon

R4-2510981	Discussion on UE and BS demodulation and CSI performance requirements for 7MHz channel bandwidth	ZTE Corporation, Sanechips

R4-2510982	Simulation summary for 7MHz	ZTE Corporation, Sanechips

R4-2511315	Release independence of 7 MHz channel bandwidth	Nokia

R4-2511479	Topic summary for [116][333] NR_7MHz_demod	Moderator (Huawei)

R4-2511540	CR to TS 37.105: 7MHz channel bandwidth introduction	Huawei, HiSilicon

R4-2511541	CR to TS 37.145-1: 7MHz channel bandwidth introduction	Huawei, HiSilicon

R4-2511542	CR to TS 37.145-2: 7MHz channel bandwidth introduction	Huawei, HiSilicon

R4-2511544	CR to TS 38.176-2: restriction of 7MHz channel bandwidth introduction	Huawei, HiSilicon

R4-2512618	(NR_FR1_7MHz_BW-Perf) DraftCR on 38.101-4 for adding UE performance requirements for 7 MHz CBW	Nokia

R4-2512619	Way Forward for [116][333] NR_7MHz_demod	Huawei

R4-2512824	Big CR on 7 MHz Channel Bandwidth for bands n5 and n26	Qualcomm

R4-2512829	CR to 38.141-1 on introduction of RF requirements for 7MHz channel bandwidth	Nokia

R4-2512830	Running CR to TS 38.104 - Introduction of 7 MHz channel BW	Ericsson

R4-2512831	CR to TS 38.141-2: 7MHz channel bandwidth introduction	Huawei, HiSilicon

R4-2512850	Release independence of 7 MHz channel bandwidth	Nokia