3GPP TSG RAN #110			RP-253481

Baltimore, USA, Dec. 8-11, 2025

Agenda Item:	8.2.1.4

Source: 	LG Electronics

Title: 	Considerations for Aerial UEs and NTN for 3D mobility

Document for:	Discussion and decision

Introduction

In this contribution, we discuss considerations for 3D mobility, including UAV/UAM/ATG/NTN.

Discussion

- 2.1	Aerial UE support for 3D mobility (UAV/UAM/ATG)

The low-altitude economy enabled by unmanned aerial vehicles (UAVs) is on the verge of exponential growth. For example, UAV-based delivery services have already been commercialized in several provinces and cities in China. In addition, urban air mobility (UAM) pilot services are being prepared in multiple countries, including Korea and China. In NR, several features were introduced In Rel-17 and Rel-19 to support UAV, which includes (but not limited to):

- Altitude-based measurement beam set switching in RRC_IDLE/RRC_CONNECTED

- Altitude-based measurement report triggering

- Measurement report triggering reduction based on the number of triggered cells

- Altitude-based CHO execution

- Altitude-based frequency priority switching for cell reselection

Supporting air-to-ground (ATG) user equipment (UEs) in 3GPP networks would significantly enhance the quality of experience for passengers on board aircraft, provided that ATG UEs can be connected to ground gNBs with a good quality. NR supports features related to Air-To-Ground UEs, which includes (but not limited to):

- RF and RRM requirements for ATG UEs

- Handling of timing and synchronization (similar to NTN)

- ATG-specific system information to provide reference location of gNBs (similar to NTN)

These developments clearly indicate substantial monetization opportunities for mobile network operators with the support of aerial UEs. Given that aerial UEs are already supported in 5G, it is essential that 6G continues to meet their requirements.

To enable 3GPP mobile network operators to fully capitalize on monetization opportunities associated with aerial UEs, support for such UEs should be available no later than Day 1 of 6G deployment. The features already specified in NR for aerial UEs should serve as the baseline for features supported on 6G Day 1. Furthermore, a 6G RAN (6GR) study is recommended to review these features and investigate potential enhancements to support advanced services with more stringent performance requirements for aerial UEs.

Proposal 1: 6G supports features related to aerial UEs, including UAV, UAM, ATG use cases from Day-1.

Proposal 2: The features introduced for UAV and ATG in NR are considered as baseline for 6G support of aerial UEs.

- 2.2	GNSS-independent and multi-connectivity operation for NTN

To ensure resilient service provision of 6G NTN, it is desirable for the radio design to support GNSS-independent operation. This allows, for example, 6G NTN-based communication to be supported without any issues even for UEs with GNSS capability whose location information is unavailable due to GNSS spoofing/jamming and UEs without GNSS capability (e.g., IoT devices) due to increased implementation complexity/battery consumption.

Proposal 3: GNSS-independent operation shall be supported for 6G NTN from Day-1.

Since NR NTN was introduced after TN standardization had already progressed significantly, it was defined by reusing TN-related functions as much as possible without sufficient consideration of its performance optimization and UE implementation complexity. This means that simply reusing NR NTN functions for 6G NTN would negatively impact its necessity and commercialization. One of the enhancement techniques that can be considered in 6G NTN is multi-connectivity between NTN and NTN or between TN and NTN. This can provide benefits such as improved connection stability (by avoiding frequent handovers), increased throughput, etc. In the former case, the NTNs may be located in different orbits (e.g., GSO and NGSO). In both cases, the NTNs, or the TN and NTN, may have different carrier frequencies, which requires features such as dual connection or carrier aggregation for 6G NTN.

Proposal 4: Spectrum aggregation for multi-connectivity between NTN and NTN or between TN and NTN shall be supported in Day-1.

Summary

In this contribution, we discussed the considerations for 3D mobility, including UAV/UAM/ATG/NTN.

Proposal 1: 6G supports features related to aerial UEs, including UAV, UAM, ATG use cases from Day-1.

Proposal 2: The features introduced for UAV and ATG in NR are considered as baseline for 6G support of aerial UEs.

Proposal 3: GNSS-independent operation shall be supported for 6G NTN from Day-1.

Proposal 4: Spectrum aggregation for multi-connectivity between NTN and NTN or between TN and NTN shall be supported in Day-1.