3GPP TSG RAN Meeting #110                                          RP-253607

Baltimore, USA, December 8-11, 2025

Source:	ZTE Corporation, Sanechips

Title:	Views on 6G NTN services

Agenda Item:	8.2.1.4

Document for:    Discussion and Decision

- Introduction

In RAN#107 meeting, the revised SID for study on 6G scenarios and requirements was endorsed with the following objectives related to new/exisiting services and NTN [1].

This study item aims to ----------------------------------------------- Excerpt from SID----------------------------------------------- - Develop 3GPP requirements for 6G Radio for improvement of existing services and for new services. - Determine the applicability of legacy services to 6G Radio, and define radio requirements for these, as appropriate. - Develop 3GPP requirements for 6G Radio for these practical deployment scenarios to ensure substantial gains in all relevant bands: overall performance, user experience, TCO reduction including at least: - Aim at a harmonized 6G Radio design for TN and NTN, including their integration ----------------------------------------------- Excerpt from SID -----------------------------------------------

In this contribution, views on 6G NTN service are elaborated with proposed TP for TR 38.914[2].

- Discussion on requirements for 6G NTN

In 5G NR, NTN with various use cases and assumptions are supported, e.g., data service for handheld device and VSAT UE (e.g., FR2-NTN), IoT service for NB-IoT Device. For 6G, NTN is also considered as one promising way to provide the ubiquitous coverage with use cases defined in TR 22.870 [3]. Among all these use cases, support of basic services, e.g., voice, SMS, should be prioritized at least for all types of devices, e.g., IoT, handheld and VSAT, to ensure always-on connectivity. Additionally, eMBB service with improved UE experience compared to 5G is also important to further boost the commericalization of NTN network for both smart-phone and VSAT.

Proposal 1: 6GR should provide basic services, e.g., voice and SMS for all devices, and eMBB for handheld and VSAT devices.

As mentioned in [3], more scenarios with various assumption on satellite are considered for future deployment. Comparing to 5G, impacts of typical satellite characteristics, e.g., transmission power, beam capabilities, should be prioritized at the beginning of 6GR study to provide better performance, e.g., coverage, service availability. Meanwhile, more complicated and dynamic typology are forseen in future, e.g., (v)LEO and multi-orbits, which requires comphrensive study to support efficient network scheduling, mobility and mangement.

Proposal 2: To enable NTN service in 6G, typical satellite setup, e.g., constellation and satellite capability should be considered in 6GR design.

Proposal 3: 6GR should be able to support mobility design considering various and dynamic network topologies, e.g., intra-NTN across same or different orbits, and TN-NTN.

Regarding details, the following aspects are highlighted in the agreement achieved in RAN1#122-bis.

Agreement - The aspects to consider for supporting NTN include, but not limited to - Initial access, including cell search and SSB periodicity - Coverage - Duplexing - Capacity - Signalling overhead - GNSS-less/resilient/based operation - Large/varying doppler and propagation delay - Beamforming / beam management / beam hopping

In 6GR design, it should be noticed that “harmonized design” should be guaranteed for each of the aspects. For example, in initial access procedure, to minimize “NTN specific design”, the pre-compensation based approach can still be kept as baseline to avoid over-complicated RACH design. Regarding the necessity of GNSS-resilience operation, in addition to impacts on PHY layer, the performance of other procedures, e.g., mobility, should also be well investigated since measurement-based approach is not applicable for NTN cases due to inaccuracy and margin of measurement.

Observation 1: The GNSS-based approach is the basis to enable the common design between TN and NTN.

For others, e.g., HARQ, coverage, the design of 6GR should also take NTN characteristics into account. Moreover, as declared by many stakeholders, 5G based NTN will be deployed in upcoming years before 2030. This means that in future, along with updates of satellites (e.g., LEO), the co-existence of 5G and 6G based services, especially over the same frequency is still foreseen since available spectrum for NTN is limited. In this case, the solution, e.g., MRSS, can be studied along with TN.

Proposal 4: 6GR with harmonized design should be ensured for TN and NTN with minimized NTN-specific feature, especially for the basic functionality.

Based on the discussion above, the TP in Proposal 5 is provided for TR 38.914.

Proposal 5: Capture the following TP in Section 5.4 for TR 38.914.

--------------------------------------------- Start of Text Proposal ------------------------------------------------- ## 5.4 Requirements of New and Existing Services ### 5.4.x Non-terrestrial network 6GR shall support voice, eMBB and IoT service in NTN with various devices, e.g., handheld, VSAT. 6GR shall support non-terrestrial network (NTN) based on harmonized design with Terrestrial network (TN) including: - 6G Radio design shall support various satellite constellations(e.g., LEO, MEO, GEO) with different satellite capability and charateristics, e.g., beam hopping. - 6G Radio design shall support GNSS-based operation with resilience for the non-terrestrial network. - 6G Radio design shall support seamless service for TN-NTN, NTN-NTN mobility. --------------------------------------------- End of Text Proposal-------------------------------------------------

- Conclusion

In this contribution, our views on 6G NTN services are provided above with following proposals:

Proposal 1: 6GR should provide basic services, e.g., voice and SMS for all devices, and eMBB for handheld and VSAT devices.

Proposal 2: To enable NTN service in 6G, typical satellite setup, e.g., constellation and satellite capability should be considered in 6GR design.

Proposal 3: 6GR should be able to support mobility design considering various and dynamic network topologies, e.g., intra-NTN cross same or different orbits, TN-NTN.

Observation 1: The GNSS-based approach is the basis to enable the common design between TN and NTN.

Proposal 4: 6GR with harmonized design should be ensured for TN and NTN with minimized NTN-specific feature, especially for the basic functionality.

Proposal 5: Capture the following TP in Section 5.4 for TR 38.914.

--------------------------------------------- Start of Text Proposal ------------------------------------------------- ## 5.4 Requirements of New and Existing Services ### 5.4.x Non-terrestrial network 6GR shall support voice, eMBB and IoT service in NTN with various devices, e.g., handheld, VSAT. 6GR shall support non-terrestrial network (NTN) based on harmonized design with Terrestrial network (TN) including: - 6G Radio design shall support various satellite constellations(e.g., LEO, MEO, GEO) with different satellite capability and charateristics, e.g., beam hopping. - 6G Radio design shall support GNSS-based operation with resilience for the non-terrestrial network. - 6G Radio design shall support seamless service for TN-NTN, NTN-NTN mobility. --------------------------------------------- End of Text Proposal-------------------------------------------------

- References

- RP-250810 Revised SID: Study on 6G Scenarios and requirements  CMCC, Verizon, NTT DOCOMO, INC., Deutsche Telekom

- RP-252882 3GPP TR 38.914 V0.2.0 Study on 6G Scenarios and Requirements (Release 20)

- RP-253198, General views on 6G deployment scenarios, ZTE Corporation, Sanechips.