3GPP TSG-RAN WG2 Meeting #131bis	R2-2506801

Prague, Czech Republic, October 13th – 17th, 2025

Source:	vivo, NTT DOCOMO, INC.

Title:	Considerations on 6G Data Collection and Data Transfer

Agenda Item:	10.3.3

Document for:	Discussion and Decision

- Introduction

In RAN#108, one of the objectives of the approved SID about “study on 6G Radio” is to support various services and functionalities, which includes the following content [1]. In this contribution, we will discuss the requirements, gap analysis and potential solutions on the data collection and data transfer.

(3) Radio interface protocol architecture and procedures for 6GR [RAN2, RAN1, RAN4, RAN3] a) User Plane architecture and protocol design [RAN2] b) Control Plane architecture (including RRC states) and protocol design [RAN2, RAN3] c) Access stratum security aspects, in alignment with requirements from SA3 [RAN2] d) Radio signalling framework for UE capabilities, aiming at improvements and simplification compared to 5G NR [RAN2, RAN1, RAN4] e) Data transfer design to support various types of data (e.g., AI/ML, Sensing, etc.) [RAN2, RAN3] … (6) Radio Access Network architecture, interface protocols and procedures considering support of various services and functionalities (e.g., AI/ML, sensing, etc.). [RAN3, RAN2]

- Discussion

  - Use cases and requirements

    - General

To better serve the users and manage the network with diverse service types, the 3GPP system need to collect related data and support the data transfer between different entities (e.g., UE, RAN node, CN node). The term “6G System Data” and the requirements for data collection and transfer have been captured in TR 22.870 [2].

6G System Data: the data that is controlled by the 6G system and can be generated or collected by the 6G system. NOTE 1: 6G system data is different from traditional user traffic data which is application level data being transmitted through the 3GPP system for user related services.

[PR 5.9.2.2-1] The 6G system shall support mechanisms for 6G System Data collection and consumption minimizing the impact to 6G services. [PR 5.9.2.2-2] Subject to operator’s policy, regulation and user consent, the 6G system shall support processing of 6G System Data. NOTE: examples of data processing are use case dependant, e.g. data fusion, data anonymization and data analysis. [PR 5.9.2.2-3] Subject to user consent, regulation and operator's policy, the 6G system shall support secure means to expose 6G System Data to authorized trusted third-party, authorized network function or authorized UE. [PR 5.9.2.2-4] The 6G system shall support security and privacy protection of the 6G System Data. [PR 5.9.2.2-5] The 6G system shall be able to provide charging and accounting mechanisms for 6G System Data. [PR 5.9.2.2-6] Subject to regulation, user consent and operator policy, the 6G system shall support storage and retrieval of 6G System Data. [PR 5.9.2.2-7] Subject to regulation and operator policy, the 6G system shall support transfer 6G System Data between different data providers and data consumers within the 6G system.

According to the following WT#5 in the “Study on Architecture for 6G System” [3], RAN should coordinate with SA2, SA3 and SA5 to study a data framework for all aspects related to efficient and scalable data handling.

WT#5: Study data framework for all aspects related to efficient and scalable data handling including, for example, data collection, distribution, processing, storage, data access and data exposure, with consideration of access control/user consent and privacy where relevant. The example of data may include data for AI and Sensing. This WT can also study any potential enhancements on system and procedure needed for user consent framework. NOTE 6: The work split with SA3, SA5 and RAN WGs will require TSG coordination

Several documents in RAN #109 [4~14] address this topic. Different terms (e.g., the transport of new service data, the generation, transport and/or visibility of new service data, data plane, data transfer, data collection, unified data collection framework, model transfer/delivery, data share, etc.) are used to elaborate on the requirements and solutions across these contributions. To ensure the efficiency and consistency of subsequent discussions on this topic within RAN WG(s), it is proposed that RAN2 unify the definitions of the relevant terms before proceeding with in-depth technical deliberations. This term unification will reduce ambiguity, align the understanding of all participants, and provide a solid foundation for the smooth advancement of follow-up work. Taking UE as a data provider, the examples of data collection and data transfer are shown in Figure 2.1-1 and Figure 2.1-2.

Figure 2.1-1: UE as data provider: an example of data collection

Figure 2.1-2: UE as data provider: an example of data transfer

Proposal 1：Adopt the following definitions of terms.

- Data collection refers to the data provider (e.g., UEs or RAN nodes) collecting data (e.g., by measurements) based on data collection configuration (e.g., measurement configuration).

- Data transfer refers to the data provider (e.g., UEs or RAN nodes) transferring data to other entities (e.g., RAN nodes or CN nodes) based on transfer configuration.

    - Sensing

Twenty-four sensing use cases (UCs) are captured in the SA1 requirements TR22. 870 [3], such as low-altitude unmanned aerial vehicle (UAV) supervision, environment object reconstruction, sensing assisted communication in industrial park, etc. There are six potential sensing modes as shown in Figure 2.1-3.

Figure 2.1-3: Sensing modes

In TS 22.137 [15], the basic procedure of sensing has been described, including capability reporting, data collection, data transmission, data processing, and capability exposure. As shown in Table 2.1-1, the data path and termination for sensing data transfer is concluded based on the combination of use cases and sensing modes.

Table 2.1-1: The data path for sensing data transfer

| Data transfer path and termination | An example use case |
| --- | --- |
| UE -> Base Station (BS) | Sensing mode 2/4/6 for the sensing assisted communication in industrial park, etc. Sensing data is used by BS to enhance the communication performance, e.g., optimizing the beam management procedure. |
| UE -> CN | Sensing mode 2/4/6 for the environment object reconstruction, etc. |
| BS -> CN | Sensing mode 1/3/5 for the low-altitude UAV supervision, etc. |

For the evaluation of sensing data volume, the sensing data transmitted by BSs or UEs requires further study and discussion. From our understanding, the potential sensing parameters can be time/frequency-domain samples of original received signals from each antenna port, channel impulse response obtained by channel estimation results, delay, Doppler, angle, signal strength, etc. In addition to the measured parameters, the volume of sensing data also depends on the UC. Some evaluations in [16] and [17] are shown in Table 2.1-2. It indicates that the sensing data rate can reach an order of magnitude of 10 Mbps ~ Gbps. For UE-BS data transfer, the potential size of data packets may exceed the L2 sublayer limit, and the RAN must address this issue. To improve data transfer efficiency, the RAN should support mechanisms, such as data compression to reduce the spectrum overhead.

Table 2.1-2: The data rate for sensing data transfer

| Sensing measurements | An example data rate |
| --- | --- |
| Time/frequency-domain samples of original received signals | 480Gbps [17] |
| Delay/angular/Doppler spectrums | 69Mbps~3.2Gbps [16] |
| Point cloud | 14.6M bits [17] |

Observation 1: Data transfer for sensing is characterized by a high data rate (driven by large volume of sensing measurements and real-time processing requirements for some use cases) and involves multiple transfer combinations (e.g., UE-BS, UE-CN, and BS-CN).

    - AI/ML

Based on the TR 38.843, the potential options of UE-sided data collection are shown in Figure 2.1-4. The potential options of dataset/model parameters sharing for two-sided model are shown in Figure 2.1-5.

Figure 2.1-4: Potential options for UE-sided data collection

Figure 2.1-5: Potential options for dataset/model parameters sharing

As shown in Table 2.1-3, the data path and termination for AI/ML data transfer is concluded based on UE-sided data collection and two-sided dataset/model parameters sharing.

Table 2.1-3: The data path for AI/ML data transfer

| Data transfer path and termination | An example use case |
| --- | --- |
| UE -> BS | UE-sided data collection Option 3. |
| UE -> CN | UE-sided data collection Option 2. |
| BS -> CN | Two-sided dataset/model parameter sharing Option 1 |
| BS -> UE | Two-sided dataset/model parameter sharing Option 2 |

Regarding the data volume of dataset/model parameters sharing, taking CSI compression as an example, the typical data volume for different options is shown in Table 2.1-4.

Table 2.1-4: Data size for dataset/parameter sharing

| Option | Typical data size |
| --- | --- |
| 1. Dataset sharing consisting of {(Target CSI, CSI feedback)} | 225 MB |
| 2. Encoder parameter sharing | 11.6 MB |
| 3. Encoder parameter sharing + dataset sharing consisting of {target CSI} | 161.6 MB |

It can be observed that AI/ML also require data transfer with large data volume. If data transfer continuity during mobility is not supported, and a handover occurs before data transfer is fully completed, the substantial amount of data would need to be retransmitted from the beginning. This would affect data transmission efficiency and increase transmission latency. Therefore, data transfer continuity during mobility must be supported, i.e., the UE should be able to continue receiving or transmitting data when serving cell changes.

Observation 2: Data transfer for AI/ML is characterized by large data volume with a relaxed latency requirement and supports multiple transfer combinations (e.g., UE-BS, UE-CN, BS-CN).

    - Typical data collection and transfer mechanisms in NR

Table 2.1-5 lists some typical mechanisms in NR to collect and transfer specific data type from UE to another entity. It can be observed that different mechanisms are defined to accommodate different requirements, such as varying data size, data content and data termination entities.

Table 2.1-5: Typical data collection mechanisms in NR

| Mechanism | Data size | Data content | Radio Protocol and Radio Bearer | Transfer path and Termination |
| --- | --- | --- | --- | --- |
| Logged MDT | per report<9KB Buffer <64 KB | L3 cell/beam measurements | SRB2 over CP | UE->gNB->OAM |
| Immediate MDT | per report<9KB | L3 cell/beam measurements | SRB1 over CP | UE->gNB->OAM |
| QoE | per report<9KB Buffer <128 KB | Application layer measurement | SRB4/5 over CP | UE->gNB->OAM |
| csi-Logged Measurement | per report<9KB Buffer <64 KB | channel measurement | SRBx over CP | UE->gNB / UE->gNB->OAM |
| LPP for positioning | per report<9KB with LPP segmentation | PRS measurement | SRB2 over CP/ DRB over UP | UE->LMF |

    - Summary of requirements

If each type of 6G system data (e.g., sensing data, AI/ML data) is collected and transferred by independent mechanisms, it results in high standardization overhead. The lack of coordination between these network functions may lead to duplicated data collection. A unified data framework of 6G RAN can enable efficient data collection and transfer within 3GPP system.

Proposal 2: Study a unified data framework in 6G RAN for data transfer and collection of various types of data.

- unified collection of standardized data via use case agnostic procedure, and enables the data to be reused by different data consumers to avoid redundant data collection.

- unified data transfer between peer data transfer entities (e.g., UE-RAN, UE-CN), which reduces the high complexity caused by supporting multiple schemes simultaneously.

Based on the above analysis, the unified data framework shall meet the common requirements for 6G system data.

Proposal 3: The data transfer of 6G RAN shall support various data types with different transmission requirements:

- Large data volume with various requirements on data rate, priority, latency, etc.

- Flexible termination of data at specific node (e.g., BS, CN) as needed.

- Continuity during mobility (i.e., resume transmission of data across BSs).

To reduce the overhead of large-volume data transmission, data compression can be employed to decrease the data volume. Specifically, if data transfer supports lossy compression of data (e.g., using a lower number of quantization bits) based on the requirements of data consumers, the data volume will be significantly reduced. Unlike signalling, encryption and integrity protection can be optional for 6G system data. For data that does not require encryption or integrity protection, on-demand security mechanisms can reduce protocol processing delay.

Proposal 4: The data transfer of 6G RAN shall support downlink/uplink lossy/lossless compression and on-demand security to improve data transfer efficiency.

  - Gap analysis

Based on the 6G data framework requirements outlined in Section 2.1.5 and the Radio Protocol Architecture of NR, the gaps of the existing control plane (CP) and user plane (UP) are analyzed in Table 2.2-1 and Table 2.2-2.

Table 2.2-1: Analysis of gaps for the existing CP

| Requirement aspect | Status in NR and corresponding Gaps |
| --- | --- |
| 1. Large data volume with various requirements (e.g., priority) | Current status: - A single NR RRC message size cannot exceed 9KB. Thus, RRC or higher layer segmentation is needed to transfer data that cannot fit in a single RRC message. NR RRC message only supports up to 5 segments in DL and 16 segments in UL, which amounts to a maximum of 45 KB of downlink data size and 144 KB of uplink data size. - NR RRC message can be transferred via SRBs with different QoS settings (e.g., SRB 1, 2, 4), while NAS messages are transmitted via SRB2 (after SRB2 has been established). The QoS settings (e.g., priority) of SRB2 is common, meaning it applies to all messages transmitted via SRB2. Gaps: - If data is transferred via RRC: The RRC segmentation in NR does not support downlink data exceeding 45 KB or uplink data exceeding 144 KB. - If data is transferred via NAS: Flexible settings (e.g., priority) is not supported, which will have an impact on the other signaling transmission. |
| 2. Continuity during mobility | Current status: - During mobility in RRC_CONNECTED, NR only supports UP data continuity. Specifically, a data forwarding tunnel can be established between the source and target nodes. However, for CP transmission, if messages are not fully transferred before handover, the transmission will be stopped and new transmission can be restarted after mobility completion. Gaps: - Data transfer continuity during mobility is not supported. |
| 3. Flexible termination | Current status: - RRC operates between the UE and the gNB, and NAS operates between the UE and the CN entity. Gaps: - No Gaps. |
| 4. Data compression and On-demand security | Current status: - UDC is not supported for SRB. - Integrity protection and integrity verification are mandatorily configured for signaling over SRB. Gaps: - Data compression is not supported. - On-demand security is not supported. |

Table 2.2-2: Analysis of gaps for the existing UP

| Requirement aspect | Current status and Gaps |
| --- | --- |
| 1. Large data volume with various requirements (e.g., priority) | Current status: - The upper layer (e.g., IP) ensures the PDCP SDU does not exceed 9KB. - Priority settings can be flexible among different DRBs. Gaps: No Gaps if segmentation is up to implementation of upper layer. |
| 2. Continuity during mobility | Current status: - During mobility in RRC_CONNECTED, a data forwarding tunnel can be established between the source and target nodes. Gaps: No Gaps |
| 3. Flexible termination | Current status: - NR user plane is point-to-point transmission and is transparent to gNB. - PDU session terminated at gNB is not supported. Gaps: - Data transfer terminated at gNB is not supported. |
| 4. Data compression and On-demand security | Current status: - Only UL lossless compression (i.e., UDC) is supported in PDCP. - Integrity protection can be optionally configured per DRB. Gaps: - DL Lossless data compression is not supported. - Lossy compression is not supported. |

It can be observed in Table 2.2-1 and Table 2.2-2 that the data transfer schemes based on the existing NR Radio Protocol Architecture (including CP and UP) cannot fulfil the requirements of the 6G data framework.

Observation 3: The potential data transfer schemes based on the existing NR Radio Protocol Architecture (including CP and UP) cannot fulfil the requirements of the 6G data framework:

- For CP-based solution:

- RRC is not suitable for transmitting large-size data due to the limited segmentation number.

- Flexible priority setting for NAS transmission is not supported.

- Data transfer continuity during mobility is not supported.

- Data compression is not supported.

- On-demand security is not supported, e.g., integrity protection is mandatory.

- For UP-based solution:

- Data transfer terminated at gNB is not supported.

- DL Lossless and lossy data compression is not supported.

  - Potential solutions

    - General

From the perspective of 6G data collection and data transfer requirements, transmission of configuration and data between different nodes (e.g. UE-BS, UE-CN, BS-CN, etc.) should be considered.  Among them, the transmission between UE and CN may be carried out in the form of Uu containers. The functions and characteristics of 6G unified data framework in RAN may include:

- Data collection configuration, especially between BS and UE, which is determined by use cases and highly overlapping with the exist RRC configurations such as RRM measurement and reference signals.

- Data transfer configuration, especially between BS and UE, which may be independent from data collection configurations. Data collection configuration is used to define the behaviour of collection data (e.g. then storing for future use) and data transfer configuration to define how to report the data.

- RB management for data transfer, which may involve separate/specific RB(s) addition/modification/release procedures since data transfer may have different QoS or transmission requirements (e.g. termination entities) from current SRB and DRB.

- Data transfer with proper QoS controlling, segmentation & reassembly if packet size is huge, on-demand pre-processing, e.g., compression based on source data (even lossy), service continuity, routing to different end points, etc;

Figure 2.3-1 Solutions for 6G Unified Data Framework

Observation 4: UE-CN data transfer consists of two parts: the UE-BS air interface and the BS-CN backhaul. To support UE-CN data transfer, the 6G RAN shall enable the BS-CN data transfer as part of the UE-CN data transmission chain.  Furthermore, the 6G RAN shall also support the separate data transfer between BS and CN.

    - Solution for data collection

In current 5G AI/ML data collection designs, the network may separately configure the UE to collect data for two use cases. Based on the network's configuration, the UE collects data for the network. These two use cases may share common measurement results, such as RSRP. If data collection is use-case-agnostic, the UE could perform measurement common to multiple use cases only once.

Data collection configuration between BS and UE can reuse RRC layer to be transmitted since the degree of overlap between data collection configuration and current RRC configuration, e.g., RRM and measurement, is relatively high, which makes reusing highly efficient.

Proposal 5: Reusing RRC layer to transmit data collection configuration between BS and UE can be a starting point for 6G unified data collection in RAN.

    - New functions and new RB type for data transfer

In current 5G, lots of data collection/transfer function are achieved via RRC layer, e.g., MDT, positioning etc. In 6G, there will be huge new data collection/transfer requirements such as from the use cases of AI/ML and sensing. It would not be a good choice to execute all or most of the control functions for data collection in the RRC layer, which is not conducive to the expansion of new cases and clear functional split, and also leads to an explosion of RRC functions. Hence, 6G needs a new functional entity, e.g., DSAP (Data Service APplication layer), for data collection management and higher-efficient data transfer.

New DSAP layer is used to process data transfer requirements from AI/ML function, Sensing function, and so on. Data collection configuration can re-use current RRC configuration, e.g., RRM and measurement, to be transmitted between BS and UE. Furthermore, data transfer configuration, e.g., transmitted data and requirements, can be sent either via RRC layer (Option 1) or DSAP layer (Option 2). Selected data can be transmitted between BS and UE via DSAP layer. Data between CN and UE can also be transmitted in the form of Uu DSAP container. There is also a new functional entity, e.g. DSAP-NG, between BS and CN node for data transfer and related configuration.

New DSAP layer can also be used for data Segmentation and reassembly. For example, when the data size of AI model or sensing measurement is relatively large, e.g. larger than 9000 bytes or 1500 bytes, which is not suitable for direct Uu L2 or IP network transmission. An appropriate data PDU size can be obtained via segmentation and reassembly function of DSAP layer. Data compression algorithms may also be considered in DSAP layer, e.g. lossy compression based on the characteristics of the data source.

Furthermore, different data transfer requirements may have different transmission QoS parameters, e.g., higher reliability for AI model transfer, strict delay requirements for some sensing data, priority distinction, etc. DSAP layer can also be used to route/map different data type to a suitable radio bearer for Uu transmission and QoS guarantee and differentiate multiple data into different terminated entities, e.g., BS, CN node or another OTT server.

Figure 2.3-2 Example for New DSAP layer

Hence, we propose:

Proposal 6: Study new functions of RAN for data transfer. These functions form a new protocol (e.g., DSAP layer) in RAN and include at least:

- Segmentation, reassembly and on-demand compression.

- Routing for multiple termination entities and mapping for different QoS requirements.

- Data transfer configuration, e.g., in cooperation with RRC layer.

In current 5G system, SRB is used to carry signalling (i.e. SRB1/2/3) and data (i.e. SRB4/5 for application layer measurement report information) between gNB and UE and DRB is used to carry user’s service packets to/from UPF. The data type transmitted by each SRB are specified and transmission parameters of SRB are pre-defined or configured by gNB. Ciphering and Integrity Protection functions of SRB are enabled by default. Flushing buffer and re-establishment of SRB’s L2 sublayers are performed upon handover. These characteristics make SRB unsuitable for 6G system data transmission, e.g., requested from 6G AI and sensing.

DRB is used to carry QoS flow transmission to/from UPF, which is unsuitable for data transfer between UE and gNB. Furthermore, the protocol stack of DRB has SDAP sub-layer, which performs mapping between QoS flows and DRB and also reflective QoS function. For 6G data transfer function, even between CN and UE there may be neither QoS flow granularity nor through UPF since the purposes and mechanisms of data transfer and user’s service are different. SA2 is also researching and promoting a unified data collection/transfer framework, which may be separate from legacy UP. From the perspective of user privacy, the transmission tunnel and control node of 6G data transfer need to be separate from legacy UP. Hence, legacy DRB is also unsuitable for 6G system data transmission, e.g., requested from 6G AI and sensing.

New Uu radio bearer type, e.g., TRB (Data Transfer Radio Bearer), can be introduced for 6G data transfer requirements. These new TRBs will have configurable transmission attributes, e.g., transmission mode, priority, PBR (Prioritized Bit Rate), PDB (Packet Delay Budget), cipher, integrity protection, service continuity, etc. TRB can on-demand perform addition/modification/release by RRC procedure, e.g., RRC Reconfiguration. For a 6G UE, a configuration with new TRB without any DRB/MRB is supported, i.e., a UE only with TRB and signalling SRBs can stay in RRC_CONNECTED state.

TRB may have different protocol stack from SRB and DRB, e.g., no need for SDAP sub-layer of UP which is up to SA2 final decision for QoS framework and transmission tunnel, and requiring new compression algorithms aimed for special use cases, e.g., lossy compression based on the characteristics of the data source.

Proposal 7: Study a new radio bearer type (e.g., TRB) dedicated for data transfer in 6G RAN, addressing the limitations of existing SRB and DRB.

If new protocol layer and new RB type for 6G data framework are widely recognized and consensus is reached on them in RAN2, this will indicate the introduction of a DP (Data Plane) in addition to the existing CP and UP.

- Conclusion

In this paper, we discuss 6G data collection and data transfer. we have the following observations and proposals:

Proposal 1：Adopt the following definitions of terms.

- Data collection refers to the data provider (e.g., UEs or RAN nodes) collecting data (e.g., by measurements) based on data collection configuration (e.g., measurement configuration).

- Data transfer refers to the data provider (e.g., UEs or RAN nodes) transferring data to other entities (e.g., RAN nodes or CN nodes) based on transfer configuration.

Observation 1: Data transfer for sensing is characterized by a high data rate (driven by large volume of sensing measurements and real-time processing requirements for some use cases) and involves multiple transfer combinations (e.g., UE-BS, UE-CN, and BS-CN).

Observation 2: Data transfer for AI/ML is characterized by large data volume with a relaxed latency requirement and supports multiple transfer combinations (e.g., UE-BS, UE-CN, BS-CN).

Proposal 2: Study a unified data framework in 6G RAN for data transfer and collection of various types of data.

- unified collection of standardized data via use case agnostic procedure, and enables the data to be reused by different data consumers to avoid redundant data collection.

- unified data transfer between peer data transfer entities (e.g., UE-RAN, UE-CN), which reduces the high complexity caused by supporting multiple schemes simultaneously.

Proposal 3: The data transfer of 6G RAN shall support various data types with different transmission requirements:

- Large data volume with various requirements on data rate, priority, latency, etc.

- Flexible termination of data at specific node (e.g., BS, CN) as needed.

- Continuity during mobility (i.e., resume transmission of data across BSs).

Proposal 4: The data transfer of 6G RAN shall support downlink/uplink lossy/lossless compression and on-demand security to improve data transfer efficiency.

Observation 3: The potential data transfer schemes based on the existing NR Radio Protocol Architecture (including CP and UP) cannot fulfil the requirements of the 6G data framework:

- For CP-based solution:

- RRC is not suitable for transmitting large-size data due to the limited segmentation number.

- Flexible priority setting for NAS transmission is not supported.

- Data transfer continuity during mobility is not supported.

- Data compression is not supported.

- On-demand security is not supported, e.g., integrity protection is mandatory.

- For UP-based solution:

- Data transfer terminated at gNB is not supported.

- DL Lossless and lossy data compression is not supported.

Observation 4: UE-CN data transfer consists of two parts: the UE-BS air interface and the BS-CN backhaul. To support UE-CN data transfer, the 6G RAN shall enable the BS-CN data transfer as part of the UE-CN data transmission chain.  Furthermore, the 6G RAN shall also support the separate data transfer between BS and CN.

Proposal 5: Reusing RRC layer to transmit data collection configuration between BS and UE can be a starting point for 6G unified data collection in RAN.

Proposal 6: Study new functions of RAN for data transfer. These functions form a new protocol (e.g., DSAP layer) in RAN and include at least:

- Segmentation, reassembly and on-demand compression.

- Routing for multiple termination entities and mapping for different QoS requirements.

- Data transfer configuration, e.g., in cooperation with RRC layer.

Proposal 7: Study a new radio bearer type (e.g., TRB) dedicated for data transfer in 6G RAN, addressing the limitations of existing SRB and DRB.

- Reference

- RP-251881, New SID: Study on 6G Radio, RAN#108, June 2025.

- S1-253370, 3GPP TR 22.870, Study on 6G Use Cases and Service Requirements, September 2025.

- SP-250806, Study on Architecture for 6G System, SA#109, June 2025.

- RP-252089, Discussion on 6G RAN Architecture, September 2025.

- RP-251995, TP on 6G Requirements for architecture and migration, September 2025.

- RP-252003, Discussion on 6G requirements for data services, September 2025.

- RP-252224, 6G RAN Architecture Requirements, September 2025.

- RP-252080, Requirements for architecture and migration, September 2025.

- RP-251985, Requirements for AI enablers in 6G radio and RAN, September 2025.

- RP-252031, Views on 6G UE data collection and transfer, September 2025.

- RP-252132, Consideration on supporting new AI services for 6G, September 2025.

- RP-252146(CATT) Views on data transfer design, September 2025.

- RP-252204, Requirements for data collection, September 2025.

- RP-252326, AI/ML Framework for 6G, September 2025.

- 3GPP TS 22.137, Service requirements for Integrated Sensing and Communication, March 2024.

- R1-2506093, Discussion on Measurement and Evaluation for ISAC, August 2025.

- R1-2506131, Discussion on 5G-A ISAC evaluation, August 2025.