3GPP TSG-RAN WG2 Meeting #131bis		R2-2506828

Prague, Czech Republic, 13th – 17th Oct. 2025

Source:	TCL

Title:	Discussion on User Plane for 6G RAN

Agenda Item:	10.3.1

Document for:	Discussion and Decision

- Introduction

In RAN#109 meeting, the revised SID for 6G radio was agreed [1]. Based on the revised SID, some requirements in 6G are as follows.

1) 6G RAN should at least support the following services, i.e., mobile broadband communication, immersive communication, IoT, sensing, AI, voice, and regulatory services, etc.

2) In 6G, it is essential to study system simplification. Transmission efficiency and latency should be further optimized.

In this contribution, we will discuss the user plane for 6G radio access technology.

- Discussion

It can be seen from the 6G services that have achieved consensus in RAN meetings so far that the main 6G services are essentially the same as those of 5G—for example, mobile broadband communication, immersive communication, IoT, sensing, AI, voice, and regulatory services, etc. Compared to 5G, 6G will further strengthen performance and expand the breadth and depth of services in these areas.

Specifically, in the current 5G user plane process, it is mainly divided into two parts: the transmission of control information and the transmission of data. Control information (i.e., RRC signaling) is processed and transmitted through the RRC layer, PDCP layer, RLC layer, MAC layer, and physical layer, while data is processed and transmitted through the SDAP layer, PDCP layer, RLC layer, MAC layer, and physical layer. The existing 5G user plane process can already satisfy the service requirements of various 5G scenarios (including highly parallel processing, low latency, high reliability, etc.). Considering that 6G’s service requirements are an extension of those of 5G, the 5G RAN user plane process can serve as the baseline for the 6G user plane process, i.e., most 6G RAN user plane functions can be carried over from 5G.

Proposal 1: The 5G RAN user plane process can serve as the baseline for the 6G user plane process; i.e., most 6G RAN user plane functions can reuse 5G technology.

On this basis, considering that 6G will involve more specialized service scenarios and stricter performance requirements, the design of the user plane also needs to consider the following issues:

Considering that 6G has much more stringent communication requirements (for example, the need for even lower latency and higher reliability), and at the same time 6G aims to reduce processing overhead and air interface resource overhead while enhancing performance, the user plane process—as the core part of transmission—will inevitably require further design improvements to address the above requirements.

Observation 1: The 6G user plane design needs to further reduce latency and improve reliability, while also considering how to further reduce processing overhead and air interface resource overhead.

From the view of reliability, one of the core features of the 5G NR user plane is the use of a two-layer retransmission scheme to ensure reliable data delivery. Specifically, the MAC layer employs Hybrid Automatic Repeat request (HARQ), where the transmitter performs rapid retransmissions with incremental redundancy, and the receiver uses soft combining of multiple transmissions to improve decoding success probability. Meanwhile, the RLC sublayer (in Acknowledged Mode, AM) performs an ARQ function to retransmit any data that HARQ did not successfully deliver. This layered design aims to balance low latency and high reliability requirements: HARQ corrects most errors on the order of milliseconds, while RLC ARQ compensates for residual errors not corrected by HARQ over a longer timescale, with the two mechanisms complementing each other to meet overall reliability needs. It has been proven in practice that 5G, by combining fast error correction at the PHY/MAC layer with slower but reliable recovery at the RLC layer, effectively achieves a high transport block success rate, meeting the requirements of broadband data transmission and even URLLC services.

Observation 2: The transmission reliability of 5G RAN is primarily ensured by HARQ and ARQ. HARQ at the MAC layer provides extremely fast feedback and retransmission, whereas ARQ at the RLC layer offers higher reliability assurance. HARQ rapidly corrects most errors on a millisecond timescale, while RLC ARQ compensates for the residual errors that HARQ fails to correct over a longer timescale. These two mechanisms complement each other.

However, considering the higher requirements of 6G, the current 5G HARQ+ARQ scheme has many limitations. First, the latency associated with RLC ARQ feedback and retransmissions is high — typically on the order of tens of milliseconds or more. This is because the RLC must send status reports and schedule new retransmission PDUs, a process which significantly prolongs the end-to-end delay of data that requires reliable delivery. For services requiring extremely low latency, this additional delay is unacceptable. Second, the RLC ARQ mechanism introduces considerable protocol overhead: the receiver must periodically send status reports (ACK/NACK) over the air interface, which occupy radio resources and increase processing load. In scenarios with poor link quality or high-reliability requirements, status reports become very frequent and their length may increase, consuming valuable air interface resources and adding further overhead. Moreover, unlike HARQ, RLC ARQ cannot take advantage of soft combining; in other words, different ARQ retransmissions of the same data cannot be soft-combined to improve decoding success. In summary, multiple retransmissions driven by status report feedback inherently add a round-trip latency cost for each retransmission. These inefficiencies imply that while the 5G scheme is robust, if applied directly to the much more stringent low-latency and massive connectivity requirements of 6G, the existing 5G scheme would inevitably appear sluggish and cumbersome, and would need improvement.

Observation 3: The coordination of HARQ and ARQ in the current 5G system still has the following issues:

- The latency of RLC ARQ feedback and retransmissions is excessively long, increasing the end-to-end transmission delay for data requiring reliable delivery.

- The RLC layer’s status report feedback must be transmitted over the air interface, and this feedback mechanism itself incurs transmission overhead. In scenarios with poor link quality or high reliability requirements, frequent status reports will occupy valuable air interface resources.

Proposal 2: In 6G, it is necessary to further shorten the retransmission latency while ensuring transmission reliability, and to further reduce the overhead of reception status report feedback.

Looking back at the 5G user plane process, we can see that compared to 4G, its differences mainly stem from 5G’s low-latency and high-bandwidth service requirements. In other words, many of the new designs in the 5G user plane process were introduced to meet the needs for low latency and high bandwidth. For example, 5G adjusted the position of reordering, modified the RLC encapsulation method, and adjusted the positions of concatenation in 4G RLC.

However, in 6G, considering that future communications will include a large amount of communication between AI entities (e.g., token communication), these forms of communication may not necessarily require low latency or high bandwidth. Therefore, low latency and high bandwidth might no longer be the only important service scenarios for 6G.

Proposal 3: The 6G user plane process design should be flexibly adapted to different requirements of services.

- Conclusion

In this contribution, we discuss the user plane for 6G radio access technology. Based on the discussion, we have the following observations and proposals.

Proposal 1: The 5G RAN user plane process can serve as the baseline for the 6G user plane process; i.e., most 6G RAN user plane functions can reuse 5G technology.

Observation 1: The 6G user plane design needs to further reduce latency and improve reliability, while also considering how to further reduce processing overhead and air interface resource overhead.

Observation 2: The transmission reliability of 5G RAN is primarily ensured by HARQ and ARQ. HARQ at the MAC layer provides extremely fast feedback and retransmission, whereas ARQ at the RLC layer offers higher reliability assurance. HARQ rapidly corrects most errors on a millisecond timescale, while RLC ARQ compensates for the residual errors that HARQ fails to correct over a longer timescale. These two mechanisms complement each other.

Observation 3: The coordination of HARQ and ARQ in the current 5G system still has the following issues:

- The latency of RLC ARQ feedback and retransmissions is excessively long, increasing the end-to-end transmission delay for data requiring reliable delivery.

- The RLC layer’s status report feedback must be transmitted over the air interface, and this feedback mechanism itself incurs transmission overhead. In scenarios with poor link quality or high reliability requirements, frequent status reports will occupy valuable air interface resources.

Proposal 2: In 6G, it is necessary to further shorten the retransmission latency while ensuring transmission reliability, and to further reduce the overhead of reception status report feedback.

Proposal 3: The 6G user plane process design should be flexibly adapted to different requirements of services.

- References

- RP-252476, Revised SID: Study on 6G Radio