3GPP TSG-RAN WG2 Meeting #131bis                                                                R2-2506836

Prague, Czech Republic, October 13th – 17th, 2025

Agenda item:	8.9.2

Source:	vivo

Title:	Discussion on RIL V211

Document for:	Decision

# Introduction

According to the outcome of the ASN.1 review of IoT NTN [1], the following issues of S&F are still open and need to be discussed.

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| V211 | IoTNTN | 1 | Indicate the regenerative operation mode to NAS | Yes, R2-2506836 | vivo (Stephen) |  | v003 | ToDo |

In the contribution, we will continue to analyse the above open issues and give our views correspondingly.

# Discussion

According to the RRC spec, UE will forward the received S&F mode indication to upper layers upon acquiring it from SIB1(-NB). And upon receiving the S&F transition time, the UE shall forward the corresponding time information to upper layers as well [2].

#### 5.2.2.7 Actions upon reception of the SystemInformationBlockType1 message Upon receiving the SystemInformationBlockType1 or SystemInformationBlockType1-BR either via broadcast or via dedicated signalling, the UE shall: Skip unrelated part 1> if in RRC_CONNECTED while T311 is not running, and the UE supports multi-band cells as defined by bit 31 in featureGroupIndicators: 2> disregard the freqBandIndicator and multiBandInfoList, if received, while in RRC_CONNECTED; 2> forward the cellIdentity to upper layers; 2> forward the trackingAreaCode to upper layers; 2> forward the trackingAreaList to upper layers, if present; 2> indicate to upper layers that the cell is operating in Store and Forward mode, if sf-OperationMode is present 1> else: 2> if UE is IAB-MT and if iab-Support is not provided for the selected PLMN nor the registered PLMN nor PLMN of the equivalent PLMN list: 3> consider the cell as barred for IAB-MT in accordance with TS 36.304 [4]; 3> perform barring as if intraFreqReselection is set to allowed, and as if the csg-Indication is set to FALSE; 2> else: 3> if the frequency band indicated in the freqBandIndicator or freqBandIndicatorAerial is part of the frequency bands supported by the UE and it is not a downlink only band; or 3> if the UE supports multiBandInfoList, and if one or more of the frequency bands indicated in the multiBandInfoList or multiBandInfoListAerial are part of the frequency bands supported by the UE and they are not downlink only bands: Skip unrelated part 4> indicate to upper layers that the cell is operating in Store and Forward mode, if sf-OperationMode is present; Skip unrelated part

#### 5.2.2.39 Actions upon reception of SystemInformationBlockType31 Upon receiving SystemInformationBlockType31 (SystemInformationBlockType31-NB), the UE shall: 1> start or restart timer T317 with the duration ul-SyncValidityDuration from the subframe indicated by epochTime; 1> forward the t-ModeSwitching to upper layers, if present.

Let us consider a possible scenario, the network operates in S&F mode in time T1, i.e., the sf-OperationMode is present in the SIB1. Upon UE receiving the SIB1, UE indicates the the upper layers that the cell is operating in S&F mode. Then, after UE acquires the SIB31 later, UE will forward the t-ModeSwitching to upper layers. After some time, network changes to the regeneration payload mode in time T2, i.e., the sf-OperationMode is not present in the SIB1. System information modification indication will be indicated and UE will re-acquire the SIB1. Upon receiving the updated SIB1, UE does not indicate any information to upper layers since sf-OperationMode is not present in the SIB1. The t-ModeSwitching is optionally present in the SIB31. That is, if the t-ModeSwitching does not change, the field may probably not present in the SIB31, then UE will not forward any time information to upper layer. Under such a case, the upper layer may misunderstand that the cell is still operating in the S&F operation mode even if it has already changed to the regeneration payload mode.

Figure 1: Changes of the cell operation mode and the corresponding impacts on NW and UE

Observation: The upper layer may misunderstand the cell still operates in S&F mode since no information is forwarded to upper layer even if the cell changes from S&F operation mode to regeneration payload mode, i.e., the sf-OperationMode is not present in SIB1 and the t-ModeSwitching is not present in SIB31.

To address the issue, two possible solutions can be considered.

Option 1: UE indicates to the upper layer the cell operates in regeneration payload mode if the sf-OperationMode is not present in the SIB1.

Based on option 1, the upper layer can explicitly awares the cell operation mode.

Option 2: Add restrictions that the t-ModeSwitching is always present in SIB31 if the cell can operate in S&F mode.

Based on option 2, UE will always forward the t-ModeSwitching to upper layer. Upon receiving both the indication that the cell operates in S&F mode and t-ModeSwitching, the upper layer considers the cell operates in S&F mode and the corresponding sf-OperationMode indicates the time from S&F mode to regeneration payload mode. Upon upper layer only receives the t-ModeSwitching, it considers the cell operates in regeneration payload mode, and the corresponding sf-OperationMode indicates the time from regeneration payload mode to S&F mode.

From our perspective, both of the options can work well. The corresponding TPs for both options are provided in Annex 1 and Annex 2, respectively.

Proposal: (RIL V211) RAN2 to further decide which option is taken to indicate the upper layer that the cell operates in the regeneration payload mode.

- Option 1: UE indicates to the upper layer that the cell operates in regeneration payload mode if the sf-OperationMode is not present in the SIB1.

- Option 2: Clarify t-ModeSwitching is always present in SIB31 if the cell is capable of S&F mode.

# Conclusion

In this contribution, we have further discussed the RIL V211 issue regarding S&F mode indication. And we make the following observation and proposal:

Observation: The upper layer may misunderstand the cell still operates in S&F mode since no information is forwarded to upper layer even if the cell changes from S&F operation mode to regeneration payload mode, i.e., the sf-OperationMode is not present in SIB1 and the t-ModeSwitching is not present in SIB31.

Proposal: (RIL V211) RAN2 to further decide which option is taken to indicate the upper layer that the cell operates in the regeneration payload mode.

- Option 1: UE indicates to the upper layer that the cell operates in regeneration payload mode if the sf-OperationMode is not present in the SIB1.

- Option 2: Clarify t-ModeSwitching is always present in SIB31 if the cell is capable of S&F mode.

# Reference

- R2-2507060, RIL status on IoT NTN Ph3, Huawei, HiSilicon.

- TS38.331, NR; Radio Resource Control (RRC); Protocol specification.

# Annex 1: Option 1 on V211

#### 5.2.2.7	Actions upon reception of the SystemInformationBlockType1 message

Upon receiving the SystemInformationBlockType1 or SystemInformationBlockType1-BR either via broadcast or via dedicated signalling, the UE shall:

1>	if the upper layers indicate the selected core network type as 5GC:

2>	if the cellAccessRelatedInfoList-5GC contains an entry with the plmn-Identity or plmn-Index of the selected PLMN:

3>	in the remainder of the procedures use plmn-IdentityList, trackingAreaCode, and cellIdentity for the cell as received in the corresponding cellAccessRelatedInfoList-5GC containing the selected PLMN;

1>	else if the cellAccessRelatedInfoList contains an entry with the PLMN-Identity of the selected PLMN:

2>	in the remainder of the procedures use plmn-IdentityList, trackingAreaCode, trackingAreaList and cellIdentity for the cell as received in the corresponding cellAccessRelatedInfoList containing the selected PLMN;

1>	if in RRC_IDLE or in RRC_CONNECTED while T311 is running; and

1>	if the UE is a category 0 UE according to TS 36.306 [5]; and

1>	if category0Allowed is not included in SystemInformationBlockType1:

2>	consider the cell as barred in accordance with TS 36.304 [4];

1>	if in RRC_CONNECTED while T311 is not running, and the UE supports multi-band cells as defined by bit 31 in featureGroupIndicators:

2>	disregard the freqBandIndicator and multiBandInfoList, if received, while in RRC_CONNECTED;

2>	forward the cellIdentity to upper layers;

2>	forward the trackingAreaCode to upper layers;

2>	forward the trackingAreaList to upper layers, if present;

2>	indicate to upper layers that the cell is operating in Store and Forward mode, if sf-OperationMode is present;

2>	indicate to upper layers that the cell is operating in normal mode, if sf-OperationMode is not present;

1>	else:

2>	if UE is IAB-MT and if iab-Support is not provided for the selected PLMN nor the registered PLMN nor PLMN of the equivalent PLMN list:

3>	consider the cell as barred for IAB-MT in accordance with TS 36.304 [4];

3>	perform barring as if intraFreqReselection is set to allowed, and as if the csg-Indication is set to FALSE;

2>	else:

3>	if the frequency band indicated in the freqBandIndicator or freqBandIndicatorAerial is part of the frequency bands supported by the UE and it is not a downlink only band; or

3>	if the UE supports multiBandInfoList, and if one or more of the frequency bands indicated in the multiBandInfoList or multiBandInfoListAerial are part of the frequency bands supported by the UE and they are not downlink only bands:

4>	forward the cellIdentity to upper layers;

4>	forward the trackingAreaCode to upper layers;

4>	forward the trackingAreaList to upper layers, if present;

4>	forward the PLMN identity to upper layers;

4>	if in RRC_INACTIVE and the forwarded information does not trigger message transmission by upper layers:

5>	if the serving cell does not belong to the configured ran-NotificationAreaInfo:

6>	initiate an RNA update as specified in 5.3.17.2;

4>	forward the ims-EmergencySupport to upper layers, if present;

4>	forward the eCallOverIMS-Support to upper layers, if present;

4>	indicate to upper layers that the cell is operating in Store and Forward mode, if sf-OperationMode is present;

4>	indicate to upper layers that the cell is operating in normal mode, if sf-OperationMode is not present;

4>	if the UE is capable of 5G NAS:

5>	forward the ims-EmergencySupport5GC to upper layers, if present;

5>	forward the eCallOverIMS-Support5GC to upper layers, if present;

5>	forward cp-CIoT-5GS-Optimisation to upper layers, if present for the selected PLMN;

5>	forward up-CIoT-5GS-Optimisation to upper layers, if present for the selected PLMN;

4>	if the UE is aerial UE and for the frequency band selected by the UE (from freqBandIndicatorAerial or multiBandInfoListAerial), the freqBandInfoAerial or the multiBandInfoListAerial is present and the UE capable of multiNS-Pmax does not support any of the additionalSpectrumEmission in the NS-PmaxListAerial within the freqBandInfoAerial or multiBandInfoListAerial:

5>	consider the cell as barred in accordance with TS 36.304 [4];

5>	perform barring as if intraFreqReselection is set to notAllowed, and as if the csg-Indication is set to FALSE, upon which the procedure ends;

4>	else if the UE is aerial UE and for the frequency band selected by the UE (from freqBandIndicatorAerial or multiBandInfoListAerial), the freqBandInfoAerial or the multiBandInfoListAerial is present and the UE capable of multiNS-Pmax supports at least one additionalSpectrumEmission in the NS-PmaxListAerial within the freqBandInfoAerial or multiBandInfoListAerial:

5>	apply the first listed additionalSpectrumEmission which it supports among the values included in NS-PmaxListAerial within freqBandInfoAerial or multiBandInfoListAerial;

5>	if the additionalPmax is present in the same entry of the selected additionalSpectrumEmission within NS-PmaxListAerial:

6>	apply the additionalPmax;

5>	else:

6>	apply the p-Max;

4>	else if, for the frequency band selected by the UE (from freqBandIndicator or multiBandInfoList), the freqBandInfo or the multiBandInfoList-v10j0 is present and the UE capable of multiNS-Pmax supports at least one additionalSpectrumEmission in the NS-PmaxList within the freqBandInfo or multiBandInfoList-v10j0:

5>	apply the first listed additionalSpectrumEmission which it supports among the values included in NS-PmaxList within freqBandInfo or multiBandInfolist-v10j0;

5>	if the additionalPmax is present in the same entry of the selected additionalSpectrumEmission within NS-PmaxList:

6>	apply the additionalPmax;

5>	else:

6>	apply the p-Max;

4>	else:

5>	apply the additionalSpectrumEmission in SystemInformationBlockType2 and the p-Max;

3>	else:

4>	consider the cell as barred in accordance with TS 36.304 [4]; and

4>	perform barring as if intraFreqReselection is set to notAllowed, and as if the csg-Indication is set to FALSE;

Upon receiving the SystemInformationBlockType1-NB, the UE shall:

1>	if the upper layers indicate the selected core network type as 5GC:

2>	in the remainder of the procedures use plmn-IdentityList, trackingAreaCode, and cellIdentity for the cell as received in the cellAccessRelatedInfo-5GC;

1>	else:

2>	in the remainder of the procedures use plmn-IdentityList, trackingAreaCode, trackingAreaList and cellIdentity for the cell as received in the cellAccessRelatedInfo;

1>	if the frequency band indicated in the freqBandIndicator is part of the frequency bands supported by the UE; or

1>	if one or more of the frequency bands indicated in the multiBandInfoList are part of the frequency bands supported by the UE:

2>	forward the cellIdentity to upper layers;

2>	forward the trackingAreaCode to upper layers;

2>	forward the trackingAreaList to upper layers, if present;

2>	indicate to upper layers that the cell is operating in Store and Forward mode, if sf-OperationMode is present;

2>	indicate to upper layers that the cell is operating in normal mode, if sf-OperationMode is not present;

2>	if attachWithoutPDN-Connectivity is received for the selected PLMN:

3>	forward the attachWithoutPDN-Connectivity to upper layers;

2>	else:

3>	indicate to upper layers that attachWithoutPDN-Connectivity is not present;

2>	if the UE is capable of 5G NAS:

3>	forward ng-U-DataTransfer to upper layers, if present for the selected PLMN;

3>	forward up-CIoT-5GS-Optimisation to upper layers, if present for the selected PLMN;

2>	if, for the frequency band selected by the UE (from freqBandIndicator or multiBandInfoList), the freqBandInfo is present and the UE capable of multiNS-Pmax supports at least one additionalSpectrumEmission in the NS-PmaxList within the freqBandInfo:

3>	apply the first listed additionalSpectrumEmission which it supports among the values included in NS-PmaxList within freqBandInfo;

3>	if the additionalPmax is present in the same entry of the selected additionalSpectrumEmission within NS-PmaxList:

4>	apply the additionalPmax;

3>	else:

4>	apply the p-Max;

2>	else:

3>	apply the additionalSpectrumEmission in SystemInformationBlockType2-NB and the p-Max;

1>	else:

2>	consider the cell as barred in accordance with TS 36.304 [4]; and

2>	perform barring as if intraFreqReselection is set to notAllowed.

No UE requirements related to the contents of SystemInformationBlockType1-MBMS apply other than those specified elsewhere e.g. within procedures using the concerned system information, and/ or within the corresponding field descriptions.

# Annex 2: Option 2 on V211

#### –	SystemInformationBlockType31

The IE SystemInformationBlockType31 contains satellite assistance information for the serving cell. SystemInformationBlockType31 is only signalled for an NTN cell.

SystemInformationBlockType31 information element

-- ASN1START

SystemInformationBlockType31-r17 ::= SEQUENCE {

servingSatelliteInfo-r17		ServingSatelliteInfo-r17,

lateNonCriticalExtension		OCTET STRING					OPTIONAL,

...,

[[	servingSatelliteInfo-v1820	ServingSatelliteInfo-v1820		OPTIONAL	-- Need OR

]],

[[	servingSatelliteInfo-v19xy	ServingSatelliteInfo-v19xy		OPTIONAL	-- Need OR

]]

}

ServingSatelliteInfo-r17 ::=	SEQUENCE {

ephemerisInfo-r17			CHOICE {

stateVectors				EphemerisStateVectors-r17,

orbitalParameters			EphemerisOrbitalParameters-r17

},

nta-CommonParameters-r17			SEQUENCE {

nta-Common-r17					INTEGER (0..8316827)		OPTIONAL,	-- Need OP

nta-CommonDrift-r17				INTEGER (-261935..261935)	OPTIONAL,	-- Need OP

nta-CommonDriftVariation-r17	INTEGER (0..29479)			OPTIONAL	-- Need OP

},

ul-SyncValidityDuration-r17		ENUMERATED {s5, s10, s15, s20, s25, s30, s35, s40,

s45, s50, s55, s60, s120, s180, s240, s900},

epochTime-r17					SEQUENCE {

startSFN-r17					INTEGER (0..1023),

startSubFrame-r17				INTEGER (0..9)

}																OPTIONAL,	-- Need OP

k-Offset-r17					INTEGER (0..1023),

k-Mac-r17						INTEGER (1..512)				OPTIONAL,	-- Need OP

...

}

ServingSatelliteInfo-v1820 ::=	SEQUENCE {

satelliteId-r18				SatelliteId-r18					OPTIONAL,	-- Need OR

referenceLocation-r18			CHOICE {

fixedReferenceLocation-r18			ReferenceLocation-r18,

movingReferenceLocation-r18			ReferenceLocation-r18

}															OPTIONAL,	-- Need OR

distanceThresh-r18				INTEGER(0..65535)			OPTIONAL	-- Need OR

}

ServingSatelliteInfo-v19xy ::=	SEQUENCE {

t-ModeSwitching-r19			TimeOffsetUTC-r17				OPTIONAL	-- Need OR

}

-- ASN1STOP

SystemInformationBlockType31 field descriptions

distanceThresh Distance from the serving cell reference location and is used in location-based measurement initiation in RRC_IDLE (as specified in TS 36.304 [4]) and RRC_CONNECTED. Each step represents 50m.

epochTime Epoch time of the satellite ephemeris data and common TA parameters, see TS 36.213 [23]. This field also indicates the epoch time for the reference location of earth moving cells if present. The reference point for epoch time of the serving satellite ephemeris and Common TA parameters is the uplink time synchronization reference point of the serving cell. epochTime is the starting time of a DL subframe indicated by startSFN and startSubframe. For serving cell, the epochTime is no earlier than the frame where the last repetition of the message indicating the epochTime is transmitted. If the field is absent, the epoch time is the starting time of the DL subframe corresponding to the end of the SI window during which the SI message carrying SIB31(-NB) is transmitted. E-UTRAN always includes epochTime when SIB31(-NB) is provided through dedicated signalling. In case of handover or conditional handover, this field is based on the timing of the target cell, i.e. the startSFN and startSubFrame number indicated in this field refers to the SFN and sub-frame of the target cell, and UE considers the target cell epoch time (indicated by the startSFN and startSubFrame in this field) to be the frame nearest to the frame where RRCConnectionReconfiguration message is received. In case of handover or conditional handover, the reference point for epoch time of the target NTN payload ephemeris and Common TA parameters is the uplink time synchronization reference point of the target cell.

k-Mac Scheduling offset used when downlink and uplink frame timing are not aligned at the eNB, see TS 36.213 [23]. Unit in ms. If the field if absent, the UE uses the (default) value of 0.

k-Offset Scheduling offset used in the timing relationships in NTN, see TS 36.213 [23]. Unit in ms.

nta-Common Network-controlled common TA, see TS 36.213 [23]. Unit of μs. Step of 32.55208 ×10-3 μs. Actual value = field value * 32.55208 ×10-3. If the field is absent, the UE uses the (default) value of 0.

nta-CommonDrift Drift rate of the common TA, see TS 36.213 [23]. Unit of μs/s. Step of 0.2 ×10-3 μs/s. Actual value = field value * 0.2 ×10-3. If the field is absent, the UE uses the (default) value of 0.

nta-CommonDriftVariation Drift rate variation of the common TA, see TS 36.213 [23]. Unit of μs/s2. Step of 0.2 ×10-4 μs/s2. Actual value = field value * 0.2 ×10-4. If the field is absent, the UE uses the (default) value of 0.

orbitalParameters Instantaneous values of the satellite orbital parameters. The signalled values are valid at least for the duration as defined by ul-SyncValidityDuration and epochTime.

referenceLocation Reference location of the NTN (quasi-)earth fixed cell or earth moving cell, used in location-based measurement initiation in RRC_IDLE (as specified in TS 36.304 [4]) and RRC_CONNECTED if distanceThresh is also configured. If configured by an earth moving cell, the broadcast reference location corresponds to the epoch time and is also used in the evaluation of Event D2 and CondEvent D2, and the UE derives the real-time reference location based on the serving satellite ephemeris, see TS 36.304 [4].

stateVectors Instantaneous values of the satellite state vectors. The signalled values are valid at least for the duration as defined by ul-SyncValidityDuration and epochTime.

t-ModeSwitching If sf-OperationMode is present in SIB1, this field indicates the time information on when an NTN cell is going to switch from the Store and Forward Satellite operation mode to the normal mode; otherwise, this field indicates the time information on when an NTN cell is going to switch from the normal mode to the Store and Forward Satellite operation mode. The field is always present if the cell can operate in the Store and Forward Satellite operation mode.

ul-SyncValidityDuration Validity duration of the satellite ephemeris data and common TA parameters, i.e. maximum time duration (from epochTime) during which the UE can apply the satellite ephemeris without acquiring new satellite ephemeris, see TS 36.213 [23]. Unit in second. Value s5 corresponds to 5 seconds, value s10 corresponds to 10 seconds and so on. The ul-SyncValidityDuration is only updated when at least one of epochTime, nta-CommonParameters, ephemerisInfo is updated.