3GPP TSG-RAN WG2 Meeting #131bis 	 R2-2506837

Prague, Czech Republic, October 13th – 17th, 2025

Agenda item:	8.9.2

Source:	vivo

Title:	Discussion on RIL S901 and RIL V214

Document for:	Decision

# Introduction

In the previous meeting, no consensus was achieved on whether the TA report can be included in CB-Msg3-EDT [1]. And no conclusion is made during the post email discussion. In this contribution, our understanding of supporting TA reporting for CB-Msg3-EDT will be provided, which is associated with RIL S901 and RIL V214 [2].

Proposal 7: For TA report in CB-Msg3 EDT, RAN2 to select one of below options. Option 1 - TA report is not included for CB-Msg3-EDT – RAN2 clarifies the RRC procedures. [4/8] Option 2 - TA report is included in the CB-Msg3-EDT according to the RRC procedures – RAN2 clarifies the ta-Report field description. [4/8] Continue the discussion in the post email discussion

# Discussion

Currently, the specification limits TA reporting to the RACH procedure for IDLE UEs. However, with the introduction of CB-Msg3-EDT, there arises a need to re-evaluate this restriction. Specifically, this restriction leads to increased scheduling delays in the CB-Msg3-EDT procedure, particularly considering that CB-Msg3-EDT allows continued scheduling of uplink and downlink data after contention resolution until an RRC message is received. Allowing TA reporting in CB-Msg3-EDT would mitigate these delays and improve overall procedure efficiency.

TS 36.331 5.3.3.2 1> if UE supports timing advance reporting and ta-Report is included in SystemInformationBlockType2: 2> instruct the associated MAC entity to trigger Timing Advance reporting; TS 36.331 6.3.2 ta-Report When this field is included in SystemInformationBlockType2, it indicates reporting of timing advance is enabled during Random Access due to RRC connection establishment, RRC connection resume or RRC connection reestablishment. When this field is included in MobilityControlInfo, it indicates TA reporting is enabled during Random Access due to handover, see TS 36.321 [6], clause 5.4.9. TS 36.331 5.4.9 The UE may be configured to report information about timing advance during a Random Access procedure and in RRC_CONNECTED Mode. The Timing Advance reporting procedure is used in a non-terrestrial network to provide the eNB with an estimate of the UE's Timing Advance, see TTA in TS 36.211 [7] clause 8.1. Timing Advance reporting shall be triggered if any of the following events occur: - if triggered by upper layers; - upon configuration of offsetThresholdTA by upper layers, if the UE has not previously reported Timing Advance value to current Serving Cell; - if the variation between current information about Timing Advance and the last reported information about Timing Advance is equal to or larger than offsetThresholdTA, if configured.

The current specification's limitation of TA reporting to the RACH procedure lacks technical justification. We understand that enabling TA reporting in the CB-Msg3-EDT procedure when configured via SIB2 is technically feasible based on existing procedural text. Thus, it is proposed that RAN2 confirms that TA report can be transmitted in the CB-Msg3-EDT procedure when TA-report is configured in SIB2, in line with the existing RRC and MAC procedural mechanisms. This change would address unnecessary scheduling delays in CB-Msg3-EDT, improving the efficiency of the overall procedure without introducing technical inconsistencies.

In summary, we propose RAN2 to clarify TA report can be reported in CB-Msg3-EDT. The corresponding modification is just to add the CB-Msg3-EDT into the TA report description part. No procedure text change is required. Such a simple modification would reduce scheduling delays in and align with the procedural texts and the field description of RRC and MAC specs.

Proposal 1: (RIL S901) RAN2 to capture in the RRC specification that when ta-Report or ntn-TA-Report is included in SystemInformationBlockType2(-NB), reporting of timing advance is enabled during Random Access or CB-Msg3-EDT.

Proposal 2: (RIL S901) RAN2 to capture in the MAC specification that the UE may be configured to report information about timing advance during a Random Access procedure or a CB-Msg3-EDT procedure.

# 3 Conclusion

In this contribution, we have discussed the RIL issues S901 and V214. And we made the following proposals,

Proposal 1: (RIL S901) RAN2 to capture in the RRC specification that when ta-Report or ntn-TA-Report is included in SystemInformationBlockType2(-NB), reporting of timing advance is enabled during Random Access or CB-Msg3-EDT.

Proposal 2: (RIL S901) RAN2 to capture in the MAC specification that the UE may be configured to report information about timing advance during a Random Access procedure or a CB-Msg3-EDT procedure.

# 4 Reference

[1] R2-2506702, RAN2#131 Meeting Report.

[2] R2-2507060, RIL status on IoT NTN Ph3, Huawei, HiSilicon.