3GPP TSG-RAN WG2 Meeting #131bis                                                   R2-2506843

Prague, Czech Republic, Oct. 13th-17th 2025

Source:	CATT

Title:	Further Discussion on Control Plane Leftover Issues

Agenda Item:	8.13.2

Document for:	Discussion and Decision

Introduction

In RAN2#131 meeting, most of issues had been well handled. In the current contribution, we further share some views on the leftover issues for control plane procedure:

- Paging related leftover issues.

- Fine tuning of extended timer values covering both Uu and PC5 hops.

Discussion

  - Paging related leftover issues

In RAN2#129 meeting, some agreements had been reached for Rel-19 multi-hop U2N relay case. Now, we focus on the below FFS:

Agreements:

An intermediate relay UE in RRC_CONNECTED releases its paging-related information in the parent UE at least for itself.  FFS whether it also releases it for the child UEs and how the child UEs receive the paging when the intermediate relay UE is in RRC_CONNECTED.

In RAN2#129bis meeting, the below agreements were reached:

Agreements:

Support the child UE relying on the parent relay UE for paging monitoring.

Strive to minimize spec impact to support intermediate relay UEs in coverage monitoring paging for a child UE on Uu interface, while avoiding duplicated paging delivery to the remote UE due to double-monitoring by upstream UEs.

With the above green marked part agreement, it is clear that the RRC_CONNECTED intermediate relay UE monitoring paging for its child UE(s) is already agreed. The leftover issue is whether we need to further specify how to avoid duplicated paging delivery to the remote UE due to double-monitoring by the upstream UEs.

In RAN2#131 meeting, the proponent company raised that the situation is similar to what we discussed in the system information part. Once the related intermediate relay UE enters into RRC_CONNECTED,  it can receive the paging-related information through the RRC signalling, that is to say, it can release its paging-related information in the parent UE for the child UEs if there is any. This approach can avoid the duplicated paging delivery issue easily. But considering there is no strong motivation to further add any specification impacts to achieve this function, we propose:

Proposal 1: Once the intermediate relay UE enters into RRC_CONNECTED mode, it is left to intermediate relay UE implementation to release the paging-related information in the parent for its child UE(s) if there is any to avoid the duplicated paging delivery.

Fine tuning of extended timer values covering both Uu and PC5 hops

One leftover issue was raised that related to fine tuning of extended timer values covering both Uu and PC5 hops. There are three possible options to handle this issue shared by rapporteur:

- Option 1 -  Instead of multiplying the timers in UE-TimersAndConstantsRemoteUE  with the hop count we multiply the UE-TimersAndConstants with the hop count ( eg t300 * hop count) that includes only the Uu hop count.

- Option 2 – We introduce a new timer for a PC5 hop, e.g., t300-PC5/t301-PC5/t319-PC5 and calculate the value as t300 for n-hop relay = t300 + t300-PC5*n.

- Option 3 – We multiply the timers in  UE-TimersAndConstantsRemoteUE  with the hop count as currently captured in the CR.

After the discussion of RAN2#131 meeting, Option2 was excluded, and whether to multiply the Rel-17 timers or the legacy Uu-only timers can be further discussed[2]:

Agreements:

Exclude the option of defining new multihop timers for T300/T301/T319.

No change to the running CR now in this respect; whether to multiply the Rel-17 timers or the legacy Uu-only timers can be confirmed in ASN.1 review.  RRC-13 can be closed.

We further share our views on the candidate options: For using Rel-17 timers, Rel-17 timers already consider two hops, if RAN2 finally adapts this as final solution, this will cause unnecessarily prolonged timers in multi-hop configurations. For using Uu-only timers, at least it is more reasonable to use as a multiplier with simplification for the case what we are talking about. Hence, we propose:

Proposal 2:  RAN2 agrees to multiply the timers in ue-TimersAndConstants with the hop count (eg t300 * hop count) for multi-hop relay T300/T301/T319 cases.

Conclusion

According to the analysis in section 2, it is proposed:

Paging related leftover issues

Proposal 1: Once the intermediate relay UE enters into RRC_CONNECTED mode, it is left to intermediate relay UE implementation to release the paging-related information in the parent for its child UE(s) if there is any to avoid the duplicated paging delivery.

Fine tuning of extended timer values covering both Uu and PC5 hops

Proposal 2:  RAN2 agrees to multiply the timers in ue-TimersAndConstants with the hop count (eg t300 * hop count) for multi-hop relay T300/T301/T319 cases.

Reference

- R2-2504674	Report from session on positioning and sidelink relay	Session chair (MediaTek)	Report

- R2-2506204	Report from session on positioning and sidelink relay	Session chair (MediaTek)	Report