3GPP TSG-RAN WG2 Meeting #131bis	R2-2506874

Prague, Czech Republic, Oct. 13th-17th

Agenda Item:	9.7

Source: 	CATT

Title: 	Discussion on supporting IMS voice call over GSO for NB-IoT

Document for: 	Discussion and Decision

1	Introduction

This contribution discusses enhancements to NB-IoT NTN that enable IMS voice call over GSO, in line with the objective in the WID [1]. LSs [2][3] from SA2 and SA4 are also discussed.

### 4.1 Objective of SI or Core part WI or Testing part WI The aim of this work item is to specify the enhancement of NB-IoT-NTN to support IMS voice call over GSO with the following objectives: - Down-select between CP and UP solutions for voice support over NB-IoT-NTN until RAN#110 [RAN2] - Coordination with SA is required - The objective can be updated with further details based on the outcome of the above down-selection - Support of semi-persistent scheduling for NB-IoT-NTN for DL and UL data transmission for voice packets [RAN2, RAN1] - Support of necessary modifications to RRC connection setup procedure for NB-IoT-NTN [RAN2] - Support of necessary modifications for emergency call for voice over NB-IoT-NTN [RAN2] - Study and if feasible, specify UE transmit power higher than PC1 (e.g. up to 37dBm) for NB-IoT-NTN [RAN4] - Note: The enhancements to support voice over NB-IoT-NTN via GSO in this work item may be also applicable to NGSO cases without additional specification enhancements - Note: Coordination with SA4 is expected

2	Discussion

2.1	CP vs UP solution

The key issue on supporting IMS voice call over GSO for NB-IoT is to down select between CP and UP solutions. This section gives analysis on CP solution and UP solution with considering the spec impacts and also gives our answers on the questions from SA2 and SA4 [2][3].

- Background

As supported by LTE, the IMS voice call service is composed of IMS signalling and voice data, both of them are regarded as user data. Reference to TS 23.203 [4], the IMS signalling has QCI 5 with Non-GBR resource type, while the voice data has QCI 1 with GBR resource type. This implies that the IMS signalling and voice need to be carried in 2 EPS bearers which are mapped to 2 DRBs over the Uu.

| QCI | Resource Type | Priority Level | Packet Delay Budget (NOTE 13) | Packet Error Loss Rate (NOTE 2) | Example Services |
| --- | --- | --- | --- | --- | --- |
| 1 (NOTE 3) | GBR | 2 | 100 ms (NOTE 1, NOTE 11) | 10-2 | Conversational Voice |
| 5 (NOTE 3) | Non-GBR | 1 | 100 ms (NOTE 1, NOTE 10) | 10-6 | IMS Signalling |

Observation 1: For legacy LTE UE, the IMS signalling and voice data are carried in 2 EPS bearers which are mapped to 2 separate DRBs over the Uu interface.

Considering the NB-IoT UE has reduced capability compared with normal LTE UE, how to support the IMS voice call for NB-IoT UE needs to be discussed. The following gives spec impact analysis of CP solution and UP solution.

- Analysis of CP solution and UP solution

For CP solution, the IMS signalling and the voice data are contained in NAS message which is carried in SRBs over the Uu. For NB-IoT, SRB1bis is for NAS messages prior to activation of security, while SRB1 is used after AS security is achieved. An NB-IoT UE that only supports the Control Plane CIoT optimisation only establishes SRB1bis [5]. To support IMS voice call, some impacts/issues are identified:

- Considering the IMS signalling and the voice data have different requirements on delay and packet error loss rate, it is reasonable to map them into 2 separate SRBs over the Uu. As both IMS signalling and voice data are contained in NAS packet and protected by NAS layer security, there is no need to support AS layer security additionally. Therefore, apart from SRB1bis, it is better to introduce a new SRB, and the new SRB can be established with no PDCP entity.

- The voice data has relatively loose packet loss rate requirement but high delay requirement. RLM UM mode is more suitable and usually used. However, RLC AM is the only applicable RLC mode for SRB [5]. RAN2 could consider supporting RLC UM mode for the SRB which bears the voice data. This issue is also identified by SA2 in the LS [2].

- To map the NAS packets containing IMS signalling and voice data to the corresponding SRBs, the NAS layer should be able to differentiate the content of the NAS packets. How to achieve that needs to be discussed with CT1 involved.

Additionally, at present, the QoS control mechanism can only applied to EPS bearer over DRB. If CP solution is adopted, how to support QoS control for the IMS voice call service over SRB is unclear.

Observation 2: If CP solution is adopted for IMS voice call, the following issues/impacts need to be considered:

- Consider introducing a new SRB, so that the IMS signalling and the voice data are carried over the existing SRB1bis and the new introduced SRB separately;

- Consider supporting RLC UM mode for the new introduced SRB;

- The NAS layer should be able to differentiate the IMS signalling and voice data to map different packets to corresponding SRBs. How to achieve that needs to be discussed with CT1 involved;

- QoS control for the IMS voice call service over SRB needs to be specified.

For UP solution, the IMS signalling and voice data are carried in DRBs over the Uu, which can be supported naturally. One noteworthy issue is, if 2 DRBs are used to separately carry the IMS signalling and the voice data to differentiate their QCI requirements, there is no DRB left for other service when an IMS voice call service is on-going, as the NB-IoT can support at most 2 DRBs currently. One way to solve this issue is to support one additional DRB for NB-IoT, but this method unavoidably improved the complexity of UE. From our perspective, it is not common case for the NB-IoT having IMS voice call service and another service at the same time; and also the data rate supported by the NB-IoT is also difficult to support the simultaneous execution of IMS voice call services and other services. Maybe we can just keep the current NB-IoT capability as it.

Observation 3: NB-IoT currently supports at most 2 DRBs. For UP solution, the IMS signalling and the voice data can be carried in the existing 2 DRBs, allowing the UP solution to be supported naturally.

Observation 3a: If these 2 DRBs are used to separately carry the IMS signalling and voice data, there is no DRB left for other service during an on-going IMS voice call. Therefore, the necessity for an additional DRB can be discussed.

With the above analysis, the CP solution needs significant spec impacts and cross group work. It is proposed to go with the UP solution for NB-IoT to support IMS voice call over GSO. On basic of UP solution, we can further discuss whether to support one additional DRB so that parallel service of IMS voice call and other service can be supported.

Proposal 1: UP solution is preferred for NB-IoT to support IMS voice call over GSO.

Proposal 1a: Further discusses whether to support one additional DRB to enable parallel service of IMS voice call and other service.

- Discussion on the LS from SA2

This section discusses the questions in SA2’s LS [2].

During the voice call, if enough consecutive packets are lost or erroneously decompressed, the compressor can find itself leave Second Order (SO) state and enter First Order (FO) state, and then if another enough consecutive packets are lost or erroneously decompressed, the compressor can find itself leave First Order state (FO) and enter the Initialization and Refresh (IR) state, causing gaps in the transmission of the voice packets.Some companies indicated in SA2 that the number of consecutive lost packets that will trigger the compressor to fall back from SO state to FO state is around 16 (e.g., when UO-0 header with 4 bits SN is used in SO state), and the number of consecutive lost packets that will trigger the compressor to fall back from FO state to IR state is around 64 (e.g., when UOR-2 header with 6 bits SN is used in FO state). Question 1 (To RAN2): Does RAN2 have any observation on how many consecutive packets lost or erroneously decompressed will trigger the RoHC state fall back at the compressor when using RoHC?

In the PDCP spec [6], the ROHC framework is introduced with reference to IETF RFC files [7] without including any implementation details. In the IETF RFC files, it only specified that the compressor falls back from SO state to FO state when the CRC check of k_1 out of the last n_1 decompressed packets have failed in decompressor, and the compressor falls back from FO state to IR state when the CRC check of k_2 out of the last n_2 decompressed packets have failed in decompressor. The appropriate values for k_1, n_1, k_2, and n_2, are related to the residual error rate of the link. Therefore, the number of consecutive packets lost or erroneously decompressed will trigger the RoHC state fall back at the compressor is related to the residual error rate of the link and not clearly specify in the IETF RFC files.

Observation 4: The number of consecutive packets lost or erroneously decompressed that will trigger ROHC state fall back is not specified in RAN2’s spec. According to the IETF RFC files, the number of consecutive packets lost or erroneously decompressed will trigger the ROHC state fall back is defined as variables, e.g., k_1 and k_2, these variables are related to the residual error rate of the link and not clearly specified.

Voice packets of different sizes can be observed over the NB-IoT NTN link e.g. due to talk/silence transitions, or occasionally RoHC state change, etc. The size difference of voice packets during talk/silent transition is described in S2-2506371/S2-2507021/S2-2507102 (neither endorsed nor agreed). More information on the RoHC related issues is described in S2-2506371, S2-2507021 and S2-2507102 that are neither endorsed nor agreed by SA2. Question 3 (To RAN2): Can RAN2 confirm whether scheduling methods for support of IMS voice over NB-IoT NTN can handle the voice packets of different sizes changing dynamically?

For NB-IoT, the dynamic resource allocation is support for both DL and UL transmission as basic. For DL, the network knows the data volume of the IMS voice call service and scheduling corresponding resource for the NB-IoT dynamically. For UL, NB-IoT can make use of SR and BSR [8][9] to report its buffer status, so that the network can assign the needed UL grant towards the UE. Additionally, per guidance of the WID [1], RAN2 will support SPS for NB-IoT to efficient the scheduling procedure for IMS voice call. With the existing scheduling mechanisms and the SPS to be supported, the NB-IoT can handle the voice packets of different sizes changing dynamically.

Observation 5: With the existing mechanisms, including dynamic resource allocation, SR and BSR, and the SPS to be supported, the NB-IoT can handle the voice packets of different sizes changing dynamically.

SA2 has observed that RAN2 has enforced the limitation that the capability of a NB-IoT UE supports a maximum 2 DRBs. In order for SA2 to reach a conclusion to use the user plane and DRBs to support IMS voice, SA2 would like to understand whether NB-IoT Access Stratum protocols can support more than 2 DRBs (e.g. 3 DRBs) in Rel-20, then the UE can support simultaneous voice and other services even during the call. Question 6 (To RAN2): Is it feasible to support more than 2 DRBs for a UE accessing NB-IoT in Rel-20?

It is feasible to support more than 2 DRBs for NB-IoT. We understand that SA2 raises this question is intend to support other service when an IMS voice call service is on-going. As analysed in section 2.1, RAN2 could further discuss whether it is essential to introduce one additional DRB to enable parallel service of IMS voice call and other service or just keep the current NB-IoT capability as it (proposal 1a).

Observation 6: It is feasible to support more than 2 DRBs for NB-IoT.

Currently, SRBs are only supported over RLC Acknowledged Mode (AM) (TS 36.331), and the possible impacts of this are discussed in S2-2507107, which is not endorsed or agreed by SA2. In order for SA2 to be able to evaluate between user-plane and control-plane solutions to carry voice media, the use of RLC Unacknowledged Mode (UM) for SRBs, SA2 has the following question: Question 9 (To RAN2): Can RAN2 provide feedback on whether it is possible to define a new SRB that will be used to carry voice media? Question 10 (To RAN2): If the answer to the above question is yes, can RAN2 confirm whether such SRB(s) are defined, whether it is technically feasible to support and configure RLC UM for such SRB(s) for example: in NB-IoT deployments over GEO satellite, when SRBs are used to carry voice media?

It is feasible to define a new SRB for NB-IoT and it is also technically feasible to support RLC UM for SRBs. However, support a new SRB and will increase the device complexity. And as analysed in section 2.1, the CP solution requires significant impacts, we tend not to go for the CP solution.

Observation 7: It is feasible to support a new SRB for NB-IoT and it is also feasible to support RLC UM for SRBs.

Reply the above answers to SA2 and show RAN2’s preference on the UP solution.

Proposal 2: Reply an LS to SA2 providing RAN2’s answers to the questions in S2-2507636 and showing our preference for the UP solution:

- Answer to Q1: According to the IETF RFC files, the number of consecutive packets lost or erroneously decompressed will trigger the ROHC state fall back is defined as variables, e.g., k_1 and k_2, these variables are related to the residual error rate of the link and not clearly specified.

- Answer to Q3: Scheduling methods for support of IMS voice over NB-IoT NTN can handle the voice packets of different sizes changing dynamically

- Answer to Q6: Supporting more than 2DRBs for NB-IoT is feasible.

- Answer to Q9: Supporting a new SRB for NB-IoT is feasible.

- Answer to Q10: RLC UM mode for SRBs is feasible.

- From RAN2’s perspective, the IMS voice call can be naturally supported by the UP solution, whereas the CP solution requires significant spec impacts and cross group work. The UP solution is preferred for NB-IoT to support IMS voice call over GSO.

- Discussion on the LS from SA4

This section discusses the questions in SA4’s LS [2].

# To SA2 and RAN2: ACTION: SA4 kindly asks SA2 and RAN2 to comment on - the different options among UP/CP and IP/Non-IP and the respective overall packet overhead (including RTP/UDP/IP with RoHC, PDCP, RLC and MAC and any potential AS layer optimization if applicable), and if there is any preferred option, and - specifically, whether a packet overhead of 1 byte of MAC header is realistic.

Reference to TS 36.321 [9], a MAC PDU header consists of one or more MAC PDU subheaders; each subheader corresponds to either a MAC SDU, a MAC control element or padding. A MAC PDU subheader consists of the header fields R/F2/E/LCID/(R/R/eLCID)/(F)/(L) with length 1~4 bytes. Considering the L field is present in the MAC PDU subheader except for the last subheader in the MAC PDU and fixed sized MAC control elements, and the MAC SDU containing the IMS voice call data typically has a variable length, the 2 bytes R/F2/E/LCID/F/L is the minimum length MAC subheader that can be used. To fill up the assigned TB, padding is added at the end of the MAC PDU, and a 1 byte R/F2/E/LCID type MAC PDU subheader is needed to be added in the MAC header. Then, the MAC PDU already has a 2 bytes or 3 bytes (with padding) length MAC header, besides the case of multiplexing several MAC SDUs and MAC CEs. Obviously, 1 byte MAC header is not realistic.

Figure 6.1.2-3: Example of MAC PDU consisting of MAC header, MAC control elements, MAC SDUs and padding

For the first question, since the packet overhead of the RTP/UDP/IP is out of RAN2 scope, this contribution only gives the packet overhead of RRC and Layer 2. Reference to [5, 6, 9, 10], the following gives the minimum protocol header size of each layers.

Table 1. Minimum protocol header size of each AS layers

|  | ## CP | ## UP |
| --- | --- | --- |
| ## RRC | ## UL: 2 bytes ## DL: 2 bytes | ## 0 byte |
| ## PDCP | ## 0 byte | ## 1 byte |
| ## RLC | ## UM: 1 byte ## AM: 2 bytes | ## UM: 1 byte ## AM: 2 bytes |
| ## MAC | ## 2bytes or 3 bytes (with padding) | ## 2bytes or 3 bytes (with padding) |

As for the preference among UP/CP and IP/Non-IP, considering the main impacts of IP/Non-IP is on SA2, we can only show our preference on the UP/CP solution. It is proposed:

Proposal 3: Reply an LS to SA4, gives RAN2’s answers on the questions in S4-251584 and show our preference on the UP solution:

- Minimum protocol header size of each AS layers is given in Table 1.

- 1 byte MAC header is not realistic.

- From RAN2’s perspective, the UP solution is preferred for NB-IoT to support IMS voice call over GSO.

2.2	Semi-persistent scheduling

The IMS voice call service is composed of talk spurt and silent period, and the data packets of IMS voice call service within the talk spurt and silent has periodic characteristics. Support SPS for NB-IoT is meaningful to efficient the scheduling procedure for IMS voice call. Take VoLTE AMR as example, the packet interval of talk spurt is 20ms and the packet interval of silent period is 160ms [11]. In case of IMS voice call over GSO, SA4 also considers defining other packets intervals for NB-IoT. Since the candidate values of SPS interval is closely related to the packets interval of IMS voice call service, it is proposed to send an LS to SA4 to ask for their progress before RAN2 determines the SPS parameters.

Observation 8: For VoLTE AMR, the packet interval of talk spurt is 20ms and the packet interval of silent period is 160ms. In case of NB-IoT supports IMS voice call over GSO, SA4 also considers defining other packets intervals.

Proposal 4: The candidate values of SPS interval is closely related to the packets interval of IMS voice call service. In case of IMS voice call over GSO for NB-IoT, send an LS to SA4 asking:

- Whether talk spurt and silent period support different packet intervals?

- The candidate values of supported packets intervals.

2.3	Other issues

Some other issues are identified for NB-IoT supports IMS voice call over GSO.

- Support of necessary modifications to RRC connection setup procedure for NB-IoT-NTN

Since the IMS voice call is supported for NB-IoT, the NB-IoT UE needs to know whether a GSO cell supports the IMS voice call service, so that it could decide whether to initiate the IMS voice call service when camping on this cell. An indication for that use can be introduced in SIB2-NB.

Proposal 5: Introduce an indication in SIB2-NB to indicate network supports IMS voice call over GSO for NB-IoT.

If the UE determines the cell supports the IMS voice call over GSO and a requirement is received from the upper layers, it initiates a RRC connection setup procedure. For network information, an establishment cause for IMS voice call can be added in the establishementCause-NB IE. An example TP is shown as following.

#### – EstablishmentCause-NB The IE EstablishmentCause-NB provides the establishment cause for the RRC connection request or the RRC connection resume request as provided by the upper layers. EstablishmentCause-NB information element -- ASN1START EstablishmentCause-NB-r13 ::= ENUMERATED { mt-Access, mo-Signalling, mo-Data, mo-ExceptionData, delayTolerantAccess-v1330, mt-EDT-v1610, mo-VoiceCall-v20xx, spare1} -- ASN1STOP

Proposal 6: Add an establishment cause for IMS voice call in the establishementCause-NB IE.

- Support of necessary modifications for emergency call for voice over NB-IoT-NTN

If emergency call is supported for NB-IoT, the network should also indicate that to UE. For LTE, the UE can perform cell (re)selection procedure with considering whether the cell supports emergency call, to prevent UE camping on a cell does not support IMS emergency call (shown as the yellow highlight text). For NB-IoT, camp on any cell state is supported in Rel-19, after the emergency call service is supported in Rel-20, the following text could also be applicable to NB-IoT. For this purpose, an indication of cell supports emergency call can be introduced in SIB1-NB.

### 5.2.9 Camped on Any Cell state In this state, the UE shall perform the following tasks: - monitor the paging channel of the cell as specified in clause 7 according to information sent in system information; - monitor relevant System Information as specified in TS 36.331 [3]; - perform necessary measurements for the cell reselection evaluation procedure; - execute the cell reselection evaluation process on the following occasions/triggers: 1) UE internal triggers, so as to meet performance as specified in TS 36.133 [10]; 2) When information on the BCCH or BR-BCCH used for the cell reselection evaluation procedure has been modified; - regularly attempt to find a suitable cell trying all frequencies of all RATs that are supported by the UE. If a suitable cell is found, UE shall move to camped normally state; - if the UE supports voice services and the current cell does not support emergency call as indicated in System information specified in TS 36.331 [3], the UE should perform cell selection/ reselection to an acceptable cell of any supported RAT regardless of priorities provided in system information from current cell, if no suitable cell is found. NOTE: The UE is allowed to not perform reselection to an inter-frequency E-UTRAN cell in order to prevent camping on a cell on which it cannot initiate an IMS emergency call.

Proposal 7: Introduce an indication in SIB1-NB to indicate network supports IMS emergency call over GSO for NB-IoT.

In Rel-19, we support PWS for NB-IoT NTN. On basic of that, to enable the NB-IoT UE camps on cell to obtain PWS service without PLMN limitation, the applicable cell concept [12] is extended to NB-IoT. However, in Rel-19, the NB-IoT does not support emergency call service, that the definition of acceptable cell excludes emergency call for NB-IoT. Since the emergency call service is support for NB-IoT UE in Rel-20, the definition of acceptable cell for NB-IoT can be updated accordingly.

Acceptable Cell: A cell that satisfies certain conditions as specified in 4.3. Except for NB-IoT, Athe UE can always attempt emergency calls on an acceptable cell, but restriction as in 5.3.3 apply. acceptable cell: An "acceptable cell" is a cell on which the UE may camp to obtain limited service (originate emergency calls (except for NB-IoT) and receive ETWS and CMAS notifications), and it is not applicable to RRC_INACTIVE state. Such a cell shall fulfil the following requirements, which is the minimum set of requirements to initiate an emergency call and to receive ETWS and CMAS notification in a E-UTRAN network: - The cell is not barred, see clause 5.3.1; - The cell selection criteria are fulfilled, see clause 5.2.3.2;

Proposal 8: Update the definition of acceptable cell in TS 36.304 for NB-IoT to support emergency call service.

- ROHC enhancement

For UP solution, the ROHC is performed in the PDCP layer for header reduction (for CP solution, the header reduction mechanism is out RAN2’s scope). In LTE, the IMS voice call data is with RTP header, and the profile 0x0001 or 0x0101 can be used. At present, the NB-IoT only supports the following profiles with yellow highlight [5, 6]. If the UP solution is adopted, the NB-IoT supported ROHC profiles may need to be extended. However, whether the profile 0x0001 or 0x0101 can be directly used by NB-IoT is also pending SA2’s selection on IP solution or non-IP solution. RAN2 can postpone the discussion on ROHC enhancement when there is more progress on these aspects.

Table 5.5.1.1: Supported ROHC protocols and profiles

| Profile Identifier | Usage: | Reference |
| --- | --- | --- |
| 0x0000 | No compression | RFC 5795 |
| 0x0001 | RTP/UDP/IP | RFC 3095, RFC 4815 |
| 0x0002 | UDP/IP | RFC 3095, RFC 4815 |
| 0x0003 | ESP/IP | RFC 3095, RFC 4815 |
| 0x0004 | IP | RFC 3843, RFC 4815 |
| 0x0006 | TCP/IP | RFC 6846 |
| 0x0101 | RTP/UDP/IP | RFC 5225 |
| 0x0102 | UDP/IP | RFC 5225 |
| 0x0103 | ESP/IP | RFC 5225 |
| 0x0104 | IP | RFC 5225 |

Observation 9: Whether the ROHC enhancement is needed for NB-IoT depends on the down selection between CP and UP solution, and between IP and non-IP solution.

Proposal 9: Postpone the discussion on ROHC enhancement until the CP vs UP solution, IP vs non-IP solution have been finalized.

3	Conclusion

This contribution discusses how to support IMS voice call over GSO for NB-IoT, the observations and proposals are listed as follows:

CP vs UP solution

Observation 1: For legacy LTE UE, the IMS signalling and voice data are carried in 2 EPS bearers which are mapped to 2 separate DRBs over the Uu interface.

Observation 2: If CP solution is adopted for IMS voice call, the following issues/impacts need to be considered:

- Consider introducing a new SRB, so that the IMS signalling and the voice data are carried over the existing SRB1bis and the new introduced SRB separately;

- Consider supporting RLC UM mode for the new introduced SRB;

- The NAS layer should be able to differentiate the IMS signalling and voice data to map different packets to corresponding SRBs. How to achieve that needs to be discussed with CT1 involved;

- QoS control for the IMS voice call service over SRB needs to be specified.

Observation 3: NB-IoT currently supports at most 2 DRBs. For UP solution, the IMS signalling and the voice data can be carried in the existing 2 DRBs, allowing the UP solution to be supported naturally.

Observation 3a: If these 2 DRBs are used to separately carry the IMS signalling and voice data, there is no DRB left for other service during an on-going IMS voice call. Therefore, the necessity for an additional DRB can be discussed.

Proposal 1: UP solution is preferred for NB-IoT to support IMS voice call over GSO.

Proposal 1a: Further discusses whether to support one additional DRB to enable parallel service of IMS voice call and other service.

Discussion on the LS from SA2

Observation 4: The number of consecutive packets lost or erroneously decompressed that will trigger ROHC state fall back is not specified in RAN2’s spec. According to the IETF RFC files, the number of consecutive packets lost or erroneously decompressed will trigger the ROHC state fall back is defined as variables, e.g., k_1 and k_2, these variables are related to the residual error rate of the link and not clearly specified.

Observation 5: With the existing mechanisms, including dynamic resource allocation, SR and BSR, and the SPS to be supported, the NB-IoT can handle the voice packets of different sizes changing dynamically.

Observation 6: It is feasible to support more than 2 DRBs for NB-IoT.

Observation 7: It is feasible to support a new SRB for NB-IoT and it is also feasible to support RLC UM for SRBs.

Proposal 2: Reply an LS to SA2 providing RAN2’s answers to the questions in S2-2507636 and showing our preference for the UP solution:

- Answer to Q1: According to the IETF RFC files, the number of consecutive packets lost or erroneously decompressed will trigger the ROHC state fall back is defined as variables, e.g., k_1 and k_2, these variables are related to the residual error rate of the link and not clearly specified.

- Answer to Q3: Scheduling methods for support of IMS voice over NB-IoT NTN can handle the voice packets of different sizes changing dynamically

- Answer to Q6: Supporting more than 2DRBs for NB-IoT is feasible.

- Answer to Q9: Supporting a new SRB for NB-IoT is feasible.

- Answer to Q10: RLC UM mode for SRBs is feasible.

- From RAN2’s perspective, the IMS voice call can be naturally supported by the UP solution, whereas the CP solution requires significant spec impacts and cross group work. The UP solution is preferred for NB-IoT to support IMS voice call over GSO.

Discussion on the LS from SA4

Proposal 3: Reply an LS to SA4, gives RAN2’s answers on the questions in S4-251584 and show our preference on the UP solution:

- Minimum protocol header size of each AS layers is given in Table 1.

- 1 byte MAC header is not realistic.

- From RAN2’s perspective, the UP solution is preferred for NB-IoT to support IMS voice call over GSO.

Semi-persistent scheduling

Observation 8: For VoLTE AMR, the packet interval of talk spurt is 20ms and the packet interval of silent period is 160ms. In case of NB-IoT supports IMS voice call over GSO, SA4 also considers defining other packets intervals.

Proposal 4: The candidate values of SPS interval is closely related to the packets interval of IMS voice call service. In case of IMS voice call over GSO for NB-IoT, send an LS to SA4 asking:

- Whether talk spurt and silent period support different packet intervals?

- The candidate values of supported packets intervals.

Other issues

Proposal 5: Introduce an indication in SIB2-NB to indicate network supports IMS voice call over GSO for NB-IoT.

Proposal 6: Add an establishment cause for IMS voice call in the establishementCause-NB IE.

Proposal 7: Introduce an indication in SIB1-NB to indicate network supports IMS emergency call over GSO for NB-IoT.

Proposal 8: Update the definition of acceptable cell in TS 36.304 for NB-IoT to support emergency call service.

Observation 9: Whether the ROHC enhancement is needed for NB-IoT depends on the down selection between CP and UP solution, and between IP and non-IP solution.

Proposal 9: Postpone the discussion on ROHC enhancement until the CP vs UP solution, IP vs non-IP solution have been finalized.

4	Reference

- RP-251867	New WI: Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 4.

- R2-2506747	LS on issues related to support of IMS voice over NB-IoT NTN connected to EPC (S2-2507636; contact: Qualcomm) 	LS in	Rel-20	FS_ULBC	To: RAN2, SA4, CT1, SA3, SA1, RAN1.

- R2-2505068	LS on the RAN simulation assumptions for ULBC (S4-251584; contact: Qualcomm)	SA4	LS in	Rel-20	FS_ULBC	To:RAN1, RAN2, RAN4, SA2, CT1	Cc:SA1.

- TS 23.203	Policy and charging control architecture.

- 3GPP TS 36.331: "Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Resource Control (RRC) protocol specification".

- 3GPP TS 36.323: "Evolved Universal Terrestrial Radio Access (E-UTRA); Packet Data Convergence Protocol (PDCP) specification".

- IETF RFC 5795: "The RObust Header Compression (ROHC) Framework". 3095.

- 3GPP TS 36.300: " Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Overall description; Stage 2".

- 3GPP TS 36.321: "Evolved Universal Terrestrial Radio Access (E-UTRA); Medium Access Control (MAC) protocol specification".

- 3GPP TS 36.322: "Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Link Control (RLC) protocol specification".

- S2-2507021	Discussion paper on IMS voice packets transportation options.

- R2-2506556	Introduction of IoT NTN Enhancements.