3GPP TSG-RAN WG2 Meeting #131bis			R2-2506905

Prague, Czech Republic, Oct. 13th-17th

Agenda item:	10.3.1

Title:	Consideration on 6GR user plane

Source:	CMCC

Document for:	Decision

Introduction

RAN#109 approved the revised 6G SID [1], where one of the primary responsibilities of RAN2 is the study of 6G user plane architecture and protocol design.

In NR, the UP mainly comprises 4 radio protocols, i.e., SDAP, PDCP, RLC, and MAC. This contribution first reviews the architecture and functionalities of the 5G UP, and then discusses the requirements imposed on the 6G UP by new services and device types, as well as the limitations observed in the 5G UP. Finally, it identifies aspects that may need to be further considered within RAN2.

Discussion

As indicated in the SID, 6G will put stronger focus on user experience. Accordingly, UP studies should prioritize user satisfaction rather than only rate or latency, and the traditional UP architecture may be challenged by emerging services.

  - Motivation of 6GR UP

Main Functions of the 5G UP

The main functions of the 5G UP and the corresponding L2 sublayers that provide these functions are summarized in the following table:

Table 2.1-1: UP main functions and the responsible entity

| Entity Function | MAC | RLC | PDCP | SDAP |
| --- | --- | --- | --- | --- |
| Resource matching | Mapping between logical channels and transport channels Multiplexing | Segmentation and reassembly of RLC SDUs Re-segmentation of RLC SDU segments |  | Mapping between a QoS flow and a DRB Reflective QoS flow to DRB mapping. |
| Reliable transmission | HARQ | ARQ for AM | Duplication |  |
| Discarding |  | RLC SDU discard | Timer/PSI based SDU discard PDU Set discard Duplicate discarding |  |
| Prioritization and resource allocation | Scheduling information reporting Priority handling Resource selection |  |  |  |
| Delivery |  |  | Reordering and in-order delivery Out-of-order delivery Routing |  |
| Compression and decompression |  |  | Header/ UL data compression and decompression |  |
| Duplication detection |  | Duplicate detection for AM |  |  |
| Security |  |  | Ciphering and deciphering Integrity protection and verification |  |

It can be observed that some 5G functions are provided across multiple L2 sublayers. In some cases, this is inherent to the layered architecture itself, e.g., resource mapping, while in other cases, certain functions may be somewhat redundant, for example, retransmission and discard.

Proposal 1: The 5G UP functionalities are proposed to form the baseline for the 6G UP.

Requirements for 6G and Key Issues from 5G

As illustrated in [2], current 5G services, while enabling high-speed connectivity and low-latency interactions, struggle to fully support emerging scenarios such as immersive extended reality (XR) experiences, intelligent sensing, and satellite-terrestrial integration. Therefore, 6G is expected to become a foundational infrastructure for the verticals, smart societies, and human-centric intelligent ecosystems. In our view, Integrated Sensing and Communication (ISAC), AI/ML-based services and immersive communication will be the key services in 6GR to unlock the new capabilities to support the next era of digital transformation.

Requirement for 6GR ISAC

ISAC has been identified as one of the six usage scenarios for IMT-2030, providing sensing services for low-altitude economy, traffic surveillance, intrusion detection and micro-deformation detection while also assisting communication services. TR 22.837[3] identify a very wide range of ISAC use cases, and requirements for such are defined in TS 22.137[4].

Requirement for 6GR immersive communication

Immersive Communication has been described by ITU-R as a usage scenario that extends the enhanced Mobile Broadband (eMBB) of IMT-2020 and covers use cases that provide a rich and interactive video (immersive) experience to users, including interactions with machine interfaces [5]. Immersive communication use cases will continue to drive requirements on mobile networks. Mixed Reality Gaming is taken as an example, and its performance requirements are shown in Table 3.

Table 3: Performance of Mixed Reality Gaming

| Profile | Characteristic parameter |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Bit rate down link | Bit rate up link | Application latency | Communication service availability | Overall user density | Activity factor | UE speed |
| Mixed Reality gaming | [50] Mbit/s (note 1) | [50-100] Mbit/s (note 3) | [20] ms (note 2) | Dense Urban [99]% Urban [99]% Rural [98]% | Dense Urban [25,000] /km2 Urban [1000-10,000]/km2 Rural [100]/km2 | 2% (note 4) | 5 km/h |
| NOTE 1: Remote or split rendering for gaming applications, such as rendering of 360-degree video tiles in the cloud and transmitting the images to device, NOTE 2: The local encoding and processing on the glasses is assumed NOTE 3: envisioned 1080P resolution 2 grayscale cameras 1920 x 1080 x 8 bit, a depth sensor 1920 x 1080 x 8 bit with 60 FPS means 1920 * 1080 * 24 * 60 bps = 3Gbps uncompressed. Depending on compression ratios that would lead to 50 Mbps to 100 Mbps NOTE 4: Activity factor 2% is based on 20% uptake of XR service and 10% activity factor for the XR application. |  |  |  |  |  |  |  |

Requirement for 6GR AI/ML

In 5G-A, 3GPP RAN has investigated various AI for RAN use cases that are aimed to optimize network performance through AI/ML function, for example, AI/ML based CSI feedback enhancement, beam management and positioning enhancement and AI/ML for mobility. For 6G, the potential new use cases of AI for 6G RAN could be identified and investigated, e.g. Joint source/channel coding (JSCC), DMRS design or AI receiver, AI/ML based mobility.

Furthermore, RAN for AI is expected to be investigate and support in 6GR, e.g. RAN provides computing resources, RAN related information or AI data for AI services/applications. AI agent, XR rendering and token communication could be considered as the potential RAN for AI use cases, and the requirements of these use cases are summarized in Table 2.

Table 2: Requirements for RAN for AI use cases

| Use Cases | Uplink KPI |  | Downlink KPI |  |  |
| --- | --- | --- | --- | --- | --- |
|  | E2E Latency | Date Rate | E2E Latency | Date Rate | Load |
| XR rendering | 100 ms | 1.5 Mbit/s | 100 ms | 150 Mbit/s | 1.5 MByte/‌frame |
| Embodied AI/ AI agent | 10 ms | 4.7 Mbit/s | 10 ms | 320 Mbit/s | 40 kByte |

As can be observed, there exists functionality overlap among L2 sublayers, e.g., MAC, RLC and PDCP have the capability to re-transmit data and RLC and PDCP have very similar windowing mechanism, which suggests potential redundancy that could be consolidated within sub-layer integration.

With the emergence of new service types, e.g., multi-modal, immersive service, and Token Communication, UP requirements differ from those of conventional services. In particular, the impact of application-layer techniques, e.g., AL-FEC, on the UP should also be taken into account.

With the rise of AI services driven by large AI/ML models such as large language models and visual language models, these mobile AI applications not only create new traffic for mobile networks, but also bring unique transmission characteristics. We noticed that tokens, as the basic unit of AI information processing and generation, can be transformed from various modalities of data such as text, images, videos, etc.:

Figure 2.1-1: Service flow of TokCom

In the current network, tokens are not distinguished from other traffic, but we have noticed that tokens generated by AI/ML models/used for AI/ML model processing have special characteristics, including:

Table 2.1-2: Characteristic for TokCom

| Error tolerance | Tokens are resilient to a range of transmission errors; i.e. certain percentage of successfully received tokens may be enough for the AI/ML processing to reach the required accuracy level. The post-processing capability of error tokens can also compensate for errors |
| --- | --- |
| Token importance differentiation | There is differentiated importance for different tokens, i.e. the tokens for different modality data or different tokens for the same modality data may contribute differently to the AI/ML model processing and QoE of users. |
| Token traffic pattern | Tokens may arrive in a streaming or burst manner, where each token can be represented as a bit sequence with length determined by the size of the vocabulary or codebook or knowledge base. In a specific AI service session with the same vocabulary, the size of the data unit (Token ID) formed after tokenization is fixed, which means that the traffic pattern of token communication depends primarily on the size of the original information and the design of the vocabulary. |
| Token context relevance | Streaming transmission adopts a method of token generation and real-time transmission. However, in the event of packet loss, relying on a retransmission mechanism can result in subsequent tokens that have already arrived being unable to be processed and having to wait for retransmission, which may cause a disruption in user experience. |

Observation 1: The transmission of tokens has potential characteristics such as fault tolerance, differential importance, special business models, and context dependency, which may involve technical optimization of RAN2.

The following sections outline the issues exposed in the 5G UP after commercial deployment, as well as the requirements imposed by new services and techniques, highlighting key directions to be considered for 6GR UP.

Proposal 2:  RAN2 fundamental functionalities for data handling should support of new services, e.g., ISAC, AI/ML-based services and immersive communication, taking User experience (QoS/QoE) improvement into consideration.

  - Key aspects on 6GR UP

From CMCC’s perspective, challenges of new services and techniques and limitation faced in 5G should be addressed in 6G, as presented in the following subsections.

    - L2 sublayer consolidation for reorganization and reduction of redundant functionalities across protocol layer

As noted above, functional redundant exists across L2, e.g., both RLC and PDCP implement windowing, while retransmission occurs in both MAC HARQ and RLC ARQ. Such functions could be considered for consolidation. In addition, the presence of multiple sublayers introduces significant inter-layer overhead, such as repeated data copying and nested headers. Therefore, consolidation and reorganization of UP functions should be considered.

Segmentation/Reassembly of SDU migrated to MAC layer

The logical channel multiplexing functionality was inherited from UMTS and LTE. In UMTS, without BSR and each DCH carries variable rate of voice data, multiplexing different DCHs data into one MAC PDU based on the Transport Format Combination (TFC) Selection mechanism can achieve high efficiency. In LTE, the LCP procedure also introduced to guarantee minimum bit rate of each RB (i.e. PBR) to avoid starvation problem. In the beginning of NR, to support the ultra low latency service requirement of URLLC services the concatenation function at RLC layer has been consolidated with multiplexing at MAC layer.

In NR, the consolidation of the concatenation function at RLC layer and multiplexing function at MAC layer is regarded as be enough in that URLLC services are always consisted of small size data.

However, for XR service are always consisted of big size data, it is necessary to remove the segmentation function from the RLC layer to MAC layer as well to further reducing the processing latency due to inter-layer communication.

Observation 2a-1: Migrating the segmentation function from the RLC layer to MAC layer would achieve the following gains can be obtained:

- Reduced Processing Latency

  - Eliminates the processing latency due to inter-layer communication with RLC

- Reduced header overhead:

  - Fewer segmentation reduce the total size of header field

- Simplified L2 PDU construction

  - For each UL grant, MAC doesn’t need to interact with RLC layer, which makes PDU construction simple enough.

- Facilitated the consolidation of ARQ and HARQ

Furthermore, considering token communication and AL-FEC, the application layer, or AI-native devices, has the capability to recover data that is close to or identical to the original from incomplete or partially received data. Hence, data delivery on RLC SDU instead of MAC PDU is appreciated. With the increase in video resolution and data volume, jumbo packets are becoming a significant portion of traffic. Hence, RLC segmentation and reassembly may no longer be necessary, and such functionality could instead be handled by the MAC entity.

In addition, submitting data to the MAC entity strictly at the granularity of a MAC PDU may conflict with these capabilities. Specifically, when only a few bits in a MAC PDU are erroneous, retransmitting rather than delivery consumes additional energy and time unnecessarily, reducing overall transmission efficiency.

Figure 2.2-3: Example MAC PDU segmentation with AL-FEC

However, the current MAC specifications do not support segmented or partial delivery of MAC PDUs. As a result, the MAC entity may retransmit entire PDUs even when only a small portion contains errors, which is inefficient in the context of Token communication or AL-FEC-enabled services. Therefore, it is necessary to investigate enhancements to MAC-layer data delivery that allow finer-granularity transmission and retransmission, better aligning with Token communication principles and supporting efficient operation of AL-FEC services.

Observation 2a-2: The data delivery is on MAC PDU level, which does not align with token communication, AL-FEC, indicating the need for finer-granularity and more flexible MAC layer data handling.

Combined ARQ and HARQ procedures

For the protocol stack of 5G user plane, there are four sublayers in L2 and some of the functions in different sublayers are duplicate, e.g., SN allocation and maintenance both in PDCP and RLC sublayers, data retransmission (ARQ and HARQ) both in RLC and MAC sublayers. From our point of view, a relatively large number of sublayers and duplicate functions may result in reduced L2 processing performance, such as increased header overhead, higher processing latency, and greater complexity.

With the 4ms UP latency for immersive service [2], L2 retransmission should be taken into consideration. L2 retransmission functionalities are distributed across MAC, RLC, and PDCP layers to ensure reliable data delivery. However, the presence of multiple retransmission procedures can introduce additional L2 latency. The relative impact of each layer depends on channel conditions: when the channel is favourable, MAC-layer HARQ dominates the retransmission delay, whereas under higher error rates, RLC ARQ becomes the primary contributor to delay. This layered retransmission structure, while improving reliability, may therefore lead to suboptimal latency performance for delay-sensitive services. To address this, it is recommended to investigate more efficient and coordinated retransmission schemes that can reduce latency while maintaining reliability, potentially through cross-layer optimization or adaptive retransmission strategies.

Figure 2.2-1: Delay comparison of UL retransmission

Regarding to the above issues, a leaner or simpler user plane protocol may be considered for the design of 6G protocol stack to improve L2 processing efficiency and to meet the low latency service. Specifically, the design principle could include reducing the number of sublayers and avoiding redundant functions.

A unified SN maintenance mechanism in L2 by combination of PDCP and RLC window

In NR, both PDCP and RLC maintain respective SNs, which main purposes of each SN are as follows:

- PDCP SN

  - Generating COUNT value

  - PDCP PDU reordering (for split bearers, duplication etc.)

  - Reordering for in-order delivery

  - Data forwarding during handover or RRC re-establishment procedure

- RLC SN

  - Identify missing PDU during data transmission/reception

  - Segmentation and reassembly of RLC SDUs (only for UM and AM data transfer);

  - Re-segmentation of RLC SDU segments

We think having two different SNs is not essential for L2 operation. By consolidating different functions, we can maintain only one SN. If we use only one L2 SN through combination of PDCP and RLC window, we can save several bytes for each packet. For example, the legacy SN allocation and maintenance mechanism of the PDCP layer can be reused, where the SN length for SDUs is set to 12 bits or 18 bits.

- Since the new sub-layer no longer includes the segmentation and reassembly of SDUs, its reception operations only require reordering process; in other words, the reception operation of the new sublayer is equivalent to that of the existing 5G PDCP layer. Assuming that the existing AM and UM modes remain necessary in 6G, the transmission operations of the new sublayer will be identical to the legacy transmission process in the RLC layer.Based on the above analysis, we propose that a unified design of PDCP and RLC is one of the options for lean user plane protocol to be considered in 6G. For example, RLC could be merged into PDCP, with some of its functions migrated to the MAC layer; we refer to this integrated structure as “new sublayer”. Specifically, the following key aspects could be further studied and verified:

- Segmentation/Reassembly of SDU migrated to MAC layer: This approach enables better adaptation to transmission opportunities, as it eliminates the need to wait for notifications from the lower layer before modifying the header.

- A unified SN maintenance mechanism in L2: For example, the legacy SN allocation and maintenance mechanism of the PDCP layer can be reused, where the SN length for SDUs is set to 12 bits or 18 bits.

- Recombined the transmission and reception windows of PDCP and RLC: Since the new sub-layer no longer includes the segmentation and reassembly of SDUs, its reception operations only require reordering process; in other words, the reception operation of the new sublayer is equivalent to that of the existing 5G PDCP layer. Assuming that the existing AM and UM modes remain necessary in 6G, the transmission operations of the new sublayer will be identical to the legacy transmission process in the RLC layer.

- Combined ARQ and HARQ procedures: Enhanced HARQ with finer-granularity feedback could enable the sub-TB level retransmission, which can be performed using the fewer radio resources. This accelerated and flexible retransmission is beneficial for low-latency services.

Figure 2.2-2: Layer 2 Structure

Proposal 3: The following key aspects could be further studied and verified:

- Segmentation/Reassembly of SDU migrated to MAC layer [RAN2]

- A unified SN maintenance mechanism in L2 [RAN2]

- Recombined the transmission and reception windows of PDCP and RLC  [RAN2]

- Combined ARQ and HARQ procedures [RAN2, RAN1]

    - Scalable-Granularity LCP for Comprehensive Factors in Data Handling

Current MAC layer procedures face challenges in efficiently supporting heterogeneous traffic. In 5G, legacy logical channel scheduling mechanism is based on fixed priority and unidirectional data transmission/discarding at the output, such as selecting data for transmission/discard based on priority or additional priority which derived from same or similar QFI from QoS flows mapped into this logical channel. However, in XR services, a single logical channel may carry both delay-critical and non-delay-critical data, each with different priorities. While this issue has been recognized in R19 discussions on DSR and LCP, the current MAC LCP specification only supports prioritization across logical channels, not within them.

As a result, sub-LCH prioritization is not possible, and packets with strict latency requirements may be delayed by less time-sensitive packets within the same logical channel. This coarse granularity limits the network’s ability to handle XR traffic efficiently, as multiple QoS flows multiplexed into a single LCH cannot have their individual QoS requirements preserved at the MAC layer. Consequently, delay-critical traffic may experience latency violations, and overall resource utilization becomes less efficient.

Figure 2.2-2: Example of 5G LCP limitations in LCH mixed with DC/non-DC data and multi-modality

Furthermore, in multi-modal services, data from different modalities are often interdependent but may be distributed across multiple LCHs. Their relative priority should therefore be aligned to preserve service-level consistency. The current MAC LCP mechanism does not support this type of cross-LCH coordination. It follows that the optimal LCP granularity may differ depending on the service type and enabling a scalable, service-adaptive LCP mechanism which cansupport of evaluation of multiple factors for data transmission/discarding, e.g., based on PSI, remaining time, integration scheduling requirements of XR PDU set or data burst of token communication, inter-dependencies among PDU sets/data bursts, and QoS coordination across multiple flows.

Observation 2a: The LCP mechanism cannot differentiate priorities within a single LCH nor coordinate priorities across multiple LCHs, limiting its ability to efficiently support heterogeneous and interdependent traffic, i.e., XR, multi-modal and token communication services.

    - MAC status report for new services

Existing MAC status reporting, e.g., BSR and DSR, still works at the LCH granularity. In the R19 multi-modality study, RAN2 focused on multi-modal service across several LCHs but RAN could only obtain static information from the CN, i.e., MMSID as the correlation of QoS flows belonging to the same multi-modal service, and current MAC status reporting mechanism could not support any dynamic information reporting related to multi-modal services, e.g., the buffer size and remaining time related to a multi-modal service. such information is scattered across different LCHs. This implies that 6G UP requires a status reporting mechanism across LCHs.

Figure 2.2-4: Example of 5G BSR/DSR limitations in multi-modal service

Furthermore, with the emergence of token communication and immersive applications, MAC status reporting mechanisms also need to accommodate the traffic characteristics of token communication and immersive services. Therefore, it is necessary to investigate new BSR/DSR tables that follow new distributions and reporting dimensions that differ from LCHs.

Observation 2b: The MAC status reporting, e.g., BSR and DSR, is limited to LCH granularity and cannot support dynamic, cross-LCH information needed for multi-modal, tokenized, or immersive services.

    - Assistance control information on MAC

The issue of SR false alarm caused by energy detection has long been a challenge for gNB vendors. Due to inherent limitations in receiving SR signals, false alarm is difficult to eliminate in principle. If a UE could directly transmit BSR, this issue could be avoided; however, the PUCCH resources are insufficient to carry BSR. Another problem caused by limited control channel resources is retransmission. Retransmitting an entire TB due to a small number of errors is inefficient. To address this, RAN1 introduced CBG-level retransmission. However, the number of CBGs is constrained by the DCI payload size. Therefore, the size of CBG may still be too large relative to the actual amount of data needing retransmission. While increasing DCI bits to enable finer-grained retransmission (e.g., at the CB level) is unlikely.

Therefore, it is proposed to study the possibility of transmitting part of the control signaling within the MAC PDU, as a supplement (not a replacement) to UCI/DCI.

Observation 2c: Limited PUCCH resources and coarse-grained HARQ retransmission led to inefficiency and SR false alarms, motivating investigation of partial control signalling within the MAC PDU as a supplement to UCI/DCI.

Based on the above analysis and observation, it’s proposed that:

Proposal 4: With the issues observed in the 5G UP as well as new services, e.g., immersive, multi-modal, and token-based services, the following areas of study are recommended for RAN2:

- Scalable-granularity LCP which can support of evaluation of multiple factors for data transmission/discarding, e.g., based on PSI, remaining time, integration scheduling requirements of XR PDU set or data burst of token communication, inter-dependencies among PDU sets/data burst, and QoS coordination across multiple flows [RAN2]

- Finer-granularity of data submitting and delivery [RAN2, RAN1]

- Assistance control information on MAC [RAN2]

    - Concatenation function in PDCP

In EUTRAN, the RLC sublayer supports concatenation for UMD PDUs and AMD PDUs. For an RLC PDU that concatenates more than one SDUs, the RLC header contains an extension part (E and LI), where each data field (except the last one) corresponds to one E and LI. In NR, to reduce processing latency and simplify the RLC PDU format, the concatenation function at RLC sublayer has been removed.

In 6G, the network should support a wide range of UE types (e.g. eMBB UE, LPWA terminals, etc.) and various kinds of traffic (e.g. background traffic, voice, gaming, XR, LPWA traffic, etc.). For the traffic with large data size or high data rates (e.g. hundreds of Mbps), the protocol overhead of the AS layer is negligible compared to the amount of transmitted data. However, for the traffic with low data rates, the protocol overhead of the AS layer becomes significant. For ease of understanding, Table 2 compares the AS protocol overhead, which is defined as the ratio of the size of the AS protocol header to the total size the data, between EUTRAN and NR under different levels of concatenation. The results in the table are obtained under the following assumptions:

- EUTRAN: PDCP SDU size of 40 bytes; PDCP header overhead of 2 bytes (12-bit PDCP SN); UMD RLC PDU with 5-bit SN; MAC header overhead of 2 bytes (R/F2/E/LCID/F/L subheader with 7-bit L field).

- NR: PDCP SDU size of 40 bytes; PDCP header overhead of 2 bytes (12 bits PDCP SN); UMD RLC PDU containing a complete RLC SDU; MAC header overhead of 2 bytes (R/F/LCID/L MAC subheader with 8-bit L field).

*Note: To compare EUTRAN and NR more fairly, the overhead of the NR SDAP header is not considered, and NR integrity protection is assumed to be not configured.

Table 2.2-1. AS protocol overhead comparison (PDCP SDU size of 40 bytes)

|  | 9 PDCP PDUs concatenated | 5 PDCP PDUs concatenated | 3 PDCP PDUs concatenated | no PDCP PDU concatenated |
| --- | --- | --- | --- | --- |
| EUTRAN | 9.17% | 9.5% | 10% | 12.5% |
| NR | 12.5% | 12.5% | 12.5% | 12.5% |

From Table 2, it can be seen that for services with low data rates/small data size, EUTRAN has lower AS protocol overhead than NR because it supports concatenation in RLC sublayer. Moreover, the more PDCP PDUs that can be concatenated, the smaller the AS protocol overhead. Besides, as shown in Table 3, if the PDCP SDU size is reduced to 30 bytes, the more significant the reduction in AS protocol overhead achieved by concatenation.

Table 2.2-2. AS protocol overhead comparison (PDCP SDU size of 30 bytes)

|  | 9 PDCP PDUs concatenated | 5 PDCP PDUs concatenated | 3 PDCP PDUs concatenated | no PDCP PDU concatenated |
| --- | --- | --- | --- | --- |
| EUTRAN | 12.22% | 12.67% | 13.33% | 16.67% |
| NR | 16.67% | 16.67% | 16.67% | 16.67% |

Observation 3a: In EUTRAN, the RLC sublayer supports concatenation for UMD PDUs and AMD PDUs. In NR, to reduce processing latency and simplify the RLC PDU format, the concatenation

Observation 3b: For services with low data rates/small data size, EUTRAN has lower AS protocol overhead than NR because it supports concatenation in RLC sublayer. The more PDCP PDUs can be concatenated or the smaller the PDCP SDU size is, the more significant improvement of AS protocol overhead achieved by concatenation.

Based on above Observations, we think that it is beneficial to support concatenation in 6G to reduce the AS protocol overhead for the traffic with low data rates/small data size. Note that further considering the integrity protection, the benefit may become more obvious, since one MAC-I field can be added to a PDCP PDU that contains multiple PDCP SDUs instead of each SDU. In terms of the AS sublayer to support concatenation, since we support to merge the legacy RLC functions to PDCP sublayer and MAC sublayer as introduced in Section 2.2.1, PDCP sublayer seems to be suitable in 6G. An illustration of PDCP concatenation is depicted in the following figure.

Fig. 2.2-5. PDCP concatenation

In case of PDCP concatenation, a single PDCP PDU may contain multiple upper-layer packets (e.g. IP packets, etc.). Therefore, the transmitting PDCP entity perform concatenation and the receiving PDCP entity splits the received PDCP PDU to recover each individual upper-layer packets and forward them to the upper layer.

The specific standardization details (e.g., whether there is an impact on the PDCP header, granularity to configure PDCP concatenation, the PDCP functional view, etc.) can be discussed in details during the WI phase.

Observation 3c: Considering the integrity protection, the benefit of PDCP concatenation may be more obvious, since one MAC-I field can be added to a PDCP PDU that contains multiple SDUs.

Proposal 5: In 6G, concatenation function at PDCP sublayer is supported. The specific standardization details can be discussed during WI phase.

    - DRX

For DRX, the DRX operation was used in 5G for MAC to discontinuously monitor the PDCCH and reduce energy consumption when no data was expected to be received. Besides, the LP-WUS function also was introduced in 5G for the energy saving purpose. It seems that both functions may achieve the similar effect and LP-WUS function may be more flexible to wake up UE when data burst arrivals without regular pattern.  However, the LP-WUS function may not be optionally supported in Day-1 and the DRX operation still has an advantage when handling data with regular patterns. In 6G, the DRX operation could be considered for the case to handle the data with regular patterns.

Furthermore, the Immersive communication, which provide the interactive video experience to users, has been described as promising use cases in [3]. So, the enhancement of DRX, e.g. the configuration of multiple DRX cycles, may also be discussed to satisfy the multi-media services with deterministic experience in 6G.

Proposal 6: In 6G, RAN2 can study whether the DRX operation is still regarded as baseline or could be replaced by LP-WUS function.

    - TSN and HRLLC

In 6G [3], the use cases on the real time digital twins and Industrial communication have been identified. In the manufacturing or construction, the multiple autonomous mobile robots (AMRs) work together using the Industrial communication and a digital twin, which can virtually represent the real-world items in industry or construction sectors, collects data from multiple sources in real time. Based on these use case, the TSG RAN in [4] has held initial discussion on requirement values of TRP (technical performance requirement) items and proposes the following table for consideration.

Table 2.2-3: Technique Performance Requirements (TRPs) in [4]

| Proposed TPR items | Proposed requirement values |
| --- | --- |
| - User Plane Latency | IC: 4ms HRLLC: 1ms |
| - Control Plane Latency | 20ms |
| - Reliability | 1-10-5 with same IMT-2020 assumptions |

It should be noted in the above table, the UP latency for HRLLC is proposed as 1ms and the reliability is 99.999%, which is still strict high reliability and low latency requirement for 6G RAN.

Since the 5G RAN already proposed the same requirement for URLLC and TSN, the feature CG or SPS configuration, which introduced to match the TSN traffic pattern, and PDCP duplication in 5G to reduce the latency and enhance the reliability could be discussed in 6G day1 as baseline.

Besides, the TSN networking has been deployed and integrated with the existing 5G network. The feature accurate reference timing delivery in 5G RAN is carried out to achieve the TSN time synchronization. To support the TSN in 6G ensuring ‌smooth migration and flexible deployment, the feature accurate reference timing delivery could be discussed in 6G day1 as baseline.

On the other hand, UE aggregation technical is introduced in R18 multi-path to support applications requiring high UL bitrates on 5G terminals, in cases when normal UEs are too limited by UL UE transmission power to achieve required bitrate, especially at the edge of a cell. UE aggregation can also improve the stability and reduce delay of uplink services as well, that is, if the channel condition of a terminal is deteriorating, another terminal can be used to make up for the traffic performance unsteadiness caused by channel condition variation.

The basic protocol stack of UE aggregation is the PDCP layer of the remote UE connects to the RLC of remote UE and RLC of one or more relay UE(s) with N3C link. Without the SRAP sublayer (introduced to SL relay) over N3C, an UL PDCP PDU in the L2 MP Remote UE is delivered to RLC entity in the L2 MP Relay UE via N3C link based on UE implementation.

Proposal 7: The set of features for deterministic service, i.e., accurate reference timing provisioning, CG or SPS configuration and PDCP duplication for TSN in 5G could be introduced as baseline in 6G, particularly, duplication and split function in PDCP entity towards one or more RLC channel(s) of another UE(s) connected to the anchor UE should also be supported.

Conclusion

Observation 1: The transmission of tokens has potential characteristics such as fault tolerance, differential importance, special business models, and context dependency, which may involve technical optimization of RAN2.

Observation 2a-1: Migrating the segmentation function from the RLC layer to MAC layer would achieve the following gains can be obtained:

- Reduced Processing Latency

  - Eliminates the processing latency due to inter-layer communication with RLC

- Reduced header overhead:

  - Fewer segmentations reduce the total size of header field

- Simplified L2 PDU construction

  - For each UL grant, MAC would not need to interact with RLC layer, simplifying PDU construction

- Facilitated the consolidation of ARQ and HARQ

Observation 2a-2: The data delivery is on MAC PDU level, which does not align with token communication, AL-FEC, indicating the need for finer-granularity and more flexible MAC layer data handling.

Observation 2b: The LCP mechanism cannot differentiate priorities within a single LCH nor coordinate priorities across multiple LCHs, limiting its ability to efficiently support heterogeneous and interdependent traffic, i.e., XR, multi-modal and token communication services.

Observation 2c: The MAC status reporting, e.g., BSR and DSR, is limited to LCH granularity and cannot support dynamic, cross-LCH information needed for multi-modal, tokenized, or immersive services.

Observation 3a: In EUTRAN, the RLC sublayer supports concatenation for UMD PDUs and AMD PDUs. In NR, to reduce processing latency and simplify the RLC PDU format, the concatenation

Observation 3b: For services with low data rates/small data size, EUTRAN has lower AS protocol overhead than NR because it supports concatenation in RLC sublayer. The more PDCP PDUs can be concatenated or the smaller the PDCP SDU size is, the more significant improvement of AS protocol overhead achieved by concatenation.

Observation 3c: Considering the integrity protection, the benefit of PDCP concatenation may be more obvious, since one MAC-I field can be added to a PDCP PDU that contains multiple SDUs.

Observation 2d: Limited PUCCH resources and coarse-grained HARQ retransmission led to inefficiency and SR false alarms, motivating investigation of partial control signalling within the MAC PDU as a supplement to UCI/DCI.

Proposal 1: The 5G UP functionalities are proposed to form the baseline for the 6G UP.

Proposal 2:  RAN2 fundamental functionalities for data handling should support of new services, e.g., ISAC, AI/ML-based services and immersive communication, taking User experience (QoS/QoE) improvement into consideration.

Proposal 3: The following key aspects could be further studied and verified:

- Segmentation/Reassembly of SDU migrated to MAC layer

- A unified SN maintenance mechanism in L2

- Recombined the transmission and reception windows

- Combined ARQ and HARQ procedures

Proposal 4: With the issues observed in the 5G UP as well as new services, e.g., immersive, multi-modal, and token-based services, the following areas of study are recommended for RAN2:

- Scalable-granularity LCP which can support of evaluation of multiple factors for data transmission/discarding, e.g., based on PSI, remaining time, integration scheduling requirements of XR PDU set or data burst of token communication, inter-dependencies among PDU sets/data burst, and QoS coordination across multiple flows

- Finer-granularity of data submitting and delivery

- Assistance control information on MAC

Proposal 5: In 6G, concatenation function at PDCP sublayer is supported. The specific standardization details can be discussed during WI phase.

Proposal 6: In 6G, RAN2 can study whether the DRX operation is still regarded as baseline or could be replaced by LP-WUS function.

Proposal 7: The set of features for deterministic service, i.e., accurate reference timing provisioning, CG or SPS configuration and PDCP duplication for TSN in 5G could be introduced as baseline in 6G, particularly, duplication and split function in PDCP entity towards one or more RLC channel(s) of another UE(s) connected to the anchor UE should also be supported.

References

- RP-252476, Revised SID: Study on 6G Radio, NTT DOCOMO, CMCC, AT&T, Vodafone

- R2-2506910, General considerations on 6GR, CMCC

- 3GPP TR 22.837 Study on Integrated Sensing and Communication (Release 19)

- 3GPP TS 22.137 Integrated Sensing and Communication (Release 19)

- 3GPP TR 22.870 V0.4.0 Study on 6G Use Cases and Service Requirements (Release 20)

- RP-252120, TR 38.914, v0.1.1, Study on 6G Scenarios and requirements, CMCC

- TR 22.970, v0.3.1, Study on 6G Use Cases and Service Requirements; Stage 1(Release 20), 3GPP

- RP-251825, Reply LS to ITU-R WP5D = RP-243202 on Minimum requirements related to technical performance for IMT-2030 radio interface(s), 3GPP RAN