3GPP TSG-RAN WG2 Meeting #131bis	R2-2506923

Prague, Czech, 13th – 17th, October, 2025

Agenda item:		8.4.3

Source:		Lenovo

Title:	Remaining MAC open issues for LP-WUS

Document for:		Discussion and Decision

- Introduction

During the email discussion to collect the open issues on Rel-19 LP-WUS 38.321 CR[1], the open issues are identified, and corresponding proposals are summarized by the Rapporteur:

Proposal 1: Revisit the UE operation in LP-WUS collision for Option 1-2, which should be based on critical issues in the current RAN2 agreements. (based on contribution) Proposal 2: Confirm UE does not monitor LP-WUS when short DRX cycle is used in DRX group when secondary DRX group is configured. Proposal 3: Confirm UE enables/disables the LP-WUS monitoring only based on RRC configuration. Proposal 4: Confirm in Option 1-2 when UE is in CDRX active time but LP-WUS monitoring timer is not running, the legacy UE operation on CSI/SRS transmission in active time is applied. Proposal 5: Confirm in Option 1-2 when UE is in CDRX active time and LP-WUS monitoring timer is running, the legacy UE operation on CSI/SRS transmission in active time is applied. Proposal 6: Further discuss the necessity to explicitly describe “option1-1/1-2” based on 38.300 description. (based on contribution)

This contribution aims to present our views on the remaining MAC open issues for Rel-19 LP-WUS.

- Discussion

MAC Open Issue 1: LP-WUS Collision for option 1-2

In RAN2#130 meeting [2], it’s agreed that for Option 1-2, UE does not start the lpwus-PDCCH-MonitoringTimer in collision cases, i.e., when the UE is not able to monitor the LP-WUS occasion(s). In addition, it’s also agreed that this agreement can be further discussed if critical issue identified with this mechanism. From our side, the reason to support not to start lpwus-PDCCH-MonitoringTimer in collision cases is that we assume the LP-WUS periodicity of option 1-2 is much smaller than option 1-1, so that frequent starting the timer degrades the power saving gain. However, regarding to the LP-WUS periodicity of option 1-2, RAN1 only agrees that at least LP-WUS periodicity is no larger than long C-DRX cycle in RAN1#118bis meeting, as follows:

Agreement - Regarding the potential restriction for LP-WUS configuration in relation with C-DRX configuration for option 1-2 of LP-WUS CONNECTED mode operation, from RAN1 perspective, - Opt2: At least LP-WUS periodicity is no larger than long C-DRX cycle, FFS other restrictions - Introducing monitoring window is not precluded

That is to say, network has the flexibility to configure the length of LP-WUS periodicity for option 1-2. If the LP-WUS cycles are sparse, e.g., not much shorter than long DRX cycle, then the case may be similar to option 1-1, it’s possible that network cannot reach the UE for a long time due to the collision cases, which impacting the scheduling latency dramatically. Considering this issue, we share the concern on the current agreement to not start the lpwus-PDCCH-MonitoringTimer in collision cases, we suggest RAN2 can have further discussion on the issue, and a compromised method is that network configures whether to start the lpwus-PDCCH-MonitoringTimer based on the LP-WUS periodicity for option 1-2. In this way, both the power saving gain and scheduling latency can be guaranteed.

Proposal 1: (MAC#1) It is suggested to further discuss the UE behavior on the LP-WUS collision case for option 1-2: it is left to network configuration on whether to start the lpwus-PDDCH-MonitoringTimer in collision case for option 1-2.

MAC Open Issue 4/5: CSI/SRS transmission in C-DRX active time for option 1-2

During Rel-19 LP-WUS discussion, following agreements related to option 1-2 have been made as following:

RAN2#127bis meeting: - For Option 1-2, LP-WUS monitoring is performed at least outside legacy C-DRX Active Time. FFS if the legacy drx-onDurationTimer is started or not if the new timer is configured in Option 1-2. - In option 1-2, a new timer triggered by LPWUS is introduced. When this new timer is running, UE is in C-DRX active time. When UE is not in C-DRX active time, UE goes back to LPWUS monitoring. - When UE is in C-DRX active time, UE PDCCH monitoring behaviours related to other legacy DRX timers (except for drx-onDurationTimer) are not affected. RAN2#128 meeting: - The drx-onDurationTimer is not started with Option 1-2 LP-WUS. - For Option 1-2, network can configure whether UE reports periodic CSI/L1-RSRP during the time given by the configured drx-onDurationTimer, for the case when UE is outside C-DRX active time.

Based on above agreements, for option1-2, in the case of lpwus-PDCCH-MonitoringTimer is running, UE enters C-DRX active time, then UE PDCCH monitoring behaviors related to other legacy DRX timers (except for drx-onDurationTimer) are not affected. However, we understood the agreement cannot imply the UE behaviors on SRS transmission and CSI reporting when the lpwus-PDCCH-MonitoringTimer is running, since it is not related to other legacy DRX timers. Basically, we share the view that in this case, the legacy UE behaviors on (SP-)SRS transmission and (SP-)CSI reporting in C-DRX active time should be followed, in current version of MAC running CR, related conditions have not been captured, then it is suggested RAN2 to confirm that for option 1-2, in the case of lpwus-PDCCH-MonitoringTimer is running, UE’s behaviors on CSI/SRS transmission follows the legacy behaviors defined in C-DRX active time.

Proposal 2: (MAC#4/5) Clarification is needed for option1-2: in the case of lpwus-PDCCH-MonitoringTimer is running, UE’s behaviours on CSI/SRS transmission follows the legacy behaviours defined in C-DRX active time.

- Conclusion

Proposal 1: (MAC#1) It is suggested to further discuss the UE behavior on the LP-WUS collision case for option 1-2: it is left to network configuration on whether to start the lpwus-PDDCH-MonitoringTimer in collision case for option 1-2.

Proposal 2: (MAC#4/5) Clarification is needed for option1-2: in the case of lpwus-PDCCH-MonitoringTimer is running, UE’s behaviours on CSI/SRS transmission follows the legacy behaviours defined in C-DRX active time.

- References

R2-2506613, Open issues on Rel-19 LPWUS 38.321 CR.

RAN2#130 meeting, Chair notes.