3GPP TSG-RAN WG2 Meeting #131bis	R2-2506952

Prague, Czech Republic, October 13th-17th

Agenda Item:	10.3.1

Source:	KT Corporation

Title:	Considerations for ARQ-less 6G user plane

WID/SID:	FS_6G_Radio (Release-20)

Document for:	Discussion and Decision

Introduction

During the 3GPP 6G workshop held in March, many 3GPP member companies mentioned about possible simplifications of user plane protocol stack for 6G. This includes merging or removing existing protocol stack which has duplicated functionalities, moving some functionalities to core network or lower layers, and replacement of some of the existing feature which similar operations can be done from other layers with the target of achieving lean protocol stack. As 6G aims to support ultra-reliable, low-latency, and high-throughput communications across diverse traffic types, such as immersive XR, tactile internet, connected robotics, holographic communication, and ubiquitous AI-driven services, the user plane architecture should evolve beyond the legacy ARQ-HARQ dual-layered error recovery model used in 5G aiming to achieve lean protocol stack. These emerging services demand not only higher performance but also greater flexibility and responsiveness from the protocol stack.

In particular, the removal or deactivation of ARQ is being considered in 6G to reduce protocol overhead, minimize latency, and simplify retransmission logic. This shift places greater responsibility on HARQ as the sole mechanism for error recovery in the 6G user plane.

This contribution identifies key requirements and proposes design directions for ARQ-less 6G user plane architectures, with a focus on HARQ-based retransmissions for service-aware adaptability, robustness, efficiency, and lean protocol stack design.

Discussion

2.1	Motivations

The transition to 6G introduces stringent requirements for ultra-low latency, high reliability, and flexible support for diverse traffic types. Lessons learned from 5G indicate that the conventional layered error recovery architecture (i.e., HARQ at MAC/PHY and ARQ at RLC) while effective for throughput-oriented services, introduces latency and control overhead that can hinder performance for delay-sensitive applications [1-2].

In particular, ARQ mechanisms at the RLC layer often result in delayed recovery and increased signaling, which are not well-suited for emerging 6G use cases requiring deterministic latency and minimal protocol complexity. For example, in 5G URLLC scenarios, ARQ was frequently disabled or bypassed to meet latency budgets, but this led to challenges in maintaining reliability solely through HARQ.

To address these limitations, 6G should explore ARQ-less user plane architectures that rely exclusively on HARQ for error recovery. This shift necessitates a rethinking of HARQ retransmission control, including:

- Dynamic adaptation of HARQ behavior to traffic characteristics and QoS requirements for 6G services

- Mechanisms to detect and recover from HARQ-induced link degradation

- Fine-grained configuration and UE-assisted feedback to support service-aware retransmission strategies

These enhancements aim to improve protocol efficiency, reduce latency, and ensure robust performance across diverse 6G service scenarios.

2.2	Issues and Proposals

To support ARQ-less architectures and ensure robust error recovery in the 6G user plane, as depicted in Figure 1, the following issues are identified, and corresponding proposals are made. These aim to enhance HARQ flexibility, improve service-specific reliability, and enable adaptive retransmission control.

Figure 1. 6G user plane architecture (an example)

Necessity of ARQ-less user plane in 6G

While the conventional dual-layer error recovery (HARQ at MAC/PHY and ARQ at RLC) has been effective in 5G, it imposes limitations in 6G contexts:

- Latency conflict: RLC ARQ retransmissions introduce tens to hundreds of milliseconds of delay, which contradicts the deterministic latency targets of emerging 6G services.

- Protocol overhead: Maintaining both HARQ and ARQ increases control signaling, memory, and buffering complexity, which is inefficient for lightweight and distributed 6G architectures.

- Service diversity: 6G should accommodate extreme URLLC, eMBB, mMTC, and AI-native traffic; a uniform ARQ mechanism is not well-suited for such heterogeneous requirements.

- Deployment experience: In 5G URLLC, ARQ was often disabled to meet latency budgets, demonstrating that HARQ-only recovery is already a practical necessity.

Observation 1: Dual-layer error recovery creates unnecessary latency and complexity, which are misaligned with 6G service requirements.

Proposal 1: Introduce ARQ-less user plane operation in 6G where selected DRBs/QoS flows rely solely on HARQ-based error recovery, reducing protocol overhead and latency.

Traffic-aware HARQ configuration

HARQ serves as the primary error recovery mechanism in ARQ-less 6G architectures. To meet diverse QoS requirements, retransmission behavior should be adaptable to traffic characteristics. Therefore, traffic-aware HARQ configuration that allows differentiated retransmission strategies based on service type, bearer context, or QoS flow will be needed. This includes:

- Dynamic adjustment of retransmission behavior based on latency and reliability needs

- Early termination of retransmission upon successful decoding

- Tailored HARQ behavior for low-latency vs. throughput-oriented services

This flexibility improves resource efficiency and aligns with 6G design goals for scalable and service-aware protocol operation.

Observation 2: HARQ retransmission behavior must be adaptable to traffic characteristics in ARQ-less 6G architectures.

Proposal 2: Enable traffic-aware HARQ configuration to support differentiated retransmission strategies based on service-specific latency and reliability requirements.

HARQ-induced link degradation detection and recovery

In the absence of ARQ, repeated HARQ failures may lead to degraded link conditions or radio link failure (RLF). To maintain robust connectivity, the 6G radio system should support mechanisms to detect HARQ-related link degradation and initiate recovery actions. These may include:

- Configurable thresholds for HARQ failure detection

- Signaling options to indicate link degradation

- Proactive recovery triggers based on HARQ trends or channel quality

Such mechanisms reduce service disruption and support timely reconfiguration or handover.

Observation 3: HARQ failure may trigger RLF in ARQ-less 6G systems, requiring new recovery mechanisms.

Proposal 3: Introduce generalized mechanisms for detecting HARQ-related link degradation and initiating recovery actions, including configurable failure thresholds and proactive triggers.

Fine-grained HARQ control

Uniform HARQ parameters across all traffic types may lead to inefficiencies. To address this, 6G HARQ control should be supported at multiple levels of granularity:

- Bearer-level (e.g., DRB/SRB)

- QoS flow-level (e.g., QFI)

- Transmission block-level (e.g., PDSCH/PUSCH or RB group)

This enables precise control of retransmission behavior, ensuring that latency, reliability, and throughput targets are met per service type.

Observation 4: Per-bearer, per-QoS-flow, and per-transmission-block HARQ configuration enhances service-specific reliability.

Proposal 4: Support fine-grained HARQ control at bearer, QoS flow, and transmission block levels to optimize retransmission behavior.

UE-assisted HARQ adaptation

Adaptive HARQ control requires visibility into UE-side performance. The UE should be able to report HARQ-related status to the network, including:

- Retransmission failure trends

- Channel quality indicators

- Timing alignment status

- Resource or capability constraints

This feedback enables the network to adjust HARQ parameters dynamically, avoid unnecessary retransmissions, and trigger recovery actions when needed.

Observation 5: UE-side feedback on HARQ performance enables adaptive retransmission control and improves link robustness.

Proposal 5: Enable the UE to report HARQ performance status to the network through existing control signaling mechanisms, allowing dynamic adjustment and proactive recovery.

Conclusion

We summarize the observations and proposals in this paper as following:

Observation 1: Dual-layer error recovery creates unnecessary latency and complexity, which are misaligned with 6G service requirements.

Observation 2: HARQ retransmission behavior must be adaptable to traffic characteristics in ARQ-less 6G architectures.

Observation 3: HARQ failure may trigger RLF in ARQ-less 6G systems, requiring new recovery mechanisms.

Observation 4: Per-bearer, per-QoS-flow, and per-transmission-block HARQ configuration enhances service-specific reliability.

Observation 5: UE-side feedback on HARQ performance enables adaptive retransmission control and improves link robustness.

Proposal 1: Introduce ARQ-less user plane operation in 6G where selected DRBs/QoS flows rely solely on HARQ-based error recovery, reducing protocol overhead and latency.

Proposal 2: Enable traffic-aware HARQ configuration to support differentiated retransmission strategies based on service-specific latency and reliability requirements.

Proposal 3: Introduce generalized mechanisms for detecting HARQ-related link degradation and initiating recovery actions, including configurable failure thresholds and proactive triggers.

Proposal 4: Support fine-grained HARQ control at bearer, QoS flow, and transmission block levels to optimize retransmission behavior.

Proposal 5: Enable the UE to report HARQ performance status to the network through existing control signaling mechanisms, allowing dynamic adjustment and proactive recovery.

References

[1] 3GPP RP-251881: SID on Study on 6G Radio (Release 20).

[2] 3GPP TS 38.300 V18.6.0 (2025-06): “NR; NR and NG-RAN Overall description; Stage-2” (Release 18).