3GPP TSG RAN WG2 #131-bis          						R2- 2506956

Prague, Czech Republic, Oct. 13th-17th, 2025

Source:		Tejas Networks Ltd.

Title:		Discussion on A-IoT phase 2 deployment scenario

Agenda item:		9.2.2

Document for:        Discussion and Decision

# Introduction

3GPP TS 38.391 specified the protocols for deployment scenario 1 with topology 1 [1]. As shown in Fig. 1, the deployment topology includes a AIoT device and an AIoT reader. In topology 1, the Base station (BS) is the AIoT reader that directly communicates with AIoT devices. In topology 2, an intermediate node that is enabled with AIoT reader can communicating with AIoT devices. The intermediate node further transmits the AIoT data to the BS via Uu signalling. Thus, the BS indirectly communicates with AIoT devices via the intermediate node. The BS further communicates with the core network at the backend to receive AIoT service requests.

(a)Topology 1                                                                              (b) Topology 2

Figure1: (a) Base station directly communicates with Ambient IoT device, (b) Base station communicates with Ambient IoT device through an intermediate node.

# Discussion

A typical system-level deployment scenario of D2T2 topology is shown in Fig. 2. It can be observed that few device are deployed at the edge of coverage region of two readers. Since the readers are not in sync, few devices may receive paging messages or service requests from multiple readers or UEs in parallel. Moreover, the UE reader can be a mobile device which can move from one coverage region to another coverage region. In both the scenario whether the UE is static or mobile, since the UE readers are not at sync, one AIoT device may receive multiple service request in parallel or the device may receive another service request with different transaction ID when it is already involved in a service.

Fig. 2: System-level deployment scenario of D2T2-topology.

Example scenario:

Fig. 3: Device receives paging messages from multiple UE readers.

Consider reader 1 starts an inventory process for a specific transaction requested from the CN. Another reader starts another inventory process with some offset from the reader 1, as shown in Fig. 3. The inventory process may involve multiple paging for one transaction. If a device successfully sends the information at one paging occasion, it will not respond to the next paging occasions for the same transaction or service request from the same reader to free up the resources for the other device. However, if the device receives the same service request from another reader (say, reader 2) with different transaction ID it may respond to that reader.

The A-IoT communication between a device and multiple readers can be divided into two scenarios. Consider a scenario where the device is already involved in a service requested from reader 1, as shown in Fig. 3. If the device already responds to the initial paging msg0 from one reader it will not respond to the new paging messages or service request from the same or different reader with the same or different transaction ID, until the ongoing process is over by receiving ACK or NACK within a time window. Thus, the device will discard the subsequent service requests.

Consider another scenario where the device has completed the initial service request and goes to idle or listening mode (shown in Fig. 3) and the device has sufficient energy to complete another transaction. If the device receives another service request, it has to decide whether to respond to that request or not.

Observation 1: It can be observed that few AIoT device are deployed at the edge of coverage region of two readers. Since the readers are not in sync, few devices may receive paging messages or service requests from multiple readers or UEs at the same time. RAN2 should consider the multi-reader scenario can occur more often in D2T2 topology and it can disrupt the ongoing process of the device.

Proposal 1: RAN2 should study the Rel-20 AIoT device behaviour considering the multi-reader scenario may exist and few AIoT devices may receive paging messages or service requests from multiple readers or UEs simultaneously.

In D2T2 topology, the D2R resource allocation can be handled either by the BS or by the UE reader. If UE generates Msg2 upon receiving Msg1, it has to allocate resources for D2R upper layer data transmission. If UE act as a relay node, the D2R resources will be allocated by the BS in Msg2 and sent to the UE reader. The UE reader will simply forward the Msg2 received from the BS to the AIoT devices. Thus, the resource allocation and message signal handling procedure are different for UE reader forwarding Msg2 from BS and UE reader generating Msg2 and transmitting to devices.

Observation 2: In D2T2 topology, the D2R resource allocation can be handled either by the BS or by the UE reader.

Proposal 2: RAN2 should study the D2R resource allocation mechanism for D2T2 topology, where the D2R resources can be scheduled ither by the UE reader or by the BS.

Observation 3: In rel-19 AIoT procedure, only NAK based mechanism is supported in response to Msg3, however, ACK based mechanism also should be supported to ensure that the data is successfully received at the reader.

Proposal 3: The reader should send an ACK signal if the Msg3 is successfully received at the reader. If the device doesn’t receive ACK and it receives next R2D trigger, it considers the transaction is failed.

# Conclusion

A-IoT device behaviour in multi-reader scenario is discussed in this work in the context of one or multiple service request(s) received by the device from one or multiple base stations. We have made the following observations and proposals related to the above-mentioned topics:

Observation 1: It can be observed that few AIoT device are deployed at the edge of coverage region of two readers. Since the readers are not in sync, few devices may receive paging messages or service requests from multiple readers or UEs at the same time. RAN2 should consider the multi-reader scenario can occur more often in D2T2 topology and it can disrupt the ongoing process of the device.

Proposal 1: RAN2 should study the Rel-20 AIoT device behaviour considering the multi-reader scenario may exist and few AIoT devices may receive paging messages or service requests from multiple readers or UEs simultaneously.

Observation 2: In D2T2 topology, the D2R resource allocation can be handled either by the BS or by the UE reader.

Proposal 2: RAN2 should study the D2R resource allocation mechanism for D2T2 topology, where the D2R resources can be scheduled ither by the UE reader or by the BS.

Observation 3: In rel-19 AIoT procedure, only NAK based mechanism is supported in response to Msg3, however, ACK based mechanism also should be supported to ensure that the data is successfully received at the reader.

Proposal 3: The reader should send an ACK signal if the Msg3 is successfully received at the reader. If the device doesn’t receive ACK and it receives next R2D trigger, it considers the transaction is failed.

# References

[1] TS 38.391, Ambient IoT Medium Access Control Protocol specification, Rel-19, V1.0.0.