3GPP TSG-RAN WG2 Meeting #131bis	R2-2506957

# Prague, Czech Republic, 13-17 October 2025

Agenda Item:	10.3.2

Source:	MediaTek Inc.

Title:	Control plane architecture and 6G RRC protocol design

Document for:	Discussion, decision

# 1	Introduction

The study on 6G radio was approved in RAN#108 [1]. One of the study objectives is radio interface protocol architecture and procedures for 6G radio, including control plane architecture and protocol design. Furthermore, minimizing optionality and configuration complexity as well as addressing real-world concerns about implementation complexity are included in the justification for the study.

In this document, we discuss control plane architecture and focus on fundamental aspects of 6G RRC. The presented proposals are intended to set a foundation for the 6G RRC protocol design.

# 2	Discussion

## 2.1	Control Plane Protocol Stack

We assume 6G RRC will depend on L2 of the eventually-agreed UP stack for reliable transport of RRC signalling, similar to previous generations. It is not obvious if SDAP or a similar “QoS sublayer” will be carried forward into 6G, but if so, we think it should not be required in the control plane (also as in 5G). The details of the control plane protocol stack can be further developed after discussion of the various UP proposals (see, e.g., [2]).

Observation 1	L2 can be expected to evolve, but it seems safe to assume that RRC will use the L2 sublayers as transport as in previous generations.

The control plane protocol stack at this level of generality is shown in Figure 1. We assume the figure itself does not need to be captured in the TR at this stage, and it can be revisited when the L2 details have been discussed.

Figure 1. Approximate control plane protocol stack

Proposal 1	RAN2 to study a control plane protocol stack in which 6G RRC sits above a set of L2 sublayers drawn from the UP stack, to be determined based on the UP discussion, but not including any QoS-specific sublayer (e.g., SDAP).

## 2.2	RRC Functions

We think 5G RRC serves as a suitable functional baseline for 6G RRC (some signalling, process, and spec-practice changes may be expected), and we assume the AS/NAS functional split can also be similar to 5G, so we can derive the list of 6G RRC functions from the 5G RRC functions in TS 38.300/38.331. The list below is intentionally not exhaustive; it is intended to provide a framework for what functions to study, with expansion/adjustment anticipated in the WI phase.

Proposal 2	RAN2 to study 6G RRC with respect to the following list of functions (non-exhaustive):

- Delivery of system information (FFS broadcast vs. unicast);

- Paging;

- Connection management including establishment, reconfiguration, re-establishment, and release of an RRC connection;

- Radio bearer management (SRBs and DRBs);

- Security functions (FFS in cooperation with SA3);

- Detection of and recovery from RLF;

- UE capability signalling;

- Transport of upper-layer signalling (e.g., NAS) to/from the UE.

As compared to the 5G RRC functions, the following functions are omitted for the time being:

- Mobility (handled elsewhere for now, subject to the functional split for mobility between RRC and lower layers);

- QoS control (likely that RRC will do something, but it should follow the general QoS design and can be discussed later);

- Measurement control/reporting (similar issue to mobility);

- Access barring (FFS how this function is divided between RRC and NAS).

## 2.3	RRC Structure

There have been suggestions that 6G RRC could be split into “upper” and “lower” parts, with different parts of the UE configuration managed and signalled separately. We see this approach as inviting complexity without a clear benefit: Configurations would need to be coordinated between the two sublayers, some functions might be expected to span both parts, and messages would terminate in different ways (potentially at different nodes on the network side in case of a disaggregated RAN architecture) depending on whether they were for “upper” or “lower” RRC.

If there is a need for security for MAC CEs, as has been suggested, one way to approach applying security would be to model certain L2 signalling, such as distinguished MAC CEs requiring security, as being carried over a bearer like a distinguished SRB. We do not intend to exclude this possibility, only to avoid splitting the RRC configuration management and signalling between sublayers.

Proposal 3	6G RRC comprises a single sublayer (no upper/lower split). This does not preclude use of the bearer concept for L2 signalling requiring security.

## 2.4	RRC States

We assume upper layers will retain the state machine defining CM-IDLE/CM-CONNECTED (or similar states), which suggests that 6G RRC should maintain RRC_IDLE and RRC_CONNECTED states.

The third RRC state, RRC_INACTIVE, is not widely deployed in 5G, but it offers an attractive combination in principle of power efficiency and low latency from inactivity to data transfer. The essential characteristics of RRC_INACTIVE are UE-controlled mobility within the RNA, long(ish) DRX for RAN paging, partial UE context at the network enabling a quick resume procedure along with reduced in-network overhead, and the availability of SDT. The state can be seen as combining the availability of data transfer of RRC_CONNECTED with the power savings of RRC_IDLE. SDT is particularly significant as a way of combining these two benefits; in a world where many data transfers are small, the SDT mechanism offers significant power savings compared to coming to RRC_CONNECTED for every transmission.

We think these characteristics will remain valuable in 6G, and they should be evaluated to see how they can be enabled in a practical way for implementations, which might mean incorporating them into one of the two existing states rather than reifying them as a third state; however, the final state modelling in RAN2 may depend partly on RAN1 conclusions in relation to power-saving features. Power saving will be particularly important for low-end IoT devices.

We understand that UE context maintenance at the base stations should be less of a challenge in 6G than in 5G, thanks to the advance of virtualization. Some coordination with RAN3 will be required to make sure that context maintenance and transfer are tractable in a 6G network.

Observation 2	The characteristics of 5G’s RRC_INACTIVE state (especially SDT) will be useful in 6G.

Proposal 4	6G RRC has baseline RRC_IDLE and RRC_CONNECTED states. RAN2 to study ways to enable functionality similar to 5G's RRC_INACTIVE state in 6G RRC.

## 2.5	Connection Management Procedures

Connection management is the key function of RRC, so we see it as important to study it early in the SI in order to determine whether 5G RRC connection management procedures are suitable for re-use for 6G RRC or if they require fundamental updates/improvements. We list a core set of connection management procedures which should be studied for 6G RRC below.

One important difference we think should be included in 6G RRC compared to 5G RRC is UE initiated RRC connection release. The temporary desync of the UE RRC state between the network and the UE, caused by local RRC connection release by the UE, is seen as a problem already in 5G. UE initiated RRC connection release also offers an opportunity for power saving, because the UE may know first when further data transfer is not expected (e.g., because an application terminated), and this mechanism allows the UE to return to a power-saving state (idle/inactive) as early as possible.

Observation 3	5G does not include UE initiated RRC connection release. Without it, local RRC connection release by the UE may cause temporary desync of the UE RRC state between the network and the UE.

Proposal 5	RAN2 to study the following core list of connection management procedures of 6G RRC, including whether they can be re-used from 5G or require fundamental updates/improvements:

- RRC connection establishment;

- RRC connection reconfiguration;

- RRC connection re-establishment/recovery;

- RRC connection release (including UE initiated RRC connection release).

Other procedures not directly related to connection management will emerge over time. RRC suspend/resume is not listed at this point, because some discussion may be needed on the RRC_INACTIVE functionality (see section 2.4).

## 2.6	UE Configuration Structure

The current structure of the UE configuration carried in 5G RRC signalling is very different from its initial version from the early days of Rel-15. Feature additions, several of those already for Rel-15, such as BWPs, SUL, mTRP, unified TCI framework, RedCap, etc., have made the structure of the UE configuration very complex. While the feature additions have been specified with an eye on maintaining the structure, the initial clarity of the structure has vanished a long time ago, and it is increasingly difficult to add even relatively small features in an error-free manner and without compromising the structure and clarity even more.

Observation 4	The UE configuration structure in 5G RRC signalling has become complex due to incremental addition of new features.

The complexity of the UE configuration structure causes unnecessary implementation and verification effort for the UE and network vendors and the test equipment manufacturers in all phases of the product development, including feature implementation, local verification, and IODT/IOT. In 3GPP standardization, not only RAN2 and RAN1 suffer from difficult addition of the new features, but also RAN5 test case development is increasingly tedious and challenging to perform comprehensively and accurately. Furthermore, the complex UE configuration structure poses performance problems for procedures such as mobility, where a seemingly simple UE configuration update requires signalling of a large set of fields.

We list characteristics which we think make the current UE configuration structure complex below. The list is intentionally non-exhaustive; it is intended to provide a starting point for which aspects to address in the study:

- Deeply nested structure and complicated cross-references within the structure

- Scattering of parameters for a single functionality within the structure

- The concept of common and dedicated configurations

- Reusing and overloading of information elements for new purposes

Based on the above, we think simpler and more maintainable UE configuration structure is required for 6G RRC signalling. An early study effort in this direction would be useful to avoid designing the signalling architecture at the last minute; inevitably, there will be late additions and changes that introduce some complexity, but RAN2 should strive to design a signalling framework that is as robust as possible in the face of such changes, as well as extensions in future releases; in addition, since significant part of the UE configuration parameters are defined by RAN1 and RAN4, they should be informed early about RAN2 decisions and agreements about the UE configuration structure.

Proposal 6	RAN2 to study from the beginning improved UE configuration structure for 6G RRC as compared to 5G RRC, with a focus on simplicity and maintainability upon future extensions (such as new feature additions).

Observation 5	UE configuration structure has an inter-WG dependency to RAN1/RAN4, since significant part of the UE configuration parameters are defined by RAN1/RAN4.

## 2.7	Delta Signalling

The size of the UE configuration carried in RRC signalling, especially for RAN1 parameters, has been a significant pain point in 5G and previous generations, for both standardisation and implementation. In order to reduce the size of over-the-air RRC messages, delta signalling has been designed into the RRC protocol – an organised system for delta signalling in 36.331 and subsequent improvements in 38.331 were specified.

Even today, many years after initial 5G deployment, delta-signalling-related interoperability issues keep occurring, and the signalling ambiguities due to the current delta signalling mechanism keep requiring unnecessary implementation and verification efforts from the UE and network vendors. We list the most painful such ambiguities from UE's perspective below (non-exhaustive list):

- Functionally mandatory configuration parameters can be absent in over-the-air RRC messages/IEs (as they are usually optional ASN.1 fields for delta-signalling purposes (Need M))

- Configuration parameters which shouldn't be modified can be sent in subsequent over-the-air RRC messages/IEs with new values (as the same IEs are used for initial setup and subsequent modifications of the configuration)

- Configuration parameters can be sent with values which do not conform to other parts of the configuration (e.g., presence or values of other configuration parameters)

Note that these ambiguities are all in the “should not occur” category. In 5G, the intended usage of signalling is captured in the RRC specification using comment text tags attached to the fields, known as Cond's. In addition, field descriptions capture further aspects of the way how RRC messages and IEs are expected to be constructed by the network. These spec guidelines tend to indicate what the network “does not” do, i.e., what circumstances the UE may anticipate will never occur; in theory this means that the UE need not implement any handling for cases like the list above, but in practice such a UE would be a brittle implementation that risked difficult-to-address problems in the field. Even though a good network implementation should not violate these guidelines, the Cond's and field descriptions themselves are not always unambiguous (e.g., it is a routine standards activity to clarify after the fact which parameters can be modified under which circumstances), and a UE implementation needs to include well-behaved handling for any physically possible combination of parameters (a network bug or an attack should not cause a UE crash or security failure, of which we'll discuss more below).  Furthermore, as the Cond's and field descriptions in fact are not machine readable by ASN.1 compilers/toolsets, they are not binding to the ASN.1 implementation. This means that even with carefully crafted Cond's and field descriptions, it is still possible to construct invalid over-the-air RRC messages/IEs.

Based on the above, the UE can be sent invalid RRC signalling (refer to the bulleted list of ambiguities above), either accidentally due to implementation errors in the network, or intentionally by rogue base stations in order to probe for and/or exploit security vulnerabilities in the UE. Robust UE implementation is required to gracefully handle this kind of invalid RRC signalling, and it is a common and necessary practice to apply fuzzy testing—including exhaustive configurations of RRC parameters in combinations that are not in principle permitted—to uncover bugs, security vulnerabilities, and crashes in the UE. Since the 5G RRC signalling has a lot of ambiguity, the required effort by the UE vendors to implement the detection and graceful handling for all ambiguities is very large, even for non-critical devices such as smart phones.

Observation 6	Delta signalling introduces ambiguities in RRC signalling, which are sources of security vulnerabilities. The required effort by the UE vendors to build devices which are robust against exploitation of the ambiguities is very high.

To summarize our view, the general principle of delta signalling is worthwhile, but it needs to be examined with an eye to removing ambiguities and simplifying the implementation process (even if this causes some complexity in the specification process). Like the UE configuration structure, also the delta signalling mechanism needs to be addressed early, to avoid a rushed solution whose problems may not be immediately apparent.

In addition, since a significant part of the UE configuration parameters is defined by RAN1 and RAN4, we detect an inter-WG dependency that needs to be addressed during the study. RAN2 may need more detailed information about individual parameters from RAN1 and RAN4, e.g., in relation to when a parameter is expected to be modified, and RAN1/RAN4 may need guidance from RAN2 about the signalling format that their parameters ultimately target.

Proposal 7	RAN2 to study from the beginning more usable mechanisms for delta signalling for 6G RRC as compared to previous generations, with a focus on implementation robustness and simplicity rather than specification convenience.

# 3	Conclusion

Based on the discussion in the previous sections we have following observations and proposals:

Control plane protocol stack, RRC functions, and RRC structure:

Observation 1	L2 can be expected to evolve, but it seems safe to assume that RRC will use the L2 sublayers as transport as in previous generations.

Proposal 1	RAN2 to study a control plane protocol stack in which 6G RRC sits above a set of L2 sublayers drawn from the UP stack, to be determined based on the UP discussion, but not including any QoS-specific sublayer (e.g., SDAP).

Proposal 2	RAN2 to study 6G RRC with respect to the following list of functions (non-exhaustive):

- Delivery of system information (FFS broadcast vs. unicast);

- Paging;

- Connection management including establishment, reconfiguration, re-establishment, and release of an RRC connection;

- Radio bearer management (SRBs and DRBs);

- Security functions (FFS in cooperation with SA3);

- Detection of and recovery from RLF;

- UE capability signalling;

- Transport of upper-layer signalling (e.g., NAS) to/from the UE.

Proposal 3	6G RRC comprises a single sublayer (no upper/lower split). This does not preclude use of the bearer concept for L2 signalling requiring security.

RRC states:

Observation 2	The characteristics of 5G’s RRC_INACTIVE state (especially SDT) will be useful in 6G.

Proposal 4	6G RRC has baseline RRC_IDLE and RRC_CONNECTED states. RAN2 to study ways to enable functionality similar to 5G's RRC_INACTIVE state in 6G RRC.

Connection management procedures:

Observation 3	5G does not include UE initiated RRC connection release. Without it, local RRC connection release by the UE may cause temporary desync of the UE RRC state between the network and the UE.

Proposal 5	RAN2 to study the following core list of connection management procedures of 6G RRC, including whether they can be re-used from 5G or require fundamental updates/improvements:

- RRC connection establishment;

- RRC connection reconfiguration;

- RRC connection re-establishment/recovery;

- RRC connection release (including UE initiated RRC connection release).

UE configuration structure:

Observation 4	The UE configuration structure in 5G RRC signalling has become complex due to incremental addition of new features.

Proposal 6	RAN2 to study from the beginning improved UE configuration structure for 6G RRC as compared to 5G RRC, with a focus on simplicity and maintainability upon future extensions (such as new feature additions).

Observation 5	UE configuration structure has an inter-WG dependency to RAN1/RAN4, since significant part of the UE configuration parameters are defined by RAN1/RAN4.

Delta signalling:

Observation 6	Delta signalling introduces ambiguities in RRC signalling, which are sources of security vulnerabilities. The required effort by the UE vendors to build devices which are robust against exploitation of the ambiguities is very high.

Proposal 7	RAN2 to study from the beginning more usable mechanisms for delta signalling for 6G RRC as compared to previous generations, with a focus on implementation robustness and simplicity rather than specification convenience.

# 4	References

RP-251881:	“3GPP Work Item Description - Study on 6G Radio”, NTT DOCOMO, RAN#108

R2-2506854: "On 6G user plane functions and considerations for user plane architecture", MediaTek Inc., RAN2#131bis