3GPP TSG-RAN WG2 Meeting #131bis	R2-2506962

Prague, Czech Republic, 13th -17th Oct, 2025

Source:	vivo

Title:	Remaining open issues on R19 Ambient IoT

Agenda Item:	8.2.2

Document for:	Discussion and Decision

# Introduction

In this contribution, we will discuss the following remaining open issues regarding A-IoT MAC spec, which are listed in the email discussion [POST131][021][AIoT] MAC spec [1]:

- Checking whether there is new case for “no upper layer data available” other than long writing operation which may impact to how to set MDI field (Issue 3-7)

- Paging ID length impact from new SA2 LS in S2-2507793 (Issue 1-3)

- CT1 LS on max NAS message size (Issue 4-6)

# Discussion

## Issue 3-7: More Data Indication

According to the email discussion, open issue 3-7 on MDI setting is further updated as following [1]:

| Issue 3-7: more data indication handling | How to set “more data indication” value in case of no NAS response available (i.e., zero SDU) - Relevant agreements: - The reader determines no data available case by SDU length 0. As more data indication is mandatory, the device sets this bit to "0". - The reader, in response to 0 SDU in the device’s MAC response, may send a follow-up R2D Upper Layer Data Transfer message at a later time to schedule another D2R Upper Layer Data Transfer message from the device. - The follow-up R2D Upper Layer Data Transfer message includes the Received Data Size field with the Received Data Size field set to value 0, without including the original command. - RAN2 would like to check if there is a case where NAS doesn’t provide a response at all. If this case exists, RAN2 will discuss this issue. - Regarding the above check point, it can be up to companies’ contribution. - Status in running CR: captured. | To be discussed by company contributions |
| --- | --- | --- |

In the latest CT1 specification[2], the following description addressing the A-IoT device’s behavior upon receiving a NAS msg with specific errors, including reception of unknown/unforeseen message type and a message with certain non-semantical mandatory information element error:

## 6.3 Unknown or unforeseen message type If the AIoT device or the network receives a AIoT NAS message with message type not defined for the AIoT NAS protocol or not implemented by the receiver, it shall ignore the AIoT NAS message. NOTE: A message type not defined for the AIoT NAS protocol in the given direction is regarded by the receiver as a message type not defined for the AIoT NAS protocol, see 3GPP TS 24.007 [3]. ## 6.4 Non-semantical mandatory information element errors When on receipt of a message, a) an "imperative message part" error; or b) a "missing mandatory IE" error; is diagnosed or when a message containing a syntactically incorrect mandatory IE is received, the AIoT device shall ignore the AIoT NAS message and the network shall: a) try to treat the message (the exact further actions are implementation dependent); or b) ignore the message.

Based on the , the following can be observed:specification excerpt above

- (Issue 3-7) According to TS 24.369 v1.0.0, an A-IoT device may ignore a received NAS message when it contains errors such as an “Unknown or unforeseen message type”, “imperative message part”, “missing mandatory IE” or “containing syntactically incorrect IE”. However, it is not specified whether the device should provide a NAS response in these cases.

In the latest SA3 specification[3], the following description addresses NAS command response generation:

### 5.3.2 Security procedure on information protection during command procedure ********************the flow chart figure Figure 5.3.2-1 is omitted ********************* 1. The command procedure is initiated as specified in step 1-6 of clause 6.2.3 of TS 23.369 [2]. 2. The procedure as described in clause 5.2.2 shall be performed. The device and AIOTF acquire the KAIOTF key to be used for command protection. The derivation of KAIOTF key is specified in Annex A.3. 3. The AIOTF shall construct a AIOT NAS Command Request and protect the message based on the KCommand_enc, KCommand_int, the confidentiality and integrity algorithms for the AIoT device. The AIOTF shall send the protected Command Request containing an indication on whether cyphering is activated to NG-RAN. NOTE 1: The whole AIOT NAS Command Request message is integrity protected. If confidentiality algorithm is not null-scheme, the AIOT NAS Command Request message is partly ciphered with the exception that the selected protection algorithms are in clear text. 4. The NG-RAN shall send a R2D message containing the protected AIOT NAS Command Request as specified in as specified in TS 38.300 [3] and TS 38.391 [6]. 5. The device shall derive the KCommand_enc, KCommand_int and verify the integrity of the command message. If the verification of integrity is successful, the AIoT device shall decipher it in case it is confidentiality protected. The AIoT device shall construct a NAS Command Response and protect the message based on the KCommand_enc and KCommand_int key using the same algorithms. 6. The AIoT device shall send a D2R message containing the protected AIOT NAS Command Response to the NG-RAN as specified in as specified in TS 38.300 [3] and TS 38.391 [6]. 7. The NG-RAN shall forward the AIOT NAS Command Response containing the protected AIOT NAS Command Response to the AIOTF. 8-9. The AIOTF shall verify the integrity of the command message. If the verification of integrity is successful, the AIoT device shall decipher it in case it is confidentiality protected. Then, the AIOTF shall continue the procedure as specified in clause 6.2.3 of TS 23.369 [2]. NOTE 2: It is assumed that there is only one round of command procedure per device following an inventory procedure. Since the KAIOTF key is fresh, there is no need for additional freshness parameters for replay protection. NOTE 3: It is assumed that no new algorithms will ever be introduced for information protection during command procedure.

Based on the above specification part, the following can be observed:

- (Issue 3-7) According to TS 24.369 v1.0.0, the A-IoT device generates a NAS response after successful integrity verification of a NAS cmd message. However, the behavior of the A-IoT device behaviour upon integrity verification failure of a NAS msg is not specified.

In the latest CT1/SA3 specifications, it is explicitly specified that the AIoT device should ignore the received NAS msg in several cases. However, it is not specified whether the AIoT device should provide a NAS command response in any of these cases.. From RAN2 perspective, it can be understood that there is no NAS cmd response from upper layer to MAC after the MAC delivers a received SDU to upper layer in these cases.

Considering the above cases, in addition to the delayed NAS response case, there are scenarios where the MAC does not receive a NAS cmd response in time after delivering an SDU to upper layer at the A-IoT device:

- Case 1: The NAS response is delayed but will be provided later;

- Case 2: No NAS command response is provided due to errors or integrity verification failure of the R2D NAS msg.

If MDI is set to 0 in both cases, the reader cannot distinguish between Case 1 or Case 2. If Case 2 occurs, the reader may perform multiple rounds of D2R scheduling for the device until the device is power exhausted. In this scenario,the AIoT device will only transmit padding bits in the D2R transmissions, resulting in significant radio resource consumption.

- (Issue 3-7) if MDI is always set to 0 in D2R MAC PDU without an SDU, the reader cannot determine which of the following case ocurrs after sending a NAS cmd to A-IoT device:

- Case 1: The NAS response is delayed but will be provided later;

- Case 2: No NAS command response is provided due to errors or integrity verification failure of the R2D NAS msg.

For safety, it is preferred to double check with CT1 regarding the above observations, and to inform CT1 the related impact on D2R scheduling issue in RAN2.

- (Issue 3-7) RAN2 send an LS to CT1 (cc SA2/SA3), as in Annex draft LS, to inquire whether the NAS layer at A-IoT device does not provide response in the following R2D NAS msg error/failure cases:

- “Unknown or unforeseen message type” error

- “Imperative message part” error

- “Missing mandatory IE” error

- “Containing syntactically incorrect IE” error

- Integrity verification failure

- (Issue 3-7) If Proposal 1 is not agreeable, to avoid the risk of subsequent D2R schedulings in case where no NAS response is provided after R2D NAS msg TX, RAN2 agree to:

- Set MDI to 1 if the NAS response is delayed and will be available later;

- Set MDI to 0 if no NAS response will be provided.

## Issue 1-3: Paging ID length impact from new SA2 LS in S2-2507793

In the email discussion, open issue 1-3 on Paging ID length is described as below [1]:

| Issue 1-3: Paging ID length field | The field to indicate the paging ID length, e.g., how many bits, taking into account of CT4 and SA2 inputs. - Relevant agreements: - 8-bit length field (in unit of bit) is assumed to indicate the paging ID length, based on current SA2/CT4 conclusion. - However, in Aug meeting, SA2 sent a new LS to RAN2 in S2-2507793 on AIoT Device Permanent ID Length, indicating SA2 identified requirements for longer device ID, and the rapporteur understand this will impact the length of paging ID as well, e.g., 600 bits are needed instead of 256 bits. Therefore, in Oct meeting, RAN2 needs to discuss how to reply the SA2 LS, and how to handle the impact to paging ID length field. - Status in running CR: the field name is captured in 6.2.1.1 with the detailed format. | To be discussed by company contributions |
| --- | --- | --- |

On one hand, the filtering information is used to identify or filter multiple AIoT Devices and is constructed by one or multiple components (i.e. ID Type, PLMN Identifier, NID, third party identifier and Identification Information) of the AIoT Device Permanent Identifier as defined in SA2 TS 23.369 clause 5.7 [4]. On the other hand, the filtering information is contained by the paging ID filed and delivered by the AIoT Paging message according to AIoT MAC spec TS 38.391. Moreover, as per SA2 estimated around 600-bit for one case and around 360-bit as the maximum length of filtering information for another case, we understand that RAN2 need to adopt a 10-bit length field (in unit of bit) to indicate the paging ID length to meet both cases. Consequently, the last RAN2#123 meeting agreement on an 8-bit length field (in unit of bit) to indicate the paging ID length should be revisited and reverted.

In addition to the filtering information, SA2 also mentioned in the LS that the overall length of the paging message may need to consider other information like security parameters, etc. Given that this issue has already been addressed in a previous reply LS out to SA3 (Cc: RAN3, SA2) in [5], no additional information needs to be provided in another LS reply to SA2 this meeting. In conclusion, regarding SA2’s question on whether extending the AIoT Device permanent ID to 256-bit and 496-bit is feasible, the following proposal is suggested.

- (Issue 1-3) Regarding the SA2 LS in S2-2507793, which inquires whether extending the AIoT Device permanent ID to 256 bits and 496 bits is feasible, RAN2 to reply as follows:

- It is feasible from a signaling perspective to extend the AIoT Device permanent ID to 256 bits and 496 bits;

- The AIoT Device permanent ID is used to construct the filtering information , which contained in the paging ID filed of the AIoT Paging message as currently defined in the A-IoT MAC spec TS 38.391 V1.0.0.

- Based on SA2's estimation of approximately 600 bits and around 360 bits as the maximum length of filtering information used for 256-bit and 496-bit Device permanent ID, respectively, RAN2 has agreed to adopt a 10-bit length field (in unit of bit) to indicate the paging ID length and will update the A-IoT MAC spec accordingly.

## Issue 4-6: CT1 LS on max NAS message size

In the email discussion, open issue 4-6 on max NAS message size is described as below [1]:

| (New) Issue 4-6: CT1 LS on max NAS message size | In C1-255679 LS on the maximum supported AIoT NAS container length, CT1 asked about the maximum supported AIoT NAS container length in both R2D and D2R directions. - Background and relevant agreements: - In R1-2409250/R2-2409405 Reply LS to RAN2 on data block sizes for Ambient IoT, RAN1 indicated that from RAN1 perspective, a maximum TB size of around 1000 bits in PHY for R2D and D2R directions can be supported. Based on this, RAN2 design the SDU length and offset length as 7 bits, which is a sufficiently large value that can indicate the maximum integer number of bytes after subtracting the MAC overhead from 1000 bits. - The rapporteur understands that RAN2 can reply CT1 with the above situation, and there is no additional impact to the current MAC signaling format, | To be discussed by company contributions |
| --- | --- | --- |

CT1 requests clarification on the maximum supported AIoT NAS container length in both R2D and D2R directions. Since this question is related to the coding aspects of AIoT NAS command messages,  a definite answer should be provided. According to the current MAC PDU format[6]:

R2D Upper Layer Data Transfer message

The maximum R2D TBS value is 125 bytes, then the maximum supported length of the R2D Data SDU is 119 bytes (i.e., 125 bytes minus 6 bytes of fixed overhead).

38.391 v1.0.0 Figure 6.2.1.4-1: MAC PDU of R2D Upper Layer Data Transfer message containing Data SDU

D2R Upper Layer Data Transfer message

The maximum D2R TBS value is 125 bytes, then the maximum supported length of the D2R Data SDU can be determined as 123 bytes (i.e., 125 bytes minus 2 bytes of fixed overhead);

38.391 v1.0.0 Figure 6.2.2.2-1: MAC PDU of D2R Upper Layer Data Transfer message

Therefore, we propose that:

- (Issue 4-6) RAN2 to reply to CT1 with the following information:

- The maximum supported AIoT NAS container length in R2D direction is 119 bytes;

- The maximum supported AIoT NAS container length in D2R direction is 123 bytes.

# Conclusion

In this contribution, remaining issues on AIoT MAC spec are discussed, and we have the following observations and proposals:

Issue 3-7: More Data Indication

- (Issue 3-7) According to TS 24.369 v1.0.0, an A-IoT device may ignore a received NAS message when it contains errors such as an “Unknown or unforeseen message type”, “imperative message part”, “missing mandatory IE” or “containing syntactically incorrect IE”. However, it is not specified whether the device should provide a NAS response in these cases.

- (Issue 3-7) According to TS 24.369 v1.0, the A-IoT device generates a NAS response after successful integrity verification of a NAS cmd message. However, the behavior of the A-IoT device behaviour upon integrity verification failure of a NAS msg is not specified.

- (Issue 3-7) if MDI is always set to 0 in D2R MAC PDU without an SDU, the reader cannot determine which of the following case ocurrs after sending a NAS cmd to A-IoT device:

- Case 1: The NAS response is delayed but will be provided later;

- Case 2: No NAS command response is provided due to errors or integrity verification failure of the R2D NAS msg.

- (Issue 3-7) RAN2 send an LS to CT1 (cc SA2/SA3) , as in Annex draft LS, to inquire whether the NAS layer at AIoT device does not provide response in the following R2D NAS msg error/failure cases:

- “Unknown or unforeseen message type” error

- “Imperative message part” error

- “Missing mandatory IE” error

- “Containing syntactically incorrect IE” error

- Integrity verification failure

- (Issue 3-7) If Proposal 1 is not agreeable, to avoid the risk of subsequent D2R scheduling in case where no NAS response is provided after R2D NAS msg TX, RAN2 agree to:

- Set MDI to 1 if the NAS response is delayed and will be available later;

- Set MDI to 0 if no NAS response will be provided.

Issue 1-3: Paging ID length impact from new SA2 LS in S2-2507793

- (Issue 1-3) Regarding the SA2 LS in S2-2507793, which inquires whether extending the AIoT Device permanent ID to 256 bits and 496 bits is feasible, RAN2 to reply as follows:

- It is feasible from a signaling perspective to extend the AIoT Device permanent ID to 256 bits and 496 bits;

- The AIoT Device permanent ID is used to construct the filtering information , which contained in the paging ID filed of the AIoT Paging message as currently defined in the A-IoT MAC spec TS 38.391 V1.0.0.

- Based on SA2's estimation of approximately 600 bits and around 360 bits as the maximum length of filtering information used for 256-bit and 496-bit Device permanent ID, respectively, RAN2 has agreed to adopt a 10-bit length field (in unit of bit) to indicate the paging ID length and will update the A-IoT MAC spec accordingly.

Issue 4-6: CT1 LS on max NAS message size

- (Issue 4-6) RAN2 to reply to CT1 with the following information:

- The maximum supported AIoT NAS container length in R2D direction is 119 bytes;

- The maximum supported AIoT NAS container length in D2R direction is 123 bytes.

# Annex: Text proposal of LS to CT1

*** ***********************Start of LS draft*******************************

Title:	LS on Cases where no NAS command response is provided

Response to:

Release:	Rel-19

Work Item:	FS_Ambient_IoT_solutions

Source:	vivo [to be RAN2]

To:	CT1

Cc:	SA2, SA3

Contact person:	Kimba Dit Adamou, Boubacar

kimba@vivo.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

1	Overall description

In RAN2#131, In RAN2#131, RAN2 discussed the D2R scheduling of delayed NAS command responses and reached the following agreement..

- RAN2 would like to check if there is a case where NAS doesn’t provide a response at all. If this case exists, RAN2 will discuss this issue.

In RAN2#131bis, RAN2 further discussed the above issue based on the latest CT1 and SA3 specifications, respectively, and made the followings observations:

- In TS 24.369 V1.0.0, the following procedures for NAS message error handling by the AIoT device are specified. It is observed that the AIoT device may simply ignore the received NAS msg in the error cases described in Cause 6.3 and 6.4:

## 6.3 Unknown or unforeseen message type If the AIoT device or the network receives a AIoT NAS message with message type not defined for the AIoT NAS protocol or not implemented by the receiver, it shall ignore the AIoT NAS message. NOTE: A message type not defined for the AIoT NAS protocol in the given direction is regarded by the receiver as a message type not defined for the AIoT NAS protocol, see 3GPP TS 24.007 [3]. ## 6.4 Non-semantical mandatory information element errors When on receipt of a message, a) an "imperative message part" error; or b) a "missing mandatory IE" error; is diagnosed or when a message containing a syntactically incorrect mandatory IE is received, the AIoT device shall ignore the AIoT NAS message and the network shall: a) try to treat the message (the exact further actions are implementation dependent); or b) ignore the message.

- In TS 33.369 V1.0.0, it is observed that the AIoT device only generates NAS cmd response only upon successful integration verification of a received NAS command message. However, it is not specified whether the AIoT device should also generate a NAS command response for a received NAS message for which integrity verification has failed..

### 5.3.2 Security procedure on information protection during command procedure … … 5. The device shall derive the KCommand_enc, KCommand_int and verify the integrity of the command message. If the verification of integrity is successful, the AIoT device shall decipher it in case it is confidentiality protected. The AIoT device shall construct a NAS Command Response and protect the message based on the KCommand_enc and KCommand_int key using the same algorithms. … … NOTE 3: It is assumed that no new algorithms will ever be introduced for information protection during command procedure.

From RAN2’s perspective, it is necessary for the reader to know whether the NAS cmd response is delayed or not provided to facilitate the D2R scheduling of the possible NAS response msg by the reader. RAN2 understands that the AIoT device will not provide NAS cmd response in the above error/failure cases. For clarity, RAN2 requests that CT1 confirm whether this understanding is correct.

Question: RAN2 would like CT1 to inform RAN2 whether the AIoT device provides a NAS cmd response upon reception of an R2D NAS message with any of the following error/failure:

- “Unknown or unforeseen message type” error

- “Imperative message part” error

- “Missing mandatory IE” error

- “Containing syntactically incorrect IE” error

- Integrity verification failure

2	Actions

To CT1

ACTION: 	RAN2 kindly asks CT1 to take above into account and provides feedback on the Question above.

3	Dates of next RAN2 meetings

TSG-RAN2 Meeting #132		17 - 21 Nov 2025	Dallas, US

TSG-RAN2 Meeting #133		 9 - 13 Feb 2026	Stor-Göteborg, Sweden

***************************End of LS draft*****************************

- Reference

- R2-2507030, Summary of A-IoT MAC open issues (outcome of [POST131][021][AIoT] MAC spec).

- 3GPP 24.369 V1.0.0 (2025-09), Ambient IoT Non-Access-Stratum (AIoT NAS) protocol for 5G System (5GS); Stage 3; (Release 19).

- 3GPP TS 33.369 V1.0.0 (2025-09), Security aspects of Ambient IoT service for isolated private networks; (Release 19).

- 3GPP TS 23.369 V19.0.0 (2025-06), Architecture support for Ambient power-enabled Internet of Things; Stage 2 (Release 19).

- R2-2506465, Reply LS on security parameter in paging request message, To:SA3; Cc: RAN3, SA2.

- 3GPP TS 38.391 v1.0.0