3GPP TSG-RAN WG2 Meeting #131bis											R2-2506966

Prague, Czech Republic, Oct. 13th-17th

Agenda Item:	8.5.2

Source:	vivo

Title:	Remaining CP open issues of NES

Document for:	Discussion and Decision

Introduction

In this contribution, we will discuss the remaining CP open issues of NES, which are suggested to be further discussed during ASN.1 review.

Discussion

[V500] Correction on RRC state restriction for SIB1 reception when the UE receives SI change or PWS notification

In the last RAN2 meeting, RAN2 reached the following agreement for UE behavior after reception of SI change or PWS notification [1]:

NW may configure any Kssb upon SI change notification (including ETWS/CMAS), but NES UE still acquires the updated SIB1 without performing OD-SIB1 REQ procedure.

This is later captured in the RRC running CR as follows [2]:

<Omitted text> 2> else if SIB1 acquisition is required for the UE and ssb-SubcarrierOffset indicates that SIB1 is not scheduled in the cell: 3> if the UE has a stored valid version of od-sib1-Config for this cell as specified in clause 5.2.2.4.2x: 4> if the UE is in RRC_IDLE or in RRC_INACTIVE; or 4> if the UE is in RRC_CONNECTED while T311 is running: 5> if the SIB1 acquisition is upon receiving an indication that the system information has changed or upon receiving a PWS notification: 6> acquire the SIB1 (see clause 5.2.2.2.2), which is scheduled as specified in TS 38.213 [13]; 5> else: 6> perform the actions as specified in clause 5.2.2.3.3x; 3> else: 4> perform the actions as specified in clause 5.2.2.5. <Omitted text>

From our understanding, a UE in RRC_CONNECTED state may also receive SI change or PWS notification in the OD-SIB1 cell. However, the current running CR stops the UE in RRC_CONNECTED state from acquiring SIB1 upon reception of SI change or PWS notification in the OD-SIB1 cell.

The current running CR results in the following unexpected situations when the RRC_CONNECTED UE receives SI change or PWS notification: 1) the RRC_CONNECTED UE can not acquire SIB1 upon receiving indication of SI change or PWS notification, and performs the actions as specified in clause 5.2.2.5 to bar the cell; 2) the UE in RRC_CONNECTED while T311 is running has little chance to receive indication of SI change or PWS notification, let alone acquiring the SIB1.

Observation 1: With RRC state restriction in the running CR, the RRC_CONNECTED UE can not acquire SIB1 upon receiving indication of SI change or PWS notification in the OD-SIB1 cell, and perform the actions as specified in clause 5.2.2.5 to bar the cell.

Therefore, we propose that the specification should remove the RRC state restriction before the UE acquires SIB1 upon reception of SI change and PWS notification in the OD-SIB1 cell, and adopt the TP1 in the Annex.

During ASN.1 review, RRC rapporteur responded to this issue, commenting that he/she thinks the previous RAN2 agreement ‘NW ensures that the RRC connected UE has the latest SIB1 (e.g. dedicated RRC message to deliver SIB1 or not configure searchSpaceSIB1), as baseline. UE understands that the stored SIB1 is the latest SIB1.’ implies the spec change is not needed. We have different understanding with this agreement. This agreement is for addressing the issue on the RRC_CONNECTED UE to obtain broadcasting status of OSI before OSI requesting procedure. By sending SI change notification, the NW also ensures that the RRC connected UE will have the latest SIB1. Thus, we think the spec change is necessary.

Proposal 1: [V500] Remove the RRC state restriction before the UE acquires SIB1 upon reception of SI change and PWS notification in the OD-SIB1 cell, and adopt the TP1 in the Annex 5.1.

[V503][J002][E023] Correction on absoluteFrequencySSB FD Timing reference

This is the field description of absoluteFrequencySSB [3]:

absoluteFrequencySSB Frequency of the SSB to be used for this serving cell. SSB related parameters (e.g. SSB index) provided for a serving cell refer to this SSB frequency unless mentioned otherwise. The CD-SSB of the PCell is always on the sync raster. Frequencies are considered to be on the sync raster if they are also identifiable with a GSCN value (see TS 38.101-1 [15] or TS 38.101-5 [75]). If the field is absent, the SSB related parameters should be absent, e.g. ssb-PositionsInBurst, ssb-periodicityServingCell and subcarrierSpacing in ServingCellConfigCommon IE. If the field is absent, the UE obtains timing reference from the intra-band SpCell or intra-band SCell if applicable as described in TS 38.213 [13], clause 4.1, or from the SpCell or an SCell indicated by referenceCell, or from the reference serving cell defined in TS 38.133 [14]. This is supported in case the SCell for which the UE obtains the timing reference is in the same or different frequency band as the cell (i.e. the SpCell or the SCell, respectively) from which the UE obtains the timing reference.

When the IE absoluteFrequencySSB is absent, the UE obtains timing reference from other SpCell or SCell if applicable according to the RRC specification. From our understanding, if the UE is configured with an SCell where od-ssb-config is present, the UE is expected to use the OD-SSB for timing reference, rather than relying on other cells for timing reference. Therefore, we think the field description absoluteFrequencySSB for should be updated.

In OD-SSB case#1, the UE is configured with od-ssb-config, but the absoluteFrequencySSB is absent. Even if the OD-SSB is configured but not activated yet, the UE does not need to take other cells for timing reference. Hence, the only case that needs spec modification here is when the UE is neither configured with absoluteFrequencySSB nor with od-ssb-config, then the UE obtains timing reference from other cells if applicable. In conclusion, we propose to adopt the TP2 in annex 5.2 for this issue.

Proposal 2: [V503][J002][E023] If absoluteFrequencySSB and od-ssb are both absent for the serving cell, the UE obtains timing reference from the intra-band SpCell or intra-band SCell if applicable. Adopt the TP2 in the Annex 5.2.

[X200][A103] SMTC setup for the activated OD-SSB or the adapted SSB

During ASN.1 review procedure, companies discussed about whether/how to polish the procedural text for SMTC setup for the activated OD-SSB/SSB adaptation. Relevant agreements in RAN2 are as below[4][5][6]:

RAN2#129(202502) 2. RAN2 preference is to keep SMTC based L3 RRM framework and to introduce additional SMTC configuration according to SSB adaptation for L3 RRM measurement on SCell with SSB adaptation. 3. For L3 measurement, RAN2 assumes the adapted SSB on neighbor cell is measured based on legacy SMTC. RAN2#129bis(202504) 5. For SSB adaptation, multiple additional SMTC configurations are configured to UE via RRC, together with the mapping between SSB burst periodicity and SMTC configuration. 6. Upon reception of DCI format 2_9 for SSB burst periodicity switching, UE selects one SMTC configuration accordingly. RAN2#130(202505) 7. The UE applies the OD-SSB specific SMTC when the OD-SSB is activated and SCell is activated. This decision does not impact RAN4 discussion whether both OD-SSB and AO-SSB can be measured.

The procedural text for SMTC setup for the activated OD-SSB/SSB adaptation is as below [2]:

Section 5.5.2.10 If smtcxlist is present, when OD-SSB is activated and the serving cell is activated, the UE shall setup SMTC according to the first configured field in smtcxlist for serving cell measurements on the corresponding configured measurement object as specified in 5.5.3.1, if the SS/PBCH block reception periodicity is configured as SSB periodicity of the first OD-SSB configuration for the serving cell; the UE shall setup SMTC according to the second SMTC in smtcx-list for measurements on the corresponding MeasObjectNR if the SS/PBCH block reception periodicity is indicated as the second SSB periodicity in od-ssb-Periodicity and so on. If smtcy-SSBAdapt is present, the UE shall setup SS/PBCH block measurement timing configuration (SMTC) according to smtc1 for serving cell measurements on the corresponding configured measurement object as specified in 5.5.3.1, if DCI format 2_9 with CRC scrambled by adaptSSBPeriodInd-RNTI is not received or the received DCI format 2_9 with CRC scrambled by adaptSSBPeriodInd-RNTI indicates the SS/PBCH block reception periodicity provided by ssb-periodicityServingCell; the UE shall setup SMTC according to the first SMTC in smtcy-SSBAdapt for measurements on the corresponding MeasObjectNR if the received DCI format 2_9 with CRC scrambled by adaptSSBPeriodInd-RNTI indicates the SS/PBCH block reception periodicity provided by the first adaptive SSB periodicity in adap-SSB-BurstPeriodicityList; the UE shall setup SMTC according to the second SMTC in smtcy-SSBAdapt for measurements on the corresponding MeasObjectNR if the received DCI format 2_9 with CRC scrambled by adaptSSBPeriodInd-RNTI indicates the SS/PBCH block reception periodicity provided by the second adaptive SSB periodicity in adap-SSB-BurstPeriodicityList.

We understand the key controversial point for this part is how to implement the text properly to reflect the previous RAN2 agreement, i.e. the periodicity of the applied SMTC maps with the periodicity of the activated OD-SSB or the adapted SSB. The problem with the running CR is that it uses the wording of ‘first’, ‘second’, which leads to some ambiguity when the reader tries to comprehend. Therefore, we propose that the procedural text to be revised a bit to clearly reflect what RAN2 has agreed.

Proposal 3: On the issue of SMTC setup for the activated OD-SSB or the adapted SSB, adopt the TP3 in Annex 5.3 to reflect that the periodicity of the SMTC to be set up is the same as the periodicity of the activated OD-SSB or the adapted SSB.

[X201] measurement using servingCellMO or servingCellMO-OD

During the Asn.1 review procedure, [X201] raised the issue to further polish the procedural text for RSRP/RSRQ/SINR measurement using servingCellMO or servingCellMO-OD. Yet companies still had diversed opinion on the proposed change. Some agreed, some thought the proposed change does not work well, and some others thought no further change is needed.

The key controversies about the procedural text change is from two aspects: 1) when to apply servingCellMO or servingCellMO-OD? 2) when measurement is based on SSB, whether the measurement comes from AO-SSB or OD-SSB?

To clarify the understanding, we checked the previous RAN2#130 agreement [5]:

5. L3 RRM framework: Case 1 - In L3 measurement in OD-SSB case 1, if MAC-CE/RRC based activation / deactivation: - • The UE starts L3 measurement towards the activated OD-SSB based on configured servingCellMO after reception of the activation - • The UE stops L3 measurements after it determines the OD-SSB is deactivated implicitly or explicitly. 6. L3 RRM framework: Case 2a (AO-SSB and OD-SSB in the same frequency) - When the AO-SSB and OD-SSB have the same frequency, they share the same servingCellMO. 7. L3 RRM framework: Case 2b (AO-SSB and OD-SSB in the different frequency) - RAN2 confirms the working consumption that when AO-SSB and OD-SSB have different center frequency, introduce a new servingCellMO in ServingCellConfig to indicate MO of OD-SSB, as agreement. - When OD-SSB is activated, UE uses servingCellMO-OD to measure serving cell; when OD-SSB is deactivated, UE uses servingCellMO-AO (i.e., legacy servingCellMO) to measure serving cell.

Based on the agreement, we understand that the only situation to apply servingCellMO-OD is that OD-SSB is activated for case 2b. Therefore, we understand the motivation of the proposed change raised by Xiaomi is basically correct after further check. On top of that, we provide a TP4 to improve accuracy and readability. We propose RAN2 to adopt it.

Observation 2: servingCellMO-OD is used only for OD-SSB case#2b when OD-SSB is activated. Otherwise, servingCellMO is used.

Proposal 4: For measurement using servingCellMO or servingCellMO-OD, adopt TP4 in Annex 5.4.

Conclusion

Based on the discussion, we have the following observation and proposals:

[V500] Correction on RRC state restriction for SIB1 reception when the UE receives SI change or PWS notification

Observation 1: With RRC state restriction in the running CR, the RRC_CONNECTED UE can not acquire SIB1 upon receiving indication of SI change or PWS notification in the OD-SIB1 cell, and perform the actions as specified in clause 5.2.2.5 to bar the cell.

Proposal 1: [V500] Remove the RRC state restriction before the UE acquires SIB1 upon reception of SI change and PWS notification in the OD-SIB1 cell, and adopt the TP1 in the Annex 5.1.

[V503][J002][E023] Correction on absoluteFrequencySSB FD Timing reference

Proposal 2: [V503][J002][E023] If absoluteFrequencySSB and od-ssb are both absent for the serving cell, the UE obtains timing reference from the intra-band SpCell or intra-band SCell if applicable. Adopt the TP2 in the Annex 5.2.

[X200][A103] SMTC setup for the activated OD-SSB or the adapted SSB

Proposal 3: On the issue of SMTC setup for the activated OD-SSB or the adapted SSB, adopt the TP3 in Annex 5.3 to reflect that the periodicity of the SMTC to be set up is the same as the periodicity of the activated OD-SSB or the adapted SSB.

[X201] measurement using servingCellMO or servingCellMO-OD

Observation 2: servingCellMO-OD is used only for OD-SSB case#2b when OD-SSB is activated. Otherwise, servingCellMO is used.

Proposal 4: For measurement using servingCellMO or servingCellMO-OD, adopt TP4 in Annex 5.4.

References

- R2-2506201	Report from session on R18 SL, R18/19 MOB, and R19 NES

- ASN.1 review file for NES

- TS 38.331, V18.6.0

- R2-2501331	Report from session on V2X/SL, R19 NES and MOB

- R2-2502981	Report from session on V2X/SL, R19 NES and MOB

- R2-2504671	Report from session on R18 SL, R18/19 MOB, and R19 NES

Annex

TP1 for proposal 1

- 5.2.2.3.1	Acquisition of MIB and SIB1

The UE shall:

1>	apply the specified BCCH configuration defined in 9.1.1.1;

1>	if the UE is in RRC_IDLE or in RRC_INACTIVE; or

1>	if the UE is in RRC_CONNECTED while T311 is running:

2>	acquire the MIB, which is scheduled as specified in TS 38.213 [13];

2>	if the UE is unable to acquire the MIB;

3>	perform the actions as specified in clause 5.2.2.5;

2>	else:

3>	perform the actions specified in clause 5.2.2.4.1.

1>	if the UE is in RRC_CONNECTED with an active BWP with common search space configured by searchSpaceSIB1 and pagingSearchSpace and has received an indication about change of system information; or

1>	if the UE is in RRC_CONNECTED with an active BWP with common search space configured by searchSpaceSIB1 and the UE has not stored a valid version of a SIB or posSIB, in accordance with clause 5.2.2.2.1, of one or several required SIB(s) or posSIB(s) in accordance with clause 5.2.2.1, and, UE has not acquired SIB1 in current modification period; or

1> if the UE is in RRC_CONNECTED with an active BWP with common search space configured by searchSpaceSIB1, and, the UE has not stored a valid version of a SIB or posSIB, in accordance with clause 5.2.2.2.1, of one or several required SIB(s) or posSIB(s) in accordance with clause 5.2.2.1, and, si-BroadcastStatus for the required SIB(s) or posSI-BroadcastStatus for the required posSIB(s) is set to notBroadcasting in acquired SIB1 in current modification period; or

1>	if the UE is in RRC_IDLE or in RRC_INACTIVE; or

1>	if the UE is in RRC_CONNECTED while T311 is running:

2>	if ssb-SubcarrierOffset indicates SIB1 is transmitted in the cell (TS 38.213 [13]) and if SIB1 acquisition is required for the UE:

3>	acquire the SIB1, which is scheduled as specified in TS 38.213 [13];

3>	if the UE is unable to acquire the SIB1:

4>	perform the actions as specified in clause 5.2.2.5;

3>	else:

4>	upon acquiring SIB1, perform the actions specified in clause 5.2.2.4.2.

2>	else if SIB1 acquisition is required for the UE and ssb-SubcarrierOffset indicates that SIB1 is not scheduled in the cell:

3>	if the UE has a stored valid version of od-sib1-Config for this cell as specified in clause 5.2.2.4.2x:

4> if the SIB1 acquisition is upon receiving an indication that the system information has changed or upon receiving a PWS notification:

5> acquire the SIB1 (see clause 5.2.2.2.2), which is scheduled as specified in TS 38.213 [13];

4> else:

5>	if the UE is in RRC_IDLE or in RRC_INACTIVE; or

5>	if the UE is in RRC_CONNECTED while T311 is running:

6>	perform the actions as specified in clause 5.2.2.3.3x;

3>	else:

4>	perform the actions as specified in clause 5.2.2.5.

NOTE 1:	The UE in RRC_CONNECTED is only required to acquire broadcasted SIB1 if the UE can acquire it without disrupting unicast or MBS multicast data reception, i.e., the broadcast and unicast/MBS multicast beams are quasi co-located. The UE in RRC_INACTIVE state while T319a is running, is only required to acquire broadcasted SIB1 and MIB if the UE can acquire them without disrupting unicast data reception, i.e. the broadcast and unicast beams are quasi co-located.

NOTE 2:	UE in RRC_INACTIVE that does not support inactiveStateNTN-r17 enters RRC_IDLE upon cell reselection between TN cell and NTN cell, and initiates the NAS signalling connection recovery (see TS 24.501 [23]).

NOTE x:	It is up to UE 's implementation to check if SIB1 is currently being broadcasted for a cell before triggering SIB1 request procedure of that cell.

TP2 for proposal 2

- 6.3.2	Radio resource control information elements

- –	FrequencyInfoDL

The IE FrequencyInfoDL provides basic parameters of a downlink carrier and transmission thereon.

FrequencyInfoDL information element

-- ASN1START

-- TAG-FREQUENCYINFODL-START

FrequencyInfoDL ::=                 SEQUENCE {

absoluteFrequencySSB                ARFCN-ValueNR                                                   OPTIONAL,   -- Cond SpCellAdd

frequencyBandList                   MultiFrequencyBandListNR,

absoluteFrequencyPointA             ARFCN-ValueNR,

scs-SpecificCarrierList             SEQUENCE (SIZE (1..maxSCSs)) OF SCS-SpecificCarrier,

...,

[[

referenceCell-r18                   ServCellIndex                                                   OPTIONAL    -- Cond SSBlessSCell

]]

}

-- TAG-FREQUENCYINFODL-STOP

-- ASN1STOP

FrequencyInfoDL field descriptions

absoluteFrequencyPointA Absolute frequency position of the reference resource block (Common RB 0). Its lowest subcarrier is also known as Point A (see TS 38.211 [16], clause 4.4.4.2). Note that the lower edge of the actual carrier is not defined by this field but rather in the scs-SpecificCarrierList.

absoluteFrequencySSB Frequency of the SSB to be used for this serving cell. SSB related parameters (e.g. SSB index) provided for a serving cell refer to this SSB frequency unless mentioned otherwise. The CD-SSB of the PCell is always on the sync raster. Frequencies are considered to be on the sync raster if they are also identifiable with a GSCN value (see TS 38.101-1 [15] or TS 38.101-5 [75]). If the field is absent, the SSB related parameters should be absent, e.g. ssb-PositionsInBurst, ssb-periodicityServingCell and subcarrierSpacing in ServingCellConfigCommon IE. If the field is absent and od-ssb is not configured for this serving cell, the UE obtains timing reference from the intra-band SpCell or intra-band SCell if applicable as described in TS 38.213 [13], clause 4.1, or from the SpCell or an SCell indicated by referenceCell, or from the reference serving cell defined in TS 38.133 [14]. This is supported in case the SCell for which the UE obtains the timing reference is in the same or different frequency band as the cell (i.e. the SpCell or the SCell, respectively) from which the UE obtains the timing reference. For PCell, this field corresponds to the CD-SSB.

frequencyBandList List containing only one frequency band to which this carrier(s) belongs. Multiple values are not supported.

referenceCell Indicates the reference cell, i.e. the cell which provides the timing reference and AGC source for the inter-band SSB-less SCell. If the reference cell is an SCell or PSCell, it should be an activated SCell or activated PSCell. If this field is absent for an inter-band SSB-less SCell, the reference serving cell is defined in TS 38.133 [14].

scs-SpecificCarrierList A set of carriers for different subcarrier spacings (numerologies). Defined in relation to Point A. The network configures a scs-SpecificCarrier at least for each numerology (SCS) that is used e.g. in a BWP (see TS 38.211 [16], clause 5.3).

| Conditional Presence | Explanation |
| --- | --- |
| SpCellAdd | The field is mandatory present if this FrequencyInfoDL is for SpCell. Otherwise the field is optionally present, Need S. |
| SSBlessSCell | The field is optionally present, Need S, if the inter-band SSB-less SCell is configured for this cell and absoluteFrequencySSB is absent. It is absent otherwise, Need R. |

TP3 for proposal 3

- 5.5.2.10	Reference signal measurement timing configuration

The UE shall setup the first SS/PBCH block measurement timing configuration (SMTC) in accordance with the received periodicityAndOffset parameter (providing Periodicity and Offset value for the following condition) in the SSB-MTC configuration. The first subframe of each SMTC occasion occurs at an SFN and subframe of the NR SpCell meeting the following condition:

SFN mod T = (FLOOR (Offset/10));

if the Periodicity is larger than sf5:

subframe = Offset mod 10;

else:

subframe = Offset or (Offset +5);

with T = CEIL(Periodicity/10).

If smtc2 is present, for cells indicated in the pci-List parameter in smtc2 in the same MeasObjectNR, the UE shall setup an additional SS/PBCH block measurement timing configuration (SMTC) in accordance with the received periodicity parameter in the smtc2 configuration and use the Offset (derived from parameter periodicityAndOffset) and duration parameter from the smtc1 configuration. The first subframe of each SMTC occasion occurs at an SFN and subframe of the NR SpCell meeting the above condition.

If smtc2-LP is present, for cells indicated in the pci-List parameter in smtc2-LP in the same frequency (for intra frequency cell reselection) or different frequency (for inter frequency cell reselection), the UE shall setup an additional SS/PBCH block measurement timing configuration (SMTC) in accordance with the received periodicity parameter in the smtc2-LP configuration and use the Offset (derived from parameter periodicityAndOffset) and duration parameter from the smtc configuration for that frequency. The first subframe of each SMTC occasion occurs at an SFN and subframe of the NR SpCell or serving cell (for cell reselection) meeting the above condition.

If smtc3list is present, for cells indicated in the pci-List parameter in each SSB-MTC3 element of the list in the same MeasObjectNR, the IAB-MT shall setup an additional SS block measurement timing configuration in accordance with the received periodicityAndOffset parameter (using same condition as smtc1 to identify the SFN and the subframe for SMTC occasion) in each SSB-MTC3 configuration and use the duration and ssb-ToMeasure parameters from each SSB-MTC3 configuration.

If smtc4list is present, for cells indicated in the pci-List parameter in each SSB-MTC4 element of the list in the same MeasObjectNR, the UE shall setup an additional SS/PBCH block measurement timing configuration (SMTC) in accordance with the received offset parameter in each SSB-MTC4 configuration and use the duration parameter and periodicity (derived from parameter periodicityAndOffset) from the smtc1 configuration. The first subframe of each SMTC occasion occurs at an SFN and subframe of the NR serving cell meeting the above condition.

If smtcxlist is present, when OD-SSB is activated and the serving cell is activated, the UE shall setup an additional SS/PBCH block measurement timing configuration (SMTC) in smtcxlist for serving cell measurements on the corresponding configured measurement object as specified in 5.5.3.1, if the periodicity indicated by periodicityAndOffset of this SMTC is the same as the od-ssb-Periodicity of the OD-SSB configuration of the activated OD-SSB for the serving cell.

If smtcy-SSBAdapt is present, the UE shall setup SS/PBCH block measurement timing configuration (SMTC) according to smtc1 for serving cell measurements on the corresponding configured measurement object as specified in 5.5.3.1, if DCI format 2_9 with CRC scrambled by adaptSSBPeriodInd-RNTI is not received or the received DCI format 2_9 with CRC scrambled by adaptSSBPeriodInd-RNTI indicates the SS/PBCH block reception periodicity provided by ssb-periodicityServingCell; the UE shall setup an SMTC in smtcy-SSBAdapt for measurements on the corresponding MeasObjectNR, if the periodicity indicated by periodicityAndOffset of this SMTC is the same as the SS/PBCH block reception periodicity indicated by the received DCI format 2_9 with CRC scrambled by adaptSSBPeriodInd-RNTI.

On the indicated ssbFrequency, the UE shall not consider SS/PBCH block transmission in subframes outside the SMTC occasion for RRM measurements based on SS/PBCH blocks and for RRM measurements based on CSI-RS except for SFTD measurement (see TS 38.133 [14], clause 9.3.8).

TP4 for proposal 4

- 5.5.3.1	General

<irrelevant text omitted>

The UE shall:

1>	whenever the UE has a measConfig, perform RSRP and RSRQ measurements for each serving cell for which servingCellMO is configured as follows:

2> if absoluteFrequencySSB is configured while od-ssb is absent in ServingCellConfigCommon; or //AO-SSB only

2> if absoluteFrequencySSB and od-ssb are both configured in ServingCellConfigCommon, but OD-SSB transmission is deactivated for the serving cell; or // OD-SSB is deactivated in case#2a and case#2b

2> if absoluteFrequencySSB and od-ssb are both configured in ServingCellConfigCommon, and OD-SSB transmission is activated where the od-ssb-absoluteFrequency of od-ssb is absent or the same as absoluteFrequencySSB for the serving cell; or // OD-SSB is activated in case#2a

2> if absoluteFrequencySSB is absent while od-ssb is configured in ServingCellConfigCommon, and OD-SSB transmission is activated for the serving cell; or // OD-SSB is activated in case#1

3>	if the reportConfig associated with at least one measId included in the measIdList within VarMeasConfig contains an rsType set to ssb and ssb-ConfigMobility is configured in the measObject indicated by the servingCellMO:

4>	if the reportConfig contains a reportQuantityRS-Indexes and maxNrofRS-IndexesToReport:

5>	derive layer 3 filtered RSRP and RSRQ per beam for the serving cell based on SS/PBCH block, as described in 5.5.3.3a;

4>	derive serving cell measurement results based on SS/PBCH block, as described in 5.5.3.3;

3>	if the reportConfig associated with at least one measId included in the measIdList within VarMeasConfig contains an rsType set to csi-rs and CSI-RS-ResourceConfigMobility is configured in the measObject indicated by the servingCellMO:

4>	if the reportConfig contains a reportQuantityRS-Indexes and maxNrofRS-IndexesToReport:

5>	derive layer 3 filtered RSRP and RSRQ per beam for the serving cell based on CSI-RS, as described in 5.5.3.3a;

4>	derive serving cell measurement results based on CSI-RS, as described in 5.5.3.3;

2>	if absoluteFrequencySSB and od-ssb are both configured in ServingCellConfigCommon, and OD-SSB transmission is activated where the od-ssb-absoluteFrequency of od-ssb is different from absoluteFrequencySSB for the serving cell: // OD-SSB is activated in case#2b

3>	if the reportConfig associated with at least one measId included in the measIdList within VarMeasConfig contains an rsType set to ssb and ssb-ConfigMobility is configured in the measObject indicated by the servingCellMO-OD:

4>	if the reportConfig associated with at least one measId included in the measIdList within VarMeasConfig contains a reportQuantityRS-Indexes and maxNrofRS-IndexesToReport and contains an rsType set to ssb:

5>	derive layer 3 filtered RSRP and RSRQ per beam for the serving cell based on SS/PBCH block, as described in 5.5.3.3a;

4>	derive serving cell measurement results based on SS/PBCH block, as described in 5.5.3.3;

3>	if the reportConfig associated with at least one measId included in the measIdList within VarMeasConfig contains an rsType set to csi-rs and CSI-RS-ResourceConfigMobility is configured in the measObject indicated by the servingCellMO-OD:

4> if the reportConfig associated with at least one measId included in the measIdList within VarMeasConfig contains a reportQuantityRS-Indexes and maxNrofRS-IndexesToReport and contains an rsType set to csi-rs:

5> derive layer 3 filtered RSRP and RSRQ per beam for the serving cell based on CSI-RS, as described in 5.5.3.3a;

4> derive serving cell measurement results based on CSI-RS, as described in 5.5.3.3;

1>	for each serving cell for which servingCellMO is configured, if the reportConfig associated with at least one measId included in the measIdList within VarMeasConfig contains SINR as trigger quantity and/or reporting quantity:

2> if absoluteFrequencySSB is configured while od-ssb is absent in ServingCellConfigCommon; or

2> if absoluteFrequencySSB and od-ssb are both configured in ServingCellConfigCommon, but OD-SSB transmission is deactivated for the serving cell; or

2> if absoluteFrequencySSB and od-ssb are both configured in ServingCellConfigCommon, and OD-SSB transmission is activated where the od-ssb-absoluteFrequency of od-ssb is absent or the same as absoluteFrequencySSB for the serving cell; or

2> if absoluteFrequencySSB is absent while od-ssb is configured in ServingCellConfigCommon, and OD-SSB transmission is activated for the serving cell; or

3>	if the reportConfig contains rsType set to ssb and ssb-ConfigMobility is configured in the servingCellMO:

4>	if the reportConfig contains a reportQuantityRS-Indexes and maxNrofRS-IndexesToReport:

5>	derive layer 3 filtered SINR per beam for the serving cell based on SS/PBCH block, as described in 5.5.3.3a;

4>	derive serving cell SINR based on SS/PBCH block, as described in 5.5.3.3;

3>	if the reportConfig contains rsType set to csi-rs and CSI-RS-ResourceConfigMobility is configured in the servingCellMO:

4>	if the reportConfig contains a reportQuantityRS-Indexes and maxNrofRS-IndexesToReport:

5>	derive layer 3 filtered SINR per beam for the serving cell based on CSI-RS, as described in 5.5.3.3a;

4>	derive serving cell SINR based on CSI-RS, as described in 5.5.3.3;

2>	if absoluteFrequencySSB and od-ssb are both configured in ServingCellConfigCommon, and OD-SSB transmission is activated where the od-ssb-absoluteFrequency of od-ssb is different from absoluteFrequencySSB for the serving cell:

3>	if the reportConfig contains rsType set to ssb and ssb-ConfigMobility is configured in the servingCellMO-OD:

4>	if the reportConfig contains a reportQuantityRS-Indexes and maxNrofRS-IndexesToReport:

5>	derive layer 3 filtered SINR per beam for the serving cell based on SS/PBCH block, as described in 5.5.3.3a;

4>	derive serving cell SINR based on SS/PBCH block, as described in 5.5.3.3;

3>  if the reportConfig contains rsType set to csi-rs and CSI-RS-ResourceConfigMobility is configured in the servingCellMO-OD:

4>	if the reportConfig contains a reportQuantityRS-Indexes and maxNrofRS-IndexesToReport:

5>	derive layer 3 filtered SINR per beam for the serving cell based on CSI-RS, as described in 5.5.3.3a;

4>	derive serving cell SINR based on CSI-RS, as described in 5.5.3.3;

<irrelevant text omitted>