3GPP TSG RAN WG2 Meeting #131bis	R2-2506985

Prague, Czech Republic, Oct 13th – 17th, 2025

Agenda Item:	9.2.2

Source:	Xiaomi

Title:	Support of Topology 2 for A-IoT

Document for:	Discussion and Decision

Introduction

This contribution discusses the support of Topology 2 for A-IoT, based on the related objective in Rel-20 A-IoT phase 2 WID [1].

Discussion

General architecture and protocol stack for Topology 2

According to the latest RAN WID [1], only the RRC-based option for the topology 2 support is within the scope. Regarding the RRC-based option, it is specified in SA WID for AIoT Ph2 [2] that the Rel-19 interim conclusions in TS 23-700-13 [3] need be followed and taken as the basis for the Rel-20 work on topology 2 in SA WGs:

WT#1: Study the support of AIoT services in Topology 2, including: - Study and investigate how to support AIoT services under the RRC-based option for UE Reader connectivity. NOTE 3: Rel-19 TR solution and interim conclusion should be used as basis for rel-20 work. Solution submitted to rel-20 need to align with rel-19 TR interim conclusion. No rel-19 AIoT Device impact from this WT is expected.

The Rel-19 TR interim conclusion on the architecture and protocol stack of the RRC based option is cited as follows in Table 1 [3]. Note that the protocol stack in Figure 8.1.3.3-1 below is based on an architecture that is supported with the indirect connectivity between gNB and AIOTF, relayed by the AMF, and it is in line with the architectural assumption further agreed by SA2 in [4] (i.e. “-	Only Indirect Connectivity (i.e. NG-RAN <-> AMF <-> AIOTF) is applied.”).

Table 1. TR Interim conclusion on RRC-based Protocol stack in SA2 [3]

#### 8.1.3.3 RRC option The following principles apply: - Messages between the UE Reader and the AIoTF are delivered using RRC between UE and RRC and NGAP between gNB and AMF, and using an SBI interface between AMF and AIoTF. The related protocol stack is shown in Figure 8.1.3.3-1. Editor's note: How addressing works for UL for Option B is FFS. Editor's note: Further details how the RRC option works are FFS. Figure 8.1.3.3-1: Protocol Stack for the RRC option

Since discussions on some RAN2 features in topology 2 are likely to have dependency on the overall architecture and protocol stack design by SA2, RAN2 should confirm the protocol stack for RRC based option in SA2 TR as the baseline for further RAN2 discussion.

As some further details, it is seen that in above Figure 8.1.3.3-1 in Table 1, there is the end-to-end protocol layer between the AIoTF and the gNB, i.e. the “AIoT Reader control” shown in the figure. As a further clarification, such “AIoT Reader control” should correspond to the “NGAP AIoT information” in the below protocol stack with indirect connectivity via AMF in Rel-19 Topology 1 [5] (as cited in Table 2), where gNB and AIOTF exchanges the “NGAP AIoT information” that is relayed by AMF over N2 and AIOT3 interfaces.

Table 2. Rel-19 Topology 1 protocol stack wit indirect connectivity via AMF [5]

Legend: - AIoT Data: It is the application data exchanged between the AIoT Device and the AF. - AIoT NAS: The NAS protocol between AIoT Device and the AIOTF. - AIoT AS: It is between the AIoT Device and the NG-RAN as specified in TS 38.300 [5]. - NGAP: Application Layer Protocol between the AMF and the NG-RAN as defined in TS 38.413 [10]. - NGAP AIoT Information: It is the subset of NGAP information that the AMF transparently relays between the NG-RAN and the AIOTF, and is included in the NGAP messages and the AIOT3 messages. Figure 4.6.2.3-1: Protocol Stack between AF and AIoT Device for Indirect Connectivity via AMF option

Based on the above discussions, it is proposed that RAN2 takes the Topology 2 protocol stack in SA2 TR interim conclusion as the baseline for further discussion, with the clarification on the NGAP A-IoT information supported between the gNB and AIOTF.

Proposal 1: RAN2 takes the architecture and protocol stack for RRC-based option in TR 23.700-13, Figure 8.1.3.3-1 as the baseline for further RAN higher layer discussion on Topology 2, with the NGAP A-IoT information exchanged between gNB and AIOTF, transparent to AMF (same as Topology 1).

In the RRC-based option for Topology 2, NGAP AIoT information from the A-IoT CN is directly visible to the gNB. Further considering that the service request for a DT/DO-DTT service is informed to the NG-RAN by NGAP A-IoT information, it can also be concluded that in Topology 2 an A-IoT service procedure is triggered at the gNB by the service request from the CN, not by the request from the UE reader. This actually addresses the issue during Rel-19 RAN WG SI phase on whether an A-IoT service procedure and corresponding radio resource configuration at the gNB is triggered by A-IoT CN or by the request from UE.

Proposal 2: RAN2 confirms that the A-IoT operation from A-IoT CN is visible to the gNB (i.e. via NGAP A-IoT information). At least for a DT/DO-DTT service, the A-IoT service procedure and the corresponding radio resource configuration (on Uu and A-IoT radio interface) are triggered at the gNB by the service request from the CN, not by the request from the UE reader.

With respect to the A-IoT radio interface in Topology 2, RAN2 can further confirm that only the A-IoT MAC and A-IoT PHY are located between the device and UE reader. RAN2 can also confirm that the signalling content and functionalities of these two protocols in Rel-19 are reused as the baseline (unless enhancements are identified as needed). This is actually in line with spirit of the agreement in Rel-19 study item phase “A-IoT air interface in topology 1 between A-IoT device and reader is fully reused in topology 2, i.e. topology is transparent to the A-IoT device and there is no impact on A-IoT device” [6].

In addition, the below architectural assumption was agreed by SA2 in [4] for Rel-20 AIoT phase 2 study. Based on this, SA2 actually aims at a Topology agnostic design from the device’s perspective, so that Topology 2 should supports a backward compatible design in the A-IoT air interface and can support A-IoT operations for legacy Rel-19 devices as well. This should also be the design target for the Topology 2 design in RAN2, even if Topology 2 specific enhancements on A-IoT MAC/PHY layer are to be introduced as discussed in later sections.

- 4.1 Architectural Assumptions The following traffic types for AIoT Device are to be studied: - DT: Device-terminated; - DO-DTT: Device-originated - device-terminated triggered; and - DO-A: Device-originated - autonomous. Common architectural assumptions: - Architecture reference models defined in TS 23.369 [3] are used as the baseline architecture for this study. - The AIoT Device is agnostic to topologies, i.e. Topology 1 and Topology 2. Architectural assumptions for support of Topology 2 for Key Issue#1: - No Rel-19 AIoT Device impact is expected. - The following topology as defined in TR 38.848 [10] is assumed: - BS <--> intermediate node <--> Ambient IoT Device: Only a UE can act as an intermediate node which is under the network control - Only Indirect Connectivity (i.e. NG-RAN <-> AMF <-> AIOTF) is applied. - The support of Topology 2 in this study is based on the RRC-based option and the interim conclusions of Rel-19 in TR 23.700-13 [7] is the baseline. Architectural assumptions for support of DO-A Capable AIoT Devices: - The AIoT Device is agnostic to Topology 1 architecture types, i.e. Direct Connectivity architecture and Indirect Connectivity architecture. NOTE 1: Coordination with RAN WGs is required to determine the AIoT Device capabilities in relation to system level of functionality (considering e.g. traffic scenarios, connectivity topologies, etc.). NOTE 2: The security aspects for Ambient IoT require coordination with SA WG3. NOTE 3: The charging aspects for Ambient IoT will be studied by SA WG5.

As per above discussions, the following two proposals are given w.r.t. the A-IoT radio interface and backward compatibility in Topology 2:

Proposal 3: RAN2 confirms that the Rel-19 A-IoT radio interface protocol stack is reused between the device and the UE reader, i.e. only A-IoT MAC and A-IoT PHY protocol layers are located in the A-IoT radio interface.

Proposal 3a: RAN2 confirms that Topology 2 aims at a backward compatible design in A-IoT radio interface, and can support the DT and DO-DTT services for the legacy Rel-19 devices (i.e. inventory and inventory + command) as well.

Authorization, topology selection and UE reader selection

In Rel-19 RAN WG SI discussion, the TR conclusion to the UE reader authorization was as follows: [6]

### 6.3.6 Topology 2 aspects on the interface between UE reader and RAN For Topology 2, the architecture/protocol stack options in [151] are studied (also corresponding to the studies in clause 6.4.2.1), while no new AS layer architecture/protocol stack options will be studied: - RRC based solution: A-IoT upper layer information is explicitly forwarded via UE reader’s NR Uu RRC message. - NAS based solution: A-IoT upper layer information is transmitted over UE reader's NAS message. - UP based solution: A-IoT upper layer information is transmitted as UE reader's user plane data. It is assumed that the intermediate UE authorization is performed by upper layers, according to [151] and [152]. It is not studied, from RAN2 signalling perspective, the scenario to support bistatic mode of operation (e.g., D2T2-A1).

It is true that the detailed procedure and signalling design for UE reader authorization is up to SA2 and RAN3. However, no matter whether the UE reader selection is eventually performed by A-IoT CN or by the gNB, the awareness of which UEs are authorized UE readers is the prerequisite for the UE reader selection, meaning that the design on the UE reader authorization procedure have potential impacts on how/by whom the UE reader selection is eventually performed.

The (interim) TR conclusions related to UE reader authorization in SA2 Rel-19 study are cited as follows: [3]

### 8.1.3 Architecture to Support Topology 2 #### 8.1.3.1 General The following principles apply: […] - Subscription aspects: - The UE subscription in the UDM will be extended with UE Reader subscription information, which consists of the following: - information indicating whether the UE is allowed to operate as a UE Reader. Editor’s note: Additional subscription information, e.g. validity information, for the UE Reader is FFS. - UE Reader subscription information is available to AMF and AIoTF. - If AMF receives, as part of the subscription information, the indication that the UE is authorized to operate as a UE Reader, then AMF informs NG-RAN that the UE is authorized to operate as a UE Reader. Editor’s note: Whether and how to enable authorization to the UE is FFS. - Radio resource management for UE Reader operation: - If the gNB has received the indication that a UE is authorized to operate as a UE Reader, then the gNB may assign radio resources to the UE for UE Reader operation. - The UE Reader is assumed to request radio resources for Reader operation (e.g. taking into account assistance information received from the AIoTF) from the gNB. NOTE 1: Assistance information is further clarified in other clauses. NOTE 2: The details of UE Reader radio resource management will be defined by RAN WG2. […] #### 8.1.3.4 Interim Conclusion for authorization The following interim conclusion are agreed for the UE Reader authorization in Topology 2: - The subscription data of UE Reader includes the indication that whether the UE is allowed to act as an AIoT Reader. - The AMF provides the UE Reader authorization indication to the NG-RAN serving the UE Reader. Editor's note: How to authorize the UE Reader will be determined later.

Since only the RRC_CONNECTED UE readers are considered in Rel-20 Topology 2 support in RAN [1], based on above TR conclusions from SA2, at least the A-IoT CN (i.e. AMF and AIOTF) can be aware of the authorized UE readers in RRC_CONNECTED that is served by a gNB. In addition, SA2 also concluded in the interim conclusion that the AMF provides the UE reader authorization indication to the NG-RAN, which further enables the gNB to get aware of which RRC_CONNECTED UEs it serves are the authorized UE readers for selection.

It may not be possible to conclude right away that the UE authorization information will definitely be signalled to the gNB, as this depends on further coordination between SA2 and RAN3 on whether it is really needed for the purpose of e.g. UE reader selection, radio resource configuration, etc. However, above SA2 TR (interim) conclusion at least means that the indication of UE authorization information to the gNB is feasible from SA2’s perspective. Therefore, to facilitate further discussion on the related functionalities, it is proposed that RAN2 makes the assumption on the UE authorization related aspects as follows.

Proposal 4: RAN2 assumes that both the A-IoT CN and the gNB can be made aware of which RRC_CONNECTED UEs served by the gNB are the UE readers authorized for Topology 2.

Besides UE reader selection, the selection of Topology (Topology 1 vs. Topology 2) to be applied for an A-IoT service also needs to be considered, because selection of UE readers happens only when Topology 2 is determined to be applied. That says, the Topology selection and UE reader selection need be considered together based on UE reader authorization information.

Then, for a DT/DO-DTT service triggered by A-IoT CN, the Topology selection (i.e. Topology 1 vs. 2) and further UE reader selection (in case of Topology 2 selected) need to be performed either by the A-IoT CN (i.e. AIOTF) or by the gNB, based on the authorized UE readers available. These two alternatives are illustrated as follows with some brief flow charts.

- [Alternative 1]: A-IoT CN performed Topology selection and UE reader selection

0. The UE authorization procedure is performed among the A-IoT CN, gNB and an RRC_CONNECTED UE reader served by the gNB.

It is assumed that the AIOTF in A-IoT CN can be made aware of the authorized UE readers in RRC_CONNECTED, if needed (with details up to RAN3/SA2).

1. Upon the trigger of the A-IoT service request for a DT/DO-DTT service, A-IoT CN (e.g. AIOTF) selects the gNB, the specific Topology and the UE reader (in the case of Topology 2 selected), and it signals the service request, along with the selected Topology and UE reader indicated, to the selected gNB.

The A-IoT CN may perform the gNB selection considering the CN-related factors for Topology 1 in Rel-19 (e.g. Area info, RAN reader list, etc.), and may further consider the Topology currently supported by each gNB. Signalling details between gNB and A-IoT CN is up to RAN3.

2. Upon reception of the A-IoT service request, the gNB performs the resource configuration for Uu and A-IoT radio interface and signals the configurations to the selected UE reader, in case Topology 2 is indicated.

3. Device, UE reader and gNB perform the A-IoT operations based on the RRC based operations in A-IoT radio interface and Uu interface, and the gNB forwards the A-IoT data from/to the A-IoT CN.

- [Alternative 2]: gNB performed Topology selection and UE reader selection

0. The UE authorization procedure is performed among the A-IoT CN, gNB and an RRC_CONNECTED UE reader served by the BS.

As per the interim conclusion from SA2, a gNB can be informed of the authorized UE readers by the A-IoT CN, and it knows whether they are in RRC_CONNECTED.

1. Upon the trigger of the A-IoT service request for a DT/DO-DTT service, A-IoT CN (e.g. AIOTF) selects the gNB, and signals the service request.

Similar to Alternative 1 above, in this step the gNB selection can be performed in a similar manner for Topology 1 or further considers the Topology supported by each gNB.

2. Upon reception of the A-IoT service request, the gNB selects the specific Topology to support the A-IoT procedure and selects the UE reader in the case that Topology 2 is determined. It also signals the selected UE reader, if applied, in the response to the A-IoT CN.

2a. In the case that the UE reader is selected, gNB performs the resource configuration for Uu and A-IoT radio interface and signals the configurations to the selected UE reader.

3. Device, UE reader and gNB perform the A-IoT operations based on the RRC based operations in A-IoT radio interface and Uu interface, and the gNB forwards the A-IoT data from/to the A-IoT CN.

Comparing the above two Alternatives, it can be seen that the Alternative 2 further enables additional considerations on some radio related factors by the gNB during the UE reader selection, e.g. radio link quality, mobility state, etc. of each UE reader it serves. Actually, as per the scope in the WID [1], the handover and RLF related scenarios are to be supported for Topology 2, which is typically based on the assumption that the UE reader in Topology 2 is not necessarily a “static-form” UE and so the UE reader’s mobility along with the impacts to its Uu radio link condition needs to be fully considered. For example, when a UE reader moves to the edge of the gNB’s coverage, it thus may not be an appropriate UE reader any longer.

Observation 1: As per the WID, support of UE reader mobility is within the scope of Topology 2 work (i.e. handover and RLF). A UE reader may move to the edge of the gNB/cell coverage, and thus may not be an appropriate UE reader any longer.

Based on Observation 1 and the scope in WID, we prefer the gNB performed Topology and UE reader selection mechanism as in above Alternative 2. With details on the specific procedure and signalling design able to be left to future meetings, we propose that RAN2 agrees the general principle that there are radio/AS related reasons (e.g. UE reader Uu link quality) for the gNB to be involved in the Topology selection and UE reader selection, where the Specific radio related impacts to Topology and UE reader selection can be FFS.

Proposal 5: RAN2 confirms that there are radio/AS related reasons (e.g. UE reader’s Uu link quality) for the gNB to be involved in the Topology selection and UE reader selection. FFS on how the gNB is involved and the specific radio related impacts to the Topology and UE reader selection.

Detailed signalling exchange of upper layer information between the A-IoT CN and gNB for Topology and UE reader selection (e.g. UE reader ID, authorization related info, localizing info., etc.) is up to RAN3 and SA2, and RAN2 may depend on them for corresponding design.

Proposal 6: Detailed signalling on the upper layer information transfer between the A-IoT CN and gNB for Topology and UE reader selection is up to RAN3 and SA2.

Resource allocation for A-IoT air interface in Topology 2

During the Rel-19 study item phase, there were following two main directions proposed for resource allocation on A-IoT air interface in Topology 2:

- Direction 1: gNB directly allocates the specific A-IoT resource used for each R2D or D2R transmission (either via dynamic scheduling or configured-grant like scheduling).

- Direction 2: gNB configures a set of A-IoT resources for the UE reader which further selects/assigns the A-IoT resource actually used for each specific R2D or D2R transmission.

In TR conclusion of the Rel-19 Topology 2 study in RAN2, only the resource allocation mechanism based on the “semi-statically” allocated A-IoT radio resource by the network is recorded, and there is the following agreement on the dedicated resource configuration to the UE reader, which implies a set of A-IoT resources will be configured for the UE reader as in the Direction 2 above [7].

Agreements

- Dedicated resource configuration is only given to the UE reader via dedicated signalling. Mechanisms for shared resource pool amongst readers are not considered in this release.

From our perspective, the resource allocation in A-IoT radio interface should follow the way as in above Direction 2 with the following reasons. For direction 1, either it results in significant signaling overhead on the Uu interface and introduce additional latency to request the scheduling, if dynamically scheduling is applied; or it leads to resource under-utilization to accommodate the A-IoT data transmission, if configured-grant like scheduling is applied. By contrast, Direction 2 can reduce the resource allocation signaling between the network and the UE reader, and also provides more flexibility for the UE reader, improving the resource utilization, since the UE reader can select/schedule the R2D/D2R resource when a transmission is really triggered.

More specifically, in Direction 2, the gNB configures a set of A-IoT resource to the UE reader via dedicated signaling. Then, based on the gNB configured A-IoT radio resources:

- At the UE reader side

For a R2D transmission, the UE reader can autonomously select the resource to be used. For a D2R message, other than Msg1, UE reader autonomous selects the D2R resource and signals the selected resource to the device based on the R2D message.

- At the device side

For Msg.1, the device randomly selects the resources for the specific Msg.1 transmission following legacy Rel-19 CBRA procedure, based on the AO related information which is controlled by and signalled from the UE reader (e.g. in A-IoT paging). For other D2R transmission, it follows the D2R scheduling received for the UE reader.

It is proposed that RAN2 confirms the above principles for resource allocation in Topology 2. The Uu signaling details on how the configuration of A-IoT radio resources is indicated to the UE reader can be left for further discussed in future meetings.

Proposal 7: In Topology 2, gNB configures a set of A-IoT radio resources to the UE reader via dedicated signaling. Based on the configured A-IoT radio resources:

- UE reader autonomously selects the resource for each specific R2D/D2R transmission (other than Msg1) in the configured set of resources, and signals the selected D2R resource to the device via R2D message.

- Device selects the resources for a Msg1 transmission following legacy Rel-19 CBRA procedure, based on the AO related information controlled/signaled by the UE reader (e.g. in A-IoT paging message), whereas for other D2R transmissions it uses the D2R resources scheduled by the UE reader.

Proposal 8: FFS on the Uu signaling details on how the configuration of A-IoT radio resources is indicated in Topology 2.

Another issue which was discussed but not concluded in Rel-19 study item phase was whether the A-IoT resource configuration is triggered by the request from the A-IoT CN or from the UE, corresponding to the two alternatives in the RAN TR conclusion [6]:

For the radio resources allocation request, following alternatives are studied: - Alternative 1: BS configures/allocates a set A-IoT radio resource to the UE reader, in response to a request from the UE reader; and/or - Alternative 2: BS configures/allocates a set A-IoT radio resource to the UE reader, based on the service request from CN.

Based on Proposal 1 and related analyses, the A-IoT service procedure in the RRC-based option is triggered by the service request from the A-IoT CN for a DT/DO-DTT service, so the A-IoT resource configuration should also be triggered in the same way.

Proposal 9: For a DT/DO-DTT service, gNB configures a set of A-IoT radio resources to the selected UE reader in Topology 2, upon the reception of a service request from the A-IoT CN.

Also, to avoid the inter-UE reader interference issues, no matter for R2D or D2R, it is proposed to confirm the agreement “Mechanisms for shared resource pool amongst readers are not considered in this release” made during Rel-19 study item phase [7].

Proposal 10: Do not support the configurations of shared A-IoT resources among different UE readers by the gNB. It is up to gNB implementation to avoid the resource overlapping.

Impact on Uu due to A-IoT paging and data transmission

In Topology 1, when a service is triggered by the A-IoT CN, it signals the service request related information to the gNB reader, which then interprets related information into the fields in the A-IoT paging message (e.g. Transaction ID, paging ID, etc.). Similarly, in Topology 2, since the NGAP AIoT information is only visible at the gNB, the gNB still needs to signal the A-IoT paging related information (e.g. paging ID, etc.) to the UE reader via the DL RRC signalling, upon reception of the service request. RAN2 should further discuss the details on what specific information related to A-IoT paging should be signalled in the RRC dedicated signalling.

Proposal 11: Upon reception of A-IoT service request, the gNB signals the A-IoT paging related information (e.g. paging ID, etc.) to the UE reader via RRC dedicated signaling. FFS what specific information related to A-IoT paging needs to be signaled in Uu.

Regarding A-IoT data transmission, in Topology 2 UE reader needs to relay the D2R data received from the device to the gNB in UL, and relay the A-IoT data received from the gNB in DL as the R2D data to the devices. As the A-IoT data is transparent to UE reader and the gNB in the RRC-based Topology 2 architecture, the D2R and R2D upper layer data should be embedded respectively in the UL and DL RRC messages as containers and transferred between the gNB and UE reader. This is proposed as follows:

Proposal 12: In Topology 2, the R2D and D2R upper layer data is embedded in the DL and UL RRC messages as containers respectively, and transferred over Uu between the gNB and UE reader.

Specifically for the UL A-IoT data forwarding, another issue that is worth looking into is whether:

- The UE reader can aggregate A-IoT data received from multiple devices (e.g. inventory response and/or command response) into the same UL RRC message sent to the gNB, or;

- The UE reader has to forward the A-IoT data from multiple devices one by one.

Basically, forwarding the data from multiple devices one by one may result in signaling storm (due to e.g., signaling to request Uu scheduling) between the reader and the gNB, whereas aggregation of multiple device’s data into the same UL RRC message would have significant saving on signaling overhead at the expense of some response latency.

Actually, in Topology 1, the Inventory Report message over NGAP has already supported the aggregation of multiple devices’ inventory response. This means that the latency caused by the aggregation would not be a critical issue at least for inventory procedure. On the other hand, compared to the A-IoT forwarding over NGAP, we care more about the radio resource consumption and efficiency in the Uu interface. From the UE reader perspective in Topology 2, it also seems unusual for a UE to generate quite a number of UL RRC messages within a short period of time (e.g. due to inventory of huge number of devices), with each containing only one piece for AIoT data from one device.

Observation 2: In Topology 2, UE reader forwarding the D2R data transmissions one by one may result in signaling storm in the Uu interface.

Observation 3: In Topology 1, it is already supported to aggregate inventory responses from multiple devices into the same Inventory Report message over NGAP.

Based on the above observations and analyses, we propose to support the D2R A-IoT data aggregation in the same UL RRC message. This is at least for the inventory response transfer, similar as the NGAP design in Topology 1.

Proposal 13: In Topology 2, UE reader can aggregate the received D2R data from multiple UEs in the same UL RRC message and forward them to the gNB together. This is at least supported for the upper layer data transfer of inventory response. FFS on the command response.

Impact on A-IoT MAC behaviour at UE reader

In the A-IoT radio interface, the reader in Topology 2 is now the UE reader, so some operations in A-IoT MAC layer which was originally left to gNB reader implementation in Topology 1 now turns to be the UE behaviour and thus may need to be specified alternatively. These operations/behaviours may include, e.g. reassembly for D2R reception, feedback and retransmission scheduling, generation of related R2D message and field setting, etc.

RAN2 may need to identify what reader behaviour which were left to gNB reader implementation for Topology 1 should now be specified in the A-IoT MAC layer at the UE reader in Topology 2.

Proposal 14: RAN2 identifies which reader behaviours which were left to gNB reader implementation in Topology 1 should be specified in the A-IoT MAC layer at the UE reader in Topology 2.

Support of mobility scenarios (i.e. RLF and handover) for Topology 2

As per the objective of Topology 2 work in [1], the enhancement to the UE readers during RLF and handover is also within the scope.

RAN2 - The objective includes specification of necessary Uu procedures to enable the A-IoT operations. - RRC CONNECTED UE reader only, including RLF and handover, without specific enhancement for other temporary out-of-connection scenarios. - A-IoT radio resource of the UE reader is only valid within the serving cell. - Resource allocation to the UE Reader shall be via Uu RRC signaling.

During the Rel-19 study item phase, there were related discussion for UE reader mobility in Topology 2 for which some TR conclusions on the validity of A-IoT resource configuration were written down [6].

The radio resources used by A-IoT radio interface between the A-IoT device(s) and UE reader are controlled by the network. The radio resources, which are dedicated for a UE reader, are only configured to the UE reader via dedicated signalling. The mechanisms for shared resource pool amongst UE readers are not considered in this release. The UE reader in coverage of BS scenario is supported. The UE reader may perform the A-IoT procedure on A-IoT radio interface between the reader and the device(s), only if the radio resource configuration is valid in the cell, which is under network control. The radio resource validity across multiple cells is not supported in this release. Hence, for all options below, the resources remain valid only in the same cell in which the resources were configured. […] For the case when the radio resources are allocated semi-statically by the network, the radio resource validities are studied in the following scenarios with candidate options (down-selection can be decided in the WI phase): […] - Scenario 2: In RRC_CONNECTED, the UE reader can perform A-IoT operations using the allocated A-IoT radio resource for A-IoT radio interface, upon its temporary out of connection scenarios (e.g., RLF and handover cases): - Option 1: The UE reader considers the resources as temporarily invalid during the temporary out of connection condition. The resource may become valid again after the UE recovers from the condition. - Option 2: The UE reader considers that the resources remain valid for a time period. It needs to be further discussed on whether this time period belongs to the one in Option 2 for Scenario 1 or is a different time period. It needs to be further discussed on whether the duration of this time period is related to the ongoing A-IoT operations.

Actually, for the UE reader mobility scenarios, the key issue to address is whether/how to ensure the A-IoT service continuity when the current selected UE reader is experiencing RLF and handover, and this involves in the aspects like whether to stop/suspend/resume the on-going services, how/whether to continue using the configured A-IoT resources during the mobility, etc.

From our perspective, it is proposed to consider such service continuity and discuss potential enhancements in RAN2 for the above two cases of UE reader mobility. One the one hand, the discussion should cover how to avoid/reduce AIoT data loss during these two mobility cases; one the other hand, following the TR conclusion that resource validity across multiple cells is not supported, the discussion should also consider avoiding/lowering the interference caused to the A-IoT operations in the cell the UE switches to (which is related to above “resource validity”).

Proposal 15: RAN2 discusses how to support A-IoT service continuity in the case of RLF or handover at the UE reader. The discussion should consider how to reduce/avoid AIoT data loss and lower/avoid the interference to A-IoT operations across the cells due to UE mobility.

Conclusion

In this contribution, RAN2 impacts to support of Topology 2 for A-IOT are discussed. Observations and proposals are listed as follows:

General architecture and protocol stack for Topology 2

Proposal 1: RAN2 takes the architecture and protocol stack for RRC-based option in TR 23.700-13, Figure 8.1.3.3-1 as the baseline for further RAN higher layer discussion on Topology 2, with the NGAP A-IoT information exchanged between gNB and AIOTF, transparent to AMF (same as Topology 1).

Proposal 2: RAN2 confirms that the A-IoT operation from A-IoT CN is visible to the gNB (i.e. via NGAP A-IoT information). At least for a DT/DO-DTT service, the A-IoT service procedure and the corresponding radio resource configuration (on Uu and A-IoT radio interface) are triggered at the gNB by the service request from the CN, not by the request from the UE reader.

Proposal 3: RAN2 confirms that the Rel-19 A-IoT radio interface protocol stack is reused between the device and the UE reader, i.e. only A-IoT MAC and A-IoT PHY protocol layers are located in the A-IoT radio interface.

Proposal 3a: RAN2 confirms that Topology 2 aims at a backward compatible design in A-IoT radio interface, and can support the DT and DO-DTT services for the legacy Rel-19 devices (i.e. inventory and inventory + command) as well.

Authorization, topology selection and UE reader selection

Proposal 4: RAN2 assumes that both the A-IoT CN and the gNB can be made aware of which RRC_CONNECTED UEs served by the gNB are the UE readers authorized for Topology 2.

Observation 1: As per the WID, support of UE reader mobility is within the scope of Topology 2 work (i.e. handover and RLF). A UE reader may move to the edge of the gNB/cell coverage, and thus may not be an appropriate UE reader any longer.

Proposal 5: RAN2 confirms that there are radio/AS related reasons (e.g. UE reader’s Uu link quality) for the gNB to be involved in the Topology selection and UE reader selection. FFS on how the gNB is involved and the specific radio related impacts to the Topology and UE reader selection.

Proposal 6: Detailed signalling on the upper layer information transfer between the A-IoT CN and gNB for Topology and UE reader selection is up to RAN3 and SA2.

Resource allocation for A-IoT air interface in Topology 2

Proposal 7: In Topology 2, gNB configures a set of A-IoT radio resources to the UE reader via dedicated signaling. Based on the configured A-IoT radio resources:

- UE reader autonomously selects the resource for each specific R2D/D2R transmission (other than Msg1) in the configured set of resources, and signals the selected D2R resource to the device via R2D message.

- Device selects the resources for a Msg1 transmission following legacy Rel-19 CBRA procedure, based on the AO related information controlled/signaled by the UE reader (e.g. in A-IoT paging message), whereas for other D2R transmissions it uses the D2R resources scheduled by the UE reader.

Proposal 8: FFS on the Uu signaling details on how the configuration of A-IoT radio resources is indicated in Topology 2.

Proposal 9: For a DT/DO-DTT service, gNB configures a set of A-IoT radio resources to the selected UE reader in Topology 2, upon the reception of a service request from the A-IoT CN.

Proposal 10: Do not support the configurations of shared A-IoT resources among different UE readers by the gNB. It is up to gNB implementation to avoid the resource overlapping.

Impact on Uu due to A-IoT paging and data transmission

Proposal 11: Upon reception of A-IoT service request, the gNB signals the A-IoT paging related information (e.g. paging ID, etc.) to the UE reader via RRC dedicated signaling. FFS what specific information related to A-IoT paging needs to be signaled in Uu.

Proposal 12: In Topology 2, the R2D and D2R upper layer data is embedded in the DL and UL RRC messages as containers respectively, and transferred over Uu between the gNB and UE reader.

Observation 2: In Topology 2, UE reader forwarding the D2R data transmissions one by one may result in signaling storm in the Uu interface.

Observation 3: In Topology 1, it is already supported to aggregate inventory responses from multiple devices into the same Inventory Report message over NGAP.

Proposal 13: In Topology 2, UE reader can aggregate the received D2R data from multiple UEs in the same UL RRC message and forward them to the gNB together. This is at least supported for the upper layer data transfer of inventory response. FFS on the command response.

Impact on A-IoT MAC behaviour at UE reader

Proposal 14: RAN2 identifies which reader behaviours which were left to gNB reader implementation in Topology 1 should be specified in the A-IoT MAC layer at the UE reader in Topology 2.

Support of mobility scenarios (i.e. RLF and handover) for Topology 2

Proposal 15: RAN2 discusses how to support A-IoT service continuity in the case of RLF or handover at the UE reader. The discussion should consider how to reduce/avoid AIoT data loss and lower/avoid the interference to A-IoT operations across the cells due to UE mobility.

Reference

[1] RP-252894	Revised WI: Solutions for Ambient IoT (Internet of Things) in NR Phase 2

[2] SP-250834	NEW SID on Architecture support of Ambient power-enabled Internet of Things - Phase 2

[3] 3GPP TR 23.700-13	Study on Architecture support of Ambient power-enabled Internet of Things

[4] 3GPP TR 23.700-30	Study on Architecture support of Ambient power-enabled Internet of Things (AIoT);

Phase 2

[5] 3GPP TS 23.369	Architecture support for Ambient power-enabled Internet of Things; Stage 2

[6] 3GPP TR 38.769	Study on solutions for Ambient IoT (Internet of Things) in NR

[7] R2-2409502	Report of 3GPP TSG RAN WG2 meeting #127bis