3GPP RAN WG2 Meeting #131-bis                                                                                R2-2507035

Prague, Czech Republic, Oct. 13th – 17th, 2025

Agenda Item:	10.3.2

Source:	HONOR

Title:	Considerations of Control plane for 6G Radio

Document for:	Discussion and Decision

- Introduction

RAN#108 has approved the first version of 6G SID and RAN#109 revised the SID with some modification[1]. An initial aspect of this SID is for RAN2 to study the radio interface protocol architecture and procedures. An initial aspect of this radio interface is Control Plane architecture (including RRC states) and protocol design.

RRC is the main control plane protocol, and it provides functions such as connection management, security configuration, initial and system access, mobility, and measurements, etc.  In this contribution, we provide some initial discussion of the possible aspects needed to be addressed for design of 6G RRC.

- Discussion

  - Overview

Architecture

When designing the RRC state, in addition to considering the factors like signaling overhead and transition latency, other major factors such as mobility, data transmission and UE power saving need to be taken into consideration.

For RRC_IDLE state, it has certain advantages in terms of UE power consumption, storage overhead, etc. The RRC_CONNECTED state, it keeps the UE in an active state and enables the unicast data and signaling to be transmitted in a timely manner. From these perspectives, both RRC_IDLE state and RRC_CONNECTED state shall be supported and might be further characterized for 6G rather than directly inherited.

For RRC_INACTIVE state, undeniably, it can meet the requirements of frequent and short-duration services as well as services with low latency. In NR, the NW releases UEs with RRC_INACTIVE capability to the idle state or inactive state based on its own implementation. However, it has a relatively significant impact on power consumption. For some certain devices (such as smart glasses), power saving should be taken into consideration. Therefore, perhaps we need to focus on whether to introduce RRC_INACTIVE state based on the consideration on the new requirements and new services of 6G in the initial discussion.

Similar to NR, when considering the RRC state of 6G, the main function of RRC connection control should be taken into consideration, such as establishment/modification/suspension/resumption/release of RRC connection, during which may involve assignment/modification of UE identity (C-RNTI, fullI-RNTI, etc.) and establishment/modification/suspension/resumption/release of SRBs (except for SRB0).

Proposal 1: RAN2 to take RRC_IDLE, RRC_CONNECTED as the baseline and study whether introducing the RRC_INACTIVE state in 6G.

Function

In addition to the above functions of RRC connection control based on the design of 6G RRC states, RAN2 should discuss the functions to supported by control plane in 6G. To not start from a blank sheet when studying 6G, it is important to state what should be inherit from NR and what should be avoid. Most of the functions provided by NR RRC can be expected to be supported in 6G. As specified in TS 38.300, the main services and functions of the RRC sublayer in NR includes:

-	Broadcast of System Information related to AS and NAS;

-	Paging initiated by 5GC or NG-RAN;

-	Establishment, maintenance and release of an RRC connection between the UE and NG-RAN including:

-	Addition, modification and release of carrier aggregation;

-	Addition, modification and release of Dual Connectivity in NR or between E-UTRA and NR.

-	Security functions including key management;

-	Establishment, configuration, maintenance and release of Signalling Radio Bearers (SRBs) and Data Radio Bearers (DRBs);

-	Mobility functions including:

-	Handover and context transfer;

-	UE cell selection and reselection and control of cell selection and reselection;

-	Inter-RAT mobility.

-	QoS management functions;

-	UE measurement reporting and control of the reporting;

-	Detection of and recovery from radio link failure;

-	NAS message transfer to/from NAS from/to UE.

Proposal 2: It is proposed to support the following NR RRC functions for 6GR and take functions in NR as the baseline:

-	Broadcast of System Information related to AS and NAS;

-	Paging initiated by 5GC or NG-RAN;

-	RRC Connection Management including establishment, maintenance and release of an RRC connection between the UE and NG-RAN;

-	Mobility handling;

-	UE measurement reporting and control of the reporting.

Considering new traffic, new terminal, new services (e.g. AI/Sensing), it need to be considered how to overcome the limitations with changes compared to the way functions were specified in NR and how to design an efficient control plane.  The following paragraphs provides an introductory analysis of these functions for 6GR and the possible differences with NR.

  - Broadcast of System Information

System information contains the essential data required by the UE to access and camp on a cell. In 5G, to address network energy consumption concerns, system information is categorized into Minimum System Information (MSI) and Other System Information (OSI). Only certain system information is always broadcast periodically, while other information can be broadcast either periodically or upon UE request.

In previous discussions regarding 6G, there has been a consensus to adopt a native and joint design approach to achieve energy efficiency and savings for both the network and the UE [2]. Therefore, it is feasible to reuse some of the system information transmission mechanisms from 5G. For instance, system information will likely continue to be divided into MSI and OSI, supporting both periodic broadcasts and broadcasts triggered by UE requests. However, considering that 6G may introduce new scenarios and technologies, such as artificial intelligence (AI) applications, the specific content of different system information categories remains an open topic for discussion. Furthermore, after a UE acquires and stores the required system information, its mobility status and network configuration may change. These changes will affect the validity of the system information stored by the UE. The UE needs to determine whether the stored system information is valid according to certain rules. If it is invalid, the UE needs to follow specific procedures to update it.

Proposal 3: System information design of 6G can reuse some 5G system information transmission mechanisms, specifically:

- Inherit the concepts of system information classification: Minimum SI and Other SI. It is FFS to discuss the content of MSI based on 6G scenarios and requirements.

- Support system information broadcast periodically or upon UE request.

- Consider the existing rules of SIB validity across time domain and spatial domain as a baseline.

- Support to update system information based on system information change indication. Details need to be further discussed in normative work.

It is worth noting that in the initial 5G spec, only OSI can be broadcast in a UE-triggered manner, whereas SIB1 was always broadcast periodically. OD-SIB1 (On-Demand SIB1) was first introduced during the Release 19 discussions to further improve the efficiency of system information broadcasting and enhance energy savings. Several different scenarios were proposed for OD-SIB1 in R19, but considering time and workload constraints, only one was selected for research. Since energy efficiency remains an important issue in 6G architecture design, it is recommended to further study OD-SIB1 from 6G Day-1.

Proposal 4: It is recommended to study OD-SIB1 from 6G Day-1.

  - Initial and system access control

In 5G, cell reservation and access restrictions enable the NW to control the initial and system access. Reservation mechanism means the NW could broadcast the cell status and special reservations. By this way, the UE observes the cell reservation status when performing cell selection and reselection. 5G access restriction (i.e., Unified Access Control) considered different access categories or access identities and only allowed the selected ones to send initial access messages for load control reasons.

On one hand, 3GPP RAN usually controls RRC connection requests to protect RAN/CN nodes. For example, 5G introduced a unified access control mechanism to limit the access attempt based on the requirements from both RAN and CN. The UE would obtain the parameters provided by the camped cell before the access. Firstly, the NAS needs to map the service request to one or more access identities and access category. Then, the RRC would perform the access barring check based on the determined access identities and access category. The UAC aims to guarantee the service quality for the connected UEs and for the UEs with higher priority or authority while the overload exceeds the capacity of the system. For 6G, access control mechanism is still needed considering different service type (e.g., emergency call, MT, MO, or other 6G new services), and device type. On the other hand, barring information is carried in MIB (i.e., cellbarred) and SIB1 (barring for different feature or device type) to indicate the cell status and cell reservation. For example, the cell status as "not barred" indicates the UE could treat the cell as a candidate during the cell selection and cell reselection procedures. For 6G, similar mechanism is needed to inform the cell status in advance to prevent the UE from camping on a cell which could not provide the required service.

Proposal 5: For 6G, RAN2 should aim to design a unified access control mechanism considering different service type (e.g., emergency call, MT, MO, or other 6G new services), and device type and NW/UE capability. Barring information could be carried in system information.

  - Paging

In LTE and 5G NR systems, paging is a critical functionality for the network to notify a UE in the RRC IDLE or RRC INACTIVE state of incoming services or messages (e.g., calls, data, system Information modification, and ETWS/CMAS notification). In LTE, paging was already optimized with Discontinuous Reception (DRX) cycles by not requiring continuous monitoring of the network for UE’s power saving. Specifically, the network determines the Paging Frame (PF) and the specific Paging Occasions (POs) within that frame for a given UE in IDLE/INACTIVE mode, using parameters like the UE’s identity and the configured DRX cycle. In 5G era, new requirements such as supporting massive IoT devices, enabling URLLC, and improving security have emerged, leading to the following key enhancements in 5G paging:

- RRC_INACTIVE and RNA Paging: Unlike LTE only using tracking areas (TA), 5G introduces RNA for UEs in inactive state to enable localized paging without involving the entire TA, reducing signaling overhead and improving efficiency.

- Improved Privacy: LTE paging could reveal IMSI or static TMSI values, making users vulnerable to tracking. 5G addresses this risk by using temporary identifiers (e.g., 5G-S-TMSI / full-RNTI) that are frequently updated.

- Paging early indication (PEI): Introduced in Rel-17, DCI-based PEI informs the UE whether to monitor an upcoming PO. This prevents unnecessary wakeups and achieves distinct energy savings, particularly for IoT UEs.

In future 6G networks, low-capability device types (e.g., NB-IoT, RedCap devices and smart glasses for XR) will be considered. All these devices are battery operated with limited energy storage, making it essential to reduce their energy consumption. Meanwhile, there has been a consensus among the telecommunication industry partners that network energy saving (NES) features are crucial for future 6G network developments. Considering paging as a fundamental function in 6GR, it’s recommended to consider energy-efficient paging mechanisms in the 6G initial phrase.

Paging for UE Power saving on 6GR

As previously mentioned, PEI can help avoid unnecessary PO reception by indicating to the UE whether it should decode paging (PDCCH/PDSCH) in its PO. If the UE detects that PEI indicates it should monitor its associated PO, it will do so; otherwise, it will not monitor the PO.

A prerequisite for PEI execution is that the UE must know its subgroup ID. AMF is responsible for assigning subgroup IDs of UEs and indicates the subgroup ID of each UE via NAS signaling. This assignment is based on the UE’s location, UE’s paging probability, capacity, and network load, etc. For example, adjacent or UEs with similar capabilities (e.g., supported frequency band, power consumption mode) can be grouped together and share the same subgroup ID. In 6G, the group-based paging mechanism can consider additional factors such as UE types and service level (e.g., high-priority emergency notifications, low-priority tasks like advertisement push) to further enhance the transmission efficiency of paging messages.

Proposal 6: In 6G, the group-based paging mechanism can consider additional factors such as UE types and service level (e.g., urgent service with high priority) to further enhance the transmission efficiency of paging messages.

Paging for Network Power saving on 6GR

In addition to SSB signals, paging signal acts like an “always-on” signal, and sparse paging occasions may increase BS’s energy consumption and reduce its sleep time, even when no data transmission occurs during the paging occasions.

In 5G-A, future work on network energy saving (NES) has been conducted in Rel-18 and Rel-19, which categorizes the network power saving (NWPS) mechanism by time, frequency, spatial and power domains. For example, Rel-19 NES WI [3] considers the paging adaptation, which is achieved by clustering of paging occasions in time domain for low-load scenarios. Specifically, paging adaptation extends the paging frame interval so that the paging frames are more interspaced compared to the legacy interval, while potentially increasing the number of paging occasions within each frame to maintain the same paging capacity. This approach makes the paging occasions relatively concentrated, enabling longer cell sleeping periods for network and facilitating more clustered RACH responses to paging. Given this, 6GR can further consider the non-uniform provisioning (in time) of paging opportunities based on paging load to enhance energy saving.

Proposal 7: In 6G, RAN2 can further consider the non-uniform provisioning (in time) of paging opportunities based on paging load to enhance energy saving.

- Conclusions

In conclusion, we propose the followings for CP aspects of 6GR.

Proposal 1: RAN2 to take RRC_IDLE, RRC_CONNECTED as the baseline and study whether introducing the RRC_INACTIVE state in 6G.

Proposal 2: It is proposed to support the following NR RRC functions for 6GR and take functions in NR as the baseline:

-	Broadcast of System Information related to AS and NAS;

-	Paging initiated by 5GC or NG-RAN;

-	RRC Connection Management including establishment, maintenance and release of an RRC connection between the UE and NG-RAN;

-	Mobility handling;

-	UE measurement reporting and control of the reporting.

Proposal 3: System information design of 6G can reuse some 5G system information transmission mechanisms, specifically:

- Inherit the concepts of system information classification: Minimum SI and Other SI. It is FFS to discuss the content of MSI based on 6G scenarios and requirements.

- Support system information broadcast periodically or upon UE request.

- Consider the existing rules of SIB validity across time domain and spatial domain as a baseline.

- Support to update system information based on system information change indication. Details need to be further discussed in normative work.

Proposal 4: It is recommended to study OD-SIB1 from 6G Day-1.

Proposal 5: For 6G, RAN2 should aim to design a unified access control mechanism considering different service type (e.g., emergency call, MT, MO, or other 6G new services), and device type and NW/UE capability. Barring information could be carried in system information.

Proposal 6: In 6G, the group-based paging mechanism can consider additional factors such as UE types and service level (e.g., urgent service with high priority) to further enhance the transmission efficiency of paging messages.

Proposal 7: In 6G, RAN2 can further consider the non-uniform provisioning (in time) of paging opportunities based on paging load to enhance energy saving.

- References

- RP-252912 Revised SID: Study on 6G Radio

- 6GWS-250238_3GPP_6G_Workshop_Summary

- RP-250343, “Revised WID on enhancements of network energy savings for NR”, RAN Meeting #107, Incheon, Korea, March 12-14, 2025