3GPP TSG-RAN WG2 Meeting #131bis	R2-2507065

Prague, Czech Republic, 13th-17th Oct, 2025

Agenda Item:	9.7.2

Source:	Huawei, HiSilicon

Title:	General consideration on voice over NB-IoT NTN via GSO

Document for: Discussion and Decision

# 1	Introduction

In RAN#108, the WID [1] was approved with the following objectives for voice over NB-IoT NTN:

The aim of this work item is to specify the enhancement of NB-IoT-NTN to support IMS voice call over GSO with the following objectives:

- Down-select between CP and UP solutions for voice support over NB-IoT-NTN until RAN#110 [RAN2]

  - Coordination with SA is required

  - The objective can be updated with further details based on the outcome of the above down-selection

- Support of semi-persistent scheduling for NB-IoT-NTN for DL and UL data transmission for voice packets [RAN2, RAN1]

- Support of necessary modifications to RRC connection setup procedure for NB-IoT-NTN [RAN2]

- Support of necessary modifications for emergency call for voice over NB-IoT-NTN [RAN2]

- Study and if feasible, specify UE transmit power higher than PC1 (e.g. up to 37dBm) for NB-IoT-NTN [RAN4]

- Note: The enhancements to support voice over NB-IoT-NTN via GSO in this work item may be also applicable to NGSO cases without additional specification enhancements

- Note: Coordination with SA4 is expected

In this contribution, we provide our views on RAN2 discussion about the support of voice over NB-IoT NTN via GSO.

# 2	Discussion

2.1	General

Up to now, RAN2 has received three LSs from SA4 and SA2 [2][3][4] asking for reply on some quite detailed questions about the support of voice over NB-IoT NTN via GSO. SA WGs started this topic several meetings ahead of RAN2 and have made some preliminary progress. During the assessments of the candidate solutions, they came across some issues that need to be discussed in RAN2. One of the important evaluations is on the down-selection between CP solution and UP solution, which is also one of our objectives. Obviously, this decision should be made considering the end-to-end information. That is why SA WGs are collecting information via LSs from RAN1/RAN2/CT1.

Since this is the first meeting in RAN2 for this topic and TU is limited, RAN2 should focus on the discussion of the reply to the LSs and hopefully SA WGs can make some further progress based on our reply, which can in turn help us make the down-selection between CP and UP solution by RAN#110 (in December).

Proposal 1: RAN2 focuses on the discussion on how to reply the LSs to SA2 and SA4 in the first meeting.

2.2	Reply to SA4 LSs

RAN2 received a LS on the RAN simulation assumptions for ULBC from SA4 as follows [2]:

# To SA2 and RAN2:

ACTION: SA4 kindly asks SA2 and RAN2 to comment on

- the different options among UP/CP and IP/Non-IP and the respective overall packet overhead (including RTP/UDP/IP with RoHC, PDCP, RLC and MAC and any potential AS layer optimization if applicable), and if there is any preferred option, and

- specifically, whether a packet overhead of 1 byte of MAC header is realistic.

The above LS raises two questions. Firstly, as mentioned in the general section 2.1, down-selection between the UP and CP solution requires further analysis based on further progress in SA WGs.

Regarding the header overhead, we can provide RAN2 view on Layer 2 headers. Note that even though RoHC is performed in PDCP for UP solution, the RoHC header is not a Layer 2 header but rather a header added in CN. So, we can leave SA2 to provide comments on the aspects related to RoHC and IP/Non-IP.

- For UP solution, the L2 header comprises MAC header, RLC header and PDCP header. Considering the poor link budget the low data rate in NB-IoT NTN via GSO, we should aim at the best choice we have in the current spec, e.g., shorter SN, no concatenation, no segmentation. In this case the L2 header size would be 2B (MAC)+1B (RLC)+1B (PDCP) =4Bytes.

- For CP solution, the L2 header comprises MAC header, RLC header and RRC overhead. And following similar principle, the L2 header size would be 2B (MAC)+1B (RLC)+1B or 2B (RRC) = 4 or 5 Bytes. Note that for RRC, the uplink overhead is 1 Byte while the downlink overhead is 2 Bytes.

Proposal 2: Regarding the Layer 2 header size, RAN2 provides the following answer:

- For UP solution, the current minimum Layer 2 overhead is 2B (MAC)+1B (RLC)+1B (PDCP) = 4 Bytes.

- For CP solution, the current minimum the L2 overhead for uplink is 2B (MAC) +1B (RLC)+1B (RRC) = 4 Bytes and the current minimize the L2 header size for downlink is 2B (MAC)+1B (RLC)+2B (RRC) = 5 Bytes.

Secondly, regarding whether the 1-byte MAC header is realistic, we first note that in the SA LS the term “1-byte MAC header” should refers to a 1-byte MAC sub-header. According to TS 36.321, the existing MAC sub-header consists of the R/F/E/LCID/L fields, which is 2 Bytes. Thus, achieving a 1-byte sub-header would require defining a new MAC sub-header. It could be theoretically feasible but would be subject to functional restrictions that RAN2 needs to further consider. We provide our analysis on the existing format in the following:

- R field. Reserved bits. Can be removed if necessary. But it is better to leave at least one R bit for future proof.

- F field. The F field indicates whether the size of the MAC SDU is smaller than 128 bytes, i.e., in case the size of L field varies. As long as the voice packet is kept less than a certain size, the L field size could be constant and F is not needed.

- E field. If there is a case that MAC PDU contains both MAC CE and MAC SDU or more than one MAC SDUs, the E field must be retained. Otherwise, it can be removed.

- LCID field. This field is to indicate the LCH for the data or to identify the MAC CE type.

- L field. This field is to indicate the variable-sized packet. Since the size of voice packet may also change, the size needs to be indicated. But there may be not many different sizes for voice packet, which needs to be confirmed by SA4.

Before discussing how to design the one-byte MAC sub-header, we need to first consider the following aspects:

- whether a voice packet over NB-IoT NTN supports multiplexing.

- whether the voice packet size is variable and if yes, how many different sizes.

- whether it is essential to save one Byte more from the overall packet.

Proposal 3: Regarding the 1 Byte MAC header, RAN2 provides the following answer:

Before discussing whether to design a one-byte MAC sub-header, RAN2 needs to firstly consider the following aspects:

- whether a voice packet over NB-IoT NTN supports multiplexing.

- whether the voice packet size is variable and if yes, how many different sizes.

- whether it is essential to save one Byte more from the overall packet.

In addition, RAN2 also received another LS from SA4 asking the following [3]:

# To RAN2:

ACTION:

SA4 kindly asks RAN2 to confirm whether the 120ms and/or 240ms bundling periods would be valid for the SPS operation in NB-IoT NTN, and provide feedback, if any.

Firstly, the bundling period in the LS is not equivalent to the SPS period and RAN2 should focus on the SPS period. It is not a new issue for RAN2 if the SPS periodicity cannot divide 10240. We have discussed similar issue in URLLC and XR topics. Generally, if the SPS periodicity cannot divide 10240, there will be issue with the current formula in the MAC spec. For this issue, two solutions can be considered: 1) use multiple SPS to solve the issue, without extra spec effort; 2) use similar solution as what was specified for DRX in XR topic, e.g., introducing non-integer periodicity, which requires spec impact. As far as we know, SA4 is doing the evaluation for codec design, all the possible periodicities may need to be evaluated since performance is important to voice over GSO. We prefer RAN2 to keep open on this at this stage and wait for SA4’s evaluation results to see the final result before discussing whether solution is needed under the SPS objective.

Proposal 4: Regarding the periodicity of SPS, RAN2 provides the following answer:

From RAN2 perspective, the bundling period is not necessarily equivalent to the SPS period. If the SPS period cannot divide 10240, there may or may not be extra RAN2 spec impact to make SPS work depending on the detailed solution.

2.3 Reply to SA2 LS

In the previous meeting, RAN2 received an LS from SA2 on issues related to the support of IMS voice over NB-IoT NTN connected to EPC [3], requesting RAN2’s comments on the following issues:

1. Overall Description:

...

In order for SA2 to be able to evaluate the viability of using RoHC for support of IMS voice over NB-IoT NTN connected to EPC, SA2 has the following questions:

During the voice call, if enough consecutive packets are lost or erroneously decompressed, the compressor can find itself leave Second Order (SO) state and enter First Order (FO) state, and then if another enough consecutive packets are lost or erroneously decompressed, the compressor can find itself leave First Order state (FO) and enter the Initialization and Refresh (IR) state, causing gaps in the transmission of the voice packets.Some companies indicated in SA2 that the number of consecutive lost packets that will trigger the compressor to fall back from SO state to FO state is around 16 (e.g., when UO-0 header with 4 bits SN is used in SO state), and the number of consecutive lost packets that will trigger the compressor to fall back from FO state to IR state is around 64 (e.g., when UOR-2 header with 6 bits SN is used in FO state).

Question 1 (To RAN2): Does RAN2 have any observation on how many consecutive packets lost or erroneously decompressed will trigger the RoHC state fall back at the compressor when using RoHC?

...

Voice packets of different sizes can be observed over the NB-IoT NTN link e.g. due to talk/silence transitions, or occasionally RoHC state change, etc. The size difference of voice packets during talk/silent transition is described in S2-2506371/S2-2507021/S2-2507102 (neither endorsed nor agreed).

More information on the RoHC related issues is described in S2-2506371, S2-2507021 and S2-2507102 that are neither endorsed nor agreed by SA2.

Question 3 (To RAN2): Can RAN2 confirm whether scheduling methods for support of IMS voice over NB-IoT NTN can handle the voice packets of different sizes changing dynamically?

...

SA2 has observed that RAN2 has enforced the limitation that the capability of a NB-IoT UE supports a maximum 2 DRBs. In order for SA2 to reach a conclusion to use the user plane and DRBs to support IMS voice, SA2 would like to understand whether NB-IoT Access Stratum protocols can support more than 2 DRBs (e.g. 3 DRBs) in Rel-20, then the UE can support simultaneous voice and other services even during the call.

Question 6 (To RAN2): Is it feasible to support more than 2 DRBs for a UE accessing NB-IoT in Rel-20?

...

Currently, SRBs are only supported over RLC Acknowledged Mode (AM) (TS 36.331), and the possible impacts of this are discussed in S2-2507107, which is not endorsed or agreed by SA2.

In order for SA2 to be able to evaluate between user-plane and control-plane solutions to carry voice media, the use of RLC Unacknowledged Mode (UM) for SRBs, SA2 has the following question:

Question 9 (To RAN2): Can RAN2 provide feedback on whether it is possible to define a new SRB that will be used to carry voice media?

Question 10 (To RAN2): If the answer to the above question is yes, can RAN2 confirm whether such SRB(s) are defined, whether it is technically feasible to support and configure RLC UM for such SRB(s) for example: in NB-IoT deployments over GEO satellite, when SRBs are used to carry voice media?

The following is our analysis on the above issues:

- 2.3.1 ROHC

Regarding question 1, since ROHC algorithm is not specified in 3GPP, in response, RAN2 should clarify that the triggering of RoHC state fallback is not within the scope of 3GPP. It may depend on implementation which makes it different for different UEs/eNBs. But it may worth considering whether we should allow ROHC fallback in NB-IoT NTN GSO since it may not be possible to transmit a voice packet with a full IP header, which can be as large as tens of Bytes.

Proposal 5: RAN2 responds to SA2 that the thresholds for triggering RoHC state fallback are out of 3GPP scope.

- 2.3.2 Scheduling

Regarding question 3, it concerns whether scheduling methods for IMS voice over NB-IoT NTN can handle dynamically changing voice packet sizes, which may result from talk/silence transitions or RoHC state changes, if any.

It is considered technically possible to handle such dynamic packet size variations through scheduling enhancements, such as:

- Alt. 1: Multiple SPS profiles. Configure several Semi-Persistent Scheduling (SPS) profiles, each dedicated to a specific packet-size level; in other words, one SPS configuration for each defined packet-size range (e.g. small, medium, large).

- Alt. 2: SPS combined with dynamic scheduling. Apply dynamic scheduling in combination with SPS, so that the network adjusts resources according to the size of each incoming packet.

In the downlink the eNB naturally knows the packet size, whereas in the uplink further study is needed to determine how the eNB can know the next UL packet size to allocate the appropriate UL grant. In this sense, Alt. 1 is better for UL since NW doesn’t need to send DCI to schedule the grant which can shorten the latency.

Proposal 6: RAN2 responds to SA2 that the dynamic packet-size variations can be handled by scheduling while details needs to be further discussed in RAN2 taking the exact candidate packet sizes into account.

- 2.3.3 Extending DRB number

Regarding question 6, which addresses the current limitation of a maximum of 2 DRBs per NB-IoT UE. To evaluate the feasibility of using the user plane for IMS voice support, SA2 seeks clarification on whether this limit can be extended to 3 or more DRBs in Rel-20. The motivation for extended DRB is to support concurrent IMS voice and data services.

For this issue, RAN2 should first confirm whether it is essential to support concurrent services for NB-IoT NTN via GSO. If the motivation is valid, from RAN2 perspective it is feasible to support more than 2 DRBs for Rel-20 NB-IoT UEs, which may require extra UE capability.

Proposal 7: RAN2 responds to SA2 that, if it is confirmed that concurrent services need to be supported, supporting more than 2 DRBs are feasible, which requires extra UE capability.

- 2.3.4 New SRB for voice data

In the legacy design, the SRB can be only configured with RLC AM mode. However, the acknowledgement and retransmission process may exceed the latency budget of real-time voice services, making RLC AM unsuitable for voice data transmission. If a new SRB is designed to carry voice data, it should be configured with RLC UM with the AM restriction lifted. This is technically feasible and can be supported with current signaling.

Proposal 8: RAN2 responds to SA2 that it is technically feasible to introduce a new SRB with RLC UM mode.

# 3	Conclusion

This contribution discusses the voice over NB-IoT NTN via GEO and we have the following proposals:

Reply to SA4 LSs

Proposal 1: RAN2 focuses on the discussion on how to reply the LSs to SA2 and SA4 in the first meeting.

Proposal 2: Regarding the Layer 2 header size, RAN2 provides the following answer:

- For UP solution, the current minimum Layer 2 overhead is 2B (MAC)+1B (RLC)+1B (PDCP) = 4 Bytes.

- For CP solution, the current minimum the L2 overhead for uplink is 2B (MAC) +1B (RLC)+1B (RRC) = 4 Bytes and the current minimize the L2 header size for downlink is 2B (MAC)+1B (RLC)+2B (RRC) = 5 Bytes.

Proposal 3: Regarding the 1 Byte MAC header, RAN2 provides the following answer:

Before discussing whether to design a one-byte MAC sub-header, RAN2 needs to firstly consider the following aspects:

- whether a voice packet over NB-IoT NTN supports multiplexing.

- whether the voice packet size is variable and if yes, how many different sizes.

- whether it is essential to save one Byte more from the overall packet.

Proposal 4: Regarding the periodicity of SPS, RAN2 provides the following answer:

From RAN2 perspective, the bundling period is not necessarily equivalent to the SPS period. If the SPS period cannot divide 10240, there may or may not be extra RAN2 spec impact to make SPS work depending on the detailed solution.

Reply to SA2 LS

Proposal 5: RAN2 responds to SA2 that the thresholds for triggering RoHC state fallback are out of 3GPP scope.

Proposal 6: RAN2 responds to SA2 that the dynamic packet-size variations can be handled by scheduling while details needs to be further discussed in RAN2 taking the exact candidate packet sizes into account.

Proposal 7: RAN2 responds to SA2 that, if it is confirmed that concurrent services need to be supported, supporting more than 2 DRBs are feasible, which requires extra UE capability.

Proposal 8: RAN2 responds to SA2 that it is technically feasible to introduce a new SRB with RLC UM mode.

# 4	References

- RP-251867, New WI: Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 4.

- S4-251584, LS on the RAN simulation assumptions for ULBC, SA4.

- R2-2506754, LS on bundling period and SPS for ULBC, SA4

- S2-2507636, LS on issues related to support of IMS voice over NB-IoT NTN connected to EPC, SA2.