3GPP TSG-RAN WG2 Meeting #131bis																				R2-2507069

Prague, Czech Republic, Oct. 13th-17th

Agenda item:		10.3.2

Title: 	Consideration on 6G control plane

Source: 			ZTE Corporation, Sanechips

Document for: 	Discussion and Decision

- Introduction

The 6G study item [1] has been agreed this June in Prague with the initial discussion on physical layer started this August in Bengaluru.

-------------------------------------

- Physical Layer structure for 6GR,

  - Waveforms (OFDM-based) and modulations. 5G NR Waveforms and modulation should be considered for 6GR and is also the benchmark for other potential proposals. [RAN1, RAN4]

  - Frame structure, including compatibility with 5G NR to allow for efficient 5G-6G Multi-RAT Spectrum Sharing (MRSS). [RAN1]

  - Channel coding, using LDPC and Polar Code as baseline, considering applicable extensions to satisfy 6G requirements and characteristics with acceptable performance/complexity trade-off [RAN1]

  - Channel Bandwidth (at least minimum and maximum), Numerology, avoiding multiple numerologies for the same band / sub-range (e.g., enabling synergies among frequency bands in the ~7GHz range) [RAN1, RAN4]

  - Physical layer control, data scheduling and HARQ operation [RAN1, RAN2]

  - MIMO operation [RAN1, RAN4]

  - Duplexing [RAN1, RAN4]

  - Initial access [RAN1, RAN2, RAN4]

    - Studies on synchronization signal and raster, broadcast signals/channel and physical random access channel [RAN1, RAN4]

    - Studies on initial access procedure, random access procedures, system information and paging [RAN2, RAN1, RAN4]

  - 6GR spectrum utilization and aggregation.  [RAN1, RAN2, RAN4]

  - Other physical layer signals, channels and procedures [RAN1, RAN2, RAN4]

  - Evaluate performance of at least energy efficiency, spectrum efficiency, and coverage compared to 5G NR, and deliver the initial result at the end of study [RAN1].

    - RAN4 can be involved, if necessary, based on the LS from RAN1

- Radio interface protocol architecture and procedures for 6GR [RAN2, RAN1, RAN4, RAN3]

- User Plane architecture and protocol design [RAN2]

- Control Plane architecture (including RRC states) and protocol design [RAN2, RAN3]

- Access stratum security aspects, in alignment with requirements from SA3 [RAN2]

- Radio signalling framework for UE capabilities, aiming at improvements and simplification compared to 5G NR [RAN2, RAN1, RAN4]

- Data transfer design to support various types of data (e.g. AI/ML, Sensing, etc) [RAN2, RAN3]

-------------------------------------

In this paper, we share some consideration on the RAN2 related objectives in this 6G study item and focus on the control plane aspects, including RRC states, connection management, initial access procedure, random access, system information and paging.

- Discussion

  - Initial Access Procedure

    - Use cases for 6G initial access

The following general objectives have been agreed for 6G SID:

-------------------------------------

- Single technology framework based on a stand-alone architecture (Note1) to support the agreed existing and new services, and to satisfy the usage scenarios, requirements, deployment scenarios and design principles with acceptable performance/complexity trade-off, as determined by the RAN requirements in [RP-250810] and [TR38.914], including: [RAN1], [RAN2], [RAN3], [RAN4]

  - Ensuring appropriate set of functionalities, minimize the adoption of multiple options for the same functionality, avoid excessive configurations, excessive UE capabilities and UE capabilities reporting.

  - Energy efficiency and energy saving: both for network and device.

  - Enhanced spectral efficiency.

  - Enhanced overall coverage, focus on cell-edge performance and UL coverage.

  - Wider channel bandwidth (at least 200MHz) support for 6G deployments at least above 2 GHz, around 7 GHz.

  - Re-use of existing 5G mid-band (~3.5GHz) site grid for 6G deployments in at least around 7 GHz and targeting comparable coverage to 5G mid-band.

  - Target scalable and forward compatible design for diverse device types.

  - Improved spectrum utilization and operations taking into account diverse spectrum allocations.

  - Aim at using common 6G Radio design, which meets mobile broadband service requirements as high priority, to also meet vertical needs.

  - Aim at a harmonized 6G Radio design for TN and NTN, including their integration.

  - System simplification, including reducing configuration complexity, enabling more efficient Cell/UE management, etc.

Note1: the term stand-alone architecture does not imply any particular Core network architecture, which is up to SA2 discussion.

-------------------------------------

For initial access in 6G, following the above objectives of 6G SID, we identify the following use cases that require more efficient utilization of network resources, e.g. frequencies, time and spatial domain resources, satellite orbits, to support full coverage, high throughput with reduced energy consumption.

Figure 1. Efficient utilization of network resources to provide full coverage

To support efficient utilization of network resources to provide full coverage, we can have cell/carrier A from higher band, e.g. U6G, to provide the basic coverage while cell/carrier B, e.g. 4.9GHz, from lower band to further enhance the coverage.

Figure 2. Efficient utilization of network resources with reduced energy consumption

To reduce the overall energy consumption at both UE and network side, cell/carrier A from lower band will be deployed for basic coverage while cell/carrier B from higher band will be turned on for heavy traffic or high throughput and most of time would be in energy saving mode with omitted or reduced common signal or configuration transmission.

Figure 3. Efficient utilization of network resources and satellite orbits in NTN

For NTN multi-orbit system, to fully utilize the network resources and satellite obits according to the characteristics of NTN system with multiple orbits, UE can camp on GEO cell with large coverage and switch to Non-GEO cell for initial access for services require relatively shorter delay.

Figure 4. Load balancing and distribution

Load balancing and distribution has always been important at network side to improve the resources utilization efficiency. Instead of handover UE for this purpose, allowing distribution from initial access would be more efficient with reduced resource and energy consumption at both network and UE side.

With all the above use cases considered, also take into account the following principle and objective stated in the 6G SID:

-------------------------------------

The study should strive at dimensioning an appropriate set of functionalities, minimizing the adoption of multiple options for the same functionality, focusing on practical user experience. The study should identify principles to ensure extensibility and deliver superior performance.

Aim at a harmonized 6G Radio design for TN and NTN, including their integration.

-------------------------------------

We understand a unified multi-cell/carrier coordinated initial access procedure should be considered, e.g. with cell/carrier A (anchor cell/carrier) providing system information, including information for the initial access on cell/carrier B (assisting cell/carrier) and allowing UE to determine and trigger initial access procedure on a cell/carrier among all the anchor cell/carrier and assisting cell/carriers.

Proposal 1: A unified multi-cell/carrier coordinated initial access procedure should be supported for efficient utilization of network resources, e.g. spectrum, time and spatial domain resources, satellite orbits, to provide overall coverage, high throughput with reduced energy consumption at both network and UE side.

    - Access procedure in 5G NR

Cell search and system information acquisition: Once a UE is power on, it performs cell search procedure to acquire time and frequency synchronization with a cell and to detect the physical layer Cell ID of the cell. NR cell search is based on the primary and secondary synchronization signals, and PBCH DMRS, located on the synchronization raster. The MIB on PBCH provides the UE with parameters for monitoring PDCCH carries the SIB1. MIB and SIB1 comprises the basic information required for initial access and information for acquiring any other SI.

Initial Access: After acquiring MIB and SIB1, UE can trigger initial access from RRC_IDLE by triggering random access procedure. Two types of random access procedure are supported: 4-step RA type with MSG1 and 2-step RA type with MSGA. Both types of RA procedure support contention-based random access (CBRA) and contention-free random access (CFRA).

To improve UL coverage for high frequency scenarios, SUL can be configured. For random access in a cell configured with SUL, the network can explicitly signal which carrier to use (UL or SUL). Otherwise, the UE selects the SUL carrier if and only if the measured quality of the DL is lower than a broadcast threshold. UE performs carrier selection before selecting between 2-step and 4-step RA type. Once started, all uplink transmissions of the random access procedure remain on the selected carrier.

RRC connection establishment and Context Set up: The RRC connection establishment will be performed after random access procedure with the correct AMF selected. The AMF establishes the complete UE context by sending the Initial Context Setup Request message to the NG-RAN over NG-C. Upon successful establishment of the UE context and allocation of PDU session resources to the relevant network slice(s) when present, the NG-RAN responds with the Initial Context Setup Response message.

With all the above, UE get accessed to network and is ready for signaling or data transmission.

Observation 1: For initial access in 5G NR, up to 2 UL carriers can be configured for a cell and once the access procedure is initiated, all uplink transmissions for the access procedure remain on the selected carrier.

    - Initial Access Procedure in 6G

To meet the target of efficient utilization of network resources, e.g. spectrum, satellite orbits, time and spatial domain resources, to provide overall coverage, high throughput with reduced energy consumption, we propose the following unified initial access procedure:

Figure 5. Initial Access procedure in 6G

- Multi-cell/carrier coordinated initial access procedure

- Step 1: The anchor cell/carrier A broadcast system information and provide initial access configuration for itself and also the assisting cell/carrier B/C/D. The assisting cell/carrier can be either on TDD or FDD spectrum. For the case of FDD spectrum, the assisting cell/carrier may have both UL and DL spectrum or only have DL or UL spectrum.

- Step 2: UE select a cell/carrier and trigger initial access

  - Option 1: UE select anchor cell/carrier A and the access procedure remains in cell/carrier A

  - Option 2: UE select UL only assisting cell/carrier B and the access produce is performed via a combination with cell/carrier A and cell/carrier B, i.e. UL via cell/carrier B and DL via cell/carrier A

  - Option 3: UE select assisting cell/carrier B and the access procedure remains in cell/carrier B

With the above Multi-cell/carrier coordinated initial access procedure:

- Flexible and efficient utilization of spectrum, satellite orbits, time and frequencies resources can be reached with combination of cell/carrier A and B/C/D

- The overall network energy consumption can be reduced with common signal or configuration, e.g. SSB and system information, omitted or reduced at cell/carrier B/C/D

- The UE energy consumption can also be reduced as cell reselection would be omitted or simplified and handover for load balancing or distribution purpose would be reduced.

- Deployment and association of anchor cell/carrier and assisting cell/carrier

Figure 6. Co-located and non-co-located cell/carrier A and B deployment

For co-located cell/carrier A and B, it can be assumed that cell/carrier A and B are fully synchronized.

While for non-co-located cell/carrier A and B, fully synchronization cannot be guaranteed, which would impact the detailed solution of the unified Multi-cell/carrier coordinated initial access procedure.

- Issue 1: Cell/carrier selection

The following factors can be considered for the cell/carrier selection:

- The measurement results of cell/carrier A and B.

For co-located deployment, since A and B are fully synchronized, the cell/carrier selection can be based on measurement results of cell/carrier A while cell/carrier B can be UL only or SSB-less.

For non-co-located deployment, the cell/carrier selection should be based on the measurement results of both A and B. Note that no additional measurement is required compared to 5G NR even for non-co-located case as UE anyway perform neighbour cell measurement in 5G NR for cell reselection.

- Traffic type or service type available in cell/carrier A and B

For example, some services or slices can be available in certain cell/carriers and UE would select according to the intended service or slice for initial access.

- Network defined rule

To make it possible for network to control, balance and distribute load from initial access, rather than do it via handover, some network defined rule, e.g. selection priority, distribution factor, load condition and energy consumption status, can be considered.

- Issue 2: RACH resources configuration

As mentioned in the step 1 of the 6G initial access procedure, the basic coverage cell/carrier A will provide random access resources for cell/carrier B/C/D/F.

Although the RACH resources and SSB design for 6G is still under RAN1 discussion, we assume that the association between RACH resources and SSB would still apply in 6G and would impact the RACH resources used by UE for initial access.

- For co-located deployment, the RACH resources for cell/carrier B can be associated with cell/carrier A SSB and UE will perform beam and RACH resources selection accordingly and one single Timing Advance (TA) would be sufficient.

- For non-co-located deployment, the RACH resources for cell/carrier B would be associated with its own DL reference signal and separate TA for A and B would be required.

- Issue 3: Paging handling

To reduce the energy consumption at both UE and network side, UE is not expected to monitor paging from another cell/carrier while performing initial access in one cell/carrier.

Thus, per our understanding on the 6G initial access procedure, UE will only monitor paging from the cell/carrier that access procedure is performed on and would camp back to cell/carrier A for paging monitoring after the transmission ends.

With the above consideration, we have the following proposal for Multi-cell/carrier coordinated initial access:

Proposal 2: Multi-cell/carrier coordinated initial access need incorporation of the following functionalities.

- UE camps in cell/carrier A (anchor cell/carrier) for system information reception, including the information for the initial access on cell/carrier B (assisting cell/carrier)

- Whenever initial access is required, the UE determine the cell/carrier for initial access (e.g. preamble transmission) among all the anchor cell/carrier and assisting cells, and perform initial access on the cell/carrier selected.

- The assisting cell/carrier can be either on TDD or FDD spectrum. For the case of FDD spectrum, the assisting cell/carrier may have both UL and DL spectrum or only have DL or UL spectrum.

- The assisting cell/carrier can be either co-located or non-co-located with anchor cell/carrier. FFS whether non-co-located can be supported for the UL only assisting cell/carrier.

- Whether the assisting spectrum resource should be “cell” or “carrier” is some kind of modeling issue, which can be discussed later when we have clear view on the whole function and framework.

  - System information

    - System information in 5G NR

Figure 7: System Information Provisioning in 5G NR

System Information (SI) consists of a MIB and a number of SIBs, which are divided into Minimum SI and Other SI:

- Minimum SI comprises basic information required for initial access and information for acquiring any other SI. Minimum SI consists of:

  - MIB contains cell barred status information and essential physical layer information of the cell required to receive further system information, e.g. CORESET#0 configuration. MIB is periodically broadcast on BCH.

  - SIB1 defines the scheduling of other system information blocks and contains information required for initial access. SIB1 is also referred to as Remaining Minimum SI (RMSI) and is periodically broadcast on DL-SCH or sent in a dedicated manner on DL-SCH to UEs in RRC_CONNECTED.

- Other SI encompasses all SIBs not broadcast in the Minimum SI. Those SIBs can either be periodically broadcast on DL-SCH, broadcast on-demand on DL-SCH (i.e. upon request from UEs in RRC_IDLE, RRC_INACTIVE, or RRC_CONNECTED), or sent in a dedicated manner on DL-SCH to UEs in RRC_CONNECTED (i.e., upon request, if configured by the network, from UEs in RRC_CONNECTED or when the UE has an active BWP with no common search space configured or when the UE configured with inter cell beam management is receiving DL-SCH from a TRP with PCI different from serving cell's PCI).

| Minimum SI | MIB | Cell barred status information and essential physical layer information of the cell required to receive further system information, e.g. CORESET#0 configuration. |
| --- | --- | --- |
|  | SIB1 | The scheduling of other system information blocks and contains information required for initial access. SIB1 is also referred to as Remaining Minimum SI (RMSI) and is periodically broadcast on DL-SCH or sent in a dedicated manner on DL-SCH to UEs in RRC_CONNECTED. |
| Other SI | SIB2 | cell re-selection information, mainly related to the serving cell; |
|  | SIB3 | serving frequency and intra-frequency neighboring cells relevant for cell re-selection (including cell re-selection parameters common for a frequency as well as cell specific re-selection parameters); |
|  | SIB4 | other NR frequencies and inter-frequency neighboring cells relevant for cell re-selection (including cell re-selection parameters common for a frequency as well as cell specific re-selection parameters), which can also be used for NR idle/inactive measurements; |
|  | SIB5 | E-UTRA frequencies and E-UTRA neighboring cells relevant for cell re-selection (including cell re-selection parameters common for a frequency as well as cell specific re-selection parameters); |
|  | SIB6 | ETWS primary notification; |
|  | SIB7 | ETWS secondary notification; |
|  | SIB8 | CMAS warning notification; |
|  | SIB9 | information related to GPS time and Coordinated Universal Time (UTC) |
|  | SIB10 | Human-Readable Network Names (HRNN) of the NPNs listed in SIB1 |
|  | SIB11 | idle/inactive measurements |
|  | SIB12 | Sidelink: information related to NR sidelink communication, ranging and sidelink positioning |
|  | SIB13 | Sidelink: information related to SystemInformationBlockType21 for V2X sidelink communication |
|  | SIB14 | Sidelink: information related to SystemInformationBlockType26 for V2X sidelink communication |
|  | SIB15 | disaster roaming |
|  | SIB16 | slice-based cell reselection information |
|  | SIB17 SIB17bis | information related to TRS configuration for UEs in RRC_IDLE/RRC_INACTIVE |
|  | SIB18 | information related to the Group IDs for Network selection (GINs) associated with SNPNs listed in SIB1 |
|  | SIB19 | TN:NTN-specific parameters for NTN neighbour cells NTN: NTN-specific parameters for serving cell and optionally NTN-specific parameters for neighbour cells |
|  | SIB20 | MBS broadcast: MCCH configuration |
|  | SIB21 | MBS broadcast: information related to service continuity for MBS broadcast reception |
|  | SIB22 | ATG-specific parameters for serving cell and optionally ATG-specific parameters for neighbour cells |
|  | SIB23 | Sidelink: information related to ranging and sidelink positioning |
|  | SIB24 | MBS multicast: information required to acquire the multicast MCCH/MTCH configuration |
|  | SIB25 | NTN: TN coverage information |

In NR, the SIBs can be broadcast in a cell specific or area specific way and the SI modification will be informed to UE via paging. The UE shall apply the SI acquisition procedure upon cell selection (e.g. upon power on), cell-reselection, return from out of coverage, after reconfiguration with sync completion, after entering the network from another RAT, upon receiving an indication that the system information has changed, upon receiving a PWS notification, upon receiving request (e.g., a positioning request) from upper layers; and whenever the UE does not have a valid version of a stored SIB or posSIB or a valid version of a requested SIB based on the value tag and area id.

    - System information in 6G

- Inheritance from 5G

In 6G, we understand the following characteristics of 5G system information can still be inherited and taken as a baseline:

- The concept of RMSI, which contains the required information for initial access. To work together with the Multi-cell/carrier coordinated initial access procedure, in which the system information broadcast by anchor cell/carrier also contains initial access information of assisting cell/carrier, the contents of RMSI should be evaluated and decided carefully in 6G to reach a balance of efficiency and coverage.

- Cell and area specific system information provisioning, which is beneficial for UE to identify the validity or acquired system information and decide whether to acquire again thus would be beneficial to both UE and network energy saving.

- System information modification indication via paging or any other DL notification to reach alignment of UE and network on the system information broadcast.

- On demand system information, which helps to reduce the UE and network energy consumption.

- Energy consumption reduction

The system information is common configuration broadcast per cell and contributes a lot to the energy consumption at network side, which have been a big head ache for operators since the first commercial deployment of 5G. Various solutions have been specified in the later releases, e.g. cell DTX/DRX, on demand SIB1 but turned into paper work due to non-backward compatibility. Thus, we would like to see network energy saving considered from day 1 for system information provisioning deign.

Two possible directions can be considered from day 1 to reduce the energy consumption at network side for system information design:

- Direction 1: Cell DTX/DRX in all RRC states

Figure 8. Cell DTX/DRX in all RRC states

Cell DTX/DRX has been supported in 5G NR only in connected mode as if we support for idle and inactive state, the power saving gain would be limited as network needs to take care of the legacy UEs and would require two ways of common signal transmission.

While for 6G, since we start from scratch, we can consider this. With the cell DTX cycle defined and introduced, the common signal and system information broadcasting would also be impacted as it is expected that network would only provide system information during cell DTX on duration once Cell DTX/DRX is activated.

- Direction 2: On demand system information

Similar as the on demand SIB1 and on demand SI, we understand on demand system information mechanism should still be supported in 6G, which is helpful in reducing the energy consumption at both UE and network side.

- Support of various device types

With the objective that “Target scalable and forward compatible design for diverse device types.” the support of diverse device types should be considered in system information design.

The following agreement has been achieved in RAN#109 for the device types:

-------------------------------------

Proposal 1: Endorse the following two RAN1 agreements (with the clarification that the 2nd agreement is applicable to FR1). Companies are invited to bring contributions regarding the minimum spectrum allocation in RAN#110, while RAN1 is requested to continue the study on both the minimum spectrum allocation and the smallest maximum UE bandwidth from the 6GR design perspective. Revisit in RAN#110.

Agreement

- For the study of RAN1 6GR design, consider the minimum spectrum allocation in which 6G can operate, subject to further discussion and confirmation in RAN.

  - Note: RAN4 involvement is necessary.

Agreement

- Study the following smallest maximum supported RF and BB UE BW without spectrum aggregation for at least one low-tier device type supported by 6GR framework from physical layer perspective, subject to further discussion and confirmation in RAN

  - Opt1: 3MHz

  - Opt2: 5MHz

  - Opt3: 10MHz

  - Opt4: 20MHz

  - FFS: the UL bandwidth may be different to the DL bandwidth

  - FFS: the bandwidth value may be different for different SCS, duplex modes, and bands.

FFS: whether RF and BB UE BW are same or different

-------------------------------------

Although we are targeting to have unified and simplified system information design to support various device types, we understand it is not fully possible for UE to always operate with the maximum supported RF and BB BW of 3MHz for MIB, SIB1, and other system information reception. Since the discussion in RAN1 is still ongoing, more input from RAN1 is required on whether separate SSB/MIB/SIB are required for different device types.

- MRSS between 5G and 6G

For compatibility with 5G NR with spectrum re-farming, 5G-6G MRSS should be considered in the first release of 6G, similar as 4G-5G DSS. RAN1 is now studying how to dynamically and efficiently share the spectrum, e.g., whether/how to allow 6G PDSCH/PDCCH transmissions on NR PDCCH/DMRS symbols.

From RAN2’s perspective, the major impact would be on the system information design, especially MIB and SIB1 and the following options would be on table:

- Option 1: Separate SSB and System information with non-overlapping in time and frequency domain

- Option 2: Joint SSB and/or System information

  - Joint SSB with separate PBCH payload, i.e. separate MIB

  - Joint SSB and PBCH payload but with separate SIB1 and other SI

Observation 2: The MRSS between 5G and 6G would impact the system information design in RAN2, especially MIB and SIB1.

- Integrity protection of system information

The following requirements have been identified by SA1 in TR22.870:

5.5.3	Use case on UE selecting a base station after assessing its legitimacy

5.5.3.1	Description

The majority of the efforts in the mobile communication system security has been focused on delivering secure communication once connected, whereas security related to bootstrapping those connections are relatively less explored. A UE can interact with a Base Station without assessing its legitimacy, which opens way for diverse attacks. False Base Station (FBS) attacks are one such example and its potential to cause active and passive impacts are an alarming concern, worldwide. Addressing fundamental security issues is important and can entail a system level reappraisal of risk and expectations. This is most feasible to undertake at the beginning of a new mobile generation. Attacks mounted by a FBS is a major security flaw in the existing communication system. There is an important opportunity to address this concern in the next generation wireless systems. The sooner that security threats are addressed in the process of interaction between components of the system, the more risks can be reduced to the 3GPP system.

By increasing the level of security between the radio network entities and the UE during the initial network selection procedure, False Base Station attacks can be prevented at earlier stages of communication establishment. One such example is the introduction of an efficient Base Station authenticity verification mechanism such that the UE can verify the authenticity of the Base Station prior to starting the 6G registration procedure.

5.5.3.2	Potential New Requirements needed to support the use case

[PR 5.5.3.2-1] Subject to operator policy and regulatory requirements, the 6G system shall support a means for a UE to be able to distinguish a False Base Station from an authentic Base Station.

There has been Study on 5G security enhancements against False Base Stations (FBS) in Release 18 with TR 33.809 as output but not formally specified. We understand the solutions studied before can also be considered for 6G while from RAN2’s perspective, Integrity protection of system information should be considered.

With the above consideration, we have the following proposal for system information design in 6G:

Proposal 3: The concept of RMSI, cell and area specific system information, SI modification indication via paging and on demand system information should be inherited from 5G while the following enhancements should be studied for 6G system information design:

- Network energy saving for system information (e.g., on-demand SI, cell DTX/DRX)

- Security protection for system information (e.g., integrity protection; pending input from SA3)

- Support for various device types (e.g., whether separate SSB/MIB/SIB are required for different device types; pending input from RAN1)

- Support for 5G–6G MRSS (e.g., whether joint SSB/MIB/SIB can be considered; pending input from RAN1)

  - Access Control

- 5G overload and access control functionality

NG-RAN supports overload and access control functionality such as RACH back off, RRC Connection Reject, RRC Connection Release and UE based access barring mechanisms.

One unified access control framework as specified in TS 22.261 applies to all UE states (RRC_IDLE, RRC_INACTIVE and RRC_CONNECTED) for NR. NG-RAN broadcasts barring control information associated with Access Categories and Access Identities (in case of network sharing, the barring control information can be set individually for each PLMN). The UE determines whether an access attempt is authorized based on the barring information broadcast for the selected PLMN, and the selected Access Category and Access Identity for the access attempt

- Access control in 6G

For 6G, we understand access control for certain UE type, service type, traffic type, network type are still needed, e.g. with 6G defined access categories, access identities and control parameters.

Apart from that, we understand finer-grained control can be considered, e.g. to support access control per cell/carrier/beam/TRP/PLMN for more efficient control and utilization of network resources.

Also considering the unified initial access procedure proposed above, the access control configuration of cell/carrier B should also be considered as part of basic information for initial access and should also be provided from cell/carrier A. Thus UE can check whether the access is allowed before triggering the initial access procedure, which is also an important factor for cell/carrier selection otherwise UE may trigger the access procedure and get rejected.

Proposal 4: Similar as 5G, the access categories, access identities based access control framework can be reused in 6G. Finer granularity can be studied for different slice/PLMN.

  - RRC state and small data transmission

Figure 9. RRC states and state transition in 5G NR

| 5G NR RRC supports the following states which can be characterised as follows |  |  |
| --- | --- | --- |
| RRC_IDLE: PLMN selection; Broadcast of system information; Cell re-selection mobility; Paging for mobile terminated data is initiated by 5GC; Transfer of MBS broadcast data to the UE over MRB(s); DRX for CN paging configured by NAS. | RRC_INACTIVE: PLMN selection; Broadcast of system information; Cell re-selection mobility; Paging is initiated by NG-RAN (RAN paging); RAN-based notification area (RNA) is managed by NG- RAN; DRX for RAN paging configured by NG-RAN; 5GC - NG-RAN connection (both C/U-planes) is established for UE; The UE Inactive AS context is stored in NG-RAN and the UE; NG-RAN knows the RNA which the UE belongs to; Transfer of MBS multicast/broadcast data to the UE over MRB(s); Transfer of unicast data and/or signaling to/from the UE over radio bearers configured for SDT. | RRC_CONNECTED: 5GC - NG-RAN connection (both C/U-planes) is established for UE; The UE AS context is stored in NG-RAN and the UE; NG-RAN knows the cell which the UE belongs to; Transfer of unicast data to/from the UE; Transfer of MBS multicast/broadcast data to the UE over MRB(s); Network controlled mobility including measurements. |

The network seldom release UE to inactive state in 5G NR commercial use mainly because there has been frequent background data transmission thus network prefer to keep UE in connected state to avoid frequent connection release, establishment or resumption at the cost of more network resources consumed for keeping UE in connected mode. Small data transmission, introduced in later release addresses this issue.

Observation 3: 5G UE is mostly kept in connected state to avoid frequent connection release, establishment or resumption caused by frequent background data transmission at the cost of consuming more network resources while small data transmission feature, introduced in later release addresses this issue.

For 6G, we understand RRC INACTIVE state is still needed for UE power saving and CP latency reduction with small data transmission supported natively.

The following characteristics defined in 5G shall be considered as baseline.

- UE context is stored in both UE and NW side.

- Dedicated radio resource can be released.

- UE controlled mobility (i.e. cell re-selection) is used instead of NW controlled mobility.

- Paging will be used for NW to reach the UE in RRC INACTIVE state.

- Long DRX cycle (similar to IDLE, longer than DRX cycle in RRC_CONNECTED) will be supported.

- Small data transmission should be natively supported to address the frequent background data transmission.

Proposal 5: RRC_IDLE, RRC INACTIVE and RRC_CONNECTED states should still be supported for 6G. Small data transmission should be natively supported from day 1 to address the frequent background data transmission.

  - Paging

Paging allows the network to reach UEs in RRC_IDLE and in RRC_INACTIVE state through Paging messages, and to notify UEs in RRC_IDLE, RRC_INACTIVE and RRC_CONNECTED state of system information change and ETWS/CMAS indications through Short Messages.

While in RRC_IDLE the UE monitors the paging channels for CN-initiated paging. While in RRC_INACTIVE, the UE monitors paging channels for RAN-initiated paging and CN-initiated paging.

For paging in 6G, we understand unified and simplified design should be considered to improve efficiency and reduce energy consumption at both UE and network side.

Unified and simplified paging

In 5G NR, The Paging DRX cycles are configured by the network:

1)	For CN-initiated paging, a default cycle is broadcast in system information;

2)	For CN-initiated paging, a UE specific cycle can be configured via NAS signalling;

3)	For RAN-initiated paging, a UE-specific cycle is configured via RRC signalling;

The UE uses the shortest of the DRX cycles applicable i.e. a UE in RRC_IDLE uses the shortest of the first two cycles above, while a UE in RRC_INACTIVE uses the shortest of the three.

As we proposed above, RRC_INACTIVE state should still be supported in 6G but we are considering some sort of simplification of paging in inactive state, e.g. by having common paging cycle for CN paging and RAN paging.

PF and PO distribution for network energy saving

Paging adaption has been introduced for energy saving in 5G NR but only with extended value range of the number of PFs in a paging cycle and the number of POs in a PF.

For PF and PO distribution in 6G, we understand it should be considered together with cell DTX/DRX that we proposed to be supported in all RRC states, i.e. the PO should be within the Cell DTX on duration once the Cell DTX/DRX is activated.

Figure 10. PF and PO distribution with Cell DTX

Early indication before paging message for UE power saving

PEI and LP-WUS have been introduced in 5G NR for early indication before paging message while the design in 5G has a lot in common, e.g. the grouping of UE for the early indication while the major difference is the DL signal to send this indication.

In 6G, for UE power saving purpose, we understand similar mechanism would still be there but we expect simplified and unified solution, e.g. to support only one single solution for paging early indication.

Proposal 6: Energy saving in both network and UE side should be studied for paging design in 6G.

  - Random Access

Two types of random access procedure are supported: 4-step RA type with MSG1 and 2-step RA type with MSGA. Both types of RA procedure support contention-based random access (CBRA) and contention-free random access (CFRA) as shown on the figure below:

(a)	CBRA with 4-step RA type						(b) CBRA with 2-step RA type

(c) CFRA with 4-step RA type						(d) CFRA with 2-step RA type

(e) CFRA without network response with 4-step RA type

Figure 11: Random Access Procedures

Figure 12: Fallback for CBRA with 2-step RA type

For 6G, we understand all these access types will still be useful and should be supported.

RACH partitioning has been introduced in 5G later release but complains about the complexity have been received since then. We have identified the major usages of RACH partitioning and try to have simplified solution in 6G but with the same functions provided:

- Function 1: RACH resource isolation, e.g. for slicing, SDT, IOT, NTN

- Function 2: Early identification of certain UE type, e.g. RedCap as in 5G

For RACH resource isolation, also taken into account the initial access procedure we proposed at the beginning of this paper, we understand RACH resources for certain slices, sub-networks or services can be available in some cells or carriers so that can be isolated from others but still visible and can be selected and applied by UEs intended for the slices, sub-networks or services.

For early identification of certain UE type, we can simply have such UE type indicate in the payload of MsgA or Msg 3/5 for 4-step RA.

Proposal 7: 4-step RA type with MSG1 and 2-step RA type with MSGA should be supported in 6G. Both types of RA procedure support contention-based random access (CBRA) and contention-free random access (CFRA).

- Conclusion and proposals

Based on the analysis in previous sections, the following proposals are given:

Proposal 1: A unified multi-cell/carrier coordinated initial access procedure should be supported for efficient utilization of network resources, e.g. spectrum, time and spatial domain resources, satellite orbits, to provide overall coverage, high throughput with reduced energy consumption at both network and UE side.

Proposal 2: Multi-cell/carrier coordinated initial access need incorporation of the following functionalities.

- UE camps in cell/carrier A (anchor cell/carrier) for system information reception, including the information for the initial access on cell/carrier B (assisting cell/carrier)

- Whenever initial access is required, the UE determine the cell/carrier for initial access (e.g. preamble transmission) among all the anchor cell/carrier and assisting cells, and perform initial access on the cell/carrier selected.

- The assisting cell/carrier can be either on TDD or FDD spectrum. For the case of FDD spectrum, the assisting cell/carrier may have both UL and DL spectrum or only have DL or UL spectrum.

- The assisting cell/carrier can be either co-located or non-co-located with anchor cell/carrier. FFS whether non-co-located can be supported for the UL only assisting cell/carrier.

- Whether the assisting spectrum resource should be “cell” or “carrier” is some kind of modeling issue, which can be discussed later when we have clear view on the whole function and framework.

Proposal 3: The concept of RMSI, -	cell and area specific system information, SI modification indication via paging and on demand system information should be inherited from 5G while the following enhancements should be studied for 6G system information design:

- Network energy saving for system information (e.g., on-demand SI, cell DTX/DRX)

- Security protection for system information (e.g., integrity protection; pending input from SA3)

- Support for various device types (e.g., whether separate SSB/MIB/SIB are required for different device types; pending input from RAN1)

- Support for 5G–6G MRSS (e.g., whether joint SSB/MIB/SIB can be considered; pending input from RAN1)

Proposal 4: Similar as 5G, the access categories, access identities based access control framework can be reused in 6G. Finer granularity can be studied for different slice/PLMN.

Proposal 5: RRC_IDLE, RRC INACTIVE and RRC_CONNECTED states should still be supported for 6G. Small data transmission should be natively supported from day 1 to address the frequent background data transmission.

Proposal 6: Energy saving in both network and UE side should be studied for paging design in 6G.

Proposal 7: 4-step RA type with MSG1 and 2-step RA type with MSGA should be supported in 6G. Both types of RA procedure support contention-based random access (CBRA) and contention-free random access (CFRA).

- Reference

[1] RP-251881 New SID: Study on 6G Radio