3GPP TSG-RAN WG2 Meeting #131	bis			               R2-2507078

Prague, Czech Republic, Oct. 13th-17th, 2025

Agenda item:	8.5.3

Source:	Samsung

Title:	Handling of MAC open issues on C(LTM)

Document for:	Discussion and Decision

# Introduction

In this contribution we discuss the following MAC open issues on C(LTM).

- Inter-CU LTM: MAC-V01, V02

- Conditional intra-CU LTM: MAC-V04, V05, K01, K02, N01, O03, M01

- Event triggered L1 measurements for LTM- S02,Q01

# Discussion

## 2.1. Inter-CU LTM

MAC-V01: RACH based LTM with MIMO 2TA

How to address the mismatch issue that the TCI state ID indicated in the LTM Cell Switch Command MAC CE and TA value in the RAR within the RACH-based LTM procedure are associated with different TAG?

This was already discussed. It was agreed to leave it to network implementation. There is no additional motivation in Release 19 to specify a new procedure to handle this scenario.

Proposal 1: Rely on network implementation to avoid the issue described in MAC-V01.

MAC-V02: Shared CFRA resource in LTM CSC MAC CE

FFS whether the CFRA resource provided by LTM candidate cells and assigned by source DU in LTM CSC MAC CE is a shared resource, similar as the RACH resource for early TA acquisition.

CFRA resource in LTM CSC MAC CE is introduced in R18. So, there is no need to discuss and specify anything in R19.

Proposal 2: No need to discuss and specify anything in R19 for MAC-V02.

## 2.2. Conditional intra-CU LTM

MAC-V04, MAC-N01: How to handle the CLTM TAT of target cell when receiving an LTM Cell switch Command MAC CE with valid TA?

As per the MAC specification CLTM TAT of target cell will continue to run after receiving an LTM Cell switch Command MAC CE with valid TA. In our view this is sufficient and no additional spec changes are needed.

Note that the CLTM TAT of target cell will continue to run when RACH less CLTM is performed based on TA of target cell for which CLTM TAT of target cell is running.

Proposal 3: As per the MAC specification CLTM TAT of target cell will continue to run after receiving an LTM Cell switch Command MAC CE with valid TA. In our view this is sufficient and no additional spec changes are needed.

MAC-V05: When current serving cell is an LTM candidate cell, whether UE maintain and update its CLTM TAT based on legacy Timing Advance Command MAC CE?

This is a non-essential optimization. Maintenance of CLTM TAT for current serving cell is for subsequent CLTM. If network wants, it can send LTM Candidate Timing Advance Command MAC CE with TA value same as TA value of current serving cell and UE will then (re) start CLTM TAT.

There is nothing broken with current spec and we should not pursue this optimization in this phase of essential corrections only.

Proposal 4: UE does not update CLTM TAT based on legacy Timing Advance Command MAC CE. No change to current specification.

MAC-K01: If the source cell is an LTM candidate cell, upon Cell switch, does the UE store the valid TA and update its CLTM TAT of the source cell for possible RACH-less CLTM back to the source cell (e.g., if the CLTM TAT for the Source Cell is not running at the time upon Cell switch)?

This is a non-essential optimization. There is nothing broken with current spec and we should not pursue this optimization in this phase of essential corrections only.

Proposal 5: Do not pursue MAC-K01. If the source cell is an LTM candidate cell, upon Cell switch, UE does not store the valid TA and does not update its CLTM TAT of the source cell for possible RACH-less CLTM back to the source cell. No change to current specification.

MAC-K02: In the last meeting, it was agreed that if the UE receives more TAs beyond its capability, it’s up to UE implementation to decide which one is released. Since the NW does not know which TA is released, whether the UE needs to report currently stored TAs to the NW to avoid unnecessary RA for early UL sync?

Based on discussion in the last meeting, our understanding is that nothing else is needed, it is up to the network to avoid this.

Proposal 6: Do not pursue MAC-K02. UE does not need to report currently stored TAs to the NW to avoid unnecessary RA for early UL sync. No change to current specification.

MAC-O03: How to handle CLTM TAT if the CLTM execution condition is released but candidate configuration is kept, i.e. NW reconfig CLTM to LTM candidate.

As the CLTM can no longer be performed to the candidate cell, there is no reason to keep the CLTM TAT for that cell running.

If the cell for which CLTM TAT is running is no longer a CLTM candidate, CLTM TAT should be stopped.

Proposal 7: If the cell for which CLTM TAT is running is no longer a CLTM candidate, CLTM TAT should be stopped.

MAC-M01:

There is no clear guidance that UE will perform RACH-based LTM after CLTM recovery in stage-3 spec. In RRC spec, two different conditions (CLTM recovery, fulfilment of CLTM condition) have the same action: ##### 5.3.5.18.6 LTM cell switch execution …… 1> else (LTM cell switch triggered upon cell selection performed while timer T311 was running or upon the fulfilment of LTM cell switch execution conditions (as specified in clause 5.3.5.18.x): 2> apply the RRCReconfiguration message in ltm-CandidateConfig within LTM-Candidate IE in ltm-Config related to the LTM candidate configuration identity for the selected cell (i.e., in accordance with 5.3.5.18.x or 5.3.7.3) according to clause 5.3.5.3;. In MAC spec, either 5.18.35 (Enhanced) LTM Cell Switch Command or 5.y.3 Conditional LTM execution seems not reflect the action for CLTM recovery case. And it is not clear if CLTM fast recovery will go to 5.y.3 Conditional LTM execution, as the condition of CLTM fast recovery is not included ### 5.y.3 Conditional LTM execution The conditional LTM cell switch procedure is triggered when: - the MAC entity determines that the event for conditional LTM is satisfied based on L1 measurements as specified in clause 5.y.2; or - the event(s) for conditional LTM is satisfied based on L3 measurements indicated by upper layers.. For R18 LTM fast recovery UE does not have other stored TA value so it is natural to fallback to RACH-based LTM. While for R19 CLTM fast recovery, it is better to be specified somewhere to clarify UE’s behavior, as UE may have other stored TA and may be capable to perform RACH-less LTM. We can discuss whether to capture that “UE only could perform RACH-based LTM for CLTM recovery” in MAC spec, or keep it in stage-2 only.

Proposal 8: We are ok to clarify that UE only perform RACH-based LTM for CLTM recovery.

## 2.3 Event triggered L1 measurements

MAC-S02: Contents for MR after truncated MR is send

Open issue MAC-S02 is about the contents for event triggered MR after the UE has send a truncated MR. This was discussed during open issue discussion and rapporteur has marked it for further discussion.

UE reporting of event triggered L1 MR is specified using the following variables. - BEAM_ENTERING_LIST: includes the reference signaling resource index of LTM candidate cell(s) for each ltm-CSI-ReportConfigId, for which the L1 measurement report entering conditions have been met for TTT for the triggered L1 measurement report; - BEAM_LEAVING_LIST: includes the reference signaling resource index of LTM candidate cell(s) for each ltm-CSI-ReportConfigId, for which has been reported in the (Truncated) L1 measurement report MAC CE, and the L1 measurement report leaving conditions have been met for TTT for the triggered L1 measurement report; Handling of these variables in section 5.x.4 Measurement report while the UE sends truncated MR is not proper, as the RS whose measurements are reported in the truncated L1 MR is not removed from the above lists in the below text. 2> else if the UL-SCH resources are available for a new transmission in the serving cell and these UL-SCH resources can accommodate the Truncated L1 measurement report MAC CE plus its subheader as a result of logical channel prioritization: 3> instruct the Multiplexing and Assembly procedure to generate the Truncated L1 measurement report MAC CE associated with the ltm-CSI-ReportConfigId as defined in clause 6.1.3.x according to the measurement report information in the MR_LIST by selecting the RS(s) based on a decreasing order of the priority for the type of beam: RS(s) in BEAM_ENTERING_LIST, RS(s) in BEAM_LEAVING_LIST, RS(s) in BEAM_REPORTED_LIST, and other RS(s) not in these three lists; We raised the issue during the CR discussions, rapporteur has informed that the below note can handle the case: Note: After sending a Truncated L1 measurement report MAC CE, if the subsequent UL grant is still not big enough to fit all the remaining beams, it is up to UE implementation to select the beam(s) which were not included in the previous MAC CE However, this note seems to be covering only a specific case where the UE does not get enough resources to send the MR after sending a truncated MR, which is a corner case. In case, the UE sends a MR (not truncated MR) after sending the truncated MR, the UE needs to only include the RS(s) which are not reported in the truncated MR. It can be achieved by simply removing the RS(s) whose measurements are reported from the BEAM_ENTERING_LIST and BEAM_LEAVING_LIST, and the UE behavior will be clear. The current procedural text is also not in line with the below note, as the RS for which truncated MR is send are not removed from BEAM_ENTERING_LIST, when the leaving condition of same RS is met, UE doesn’t send the event triggered MR. NOTE Z: When a measurement report is triggered by entry condition for one or more RS(s), and included in the BEAM_ENTERING_LIST, another measurement report is triggered by leaving condition for the same RS(s), all the corresponding measurement reports are cancelled. When a measurement report is triggered by leaving condition for one or more RS(s), and included in the BEAM_LEAVING_LIST, another measurement report is triggered by entry condition for the same RS(s), all the corresponding measurement reports are cancelled. So we suggest to capture the behavior as below highlighted without any ambiguities. 2> else if the UL-SCH resources are available for a new transmission in the serving cell and these UL-SCH resources can accommodate the Truncated L1 measurement report MAC CE plus its subheader as a result of logical channel prioritization: 3> instruct the Multiplexing and Assembly procedure to generate the Truncated L1 measurement report MAC CE associated with the ltm-CSI-ReportConfigId as defined in clause 6.1.3.x according to the measurement report information in the MR_LIST by selecting the RS(s) based on a decreasing order of the priority for the type of beam: RS(s) in BEAM_ENTERING_LIST, RS(s) in BEAM_LEAVING_LIST, RS(s) in BEAM_REPORTED_LIST, and other RS(s) not in these three lists; 3>include the RS(s) from BEAM_ENTERING_LIST whose measurements are reported into BEAM_REPORTED_LIST for this ltm-CSI-ReportConfigId. 3> remove the RS(s) whose measurements are reported from BEAM_ENTERING_LIST and/or BEAM_LEAVING LIST for this ltm-CSI-ReportConfigId. If there is still a different understanding, please mark for discussion in the next meeting. [Huawei] Agree with Samsung, but with some refinement of the TP. Then, the note is useless and can be removed. [Ofinno] Agree with Samsung. We indicated this during the CR discussion as well and think the existing note for the subsequent UL grant not being enough does not address the procedure completely. [Rapp] Based on our previous discussion and conclusion, the understanding “In case, the UE sends a MR (not truncated MR) after sending the truncated MR, the UE needs to only include the RS(s) which are not reported in the truncated MR” is not correct. In this case, UE should report the whole MR MAC CE, rather than include the RS which are not reported. That is why we didn’t capture the suggested TP in the normative text. We have discussed that the beam could be removed from BEAM_ENTERING_LIST only when the whole MR MAC CE is sent. Thus current CR could work well, there is no need to introduce any optimization. Regarding the understanding “The current procedural text is also not inline with the below note, as the RS for which truncated MR is send are not removed from BEAM_ENTERING_LIST, when the leaving condition of same RS is met, UE doesn’t send the event triggered MR.” The triggered beam should be evaluated on the leaving condition before the normal MR is sent, and when the leaving condition is met, UE won’t sent the MR as captured in the note, which has been discussed based on Apple’s proposal before, which is reasonable and there is no issue foreseen. Considering several companies have same understanding, let’s discuss it to align companies’ understanding.

Once the UE has send truncated MR, the network already has the measurement results of the reference signals in the truncated MR. There is no need for retransmitting the same again in the following MR, as the network can use the previously received values. During OI discussion, many companies have agreed to this understanding. So we suggest that RAN2 can first confirm this.

Proposal 9: RAN2 confirms that there is no need to resend the measurements included in truncated MR in the following MR.

If we can confirm the above understanding, then the UE needs to remove the RS whose measurements are reported from BEAM_ENTERING_LIST and BEAM_LEAVING_LIST. UE also needs to include the RS from BEAM_ENTERING_LIST whose measurements are reported in BEAM_REPORTED list.

Proposal 10: UE removes the RS whose measurements are reported from BEAM_ENTERING_LIST and BEAM_LEAVING_LIST. UE also includes the RS from BEAM_ENTERING_LIST whose measurements are reported in BEAM_REPORTED list.

MAC-Q01: Reporting for LTM2

We also agree that MAC CR needs update to handle LTM2 correctly. According to the current procedure text, if the periodic reporting need to work, the current beam from the serving cell need to be included in the lists MR_LIST, BEAM_ENTERING_LIST, BEAM_LEAVING_LIST, and BEAM_REPORTED_LIST, for LTM2. However this may need some more updates. For e.g. Type field needs to be updated to capture that it is not set for current beam in LTM2.

Proposal 11: Update the specification to clarify the handling of LTM2 based on MAC-Q01.

# Conclusion

Proposal 1: Rely on network implementation to avoid the issue described in MAC-V01.

Proposal 2: No need to discuss and specify anything in R19 for MAC-V02.

Proposal 3: As per the MAC specification CLTM TAT of target cell will continue to run after receiving an LTM Cell switch Command MAC CE with valid TA. In our view this is sufficient and no additional spec changes are needed.

Proposal 4: UE does not update CLTM TAT based on legacy Timing Advance Command MAC CE. No change to current specification.

Proposal 5: Do not pursue MAC-K01. If the source cell is an LTM candidate cell, upon Cell switch, UE does not store the valid TA and does not update its CLTM TAT of the source cell for possible RACH-less CLTM back to the source cell. No change to current specification.

Proposal 6: Do not pursue MAC-K02. UE does not need to report currently stored TAs to the NW to avoid unnecessary RA for early UL sync. No change to current specification.

Proposal 7: If the cell for which CLTM TAT is running is no longer a CLTM candidate, CLTM TAT should be stopped.

Proposal 8: We are ok to clarify that UE only perform RACH-based LTM for CLTM recovery.

Proposal 9: RAN2 confirms that there is no need to resend the measurements included in truncated MR in the following MR.

Proposal 10: UE removes the RS whose measurements are reported from BEAM_ENTERING_LIST and BEAM_LEAVING_LIST. UE also includes the RS from BEAM_ENTERING_LIST whose measurements are reported in BEAM_REPORTED list.

Proposal 11: Update the specification to clarify the handling of LTM2 based on MAC-Q01.

# References