3GPP TSG-RAN2 WG2 Meeting #131bis	R2-2507135

Prague, Czech Republic, 13 – 17 October 2025

Agenda Item:	10.4

Study Item:	FS_6G_Radio

Source: 	Qualcomm Incorporated

Title: 	Views on Mobility and RRM for 6G

Document for:	Discussion/Decision

# Background

This paper provides our views on some of the topics that will be addressed by this agenda item:

General mobility principles, types (e.g. L3, CHO, LTM, RLM/RLF, cell reselection), and measurements. Including Inter-RAT and intra-RAT mobility.

# Discussion

## Mobility principles for 6G

### Unified mobility framework

Over the 3GPP releases in NR (i.e., from Release 15-19), 5G ended up specifying different types of mobility mechanisms.

In Rel-15, the basic RRC-triggered mobility mechanism was specified via which the UE can be triggered to perform handover to a target cell via RRCReconfigWithSync (referred to as HO command). The HO command can be sent by the source gNB either upon receiving L3 measurements from the UE (indicating that there is a better cell to serve the UE) or can be sent “blindly” without considering measurements from the UE.

In Rel-16, Conditional HO (CHO) was specified which supported pre-configuring the UE with multiple candidate cell configurations including its associated execution conditions. This enabled timely execution of handover by UE and improved mobility robustness by preparing several candidate cells in advance and pre-configuring the UE, at the expense of reserving resources at the candidate cells prior to handover.

In Rel-18 and Rel-19, LTM (which is a L1/L2 triggered mobility mechanism) was specified and incorporated several useful features such as early UL/DL sync towards the candidate cells which enabled the UE to perform RACH-less LTM cell switch with the aim to reduce handover interruption time. Just like CHO, LTM also supports pre-configuring the UE in advance with the configurations of several LTM candidate cells. Rel-19 further specified CLTM as well, which enabled the UE to be configured with CLTM execution conditions as well.

With these NR mobility mechanisms being specified over multiple releases, there were a lot of redundant functionalities added. For example, CHO/LTM/CLTM all support pre-configuring the UE with candidate cell configurations and support their own signalling mechanisms to configure the UE. Also, both CHO and CLTM support configuring UE with execution conditions for the UE to trigger mobility based on the fulfilment of the execution conditions.

Also, there are two ways for network to trigger cell switch in 5G, either via an RRC-triggered HO command or via the Cell Switch Command MAC-CE. There are a lot of commonalities in both e.g., both commands carry a target cell ID, RACH resource information, SSB related information.

Also from the RAN architecture perspective, 5G supports duplicate functionalities among gNB-CU and gNB-DU. For example, cell switch decision can be taken by gNB-CU based on L3 measurements for L3 HO whereas cell switch decision can also be taken by gNB-DU based on L1/L3 measurements for LTM.

Also, the measurement configuration framework used for the different mobility mechanisms are quite fragmented e.g., L3 HO uses the RRC structure via measConfig to collect L3 measurements whereas LTM uses the CSI resource/reporting structure to collect L1 measurements. There are also different event triggered measurement reporting mechanisms, in RRC for L3 HO and in MAC for LTM.

Mobility mechanisms specified in 5G have a lot of duplicate and fragmented functionalities:

  - Two ways for network to trigger cell switch (RRC-based HO command and MAC-CE based Cell Switch command)

  - Multiple mechanisms to pre-configure the UE with candidate cell configurations (CHO vs. LTM)

  - Different execution conditions and recovery mechanisms for CHO and LTM

  - Fragmented measurement configuration and reporting framework for L3 HO and LTM

  - Cell switch decision taken by gNB-CU based on L3 measurements for L3 HO vs. taken by gNB-DU based on either L3 or L1 measurements for LTM

To avoid the duplicate and fragmented functionalities introduced by 5G mobility, RAN2 is therefore proposed to consider the following principle for 6G mobility:

6G should strive to support a "unified" mobility framework and aim to reduce duplicate configurations and functionalities as introduced in 5G over multiple releases (e.g., L3 HO, LTM)

### Prepared cell vs. non-prepared cell

As mentioned in section 2.1.1, 5G via CHO/LTM supports pre-configuring the UE with candidate cell configurations and its associated execution conditions after preparing the candidate cells in advance. This enables mobility to a “prepared” candidate cell.

5G also supports L3 HO to a non-prepared cell i.e., sending L3 HO command to a target cell whose configuration is not provided to the UE in advance. However, it is to be noted that 5G doesn’t support LTM cell switch to a non-prepared cell as this scenario is precluded by the specifications explicitly.

5G supports intra-NR mobility to a prepared cell (i.e., via pre-configuring UE with one or more candidate cell configurations during CHO/LTM/CLTM) and to a “non-prepared” cell (i.e., sending L3 HO command to a target cell whose configurations were not provided in advance to the UE) and specified them over different releases in NR.

For 6G, we propose the unified mobility framework to support mobility to prepared cells and non-prepared cells from day-1 and hence the following is proposed:

6G should support both intra-6G mobility to a prepared cell (i.e., via pre-configuring UE with one or more candidate cell configurations before the cell switch) and intra-6G mobility to a non-prepared cell (i.e., cell switch to a target cell whose configuration is not provided prior to the cell switch) from day-1.

### RACH-less vs. RACH-based

In order to support the IMT-2020 requirement of 0 ms mobility interruption time, DAPS was specified in 5G in Rel-16. However, the implementation of DAPS didn’t get traction in 5G due to its complexities.

In RAN#109, a similar requirement of 0 ms mobility interruption time was agreed in a pCR for 38.914 on Key performance indicators for 6G:

5.1.12	Mobility interruption time

Mobility interruption time shall be sufficiently minimized to support a seamless application performance. The minimum requirement for mobility interruption time is 0 ms at least for mobility between TRxPs of a single base station. The proponent can report the Mobility interruption time in other scenarios, where applicable.

Another mechanism to reduce mobility interruption time is performing early DL/UL sync towards the candidate cells as specified in NR for LTM. This is enabled via backhaul signaling between the source cell and the candidate cells to obtain the timing and beam information prior to the handover/cell switch so that the UE can perform a “RACH-less” cell switch and thereby reducing mobility interruption time.

Also, it is to be noted that RACH-less handover was specified in NR but was limited to intra-gNB handovers for both TN and NTN and for inter-gNB handovers only in case of NTN or mobile IAB-DU migration procedure (where timing of the target/candidate cell is known aprioiri). Also, unlike RACH-less LTM, RACH-less L3 HO doesn’t use any early DL/UL sync procedure.

RACH-less handover is supported in NR for intra-gNB handovers (for both TN and NTN), but was specified for inter-gNB handovers only in case of NTN and mobile IAB-DU migration procedure. Also, unlike RACH-less LTM, RACH-less handover doesn’t support early UL/DL sync procedures.

For 6G unified mobility, we propose to support “RACH-less” intra-6G mobility from day-1 e.g., via performing early DL/UL sync towards the target/candidate cells and this can also be a good step towards addressing the IMT-2030 requirement. But RACH-less mobility might not always be possible and hence 6G should also support RACH-based mobility.

6G should support both “RACH-less” intra-6G mobility (e.g., via early DL/UL sync towards target/candidate cells) and RACH-based intra-6G mobility.

## Measurement framework for 6G

In original releases of NR (Rel-17 or earlier), L1 CSI measurements and L3 measurements had different objectives and therefore had an independent measurement resource/report framework.

L1 CSI measurements were used for obtaining the channel quality measurements of the serving cell for the purpose of DL data reception, BM, etc. L3 measurements were used for obtaining the channel quality measurements of the serving and neighbour cell for the purpose of mobility.

In 5G RRC, SSB and CSI-RS resource configurations were defined independently in two different places for L1 and L3 measurement resources, even though this resulted in some duplicate configurations in RRC signalling. For LTM, a new dedicated IE for LTM CSI report and resource was however specified in R18/R19 due to lack of neighbouring cell support in L1 CSI report framework.

To avoid such duplicate measurement resource configurations, we propose 6G to strive to define unified measurement resource configuration structure for different types of measurements (e.g., L1 CSI measurement, L3 measurement),

Further, we also propose RAN2 to study that a unified measurement report configuration structure can be supported for 6G, wherein both serving cell CSI measurement (e.g., L1 CSI measurement) and serving/neighboring cell measurement for mobility (e.g., L3 measurement) are defined under a single report configuration structure.

Such a unified measurement framework would enable a simplified RRC configuration that can be applicable to both L1 CSI measurement and L3 measurement. This can also avoid duplication of CSI-RS/SSB configurations across L1 and L3 measurements, as was the case in 5G. This can also enable a scalable and forward-compatible measurement configuration, e.g., if we have to support early neighbouring cell CSI measurements.

5G measurement framework is quite fragmented and supports different measurement resource and reporting framework for L1/L3 measurements, serving/neighboring cells, SSB vs. CSI-RS.

6G should strive to support a unified measurement configuration framework e.g., via unification of measurement resource configuration structure (for L1/L3 measurements, for serving/neighbor cells/beams, for different types of RS) and unification of measurement report configuration structure.

## Inter-RAT mobility between 6G and 5G

In RAN#109, an initial set of architecture and migration requirements was agreed in RP-252870, including the following requirement for inter-RAT mobility:

The 6G RAN architecture shall support inter-RAT mobility between the 6GR and NR.

Inter-RAT mobility between 5G and 4G was specified via UE obtaining an RRC message with the target RAT configuration. The UE performs RACH in the target cell belonging to a different RAT and then applies the target RAT’s configuration after the inter-RAT mobility is successfully completed. There were no enhancements specified in 5G to optimize the interruption time or robustness for inter-RAT mobility between 5G and 4G. For example, features that were later introduced for intra-NR mobility (e.g., conditional mobility, early UL/DL sync, LTM) were never extended to inter-RAT mobility.

Several features supported for intra-NR mobility (e.g., conditional mobility, early UL/DL sync, LTM) are not supported for inter-RAT mobility between 5G and 4G

Inter-RAT mobility between 4G and 5G was kept simple because 4G doesn’t support many of the mobility features which were introduced in NR (e.g., LTM, early UL/DL sync). Even features such as CHO were introduced for LTE quite late, only after introducing them in NR. So, inter-RAT mobility between 4G and 5G was left untouched. Also, several of the mobility features in NR were introduced over the releases and not on day-1, thereby the motivation to change inter-RAT mobility was not significant.

Unlike the above, 5G and 6G might end up sharing a lot of common mobility features (e.g., conditional mobility, RACH-less mobility). And 6G “unified” mobility may support most of the useful features from day-1 itself.

Hence, for the inter-RAT mobility between 6G and 5G, we propose RAN2 to study whether features that will be supported by intra-6G unified mobility framework (e.g., RACH-less inter-RAT mobility, inter-RAT conditional mobility) can be supported for inter-RAT mobility between 6G and 5G as well, thereby improving the robustness and interruption time for inter-RAT mobility as well. Hence, the following is proposed:

RAN2 to study whether Inter-RAT mobility between 6G and 5G can leverage the same features supported by intra-6G mobility framework e.g., inter-RAT mobility to prepared/non-prepared cell, RACH-based/RACH-less mobility.

## Redirection between 6G and 5G

In 5G, inter-system fallback towards E-UTRAN is performed when 5GC does not support emergency services, voice services, for load balancing etc. Depending on factors such as CN interface availability, network configuration and radio conditions, the fallback procedure results in either RRC_CONNECTED state mobility (handover procedure) or RRC_IDLE state mobility (redirection).

Similarly, redirection between 6G and 5G should be supported. Therefore, the following is proposed:

RAN2 should study how to support redirection between 6G and 5G.

## AI/ML and observability for 6G mobility/RRM

In Rel-19, RAN2 studied several aspects such as measurement event prediction, RRM prediction, RLF prediction to improve mobility and RRM via utilizing AI/ML techniques and will now specify some of these aspects in Rel-20 for NR.

While designing the mobility and measurement framework in 6G, RAN2 should also study whether and how to incorporate AI/ML aspects (e.g., RRM prediction, measurement event prediction, RLF prediction that were studied for NR) to further improve the mobility/RRM related KPIs.

Another thing to consider while designing the measurement and mobility framework for 6G is to have an energy efficient mechanism from both UE and NW perspective. Some aspects such as measurement skipping or efficient measurement collection/reporting (e.g., based on AI/ML predictions) can lead to higher energy efficiency, and these aspects should also be considered.

Also, it is important for an operator to have good observability of the mobility related KPIs and monitor how well the mobility mechanisms are working e.g., performance monitoring mechanisms. The performance monitoring of mobility should be designed together with the fundamental mobility/RRM framework and not added “on top” only when AI/ML is added.

We therefore propose the following:

RAN2 should study how to incorporate AI/ML aspects (e.g., RRM prediction, measurement event prediction, RLF prediction) in 6G Mobility/RRM framework and also how to enable observability of mobility related KPIs from day 1.

# Summary

Based on the discussion above, we have following observations and proposals:

2.1 Mobility principles for 6G

2.1.1	Unified mobility framework

Observation 1: Mobility mechanisms specified in 5G have a lot of duplicate and fragmented functionalities:

- Two ways for network to trigger cell switch (RRC-based HO command and MAC-CE based Cell Switch command)

- Multiple mechanisms to pre-configure the UE with candidate cell configurations (CHO vs. LTM)

- Different execution conditions and recovery mechanisms for CHO and LTM

- Fragmented measurement configuration and reporting framework for L3 HO and LTM

- Cell switch decision taken by gNB-CU based on L3 measurements for L3 HO vs. taken by gNB-DU based on either L3 or L1 measurements for LTM

Proposal 1: 6G should strive to support a "unified" mobility framework and aim to reduce duplicate configurations and functionalities as introduced in 5G over multiple releases (e.g., L3 HO, LTM)

2.1.2	Prepared cell vs. non-prepared cell

Observation 2: 5G supports intra-NR mobility to a prepared cell (i.e., via pre-configuring UE with one or more candidate cell configurations during CHO/LTM/CLTM) and to a “non-prepared” cell (i.e., sending L3 HO command to a target cell whose configurations were not provided in advance to the UE) and specified them over different releases in NR.

Proposal 2: 6G should support both intra-6G mobility to a prepared cell (i.e., via pre-configuring UE with one or more candidate cell configurations before the cell switch) and intra-6G mobility to a non-prepared cell (i.e., cell switch to a target cell whose configuration is not provided prior to the cell switch) from day-1.

2.1.3	RACH-less vs. RACH-based

Observation 3: RACH-less handover is supported in NR for intra-gNB handovers (for both TN and NTN), but was specified for inter-gNB handovers only in case of NTN and mobile IAB-DU migration procedure. Also, unlike RACH-less LTM, RACH-less handover doesn’t support early UL/DL sync procedures.

Proposal 3: 6G should support both “RACH-less” intra-6G mobility (e.g., via early DL/UL sync towards target/candidate cells) and RACH-based intra-6G mobility.

2.2 Measurement framework for 6G

Observation 4: 5G measurement framework is quite fragmented and supports different measurement resource and reporting framework for L1/L3 measurements, serving/neighboring cells, SSB vs. CSI-RS.

Proposal 4: 6G should strive to support a unified measurement configuration framework e.g., via unification of measurement resource configuration structure (for L1/L3 measurements, for serving/neighbor cells/beams, for different types of RS) and unification of measurement report configuration structure.

2.3 Inter-RAT mobility between 6G and 5G

Observation 5: Several features supported for intra-NR mobility (e.g., conditional mobility, early UL/DL sync, LTM) are not supported for inter-RAT mobility between 5G and 4G

Proposal 5: RAN2 to study whether Inter-RAT mobility between 6G and 5G can leverage the same features supported by intra-6G mobility framework e.g., inter-RAT mobility to prepared/non-prepared cell, RACH-based/RACH-less mobility.

2.4 Redirection between 6G and 5G

Proposal 6: RAN2 should study how to support redirection between 6G and 5G.

2.5 AI/ML and observability for 6G mobility/RRM

Proposal 7: RAN2 should study how to incorporate AI/ML aspects (e.g., RRM prediction, measurement event prediction, RLF prediction) in 6G Mobility/RRM framework and also how to enable observability of mobility related KPIs from day 1.