3GPP TSG-RAN WG2 Meeting #131bis	R2-2507143

Prague, Czech Republic, Oct. 13th-17th

Agenda item:	10.4

Source:		LG Electronics Inc.

Title:		Consideration on mobility aspects for 6G

Document for:	Discussion

1.	Introduction

In this contribution, we examine key pain points in mobility procedures in NR and their implications for 6G. Based on these observations, we share our initial views on 6G mobility design, focusing on a unified mobility framework, fast PCell failure recovery, enhanced cell reselection, and advanced NTN mobility, aiming to support diverse deployment scenarios and UE types from 6G Day-1.

2.	Discussion

Mobility in RRC_CONNECTED and RRM measurements

Pain point #1: Increment of UE complexity and obstacles to evolution due to diverging mobility schemes and duplicated RRM procedures

In 5G NR, various mobility schemes (e.g., L3HO, LTM, DAPS, CHO, (S)CPAC) have been introduced. Diverging mobility schemes result in low market adoption and evolution hurdles. Further, the duplicated RRM procedures (i.e., cell-based condition with RRC reporting and beam-based condition with MAC CE reporting) complicate the specification and consequently increase UE complexity. Hence, it is necessary to study a unified design for various mobility schemes, where a single mobility core function is used to support all necessary mobility schemes/scenarios and a single RRM procedure can support both beam-level measurement reporting and cell-level measurement reporting.

LTM introduced in 5G Rel-18 has been receiving a big attention thanks to its low mobility interruption/latency. LTM is further enhanced in Rel-19 and Rel-20. LTM is designed to work well in CU-DU split architecture from a network architecture perspective. In our view, this functional split in network architecture can be supported in 6G because of its economic benefits. Hence, LTM can be a baseline of unified mobility framework for 6G. Taking LTM as baseline of 6G mobility has several implications.

In terms of mobility execution procedure, in 5G LTM procedure, DU triggers mobility execution by sending LTM cell switch command, while CU provides to the UE with the candidate configurations prior to LTM cell switch command. On the other hand (i.e., in 5G L3HO procedure), CU initiates mobility execution by sending the target configuration, i.e., CU sends to the UE the target configuration and cell switch command together. LTM based unified 6G mobility implies that mobility execution procedure would be initiated by triggering a cell switch command at DU (i.e., MAC) for both CU-initiated mobility and DU-initiated mobility. To this end, the mobility execution procedure for 5G LTM can be inherited for DU-imitated mobility, while a new signaling for mobility execution, e.g., CU-DU signaling for triggering cell switch command, may be necessary for CU-initiated mobility.

Proposal 1.	Study LTM based unified mobility framework supporting all mobility schemes/scenarios (e.g., L3HO, CHO, LTM, and CLTM), with the evaluation of DU-initiated approach and CU-initiated approach for mobility execution procedure.

Pain point #2: Signaling redundancy and reduced flexibility from split MAC-centric and RRC-centric measurement frameworks

In terms of RRM procedure, LTM taken as baseline for 6G mobility implies that the measurement reporting procedure would be MAC-centric, as opposed to RRC-centric legacy L3HO. In LTM, measurement report and cell switch command are transmitted as MAC Control Element (MAC CE). While the MAC-centric mobility procedure in 5G LTM is beneficial in reducing processing and transmission delay, it has a drawback of less flexible nature of MAC CEs, compared to a full flexibility offered by RRC. Considering that measurement report procedure serves various purposes such as serving cell management for CA and mobility and network performance optimization (SONMDT), etc., less flexibility of MAC CE based measurement reporting procedure may restrict applicability of the measurement reporting. Then, if a unified RRM procedure is pursued in 6G to support both beam-level measurement reporting and cell-level measurement reporting, the appropriate approach between MAC-centric approach and RRC-centric approach should be studied.

Further, in 5G, the reference signals pool for RRC-centric measurement reporting (i.e., L3 MR) and that for MAC-centric measurement reporting (i.e., L2 MR) are separately provided to the UE. Similarly, the configurations for RRC-centric and MAC-centric measurement reporting are also separately provided. The separate configuration for RRC-centric and MAC-centric approaches can result in signaling redundancy because the same reference signals can be provided for both RRC-centric and MAC-centric measurement reporting. In addition, redundant measurement reporting for a same cell triggered by both MAC-centric and RRC-centric approaches may happen. Therefore, unified signaling structure for both MAC-centric and RRC-centric measurement reporting should be studied.

Basically, beam-level measurements are fast, less averaged measurements providing quick feedback for beam management and LTM, while cell-level measurements use a longer-term view of channel conditions with L3 filtering to smooth fluctuations, preventing unnecessary handovers and reducing overhead for RRC signaling-based mobility events. From a mobility perspective, beam-level measurement provides a better radio quality during/after mobility execution but may lead to frequent mobility (i.e., ping-pong), as opposed to cell-level measurement based mobility. In our view, mixture of beam-level and cell-level measurement can improve mobility performance. For example, joint beam-level and cell-level measurements not only prevent frequent measurement reporting but also provide robust results compared to beam-level measurement based measurement reporting.

Proposal 2.	Study a single RRM procedure supporting beam-level, cell-level, and mixture of beam- and cell-level measurement/reporting, with the evaluation of MAC-centric approach and RRC-centric approach for measurement reporting procedure.

Pain point #3: Limited mobility optimization gain due to fragmented features tied to specific mobility schemes

Various mobility optimization features have been introduced in 5G, such as RACH-less mobility, early DL/UL synchronization, L2 reset skipping, and subsequent cell switching, etc. However, the use cases of the features are limited due to the diverged mobility schemes. For example, RACH-less L3 HO is only applicable for the case where the UE uses the same timing advance value (i.e., NTA value) at the target cell as in the source cell or timing advance value (i.e., NTA value) of 0. Early synchronization procedure, L2 reset skipping, and lower layer cell switch command are applicable for LTM but not for L3 HO. Subsequent cell switch is only available for LTM and SN mobility (i.e., CPAC). With the fragmented optimizations for the diverged mobility schemes, the eventual mobility performance optimization gain seems less promising. For measurement overhead reduction, dynamic L1 measurement and reporting configuration change will be specified in Rel-20, but only a network based solution (i.e., MAC CE based approach) is considered. UE based approach may be beneficial in terms of radio resource utilization by reducing signaling between network and UE.

In 6G, it is desired that essential mobility optimization features should be supported from 6G day-1 and all the features should be applicable for the mobility core function taken as a basis for 6G mobility schemes to be supported.

Proposal 3.	Study essential mobility optimization features to support from 6G day-1, including a) RACH-less cell switch, b) minimal L2 reset, c) subsequent cell switch, and d) measurement overhead reduction.

RLF recovery

In NR, when a PCell experiences a Radio Link Failure in single cell or carrier aggregation scenarios, the UE shall perform RRC re-establishment to recover the RRC connection, which leads to significant service interruption. In dual connectivity, however, the UE can report a PCell failure via the secondary cell group, enabling recovery without RRC re-establishment. This mechanism reduces service interruption and improves overall reliability.

Pain Points of NR PCell failure recovery

- Significant service interruption caused by RRC re-establishment based recovery from PCell failure in NR without dual connectivity

For 6G, we propose extending this principle to carrier aggregation scenario where dual connectivity is not configured. Specifically, when a PCell RLF occurs, the UE could use an SCell to notify the network of the failure and receive recovery command without performing RRC re-establishment. To support this, the reliability of the SCell must be ensured, for example by applying the radio link monitoring on the SCell, similar to the monitoring of SCG link in NR dual connectivity. With this mechanism, the network can rely on the SCell-based failure report and provide reliable recovery commands.

This enhancement would enable fast PCell failure recovery in carrier aggregation scenarios, minimize service interruption compared to conventional RRC re-establishment, and improve user experience with higher reliability in multi-carrier deployments.

Proposal 4	Study PCell failure recovery without interruption in 6G carrier aggregation where the UE can utilize SCell to report PCell RLF, enabling fast recovery without RRC re-establishment.

Mobility in RRC_IDLE/INACTIVE

In LTE and NR, basic cell reselection is strictly based on frequency priorities and radio quality. UE needs to select a cell on a highest ranked cell on a frequency of a highest priority. This reselection rule is simple but has several limitations. For example, UEs are concentrated on a frequency of highest priority as provided in system information, resulting in paging congestion and access congestion. Dedicated priority may relax such concentration somehow, but its applicability for RRC_IDLE/INACTIVE is limited due to necessity of dedicated configuration. To distribute UEs across frequencies of multiple frequencies, a simple distribution mechanism can be incorporated into cell reselection rule to be developed in 6G. MCLD (multi-carrier load distribution) introduced in LTE can be considered as a starting point. Another limitation arises in a scenario when different cells/frequencies may offer different set of services (or slices) and UE’s preferred services (or slices) are different across UEs. To better address this scenario, NR introduces a slice-aware reselection. In our view, the slice-aware reselection is very complicated and testability of this feature is extremely difficult, which makes the feature hardly deployable in the real fields. In 6G, we should seek for a better reselection, which must be less complex but fully functional.

Pain point of NR cell reselection

- UEs tend to be concentrated on a frequency of highest priority as provided in system information, resulting in paging congestion and access congestion.

- Slice-aware cell reselection is too complex and its testability is bad.

Proposal 5	Study enhanced cell reselection for network to enable camping cell distribution efficiently and to support network slicing in a simpler but fully functional manner.

As described in [1], we think that 6GR needs to be designed to support not only conventional cellular scenarios but also new potential 6G scenarios such as e.g. multi-carrier single/multi-cell, intra-frequency cell-cluster massive MTRP and MRSS scenarios. Considering new potential 6G scenarios, we can study how to support idle measurement and cell (re)selection applicable to new scenarios.

For instance, if MRSS is deployed, we assume that inter-RAT mobility between 5G cell and 6G cell in MRSS could frequently occur. With this assumption, inter-RAT mobility in MRSS can be supported in a more efficient way than mobility between LTE and NR. Especially, 5G cell and 6G cell can be well coordinated for MRSS, so that inter-RAT mobility can occur during initial access e.g. based on a network command.

Proposal 6	Study idle measurement and cell (re)selection for new potential 6G scenarios e.g. a multi-carrier single/multi-cell, intra-frequency cell-cluster massive MTRP and MRSS scenarios

NTN mobility

In NTN scenario, the latency for the HO procedure could be huge due to the nature of the very large propagation delay. To be specific, when the UE receives a HO command from the serving cell, communication through the serving cell becomes impossible, and communication through the target cell can start after the connection setup procedure for the target cell is completed. Therefore, in NTN scenario, the very large propagation delay will cause huge service interruption time until the HO procedure is completed for both HO between TN and NTN and between NTN and NTN compared to TN scenario. To mitigate this inefficiency, in 5G NR, the conditional HO and RACH-less HO are introduced for the HO enhancement. Moreover, since the RSRP measurement results could be similar within a cell area in NTN scenario, the location-based handover and cell managements were introduced for NR NTN. In 6GR, the advanced HO mechanisms need to be considered from the early stage of 6GR.

Meanwhile, the location-based mechanism (e.g., location-based CHO/SMTC) introduced for NR NTN basically assumes that all the UEs knows the valid UE location (based on GNSS). However, in 6GR NTN, this assumption would be no longer valid to extend the applicable UE types for the NTN operation. Considering that the RSRP measurement-based operation would be still inefficient or infeasible in NTN scenario, some enhancement would be needed to cover the 6GR GNSS-less UE. In 6GR, the advanced radio measurement-independent mechanisms for the GNSS-less UE need to be considered from the early stage of 6GR.

In NR NTN, due to the satellite moving, even for the earth-fixed cell, the UE needs to perform the satellite switching for the same cell ID. In this case, the UE can receive SS/PBCH from the target satellite for the DL resynchronization and the UL resynchronization can be done based on the UE location and the location of the target satellite. However, in 6GR, in case of the GNSS-less UE, it would be necessary that the UEs perform PRACH transmission to the target satellite for the UL resynchronization during the satellite switching. In this case, huge number of UEs may need to transmit PRACHs to the target satellite. In case of earth-moving cell, a number of (RRC CONNECTED) UEs may need to performs the HO to the target cell associated with the target satellite. If the UE has the valid UE location information, RACH-less HO can be used, but the UL transmission for the HO confirmation could be excessive. If the UE has no valid UE location information, a number of UEs needs to transmit PRACH. It may cause the NW burden and increase satellite switching/HO failure and/or latency, especially when the bandwidth is narrow.

For the successful commercialization of the 6GR NTN, it is necessary to consider the enhanced throughput and reliability. Meanwhile, due to the satellite moving, the service interruption due to the satellite switching or HO will frequently occur. To be specific, until the completion of the satellite switching or HO, the ongoing service provision will be halt. Depending on the target scenario (e.g., voice call or emergency alert), the service continuity would be critical, so this kind of the service interruption need to be avoided. Alternatively, it can be considered that the GEO-based NTN operation is used, but it does not fit with the real-time services due to the huge propagation delay. In the real field, a number of satellite services consider the hybrid approach with a combination of GEO and LEO. To be specific, the GEO satellite is used for the service continuity and the LEO satellite is used for services that require relatively low latency and/or high throughput. In 6GR, it would be necessary to consider the multi-orbit scenario. In a similar approach, it can be considered that the multi-connection between TN and NTN. For the multi-connection between NTNs of different orbits or between TN and NTN, the relevant management mechanisms need to be carefully designed considering the NTN characteristics. For the enhanced throughput compared to 5G NR NTN, the multi-carrier operation (for both intra-satellite and inter-satellite) can be further considered in NTN scenario. According to the NTN characteristics, the large propagation delay, the limited EIRP at the satellite, and the limited number of active beams would need to be considered for the efficient multi-carrier management.

Pain Points of NR NTN mobility

- Significant service interruption, high-latency HO, simultaneous HO of large numbers of UEs, limited throughput/reliability and increased UE implementation complexity due to satellite mobility, long propagation delay, low link budget, and UE location info acquisition based on non-3GPP technology.

Proposal 7	Study mobility mechanisms that consider NTN-related aspects such as a) seamless TN-NTN/NTN-NTN mobility with low latency, b) GNSS-independent UE mobility management, c) mobility handling for large numbers of UEs, d) single/multi-cell operation based on multiple satellites (in different bands/orbits), e) multi-connection of TN and NTN.

3.	Conclusion

In this contribution, we present our view on pain points of NR to be addressed in 6G design as follows:

Paint points of NR

Mobility in RRC_CONN and RRM measurements

- Increment of UE complexity and obstacles to evolution due to diverging mobility schemes and duplicated RRM procedures

- Signaling redundancy and reduced flexibility from split MAC-centric and RRC-centric measurement frameworks

- Limited mobility optimization gain due to fragmented features tied to specific mobility schemes

RLF recovery

- Significant service interruption caused by RRC re-establishment based recovery from PCell failure in NR without dual connectivity

Mobility in RRC_IDLE/INACTIVE

- UEs tend to be concentrated on a frequency of highest priority as provided in system information, resulting in paging congestion and access congestion.

- Slice-aware cell reselection is too complex and its testability is bad.

NTN mobility

- Significant service interruption, high-latency HO, simultaneous HO of large numbers of UEs, limited throughput/reliability and increased UE implementation complexity due to satellite mobility, long propagation delay, low link budget, and UE location info acquisition based on non-3GPP technology.

Proposals

Proposal 1.	Study LTM based unified mobility framework supporting all mobility schemes/scenarios (e.g., L3HO, CHO, LTM, and CLTM), with the evaluation of DU-initiated approach and CU-initiated approach for mobility execution procedure.

Proposal 2.	Study a single RRM procedure supporting beam-level, cell-level, and mixture of beam- and cell-level measurement/reporting, with the evaluation of MAC-centric approach and RRC-centric approach for measurement reporting procedure.

Proposal 3.	Study essential mobility optimization features to support from 6G day-1, including a) RACH-less cell switch, b) minimal L2 reset, c) subsequent cell switch, and d) measurement overhead reduction.

Proposal 4	Study PCell failure recovery without interruption in 6G carrier aggregation where the UE can utilize SCell to report PCell RLF, enabling fast recovery without RRC re-establishment.

Proposal 5	Study enhanced cell reselection for network to enable camping cell distribution efficiently and to support network slicing in a simpler but fully functional manner.

Proposal 6	Study idle measurement and cell (re)selection for new potential 6G scenarios e.g. a multi-carrier single/multi-cell, intra-frequency cell-cluster massive MTRP and MRSS scenarios

Proposal 7	Study mobility mechanisms that consider NTN-related aspects such as a) seamless TN-NTN/NTN-NTN mobility with low latency, b) GNSS-independent UE mobility management, c) mobility handling for large numbers of UEs, d) single/multi-cell operation based on multiple satellites (in different bands/orbits), e) multi-connection of TN and NTN.

Reference

R2-2507141 Consideration on general aspects for 6G		LG Electronics Inc