3GPP TSG-RAN WG2 Meeting #131bis										R2-2507196

Prague, Czech Republic, October 13th – 17th, 2025

Agenda Item:	9.7.2

Source: 	Sharp

Title:  	UP Solution vs CP Solution for Voice Support over NB-IoT-NTN

Document for: 	Discussion

# Introduction

In IoT NTN Phase 4 WID [1], down-selection between CP and UP solutions is captured as follows:

- Down-select between CP and UP solutions for voice support over NB-IoT-NTN until RAN#110 [RAN2] - Coordination with SA is required - The objective can be updated with further details based on the outcome of the above down-selection

In an SA4 LS [2], SA4 asked RAN2 comments on voice service over NB-IoT-NTN, as follows:

ACTION: SA4 kindly asks SA2 and RAN2 to comment on - the different options among UP/CP and IP/Non-IP and the respective overall packet overhead (including RTP/UDP/IP with RoHC, PDCP, RLC and MAC and any potential AS layer optimization if applicable), and if there is any preferred option, and - specifically, whether a packet overhead of 1 byte of MAC header is realistic.

The LS asks two things:

(1) Preferred option among CP/UP solutions

(2) Feasibility of 1 byte of MAC header.

This document discusses how to address the issues from the WID and the SA4 LS.

# Discussion

## UP Solution vs CP Solution

The main difference between UP solution and CP solution is which radio bearer carries the voice data. Voice data is considered as user traffic. Traditionally, DRB is being used to carry user traffic. In this context, UP solution is desirable.

Voice data is considered as user traffic which is being served by DRB.

TR 23.700-19 [3] already provided rough overhead analysis for both UP solution and CP solution. Both solutions assume RTP/UDP/IP on top of 3GPP protocols, so ROHC will be common for both solutions. MAC header is also common. The only difference in header overhead may occur due to PDCP/RLC headers in UP solution and NAS/RRC/RLC headers in CP solution.

Based on the current standards without consideration of any enhancements, the UP solution seems to have total 2-byte header (1 byte in PDCP and 1-byte in RLC) and CP solution seems to have 15 bytes (11 bytes in NAS, 2-bytes in RRC and 2-byte in RLC). UP solution looks better from the overhead perspective. However, we do not see it is a big difference as we may consider enhancements for further overhead reductions for both solutions, e.g., NAS overhead reduction in other WGs or AS overhead reduction in RAN2.

UP solution has smaller overhead, but the difference can be minimized by further enhancements.

For CP solution, we expect more work in RAN2. For example, dedicated SRBs for voice traffic and potentially IMS control signaling should be defined. Also, voice data is usually served by RLC UM. SRB with RLC UM should be defined whereas DRB already allows it. Moreover, UP solution can just reuse existing PDCP PDU format and signaling for ROHC whereas CP solution assumes PDCP bypass.

CP solution requires more RAN2 work whereas UP solution can reuse existing features.

Considering the observations above, we see that UP solution is preferred.

RAN2 prefers a UP solution for voice support over NB-IoT-NTN.

## 1-byte MAC header

1-byte MAC header is possible only if the MAC PDU includes only one MAC SDU without any MAC CE nor padding, i.e., E field is set to 0. It means that if the condition is not met, MAC header is larger than 1-byte. The issue is whether the condition of 1-byte is always guaranteed. As specified in the WID, a basic assumption is that SPS is used, and the voice packet size is fixed. Ideally, it may guarantee 1-byte MAC header. However, we cannot always guarantee 1-byte.

The first reason is ROHC protocol. Both UP solution and CP solution assume to use ROHC. Depending on ROHC state, e.g., IR/FO/SO, the exact size could vary. Even if SPS is used, padding or segmentation may be possible.

The second reason is lower priority of data than MAC CE or SRB. When such messages interrupt the data transmission, segmentation may happen. It increases the size of both MAC header and RLC header.

Thus, we see that 1-byte MAC header is not realistic from RAN2 point of view.

RAN2 to reply that 1-byte MAC header is not realistic.

# Conclusions

Based on the discussion given above, RAN2 is requested to agree the following proposals:

- RAN2 prefers a UP solution for voice support over NB-IoT-NTN.

- RAN2 to reply that 1-byte MAC header is not realistic.

# Reference

[1] RP-252473	Revised WID: Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 4	vivo (Moderator)

[2] R2-2505068/S4-251583	LS on the RAN simulation assumptions for ULBC	SA4

[3] 3GPP TR 23.700-19	Study on Integration of satellite components in the 5G architecture Phase 4	v1.0.0 (2025-09)