3GPP TSG RAN WG2 Meeting #131bis                                                       R2-2507197

# Prague, Czech Republic, Oct. 13th-17th, 2025

Agenda item:	8.2.2

Source:	Ofinno

Title:	Views on remaining issue 3-7

Document for:	Discussion and decision

# Introduction

This document discusses the [issue 3-7] identified during email discussion [POST131][021][AIoT] MAC spec [1].

| Issue 3-7: more data indication handling | How to set “more data indication” value in case of no NAS response available (i.e., zero SDU) - Relevant agreements: - The reader determines no data available case by SDU length 0. As more data indication is mandatory, the device sets this bit to "0". - The reader, in response to 0 SDU in the device’s MAC response, may send a follow-up R2D Upper Layer Data Transfer message at a later time to schedule another D2R Upper Layer Data Transfer message from the device. - The follow-up R2D Upper Layer Data Transfer message includes the Received Data Size field with the Received Data Size field set to value 0, without including the original command. - RAN2 would like to check if there is a case where NAS doesn’t provide a response at all. If this case exists, RAN2 will discuss this issue. - Regarding the above check point, it can be up to companies’ contribution. - Status in running CR: captured. | To be discussed by company contributions |
| --- | --- | --- |

# Discussion

TS 24.369 capture two kind of scenarios in which a NAS command request is considered as “not successful” by A-IoT NAS device point of view:

- Scenarios (A) A-IoT NAS does not respond. The scenarios (A) include when A-IoT fails the authentication procedure as specified in 3GPP TS 33.369, in which case A-IoT devices does not respond to the A-IoT paging message, or when NAS message/IEs cannot be parsed/decoded (e.g., message being too short, syntactically incorrect mandatory IE, messages with semantically incorrect contents, unknown/unforeseen message type, or unknown/unforeseen IE in the non-imperative message part), in which case the UE ignores the IE/message.

- Scenarios (B) A-IoT NAS responds with a NAS reject message. The scenarios (B) include not supporting a command message (to write or read), a command type specific parameter being invalid, not being able to execute successfully a command (write or read), application data not being found to be read in the requested place of the user memory, or not being able to implement a write command due to running out of energy. For any of these scenarios, a NAS reject command message is sent by the A-IoT NAS device to the AIOTF.

CT1 defines a group of cases, described in this document as scenarios (A), in which the A-IoT NAS does not respond. Scenarios (A) includes failure to authentication (as specified in SA3) or a failure to parse/decode a NAS message/IEs.

CT1 defines a group of cases, described in this document as scenarios (B), in which the A-IoT NAS responds but with a NAS reject command (for write or for read). Scenarios (B) includes not supporting a command, not being able to successfully execute a command, or not being able to implement a command.

For A-IoT, a reader pre-allocates the D2R scheduling information when providing the R2D message including inventory or a command to be requested. For scenarios (A), A-IoT NAS would not generate any NAS PDU. Considering previous RAN2 agreement, A-IoT MAC could rely on sending SDU length set to 0 and more data indication (MDI) set to 0 to indicate that there is no NAS response. However, this would not help the network to differentiate/know when the failure scenarios (A) happened. On other hand, an A-IoT device (in MAC) is currently not allowed response again to a paging for the same transaction ID when it has already been received (i.e. the transaction ID is already stored in the A-IoT MAC of the device). In our understanding, RAN2 should discuss whether any action may be required in the AS layer for these failure scenarios (A); whether the failure happening in the upper layers (e.g., to authenticate or to parse/decode the NAS message) needs to be available (visible) to the A-IoT MAC of the device and/or to the Reader. It is important to highlight that the A-IoT MAC has a valid D2R scheduling configuration (to be used) and could potentially be used if required to inform of the failure to the Reader. Moreover, the A-IoT MAC of the device could allow future retries for the same transaction ID in case of the failure (e.g., by releasing the stored transaction ID and AS ID).

When the A-IoT NAS does not respond to an inventory or command due to a failure in the upper layers (e.g., to authenticate or to parse/decode the NAS message), the A-IoT MAC of the device still has a valid D2R scheduling information to be used.

- For the scenarios in which A-IoT NAS does not respond to an inventory or command due to a failure determined by the upper layers (e.g., to authenticate or to parse/decode the NAS message), RAN2 should discuss whether the A-IoT MAC needs to be aware of this failure in the upper layers and if so, whether any action is required (e.g., to release the transaction/AS ID and/or to inform the reader of the upper layer failure).

- For the scenarios in which A-IoT NAS responds with a reject command message, to discuss whether the A-IoT MAC needs be aware of the rejection (e.g., to release the stored transaction ID and allow a re-access to the same transaction).

# Conclusion

The observations captured are the following:

Observation 1.	CT1 defines a group of cases, described in this document as scenarios (A), in which the A-IoT NAS does not respond. Scenarios (A) includes failure to authentication (as specified in SA3) or a failure to parse/decode a NAS message/IEs.

Observation 2.	CT1 defines a group of cases, described in this document as scenarios (B), in which the A-IoT NAS responds but with a NAS reject command (for write or for read). Scenarios (B) includes not supporting a command, not being able to successfully execute a command, or not being able to implement a command.

Observation 3.	When the A-IoT NAS does not respond to an inventory or command due to a failure in the upper layers (e.g., to authenticate or to parse/decode the NAS message), the A-IoT MAC of the device still has a valid D2R scheduling information to be used.

The proposals captured are the following:

Proposal 1.	For the scenarios in which A-IoT NAS does not respond to an inventory or command due to a failure determined by the upper layers (e.g., to authenticate or to parse/decode the NAS message), RAN2 should discuss whether the A-IoT MAC needs to be aware of this failure in the upper layers and if so, whether any action is required (e.g., to release the transaction/AS ID and/or to inform the reader of the upper layer failure).

Proposal 2.	For the scenarios in which A-IoT NAS responds with a reject command message, to discuss whether the A-IoT MAC needs be aware of the rejection (e.g., to release the stored transaction ID and allow a re-access to the same transaction).

# Reference

- R2-2507030, Email discussion [POST131][021][AIoT] MAC spec CR and open issues, October 2025.