3GPP TSG RAN WG2 Meeting #131bis                                                       R2-2507198

# Prague, Czech Republic, Oct. 13th-17th, 2025

Agenda item:	9.2.2

Source:	Ofinno

Title:	RAN2 impacts to support D2T2 for DT and DO-DTT traffic

Document for:	Discussion and decision

# Introduction

This document discusses RAN2 impacts to support D2T2 for DT/DO-DTT traffic as captured in Rel-20 WID [1].

Specify support for Deployment Scenario 2 with Topology 2 with intermediate UE as Reader under the following conditions [RAN2-led]:

- According to D2T2-B for Device 1 with carrier-wave transmission for waveform 1 only, per TR 38.769.

  - No specification support for CW control

- According to D2T2-C for active Device(s).

  - Start after March 2026

- Traffic types DO-DTT and DT, and DO-A (only for active Device(s)).

  - RAN2 to start on DO-A for D2T2 when sufficient progress has been made on DO-A for D1T1, after a checkpoint in RAN plenary in September 2026

- Representative use cases rUC1 (indoor inventory) and rUC4 (indoor command).

- FR1 licensed spectrum in FDD, with R2D in UL spectrum, and D2R and CW in UL spectrum.

- Specify a single architecture by down-selection between RRC-based architecture and UP-based architecture, in coordination with SA2.

- The following are not supported: Coordination btwn UEs operating as UE readers; New Uu protocol layer(s); Simultaneous (time overlapping) operation of Uu and A-IoT at intermediate UE in the same band

The work is subject to the following constraints:

RAN1

- The objective includes specification of necessary power control and collision handling procedures between intermediate UE and BS. No physical layer impact for signaling A-IoT resources to the intermediate UE.

- No additional specification change on R2D and D2R for Device 1 in Rel-20

RAN2

- The objective includes specification of necessary Uu procedures to enable the A-IoT operations.

- RRC CONNECTED UE reader only, including RLF, without specific enhancement for other temporary out-of-connection scenarios.

- A-IoT radio resource of the UE reader is only valid within the serving cell.

- Resource allocation to the UE Reader shall be via Uu RRC signaling.

RAN3

- Specify signaling for Topology 2.

- The objective includes specification of UE reader authorization, including F1AP support, and UE reader selection. NOTE: F1AP/XnAP/NGAP impact is expected to be minimized.

- Inter-gNB resource coordination is not specified.

- Specify the necessary signaling support for inter-gNB RRC-connected UE Reader Mobility

# Discussion

## SA2 and RAN2 Rel-20 alignment

The following figure, from Rel-18 TR 38.848 [2], provides an overview of Topology 2 with intermediate node (i.e., UE Reader). Rel-20 A-IoT limited the scope to enable UE Reader operation when in connected mode.

Topology 2 refers to a A-IoT device having bidirectional communication with a base station via an intermediate node (intermediate UE acting as a Reader, i.e. UE Reader), and Rel-20 scope is focused in UE Reader in RRC_CONNECTED.

RAN2 studied architecture/protocol stack options, as captured in Rel-19 TR 38.769 [3]. RAN2 is task to down-select between RRC-based and UP-based architecture in coordination with SA2 for Rel-20 RAN A-IoT WI [1] as highlighted in yellow is the objective copied above. On other hand, SA2 is only focusing their study on RRC-based architecture as highlighted in blue below in the corresponding Rel-20 SA A-IoT SID [4]:

WT#1: Study the support of AIoT services in Topology 2, including:

-	 Study and investigate how to support AIoT services under the RRC-based option for UE Reader connectivity.

To enable topology 2 in Rel-20, RAN2 aims to down select between RRC-based or UP-based architectures (in coordination with SA2) while SA2 is only focus their study on RRC-based.

- For topology 2, RAN2 needs to discuss an alignment of the discussion with SA2 on the architecture study: option (1) RAN2 aligns its objective with SA2 (i.e., RRC-based approach is adopted and not UP-based approach), or option (2) RAN2 studies both approaches and informs SA2 (i.e., SA2 and RAN2 are expected to study both RRC-based and UP-based approach).

The following sections aim to identify RAN2 areas of discussion considering Rel-19 study captured in TR 38.769 [3].

## A-IoT radio resources

TR 38.769 [3] captured the following on the A-IoT radio resources used to enable Topology 2:

The radio resources used by A-IoT radio interface between the A-IoT device(s) and UE reader are controlled by the network. The radio resources, which are dedicated for a UE reader, are only configured to the UE reader via dedicated signalling. The mechanisms for shared resource pool amongst UE readers are not considered in this release. The UE reader in coverage of BS scenario is supported. The UE reader may perform the A-IoT procedure on A-IoT radio interface between the reader and the device(s), only if the radio resource configuration is valid in the cell, which is under network control. The radio resource validity across multiple cells is not supported in this release.

For the case when the radio resources are allocated semi-statically by the network, the radio resource validities are studied in the following scenarios with candidate options (down-selection can be decided in the WI phase):

-	Scenario 1: In RRC_CONNECTED, the UE reader can perform A-IoT operations using the allocated A-IoT radio resource for A-IoT radio interface:

- Option 1:	UE reader receives the resources configuration in NR Uu dedicated RRC signalling. The radio resources remain valid until the network releases them explicitly. This would be supported by the NR Uu RRC reconfiguration; and/or

- Option 2: 	UE reader receives the resources configuration in NR Uu dedicated RRC signalling, which configures a time period in which the corresponding resource can be used. The UE reader considers that the resources remain valid for that time period, unless the resource configuration is explicitly released by the network by the NR Uu RRC reconfiguration.

-	Scenario 2: In RRC_CONNECTED, the UE reader can perform A-IoT operations using the allocated A-IoT radio resource for A-IoT radio interface, upon its temporary out of connection scenarios (e.g., RLF and handover cases):

- Option 1: The UE reader considers the resources as temporarily invalid during the temporary out of connection condition. The resource may become valid again after the UE recovers from the condition.

- Option 2: The UE reader considers that the resources remain valid for a time period. It needs to be further discussed on whether this time period belongs to the one in Option 2 for Scenario 1 or is a different time period. It needs to be further discussed on whether the duration of this time period is related to the ongoing A-IoT operations.

NOTE 1:	The A-IoT radio resource can be (re)configured by UE reader’s RRC reconfiguration (including during handover procedure, after re-establishment of the UE reader, etc.)

RAN2 should continue their Rel-19 work on identified impacts on this to enable Topology 2.

- To confirm that A-IoT radio resources are under network control and can be (re)configured by gNB to the UE reader in RR_CONNECTED via RRC reconfiguration including for handover, resume or re-establishment. FFS on details.

- To discuss when/how the A-IoT radio resources become invalid or released, considering option 1) explicit network release, and/or option 2) implicit release (e.g., releasing after a period of time).

- To discuss whether/how to handle A-IoT radio resources when a UE reader is temporary out of coverage (e.g., RLF and handover), considering option 1) temporarily invalid and/or option 2) temporarily valid (i.e., valid for a period of time after being out of coverage and invalid afterwards).

TR 38.769 [3] also captured the following on how the network (BS) gets information of the A-IoT radio resources used to enable Topology 2:

For the radio resources allocation request, following alternatives are studied:

-	Alternative 1: BS configures/allocates a set A-IoT radio resource to the UE reader, in response to a request from the UE reader; and/or

-	Alternative 2: BS configures/allocates a set A-IoT radio resource to the UE reader, based on the service request from CN.

NOTE 2:	The above two alternatives can co-exist. The configuration for radio resource allocation may also include the validity criteria (if applicable, see the time period in previous option 2 of those scenarios)

Both alternatives 1 and 2 can co-exist as RAN2 concluded; if agreed, the enabling of alternative 2 should be driven by other WGs (SA2/RAN3) or leave it up to network implementation (assuming the required information is already known/visible to the gNB) while alternative 1 should be driven (studied) by RAN2. If RAN2 confirms the need for a UE reader to request A-IoT radio resources, RAN2 would need to discuss how this SR like may be done e.g., whether A-IoT specific/differentiation may be needed in the RB/LCH level.

- To discuss whether/how the UE reader requests A-IoT radio resources to the gNB and wait for SA2/RAN3 input on which A-IoT CN information (if new one) is provided.

## Energy related Assistance Information

RAN2 should confirm from the beginning of the Rel-20 on the intention to enable a mechanism for A-IoT devices to provide energy related assistance information to reader/network considering that this had a large support during Rel-19 discussion and was not considered due to the lack of time.

Energy Status Indication

R2-2506048	Ambient IoT data transmission and energy status indication	MediaTek Inc., Vodafone, Nokia, Samsung, NTT DOCOMO INC., InterDigital, T-Mobile USA	discussion	Rel-19	Ambient_IoT_Solutions-Core	R2-2506046

Proposal 1: Specify a mechanism for the device to indicate to the reader if the device can complete a requested command procedure when considering its energy status and radio conditions (taking into account both R2D and D2R directions, and with no requirement on the reader to take any specific action on the indication).

- Can consider it for Rel-20

- To enable a mechanism for a Rel-20 A-IoT device to provide energy related assistance information to the network. Details FFS.

# Conclusion

The observations captured are the following:

Observation 1.	Topology 2 refers to a A-IoT device having bidirectional communication with a base station via an intermediate node (intermediate UE acting as a Reader, i.e. UE Reader), and Rel-20 scope is focused in UE Reader in RRC_CONNECTED.

Observation 2.	To enable topology 2 in Rel-20, RAN2 aims to down select between RRC-based or UP-based architectures (in coordination with SA2) while SA2 is only focus their study on RRC-based.

The proposals captured are the following:

Proposal 1.	For topology 2, RAN2 needs to discuss an alignment of the discussion with SA2 on the architecture study: option (1) RAN2 aligns its objective with SA2 (i.e., RRC-based approach is adopted and not UP-based approach), or option (2) RAN2 studies both approaches and informs SA2 (i.e., SA2 and RAN2 are expected to study both RRC-based and UP-based approach).

Proposal 2.	To confirm that A-IoT radio resources are under network control and can be (re)configured by gNB to the UE reader in RR_CONNECTED via RRC reconfiguration including for handover, resume or re-establishment. FFS on details.

Proposal 3.	To discuss when/how the A-IoT radio resources become invalid or released, considering option 1) explicit network release, and/or option 2) implicit release (e.g., releasing after a period of time).

Proposal 4.	To discuss whether/how to handle A-IoT radio resources when a UE reader is temporary out of coverage (e.g., RLF and handover), considering option 1) temporarily invalid and/or option 2) temporarily valid (i.e., valid for a period of time after being out of coverage and invalid afterwards).

Proposal 5.	To discuss whether/how the UE reader requests A-IoT radio resources to the gNB and wait for SA2/RAN3 input on which A-IoT CN information (if new one) is provided.

Proposal 6.	To enable a mechanism for a Rel-20 A-IoT device to provide energy related assistance information to the network. Details FFS.

# Reference

- RP-251885, New WID on Solutions for Ambient IoT (Internet of Things) in NR Phase 2, Rel-20, June 2025.

- 3GPP TR 38.848, Study on Ambient IoT (Internet of Things) in RAN, Rel-18.

- 3GPP TR 38.769, Study on solutions for Ambient IoT (Internet of Things) in NR, Rel-19.

- SP-250834, SID on Architecture support of Ambient power-enabled Internet of Things - Phase 2, Rel-20.

- 3GPP TR 23.700-30, .