3GPP TSG RAN WG2 Meeting #131bis                                                             R2-2507199

# Prague, Czech Republic, Oct. 13th-17th, 2025

Agenda item:	8.12.2

Source:	Ofinno

Title:	Remaining MAC issues in MIMO

Document for:	Discussion and decision

# Introduction

According to the conclusion of the open issues discussion in the post 131 email discussion [Post131][217][MIMO_Ph5] CR for TS 38.321 (Samsung) [1], contributions are expected on the following proposals:

R2-250xxxx Report of Rel-19 MIMO MAC open issues for maintenance (Samsung) Proposal 1: For mode-B UEI reporting, PUCCH and type-1 CG PUSCH resource can be associated with different TAGs. Discuss UE behavior when the TAT for PUSCH is expired while the TAT for PUCCH is running. - Option-1: release the associated PUCCH; - Option-2a: not release PUCCH and transmit the Report Indication on PUCCH; - Option 2b: not release PUCCH and not transmit the Report Indication on PUCCH. Proposal 2: Discuss whether to transmit UEI Report Indication on PUCCH if the BWP for the associated mode-B CG PUSCH is deactivated. Proposal 5: For multipanelSchemeSDM, discuss if the UE should clear the CG/PUSCH for SP-CSI if any of the activated TCI state(s) for the CG/PUSCH for SP-CSI is associated with the TAG of the expired TAT.

In this contribution, we would like to share our views on each of the above proposals/open issues.

# Discussion

## TAT expiry of sTAG for UEIBR

In the current procedure, when the TAT for an SCell expires, all PUCCH resources and configured uplink grants associated with that SCell are released by the RRC or cleared at the MAC layer, respectively. This can also be applied to UE-initiated CSI reporting because the PUCCH/PUSCH resources cannot be used on a SCell when the TAT of the SCell expires. In particular, if a UE-initiated report configuration uses a PUCCH resource linked to the expired TAT on the SCell, that resource is released by RRC. Similarly, if a Type-1 CG for a UE-initiated report configuration is associated with the expired TAT on that SCell, it is cleared by MAC. The benefit of these behaviors is that the network can reassign the released/cleared resources to other UEs for better resource efficiency and system capacity. The above behaviors have been agreed by RAN2#131:

RAN2#131 Agreement - If the PUCCH of a UEI report configuration is pointed to a SCell whose TAT of the single sTAG is expired, this PUCCH for the SCell is released by RRC. If the type-1 CG of a UEI report configuration is pointed to a SCell whose TAT of the single sTAG is expired, this type-1 CG for the SCell is cleared as a configured UL grant. There is no MAC specification impact

For mode-B, if the PUCCH resource and the Type 1 CG are configured in the serving cell(s) associated with the same TAG, there is no issue. However, if the PUCCH resource and the Type 1 CG are configured on different serving cells associated with different TAGs, there are two cases, as illustrated in Figure 1, worth highlighting for further discussion.

- Case 1: If the TAT of the TAG (e.g., TAG 2) associated with the Type 1 CG expires while the TAT for the PUCCH-associated TAG (e.g., TAG 1) is still running, following the RAN2#131 agreement, the UE only clears the Type 1 CG. In this case, since the UE cannot use the Type 1 CG, the UE’s behavior requires clarification, specifically addressing whether the UE should transmit the PUCCH notification when the associated Type 1 CG for CSI report is unavailable.

- Case 2: The TAT of the TAG associated with the PUCCH resource (e.g., TAG 1) expires, while the TAT of the TAG associated with the Type 1 CG (e.g., TAG 2) remains running. Following the RAN2#131 agreement, the UE releases the PUCCH resource but retains the Type 1 CG. However, since the UE cannot perform the PUCCH transmission for the UEI report, the retained Type 1 CG cannot be utilized.

Figure 1: TAT expiry cases for different Cells with different TAGs

In the MAC open issues email discussion [1], the Case 1 (If the TAT for Type 1 CG/PUSCH is expired while the TAT for PUCCH is running) has been discussed, and the following three options are on the table:

- Option 1: release the associated PUCCH;

- Option 2a: not release PUCCH and transmit the Report Indication on PUCCH;

- Option 2b: not release PUCCH and not transmit the Report Indication on PUCCH.

In such case, we prefer Option 1. Because the PUCCH notification becomes useless if the associated CSI report cannot be transmitted on the PUSCH, releasing the PUCCH resource in such situations allows the network to reallocate it to other UEs, enhancing overall resource efficiency and system capacity.

However, if the majority view is to not release the PUCCH resource (perhaps due to its utility for HARQ-ACK/SR transmission), we are also fine with the Option 2b. Transmitting the PUCCH when no CG/PUSCH is available for CSI report transmission offers no benefit and leads to unnecessary UE power consumption, so the UE should not transmit the UEIRI on the PUCCH when the associated CG type 1 has been cleared even if the PUCCH is not released. This rationale aligns with the agreement reached in the previous meeting regarding the UE behaviors on PUCCH/PUSCH transmission outside the DRX Active Time. We prefer to specify it in the MAC spec instead of PHY spec because the operation of TAT is invisible to the PHY.

For mode-B UEI reporting, PUCCH and Type1 CG PUSCH can be associated with different TAGs. If the TAT (associated with a sTAG) for Type1 CG PUSCH is expired while the TAT for PUCCH is running, RAN2 to discuss and down-select one of the following options:

- Option 1: UE releases the PUCCH. (Adopt the TP in the Appendix 1).

- Option 2: UE does not release the PUCCH and not transmit the UEIRI on the PUCCH. (Adopt the TP in the Appendix 2).

Moreover, the Case 2 (If the TAT for PUCCH is expired while the TAT for Type 1 CG/PUSCH is running) should also be discussed. Given that the PUCCH resource has been released, without the PUCCH notification, the UE cannot transmit the CSI report on the CG/PUSCH. Furthermore, the Type 1 CG resources for mode B cannot be utilized for UL-SCH data transmission and even cannot be utilized for other UCI transmission. Clearing the Type 1 CG in this scenario would enable the network to reallocate these CG resources to other UEs, thereby enhancing overall resource efficiency and system capacity

For mode-B UEI reporting, PUCCH and Type1 CG PUSCH can be associated with different TAGs. If the TAT (associated with a sTAG) for PUCCH is expired while the TAT for Type1 CG PUSCH is running, the UE clears the Type1 CG PUSCH. (Adopt the TP in the Appendix 3).

## Deactivated BWP of mode-B PUSCH

There is an open issue raised by Sharp in [1] about deactivated BWP. According to the latest MIMO TS 38.331 CR, for mode B, it is possible that PUCCH resource configured in a specific UL BWP (based on a ul-BWP-Id for pucch-Resource) while Type 1 CG PUSCH resource configured in another UL BWP (based on another ul-BWP-Id for pusch-ResourceOfModeB-r19).

TS 38.331 CSI-ReportUE-IBR-r19 ::= SEQUENCE { ... reportTransmissionMode-r19 CHOICE { modeA-r19 NULL, modeB-r19 SEQUENCE { pusch-ResourceOfModeB-r19 SEQUENCE { configuredGrantConfigIndex-r19 ConfiguredGrantConfigIndex-r16, ul-BWP-Id-r19 BWP-Id, servCellIndex-r19 ServCellIndex }, … pucch-Resource-r19 SEQUENCE { … resource PUCCH-ResourceId, ul-BWP-Id-r19 BWP-Id, pucch-Cell-r19 ENUMERATED {spCell, pucch-Scell} } }

This creates an issue where a UEIBR is triggered, and the UL BWP configured with the Type 1 CG PUSCH resource for mode B UE-initiated CSI reporting is deactivated, while a different UL BWP configured with the PUCCH resource for mode B UE-initiated CSI reporting remains active. In this case, it is necessary to discuss whether the UE should transmit the PUCCH on the active UL BWP, even though the corresponding CSI report cannot be transmitted on the Type 1 CG PUSCH due to deactivated BWP. In our opinion, the UE should not transmit the PUCCH notification if the CSI report is not able to be transmitted. This stance is consistent with the agreement reached in the prior meeting concerning UE behavior for PUCCH/PUSCH transmissions outside the DRX Active Time. Also, the UE can save power by not transmitting the UEIRI on the PUCCH.

Some companies commented, in the email discussion [1], that there is no MAC impact for this proposal. However, we believe the current MAC specification’s behaviors on the clause 5.15.1 (as highlighted below) do not cover this UE behavior. The current MAC specification is to specify the UE behaviors on the DL BWP if the DL BWP is deactivated. Specifically, when a DL BWP is deactivated, the UE does not report CSI for this deactivated DL BWP because the UE cannot perform the measurement on the deactivated DL BWP. However, the issue here is about UL BWP instead of DL BWP. Moreover, “not report CSI” cannot be interpreted as “not transmit UEIRI” because the TS 38.213 clearly specifies that the CSI and the UEIRI are different UCI types. Consequently, we believe some changes in the MAC specification is needed.

TS 38.321 ### 5.15.1 Downlink and Uplink … 1> if a BWP is deactivated or the Serving Cell is PSCell of deactivated SCG: 2> not transmit on UL-SCH on the BWP; 2> not transmit on RACH on the BWP; 2> not monitor the PDCCH on the BWP; 2> not transmit PUCCH on the BWP; 2> not report CSI for the BWP; 2> not transmit SRS on the BWP; 2> not receive DL-SCH on the BWP; 2> clear any configured downlink assignment and configured uplink grant of configured grant Type 2 on the BWP; 2> suspend any configured uplink grant of configured grant Type 1 on the inactive BWP.

For mode-B UEI reporting, when PUCCH and Type1 CG PUSCH are configured in different UL BWPs, the UE does not transmit the UEIRI on the PUCCH when the Type 1 CG PUSCH is configured in an UL BWP which is deactivated. (Adopt the TP in the Appendix 4).

## Multi-panel SDM in asymmetric DL sTRP/UL mTRP

In Rel-19 asymmetric DL sTRP/UL mTRP scenarios, the UE may transmit repetitions of a PUSCH or PUCCH transmission to both the anchor TRP and the UL-only TRP, or it may perform simultaneous PUSCH/PUCCH transmission using two panels (e.g., via multipanelSchemeSDM or multipanelSchemeSFN) targeting both TRPs as shown in Figure 2.

Figure 2: STxMP schemes

In the Rel-19 asymmetric DL sTRP/UL mTRP scheme, a CG/PUSCH for SP-CSI can be associated with both TAGs of both TRPs. However, in the current spec, it’s unclear whether the UE clears the CG/PUSCH for SP-CSI if one or both of the TATs of both TAGs are expired.

One case is worthy for discussion is that an UE is configured with a multi-panel simultaneous uplink transmission SDM scheme (multipanelSchemeSDM) for PUSCH, it is possible that some layers of the CG/PUSCH transmissions are mapped to the TCI state associated with a first TAG with expired TAT, while the other layers of the CG/PUSCH transmissions are mapped to the TCI state associated with a second TAG with running TAT. Since one of the two TAGs is not valid, the network may not be able to successfully decode the partial PUSCH transmissions that composed only partial of the MIMO layers corresponding to the still-valid TAG. One option is that the UE should clear the CG/PUSCH for SP-CSI if any of the two TCI states is associated with a TAG of an expired TAT. Another option is to clarify that the UE does not transmit any CG/PUSCH for SP-CSI if any of the two TCI states is associated with a TAG of an expired TAT.

Some companies commented, in the email discussion [1], that this issue may already exist in the Rel-18 2TA and STMP. In our understanding, the Rel-18 two TA was agreed only for multi-DCI multi-TRP scheme (e.g., non-ideal backhaul case). In the Rel-18 multi-DCI multi-TRP scheme, a CG/PUSCH resource for semi-persistent CSI reporting cannot be associated with both TAGs of both TRPs because the TDM repetitions and SDM/SFN simultaneous transmissions necessitate close coordination between the two TRPs. However, in the Rel-19 single-DCI multi-TRP scheme, the difference is that a CG/PUSCH resource for semi-persistent CSI reporting can be associated with both TAGs of both TRPs (due to ideal backhaul). On the other hand, the Rel-18 STMP was only specified under single TAG assumption. Thus, this issue is introduced due to the support of the Rel-19 asymmetric DL sTRP/UL mTRP scenario, so should be in the scope of Rel-19 MIMO discussion.

When a serving cell is configured with multipanelSchemeSDM and with two TAGs, to avoid unsuccessfully decoding due to only partial PUSCH transmission/MIMO layer/TRP has valid TA, RAN2 to discuss and down-select one of the following options:

- Option 1: UE clears any CG/PUSCH for SP-CSI if any of the two activated TCI states for the CG/PUSCH for SP-CSI is associated with a TAG of an expired TAT. (Adopt the TP in the Appendix 5).

- Option 2: UE does not transmit any CG/PUSCH for SP-CSI if any of the two activated TCI states for the CG/PUSCH for SP-CSI is associated with a TAG of an expired TAT. (Adopt the TP in the Appendix 6).

# Conclusion

Based on the analysis above, we have made the following proposals:

Proposal 1	For mode-B UEI reporting, PUCCH and Type1 CG PUSCH can be associated with different TAGs. If the TAT (associated with a sTAG) for Type1 CG PUSCH is expired while the TAT for PUCCH is running, RAN2 to discuss and down-select one of the following options:

- Option 1: UE releases the PUCCH. (Adopt the TP in the Appendix 1).

- Option 2: UE does not release the PUCCH and not transmit the UEIRI on the PUCCH. (Adopt the TP in the Appendix 2).

Proposal 2	For mode-B UEI reporting, PUCCH and Type1 CG PUSCH can be associated with different TAGs. If the TAT (associated with a sTAG) for PUCCH is expired while the TAT for Type1 CG PUSCH is running, the UE clears the Type1 CG PUSCH. (Adopt the TP in the Appendix 3).

Proposal 3	For mode-B UEI reporting, when PUCCH and Type1 CG PUSCH are configured in different UL BWPs, the UE does not transmit the UEIRI on the PUCCH when the Type 1 CG PUSCH is configured in an UL BWP which is deactivated. (Adopt the TP in the Appendix 4).

Proposal 4	When a serving cell is configured with multipanelSchemeSDM and with two TAGs, to avoid unsuccessfully decoding due to only partial PUSCH transmission/MIMO layer/TRP has valid TA, RAN2 to discuss and down-select one of the following options:

- Option 1: UE clears any CG/PUSCH for SP-CSI if any of the two activated TCI states for the CG/PUSCH for SP-CSI is associated with a TAG of an expired TAT. (Adopt the TP in the Appendix 5).

- Option 2: UE does not transmit any CG/PUSCH for SP-CSI if any of the two activated TCI states for the CG/PUSCH for SP-CSI is associated with a TAG of an expired TAT. (Adopt the TP in the Appendix 6).

# Reference

R2-250xxxx Report of Rel-19 MIMO MAC open issues for maintenance (Samsung)

R1-2506253 Moderator Summary #1 on UE-initiated/event-driven beam management (ZTE)

# Appendix

## Appendix 1

## 5.2 Maintenance of Uplink Time Alignment … 1> when a timeAlignmentTimer expires: … 3> if the timeAlignmentTimer is associated with a TAG for an SCell configured with only this TAG; or 3> if the timeAlignmentTimer is associated with a TAG for an SCell, and if the SCell is configured with two TAGs and the timeAlignmentTimer associated with the other TAG is not running: 4> flush all HARQ buffers for all such SCells; 4> notify RRC to release PUCCH, if configured for all such SCells; 4> notify RRC to release SRS, if configured for all such SCells; 4> clear any configured downlink assignments and configured uplink grants for all such SCells; 5> if there is PUCCH resource configured on a serving cell referred to by pucch-Cell included in CSI-ReportUE-IBR of an associated CSI-ReportConfig that provides these configured uplink grants: 6> notify RRC to release the PUCCH resource for the serving cell; 4> clear any PUSCH resource for semi-persistent CSI reporting for all such SCells; - 4> maintain NTA (defined in TS 38.211 [8]) of this TAG.

## Appendix 2

## 5.2 Maintenance of Uplink Time Alignment … - For mode-B UE-initiated CSI reporting, if the configured grant Type 1 for mode-B UE-initiated CSI reporting is associated with a TAG of an expired timeAlignmentTimer, the UE Initiated Report Indication on the PUCCH is not transmitted.

## Appendix 3

## 5.2 Maintenance of Uplink Time Alignment … 1> when a timeAlignmentTimer expires: … 3> if the timeAlignmentTimer is associated with a TAG for an SCell configured with only this TAG; or 3> if the timeAlignmentTimer is associated with a TAG for an SCell, and if the SCell is configured with two TAGs and the timeAlignmentTimer associated with the other TAG is not running: 4> flush all HARQ buffers for all such SCells; 4> notify RRC to release PUCCH, if configured for all such SCells; 5> if there is configured grant Type 1 for mode-B UE-initiated CSI reporting configured on a serving cell by pusch-ResourceOfModeB-r19 included in CSI-ReportUE-IBR of an associated CSI-ReportConfig that provides these PUCCH resources: 6> clear the configured grant Type 1 for mode-B UE-initiated CSI reporting for the serving cell. 4> notify RRC to release SRS, if configured for all such SCells; 4> clear any configured downlink assignments and configured uplink grants for all such SCells; 4> clear any PUSCH resource for semi-persistent CSI reporting for all such SCells; - 4> maintain NTA (defined in TS 38.211 [8]) of this TAG.

## Appendix 4

### 5.15.1 Downlink and Uplink … For mode-B UE-initiated CSI reporting, if the configured grant Type 1 for mode-B UE-initiated CSI reporting is configured on an UL BWP which is deactivated, the UE Initiated Report Indication on the PUCCH is not transmitted.

## Appendix 5

5.2 Maintenance of Uplink Time Alignment … 1> when a timeAlignmentTimer expires: … 3> else if the timeAlignmentTimer is associated with a TAG for a Serving Cell configured with two TAGs, and if the timeAlignmentTimer associated with the other TAG is running, for all such Serving Cells: 4> clear any configured downlink assignment, if the activated TCI state(s) for all PUCCH resources configured for the configured downlink assignment is associated with the TAG of the expired timeAlignmentTimer; 4> if this Serving Cell is configured with multipanelSchemeSDM: 5> clear any configured uplink grant, if any of the two activated TCI states for the configured uplink grant is associated with the TAG of the expired timeAlignmentTimer; 5> clear any PUSCH resource for semi-persistent CSI reporting, if any of the two activated TCI states for the PUSCH resource is associated with the TAG of the expired timeAlignmentTimer; 4> else: 5> clear any configured uplink grant, if all of the activated TCI state(s) for the configured uplink grant is associated with the TAG of the expired timeAlignmentTimer; 5> clear any PUSCH resource for semi-persistent CSI reporting, if all of the activated TCI state(s) for the PUSCH resource is associated with the TAG of the expired timeAlignmentTimer; 4> maintain NTA (defined in TS 38.211 [8]) of this TAG.

## Appendix 6

5.2 Maintenance of Uplink Time Alignment … - If a Serving Cell is configured with multipanelSchemeSDM and with two TAGs, the UE does not transmit any configured uplink grant on the Serving Cell, if any of the two activated TCI states for the configured uplink grant is associated with a TAG of an expired timeAlignmentTimer. If a Serving Cell is configured with multipanelSchemeSDM and with two TAGs, the UE does not transmit any PUSCH for semi-persistent CSI reporting on the Serving Cell, if any of the two activated TCI states for the PUSCH resource is associated with a TAG of an expired timeAlignmentTimer.