3GPP TSG-RAN WG2 Meeting #131bis	R2-2507202

Prague, Czech Republic, Oct 13th – 17th, 2025

Agenda item:		10.3.1

Source:		Futurewei

Title:		Discussions on 6G User Plane

Document for:		Discussion and Decision

# Introduction

According to the revised 6G SID [1], RAN2 is the lead WG responsible for the following user plane (UP) aspects of 6G radio interface design:

- Radio interface protocol architecture and procedures for 6GR [RAN2, RAN1, RAN4, RAN3]

- User Plane architecture and protocol design [RAN2]

- Control Plane architecture (including RRC states) and protocol design [RAN2, RAN3]

- Access stratum security aspects, in alignment with requirements from SA3 [RAN2]

- Radio signalling framework for UE capabilities, aiming at improvements and simplification compared to 5G NR [RAN2, RAN1, RAN4]

- Data transfer design to support various types of data (e.g. AI/ML, Sensing, etc) [RAN2, RAN3]

  - On-line training has stringent delay requirements on data transfer.

In this paper, we discuss some key aspects in 6GR user plane design.

# Discussions

## On essential user plane functions

Table 1 below summarizes the essential user plane functions performed at various sublayer entities at Layer 2 in 5G NR, the respective performing entity in 5G NR, and in our opinion, whether they are applicable to 6GR or not, and if applicable to 6GR, the proposed performing entity in 6GR.

Table 1

| User plane functions | Performing entity in 5G NR | Proposed applicability to 6GR | Proposed performing entity in 6GR |
| --- | --- | --- | --- |
| mapping between QoS flow and DRB for DL and UL | SDAP | Yes | PDCP or possibly a combined PDCP/RLC layer |
| reflective QoS flow to DRB mapping for UL | SDAP | No | N/A |
| QFI marking on PDU | SDAP | Yes | PDCP or possibly a combined PDCP/RLC layer |
| header compression/ decompression/UDC | PDCP | Yes | PDCP or possibly a combined PDCP/RLC layer |
| SDU discarding | PDCP | Yes | PDCP or possibly a combined PDCP/RLC layer |
| integrity protection/verification, ciphering/deciphering | PDCP | Yes | PDCP or possibly a lower layer or a combined layer |
| maintenance of PDU SN and duplicate detection and discarding | PDCP and RLC, respectively | Yes, but possibly combined into one SN | PDCP and RLC, or possibly a combined PDCP/RLC layer |
| split bearer, DAPS, and PDCP duplication | PDCP | Not if DC is not supported in 6GR | N/A if DC is not supported in 6GR |
| reordering and in-order delivery | PDCP | Yes | PDCP or possibly a combined PDCP/RLC layer |
| out-of-order delivery | RLC and optional in PDCP | Yes | RLC and optional in PDCP, or possibly optional in a combined PDCP/RLC layer |
| error correction or data recovery through ARQ retx or auto-retx | RLC (AM only) and PDCP | Yes, but possibly combined into one | PDCP or possibly a combined PDCP/RLC layer |
| segmentation and reassembly of SDUs | RLC (AM and UM) | Yes | RLC or possibly an RLC/MAC combined layer or a PDCP/RLC combined layer |
| error correction through HARQ | MAC | Yes | MAC |
| multiplexing/demultiplexing of MAC SDUs into/from TB | MAC | Yes | MAC |
| logical channel prioritization | MAC | Yes | MAC |
| BSR/DSR | MAC | Yes | MAC |

Proposal 1. RAN2 study the essential user plane functions described in Table 1, determine whether they should be applicable to 6GR, respectively, and take those applicable functions into consideration when designing 6GR user plane architecture and protocol design.

## On user plane architecture and protocol design

The KPIs for 6GR, especially as related to the peak data rates, throughputs, and latency, are expected to be more demanding than those for 5G NR. It is important to further streamline the user plane protocols to enable more complexity-effective implementations.

Take the 5G SDAP layer for example, the reflective QoS flow to DRB mapping for UL has increased processing requirement on the UEs significantly with very limited gain and hence should not be considered for 6GR. The remaining two key functions of the 5G SDAP layer can then be moved into a 6G PDCP-like layer, which may be an enhanced PDPC layer or a combined PDCP/RLC layer.

Proposal 2. RAN study excluding a SDAP layer in 6GR user plane and moving the remaining necessary functions in the 5G SDAP layer into a 6G PDCP-like layer.

For another example, there are more than one types of error correction mechanisms being specified at various communication protocol layers today, for example, HARQ in MAC, ARQ in RLC, PDCP retransmission in PDCP. For some applications, the higher layers can further support E2E error correction, e.g., through TCP retransmission, RTP retransmission, application layer forward error correction (AL-FEC), etc. These various error correction mechanisms, when used at the same time, may interact with one and another in a way that negatively affects the overall efficiency and effectiveness for error correction. Therefore, awareness of target residual error rates at each of the different layers and the error correction capability and configurability at the other layers can be used for optimization of the use and configuration of error correction mechanisms at the different layers.

Proposal 3, RAN2 study the support of the awareness of target residual error rates, error correction capability and configurability at the different protocol layers, and based on the awareness, means for optimizing the use and configuration of the error correction mechanisms at the different layers.

In 5G NR, there are some similar functions being performed at the RLC layer and the PDCP layer, with some degree of redundancy. For examples, both layers maintain a respective PDU sequence number (SN) for duplicate detection and removal, and both layers perform PDU retransmission through ARQ retransmission or auto-retransmission for error correction or data recovery. Therefore, it may be possible to combine these two layers into one in 6GR to reduce the need for packet processing and the associated processing delay and power consumption.

Proposal 4. RAN2 study the feasibility of combining the PDCP and RLC layers into one for the user plane in 6GR and the potential impact to RAN architecture and the related intra-RAN and RAN-CN interfaces.

In RAN meeting#109, RAN Plenary endorsed a draft of TR 38.914 (Study on 6G Scenarios and Requirements), which contains the following highlighted text with respect to service-awareness [2]:

## 5.2	Requirements for architecture and migration

Editor note: 6G RAN architecture, 5G-6G migration

The RAN design for the 6G Radio Access Technologies shall be designed to fulfil the following requirements:

...

-	The 6G RAN architecture shall support enhanced service awareness in RAN.

5G RAN is mostly agnostic of the data or the goals of the data being delivered, except that we introduced XR-awareness for XR traffics half-way through the 5G standardization. RAN2 should study further enhancements in service awareness, including latency awareness and other possible types of awareness, and enable the support of enhanced service awareness and corresponding RAN handling based on the awareness from day one in 6G.

Proposal 5. RAN2 study further enhancements in service-awareness and enable the support of enhanced service awareness and corresponding RAN handling based on the awareness from day one in 6G.

## Potential inter-WG and/or inter-TSG dependencies/impacts

Dependencies/impacts are anticipated on RAN3 work for RAN architecture, RAN-CN interfaces, and CU-DU split gNB architecture, on SA2/SA4/RAN3 work for service awareness, and on SA3 work for AS security. Early alignments with these WGs on these issues are desirable.

# Conclusions

Proposal 1. RAN2 study the essential user plane functions described in Table 1, determine whether they should be applicable to 6GR, respectively, and take those applicable functions into consideration when designing 6GR user plane architecture and protocol design.

Proposal 2. RAN study excluding a SDAP layer in 6GR user plane and moving the remaining necessary functions in the 5G SDAP layer into a 6G PDCP-like layer.

Proposal 3, RAN2 study the support of the awareness of target residual error rates, error correction capability and configurability at the different protocol layers, and based on the awareness, means for optimizing the use and configuration of the error correction mechanisms at the different layers.

Proposal 4. RAN2 study the feasibility of combining the PDCP and RLC layers into one for the user plane in 6GR and the potential impact to RAN architecture and the related intra-RAN and RAN-CN interfaces.

Proposal 5. RAN2 study further enhancements in service-awareness and enable the support of enhanced service awareness and corresponding RAN handling based on the awareness from day one in 6G.

# References

RP-252912, Revised SID: Study on 6G Radio, NTT DOCOMO, et al., RAN#109.

RP-252882, TR 38.914 V0.2.0.