3GPP TSG-RAN WG2 Meeting #131bis	R2-2507204

Prague, Czech Republic, Oct 13th – 17th, 2025

Agenda item:		10.3.3

Source:		Futurewei

Title:		Discussions on 6G Common Aspects of UP and CP

Document for:		Discussion and Decision

# Introduction

According to 6G SID [1], RAN2 is the lead WG responsible for the following common aspects of 6G radio interface design:

- Radio interface protocol architecture and procedures for 6GR [RAN2, RAN1, RAN4, RAN3]

- User Plane architecture and protocol design [RAN2]

- Control Plane architecture (including RRC states) and protocol design [RAN2, RAN3]

- Access stratum security aspects, in alignment with requirements from SA3 [RAN2]

- Radio signalling framework for UE capabilities, aiming at improvements and simplification compared to 5G NR [RAN2, RAN1, RAN4]

- Data transfer design to support various types of data (e.g. AI/ML, Sensing, etc) [RAN2, RAN3]

- AI/ML for 6GR and Radio Access Network, leveraging 5G AI/ML framework, as appropriate [See TR38.843] [RAN1, RAN2, RAN3, RAN4]

- Identify Use Case(s) of interest (either existing or new) with compelling trade-off between e.g., performance, complexity, etc… 
Coordinated discussion needs to be ensured with related design areas, where needed (e.g., MIMO, Mobility, etc…)

NOTE: lead WG depends on the use case.

- AI/ML framework: Extensible AI/ML enablers based on the identified Use Case(s), including

  - LCM procedures [RAN2, RAN1, RAN3, RAN4]

  - Data collection and data management, in coordination with SA WGs [RAN2, RAN3, RAN1]

Note: NW for AI is assumed to be covered by new services

6GR and RAN design shall ensure that the 6G System can also operate without AI/ML

In this paper, we discuss the AS security aspect and the AI/ML aspect that are common for the user plane and control plane for 6G RAN.

# Discussions

## AS security aspects

### System information protection

In the 5G era, SA3 had studied the issue of rogue base station, during which time various mechanisms for protecting the system information were proposed without SA3 concluding on any specific enhancements be moved forward into the specification phase [2]. Most of these mechanisms can be summarized as adding signature to the system information message broadcasted by the base station and then having the UEs receiving the system information message to verify the signature successfully before using the system information. Part of the reason that none of these protection mechanisms was pursued is because all of them require additional computing power and energy consumption on the UEs, which can be quite demanding, and in the case of simple UEs such as IoT devices, almost prohibiting, for UE implementations.

Nonetheless, if unaddressed, the security vulnerability presented by a lack of security protection on the system information, e.g., subjection to a spoofing, replaying, or forgery attack, would continue to exist in 6G RAN. Therefore, we propose that, in 6G SI, RAN2 study the issue of system information protection and work with SA3 to explore possible solutions, bearing in mind that the diverse types of 6G UEs may have a wide range of computing capability, energy storage capacity, and overall sophistications.

Proposal 1. In 6G SI, RAN2 study the issue of system information protection and work with SA3 to explore possible solutions, bearing in mind that the diverse types of 6G UEs may have a wide range of computing capability, energy storage capacity, and overall sophistications.

### Protection of critical control information at lower layers

In 5G, security protection is supported on most RRC messages but not for control information at the lower layers, such as PDCP control PDUs, RLC control PDUs, MAC CEs, and DCIs, because the integrity protection and cyphering functions are performed only at the PDCP layer and only on PDCP data PDUs.

As MAC CEs have increasingly been used for conveying lower layer parameters or configuration indices, and activating or deactivating features, a faked MAC CE sent out by a malicious attacker, if undetected, can render the operation between the victim UE and victim gNB inoperable. In addition, certain MAC CEs, such as C-RNTI and Timing Advance Command MAC CEs, if uncyphered or not concealed, can also reveal the temporary identity and the location of the UE, which can then be used for tracking the UE. Therefore, MAC CEs should be a primary target of study for AS security enhancements in 6G.

Proposal 2. In 6G SI, RAN2 study the issue of protecting critical control information at lower layers, with protecting critical MAC CEs as a starting point, and if needed, work with SA3 to explore possible solutions.

### Security issues and requirements brought up by new services or techniques

Sensing services, without proper privacy protection, can introduce new sources of leaking user’s location information or other private information.

In RAN meeting#109, RAN Plenary endorsed the draft of TR 38.914 (Study on 6G Scenarios and Requirements), which contains the following highlighted text with respect to service-awareness [3]:

## 5.2	Requirements for architecture and migration

Editor note: 6G RAN architecture, 5G-6G migration

The RAN design for the 6G Radio Access Technologies shall be designed to fulfil the following requirements:

...

-	The 6G RAN architecture shall support enhanced service awareness in RAN.

Further enhancements in service awareness in RAN may require certain service-related information, such as traffic characteristics, be made available to the lower layers in RAN. If unprotected, that information may become visible to an eavesdropper and hence may also become a source of leaking user’s privacy.

Proposal 3. In 6G SI, RAN2 study the potential privacy issues brought up by sensing services or enhancements in service awareness in RAN.

## AI/ML Framework for 6G

In R18 and R19, RAN has done extensive study and normative work on the application of AI/ML to the NR air interface [4]. The study establishes a foundational framework for integrating AI/ML technologies, evaluates their benefits through specific use cases, and assesses the potential impact on 3GPP specifications.

The core of the study is the development of a general AI/ML framework, encompassing a detailed Life Cycle Management (LCM) process for AI/ML models. It encompasses the following functional blocks, as shown below in Figure 1 [4].

- Data Collection: Gathering data for training, inference, monitoring, and updates.

- Model Training: The process of creating a trained model from data.

- Model Delivery/Transfer: Procedures for moving a model from one entity to another (e.g., network to UE).

- Model Inference: Using a trained model to generate outputs from new inputs.

- Model Management: Oversees the operation (selection, activation, fallback) and monitoring of AI/ML models and functionalities.

- Model Storage: A logical function for storing trained and updated models.

Figure 1. Functional framework for AI/ML for NR Air Interface

Three key use cases were selected to validate the framework and quantify performance gains for the PHY air interface, CSI Feedback Enhancement, Beam Management (BM), and Positioning Accuracy Enhancements.

In R19, RAN2 has also studied the application of AI/ML to enhance UE mobility in NR systems [5]. The study's primary goals were to investigate AI/ML's potential to reduce UE measurement efforts and to improve handover (HO) performance, specifically by reducing the Handover Failure (HOF) rate. The study focused on three main use cases:

- RRM measurement prediction,

- measurement event prediction, and

- Radio Link Failure (RLF) prediction, with the latter being deprioritized.

Evaluations conducted via extensive simulations demonstrate that AI/ML models significantly outperform non-AI baseline methods (e.g., sample-and-hold) in prediction accuracy, particularly for longer prediction windows and higher UE speeds.

Both the PHY air interface project and the AI/ML-based mobility project have left some problems unsolved or areas that need further enhancements in the future releases. In the rest part of this section, we will discuss these areas/aspects and propose working directions for 6G.

### Data collection and model transfer

Data collection is a key function within the general AI/ML framework. It is defined as the process of collecting data by NW nodes, a management (e.g., training, inference, and monitoring) entity, or the UE for purposes such as AI/ML model training, data analytics, and inference.

In 5G/5G-A, data collection for both NW-side and UE-side models have been studied.

NW-side data collection supports models where inference is performed entirely at the network or for NW-side model training, monitoring, and inference. NW-side data collection needs generally consider UE support for data logging and the ability to report data periodically, based on events, or on-demand. Practical constraints include signaling overhead, energy consumption, processing power, and UE memory size.

UE-side data collection supports models where inference is performed entirely at the UE, and is essential for training, monitoring, or inference of UE-side models or UE parts of two-sided models. The primary focus for UE-side model training data transfer is the process of collecting data at the UE and delivering it to a training entity. RAN2 studied several methods for transferring training data collected by the UE; both UP and CP solutions have been considered.

- Option 1a (Direct to OTT Server): The UE collects and directly transfers data to an entity outside the MNO (e.g., an OTT server). This typically uses a UP tunnel if transferred via the 3GPP network, and no 3GPP specification impact is expected.

- Option 1b (MNO Server): The UE collects data and transfers it to a server for UE-side model training inside the MNO, which may then optionally transfer it to an OTT server. This path typically utilizes a UP tunnel.

- Option 2 (Via Core Network): The UE collects data and transfers it to the CN, which forwards it to the training server/OTT server. A CP tunnel may be used if the data volume is manageable within NAS signaling capacity.

- Option 3 (Via OAM): The UE collects data and transfers it to the OAM (via the gNB), which then forwards it to the training server/OTT server. A CP tunnel may be used if the data volume is within RRC signaling capacity.

Due to time constraints and the fact that the feasibility of some of the mechanisms listed above need confirmation from other WGs and even other TSGs (such as SA), there are some areas that need further studies in 6G. For example, for UE-side model training,

- For Option 1a, the NW has neither controllability nor visibility, which are demanded by the carriers. In addition, it is not clear how to configure/activate/deactivate data collection, as data collection happens in the UP and hence outside 3GPP.

- For Option 1b, the following additional aspects are not clear

  - Where the Server for Data Collection resides; some think inside the MNO, but others are not sure.

  - How the UP tunnel will be established (currently under SA study).

  - How controllability and visibility can be achieved.

- For Option 2 and 3, the following additional aspects are not clear

  - While CP solutions have been agreed (NAS layer for Option 2 and RRC layer for Option 3), its feasibility has not been concluded.

  - CP solution is not ideal for big-size data (multiple companies raised the issue).

  - Using CP for data collection could congest the CP for other control messages.

  - RAN2 cannot reach consensus the feasibility of UP solutions (UP tunnels), which is under SA study.

  - RAN2 cannot reach consensus on controllability and visibility.

In summary, under the data collection framework of 5G/5G-A, the CP handles tasks such as establishing connections and controlling the forwarding of UP data (commonly referred to as session management within the 5G core). The UP is designed to forward network packets to their intended destinations efficiently. Both the CP and UP operate within a communication session-centric framework; the CP sets up UP data tunnels for each session, facilitating signaling message exchange among various entities, including UEs, BSs, and CN functions, to maintain state synchronization. However, this existing CP/UP framework/collaboration for data collection may not be enough to support data collection needs in 6G.

As we move into the era of 6G, collecting data from wireless networks faces challenges from both the business side and the technical side.

From the business side, with the introduction of non-communication functions, such as sensing, into the wireless network, new applications bring massive perception data and AI data, as well as network operation data. These non-communication session data contain huge value and require efficient data collection/management mechanisms to realize data monetization.

From the technical side, 6G's new data types bring challenges to the existing user plane: the existing communication network lacks a normalized data service framework and cannot process the newly added perception data and AI model data of 6G. In addition, the data collection mechanism and processing functions in the existing architecture are rigid and lack flexibility and cannot meet the needs of "in-path computing" (i.e., processing functions exist in the relaying nodes when delivering data from one end point to another) and "arbitrary topology" (i.e., any node in the network can be the data producer/source and data consumer/destination).

Because data collection in 6G involves diverse types of data (e.g., AI/ML, sensing etc) and supports different types/levels of services (e.g., for wireless network itself, for service promoted to the application level etc), a unified framework for data collection and management is required to avoid multiple data collection systems.

Similar challenges also apply to model transfer and delivery.

Proposal 4. Support a unified framework for data collection/delivery and model transfer in 6G AI/ML.

### Two-sided models

A two-sided model is defined as a paired AI/ML model(s) over which joint inference is performed. This joint inference process occurs jointly across the UE and the NW. Specifically, the inference workflow involves the first part of inference being performed by the UE and then the remaining part being performed by the gNB, or vice versa.

General consensus of 3GPP study is that two-sided models will be essential for realizing 6G's long-term potential. In particular, two-sided models have the potential to achieve global optimization and deliver enhanced performance gains that are challenging to realize with traditional or one-sided models.

- Enhanced system performance and joint optimization (of models at both sides)

- Handling complex physical layer challenges

- Continuous learning/update at both sides supports fine-tuning and adaptation of UE and NW AI/ML-based algorithms to respond to changing network conditions

Despite their benefits, two-sided models introduce challenges that need to be studied and addressed. These challenges include, but are not limited to,

- synchronization, including model pairing/alignment,

- increased signaling overhead,

- complex inter-vendor collaboration compared to one-sided models,

- LCM (e.g. fallback procedures on both sides, switching models for one or both sides.)

Some of these challenges, such as inter-vendor collaboration, have been discussed in 5G/5G-A. However, more aspects related to two-sided models need to be further studied and concluded to ensure their successful applications in wireless networks.

Proposal 5. Support further study of two-sided models in 6G AI/ML.

### UE capability reporting and management

In 5G/5G-A, UE capabilities have been lightly studied but they were not at the center of the discussions. To enable successful implementation and management of AI/ML models, particularly those residing fully or partially at the UE side, the discussion focused on computational constraints, memory size, power level and reporting mechanisms.

The study recognizes several constraints and capabilities related to the UE hardware that must be factored into the design and evaluation of AI/ML schemes.

- AI/ML Processing Unit (APU): This unit is intended for quantifying simultaneous processing of AI/ML-related computations within a UE. In 5G/5G-A, it was used to quantify simultaneous processing of multiple CSI reports related to AI/ML use cases, such as CSI compression, CSI prediction, spatial domain beam prediction, and temporal domain beam prediction.

- Energy and Power Consumption: Energy consumption and power consumption (including computational power) are key limiting factors that must be considered when enabling AI/ML schemes. The UE's battery level is also identified as an internal condition impacting model operations.

- Memory/Storage Requirements: UE memory is a crucial practical constraint.

  - Model Storage: Evaluations consider memory storage associated with enabling AI/ML schemes, reported in terms of model size (e.g., Mbytes) and the total number of parameters.

  - Data Transfer Constraints: If control plane (CP) signaling is used for data transfer (e.g., for data collection), UE memory allocation within the Access Stratum (AS) buffer becomes a challenge. The existing Radio Resource Control (RRC) messaging framework supports segmentation up to 16 uplink segments, which amounts to a maximum capacity of approximately 144KB of collected data that can be transmitted in a single RRC procedure.

In 6G, we expect more and diverse use cases to be introduced. How to handle the impact of the UE’s internal capabilities and conditions such as computational power, memory, battery, and other hardware limitations on functionality or model operations is an ongoing area of study. Moving forward to 6G, we expect more UE capabilities, such as data pre-processing and forwarding, will be introduced and need to be studied.

Proposal 6. Support enhanced UE capability reporting and management in 6G AI/ML.

### AI/ML techniques beyond supervised learning

More AI/ML use cases are expected to be developed in 6G time frame with new AI/ML learning types.

An enhanced AI/ML framework needs to support diverse use cases, both 1-sided or 2-sided, including model (parameters) and dataset exchange when applicable. In addition, as most AI/ML models belong to vendor implementation and AI/ML technology is continuously evolving, the framework should not be limited to supporting only some types of model structures.

We believe the 6G AI/ML framework needs to support various learning techniques. In addition to supervised learning which is leveraged in all 5G NR AI/ML use cases, other learning techniques to consider may include (but not limited to):

- Online or continuous learning, e.g., reinforcement learning.

- Decentralized learning, e.g., federated learning.

- Distributed learning

Reinforcement Learning (RL) and Deep RL have the potential to be one of the major AI/ML approaches for improving high-layer functions/features. Digital Twin, as a powerful tool for RL/DRL model training/inference, is often proposed as an integrated part of a RL/DRL-based system for AI/ML applications in wireless networks. Because of this, ITU-T has adopted the following reference architecture of digital twin for 6G, as shown in Figure 2 [6].

Figure 2. A reference 6G digital twin network architecture based on ITU-T recommendation Y.3090 [6]

Proposal 7: Enable the support of more AI/ML techniques beyond supervised learning, as well as digital twin.

## Potential inter-WG and/or inter-TSG dependencies/impacts

Dependencies/impacts on SA3 work are anticipated for the AS security aspects. Early alignment with SA3 on the issues raised above is desirable.

Based on our experience with past AI/ML related studies in 5G/5G-A, most of the use cases in RAN involve collaborations among RAN1, RAN2 and RAN4. Dependencies on SA work are also anticipated for topics such as unified data collection framework, which is also a topic under SA study for 6G.

# Conclusions

Based on the discussions above, we propose the following:

Proposal 1. In 6G SI, RAN2 study the issue of system information protection and work with SA3 to explore possible solutions, bearing in mind that the diverse types of 6G UEs may have a wide range of computing capability, energy storage capacity, and overall sophistications.

Proposal 2. In 6G SI, RAN2 study the issue of protecting critical control information at lower layers, with protecting critical MAC CEs as a starting point, and if needed, work with SA3 to explore possible solutions.

Proposal 3. In 6G SI, RAN2 study the potential privacy issues brought up by sensing services or enhancements in service awareness in RAN.

Proposal 4. Support a unified framework for data collection/delivery and model transfer in 6G AI/ML.

Proposal 5. Support further study of two-sided models in 6G AI/ML.

Proposal 6. Support enhanced UE capability reporting and management in 6G AI/ML.

Proposal 7: Enable the support of more AI/ML techniques beyond supervised learning, as well as digital twin.

# References

RP-252912, Revised SID: Study on 6G Radio, NTT DOCOMO, et al., RAN#109.

TR 33.809 V18.1.0, Study on 5G security enhancements against False Base Stations (FBS).

RP-252882, TR 38.914 V0.2.0.

TR 38.843 V19.0.0, Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface,  2025-09

TR 38.744 V1.2.0, Study on Artificial Intelligence (AI)/Machine Learning (ML) for Mobility in NR, 2025-09

ITU-T recommendation Y.3090, Digital twin network - Requirements and architecture , 2022-02