3GPP TSG-RAN WG2 #131bis 	R2-2507211

Prague, Czech Republic, 13 – 17 October, 2025

Agenda Item:	9.2.2

Source:	KT Corp.

Title:	Discussion on Topology 2 for A-IoT_

Document for:	Discussion and Decision

Acronym:	Ambient_IoT_Solutions_Ph2

# 1	Introduction

Revised WID for Rel-20 Ambient IoT Phase 2 is approved. We would like to present our observations and proposals on each objective listed in [1].

- Specify support for Deployment Scenario 2 with Topology 2 with intermediate UE as Reader under the following conditions [RAN2-led]: - According to D2T2-B for Device 1 with carrier-wave transmission for waveform 1 only, per TR 38.769. - No specification support for CW control - According to D2T2-C for active Device(s). - Start after March 2026 - Traffic types DO-DTT and DT, and DO-A (only for active Device(s)). - RAN2 to start on DO-A for D2T2 when sufficient progress has been made on DO-A for D1T1, after a checkpoint in RAN plenary in September 2026 - Representative use cases rUC1 (indoor inventory) and rUC4 (indoor command). - FR1 licensed spectrum in FDD, with R2D in UL spectrum, and D2R and CW in UL spectrum. - Specify RRC-based architecture, in coordination with SA2. - The following are not supported: - Coordination between UEs operating as UE readers - New Uu protocol layer(s) - Simultaneous (time overlapping) operation of Uu and A-IoT at the intermediate UE in the same band This objective begins after RAN#109 (September 2025), and after RAN1#111 (March 2026) for RAN1. The work is subject to the following constraints: RAN2 - The objective includes specification of necessary Uu procedures to enable the A-IoT operations. - RRC CONNECTED UE reader only, including RLF and handover, without specific enhancement for other temporary out-of-connection scenarios. - A-IoT radio resource of the UE reader is only valid within the serving cell. - Resource allocation to the UE Reader shall be via Uu RRC signaling.

# 2	Discussion

## 2.1	Review of topology 2

The Topology 2 architecture consists of an A‑IoT‑enabled UE reader, an A‑IoT‑enabled gNB and an A‑IoT CN as shown in Figure 1. The UE reader reuses the NR Uu radio protocol layers and the RRC layer, while the gNB adds the xx interface to communicate with the A‑IoT CN. A‑IoT messages are conveyed between the CN and the UE reader through the gNB.

Figure 1. Logical system architecture for topology 2

To support Topology 2, three solutions for conveying A-IoT upper layer information are captured in [2].

RRC‑based solution – In this option the gNB terminates the A‑IoT upper‑layer information and uses NR RRC signalling to convey A‑IoT messages to the UE reader. The UE reader receives A‑IoT radio‑resource configuration via dedicated RRC messages, and the gNB forwards inventory or command requests, resource allocation. Only UE readers in the RRC‑Connected state are authorised to perform inventory or command procedures. The gNB can allocate or release A‑IoT radio resources during an RRC reconfiguration or as part of handover or re‑establishment, and the UE reader reports inventory and command results back to the gNB.

Figure 2. Message flow for A-IoT Inventory in Topology 2 - RRC-based solution

NAS‑based solution – Here the A‑IoT upper‑layer information is not terminated at the gNB. Instead, the gNB forwards the information transparently over NAS signalling to the UE reader. While this reduces RAN‑side processing, it still requires the gNB to allocate A‑IoT radio resources via RRC and to coordinate inventory or command procedures. Because only RRC‑based solutions are normative in Rel‑20, NAS‑based operation is treated as a study option.

UP‑based solution – In this option the upper‑layer information is treated as user‑plane data and the gNB simply transports it between the CN and UE reader. This approach minimises modifications to RRC and NAS but requires user‑plane bearers and may increase overhead. As with the NAS‑based option, it is part of Rel‑19 study.

Figure 3. Message flow A-IoT Inventory in Topology 2 - NAS/UP based solution – A-IoT CN triggered A-IoT session resource allocation

Candidate procedures for inventory and command – TR 38.769 specifies candidate procedures for A‑IoT inventory and command in Topology 2. In the RRC‑based inventory procedure, the gNB selects a UE reader, allocates A‑IoT radio resources via RRC configuration and sends an inventory request from the CN to the UE reader. The UE reader performs an Access Stratum (AS) procedure—paging, random access and data transmission—to obtain device identifiers and reports the inventory results back to the gNB, which forwards them to the CN. A similar sequence is used for A‑IoT command: the gNB triggers the command request, allocates resources, the UE reader delivers the command to devices and collects responses, and the results are reported to the gNB and CN.

Only one UE reader can use the configured A‑IoT radio resources at a time and cross‑cell validity of resources is not supported. The UE reader receives dedicated resources via RRC, and the resources remain valid until the network releases them explicitly or until the configured timer expires. When the UE reader moves to another cell, the previous resources become invalid to avoid interference.

When summarizing the contents up to now, in the RRC‑based solution, the gNB terminates the A‑IoT messages and delivers them to the UE reader via NR RRC signalling; resource allocation and release are carried out through RRC reconfiguration or during handover. This approach reuses the NR Uu interface and the RRC protocol, so it requires minimal additional protocol changes.

Proposal 1	Use the RRC‑based solution as baseline for Topology 2 implementation, while continuing to investigate NAS‑ and UP‑based approaches for future releases when their protocols and resource‑control mechanisms are fully clarified.

## 2.2	A-IoT radio resource allocation for UE reader

Considering the Topology 2 discussions up to date, the gNB must allocate dedicated A‑IoT radio resources to the intermediate UE reader via RRC signalling. These resources are valid only in the serving cell, and their validity may be indefinite until explicit release or limited by a timer. When network priorities or traffic loads change, the gNB may need to reclaim or suspend these resources, which currently lacks a standardised mechanism. To ensure efficient resource sharing without disrupting ongoing A‑IoT sessions, there is interest in studying how to pause and resume A‑IoT transmissions gracefully when the network reallocates spectrum.

Observation 1	The gNB can assign UE readers A=IoT dedicated radio resources via RRC signaling as specified WID, and these assignments are not valid across cells.

Observation 2	A‑IoT resource validity can be open‑ended or constrained by a timer, but current standards do not define how to release or reactivate the resources when network conditions change.

Proposal 2	RAN2 confirms that study the explicitly signaling for the network to release and resume A‑IoT radio resources, including time and frequency parameters, so that the UE reader can pause A-IoT radio transmissions and later restart them with minimal disruption.

Proposal 3	RAN2 confirms that study new control elements or parameters that indicate release/resume commands for A-IoT resources explicitly, enabling dynamic resource management without impacting other services.

Proposal 4	RAN2 confirms that study how a UE reader requests A‑IoT radio resources to the gNB similar to SR (Scheduling Request).

## 2.3	Handling of RLF and handover for UE reader

Radio‑link failure (RLF) and handover are critical events for an A‑IoT UE reader because the reader sits between the devices and the 5G network. Under today’s specifications, if the radio link between the reader and its serving cell degrades to the point of RLF, or if the reader moves to a new cell and a handover is triggered, the reader must tear down its A‑IoT session and release its resources. Since A‑IoT radio resources are only valid within the serving cell, the device context, identifier mappings and any buffered upper‑layer data collected during inventory or command procedures are discarded. This is highly inefficient: devices that were partway through a session must start over, and valuable data is lost. Furthermore, current specifications provide no method for transferring these contexts to another reader or to a neighbour cell. We propose to study a procedure in which the UE leader detects link degradation, transfers the A‑IoT context and collected data to the gNB or CN before disconnection, and then reconfigures resources in a new cell to continue the service. These procedures would align with existing RRC re‑establishment and handover mechanisms while preserving A‑IoT information.

Observation 3	If a UE reader discards its A‑IoT context during RLF or handover, device identifiers and upper‑layer data can be lost.

Proposal 5	RAN2 need to study the signaling that allow the UE reader to report its device context and buffered data when link quality deteriorates or a handover procedure triggered.

# 3	Conclusion

Based on the discussion in the previous sections we propose the following:

Observation 1	The gNB can assign UE readers A=IoT dedicated radio resources via RRC signaling as specified WID, and these assignments are not valid across cells.

Observation 2	A‑IoT resource validity can be open‑ended or constrained by a timer, but current standards do not define how to release or reactivate the resources when network conditions change.

Observation 3	If a UE reader discards its A‑IoT context during RLF or handover, device identifiers and upper‑layer data can be lost.

Proposal 1	Use the RRC‑based solution as baseline for Topology 2 implementation, while continuing to investigate NAS‑ and UP‑based approaches for future releases when their protocols and resource‑control mechanisms are fully clarified.

Proposal 2	RAN2 confirms that study the explicitly signaling for the network to release and resume A‑IoT radio resources, including time and frequency parameters, so that the UE reader can pause A-IoT radio transmissions and later restart them with minimal disruption.

Proposal 3	RAN2 confirms that study new control elements or parameters that indicate release/resume commands for A-IoT resources explicitly, enabling dynamic resource management without impacting other services.

Proposal 4	RAN2 confirms that study how a UE reader requests A‑IoT radio resources to the gNB similar to SR (Scheduling Request).

Proposal 5	RAN2 need to study the signaling that allow the UE reader to report its device context and buffered data when link quality deteriorates or a handover procedure triggered.

# 4	References

- RP-252894, Revised WI: Solutions for Ambient IoT (Internet of Things) in NR Phase 2, Huawei, Beijing, China, September 15-18, 2025.

- TR 38.769, Study on solutions for Ambient IoT (Internet of Things) in NR，v19.0.0.