3GPP TSG-RAN WG2 Meeting #131bis														R2- 2507218

Prague, Czech Republic, October 13 – 17th, 2025

Agenda Item:		10.3.3

Souce:	Samsung, Verizon

Title:	Considerations for Energy Saving and AS Security in 6GR

Document for:		Discussion

1. Introduction

In RAN#108, a study item on the next generation of non-backward compatible Radio Access Network (RAN), i.e., 6G Radio (6GR), was approved [1]. Based on the SID, this contribution provides a summary of Samsung’s views on UE and network energy saving in 6GR and Access Stratum security aspects.

2. Discussion

2.1 Energy-Saving Technologies in 5G

5G has introduced a range of advanced energy-saving technologies designed to optimize energy efficiency across both user equipment (UE) and network infrastructure (NG-RAN). These innovations represent a significant step forward in reducing energy consumption while maintaining high performance. Figure 1 illustrates the introduced 5G energy saving technologies in each release.

Figure 1 5G UE and Network Energy Saving Technologies

Despite significant advancements, 5G networks face multiple challenges in implementing energy-efficient solutions:

- First, the fundamental trade-off between energy efficiency and network performance. Operators are required to carefully balance power savings with critical QoS requirements including low latency and reliable coverage, while simultaneously managing different parameters across various UEs in different operational states.

- Second, periodic and always-on signaling requirements. Essential network operations such as synchronization signals, reference signals, and system information updates prevent both UEs and gNBs from entering deeper, more efficient sleep states, creating inherent limitations in power saving capabilities.

- Third, the lack of harmonization between different power-saving technologies. Energy-efficient solutions for UEs and networks have been developed separately without proper coordination, resulting in limited system-level efficiency.

- Fourth, the increasing network complexity. Modern deployments face coordination challenges in mixed macro/small cell scenarios, coexistence issues in unlicensed spectrum bands, and unique synchronization requirements when integrating NTN and diverse IoT devices - all of which complicate energy optimization efforts.

- Finally, the gradual development of new features creates challenges in adaptation. The gradual rollout of energy-saving technologies across different 5G releases creates compatibility gaps, preventing early-generation devices from utilizing newer power-saving features.

These challenges collectively impact the real-world effectiveness of 5G energy-saving technologies and highlight areas requiring further research and development. In contrast, 6G should aim to overcome these limitations through innovative and integrated approaches.

Observation 1: 5G networks face inherent limitations in energy efficiency due to fundamental trade-offs between power savings and QoS requirements, always-on signaling, and lack of harmonization between UE and network power-saving technologies.

Observation 2:5G networks also encountered challenges from increasing network complexity due to mixed deployments and heterogeneous device integration, as well as compatibility gaps arising from the gradual development of new features.

2.2 RAN1 progress on 6G Energy Saving

- Energy efficiency Agreement Study how to reuse and update reference configurations in TR 38.864 for 6G BS. Agreement Study how/whether to reuse or update the power model in TR 38.864 for evaluating BS power consumption for 6G BS. Agreement - Study metric(s) for UE energy efficiency. - Study metric(s) for BS energy efficiency. Agreement Study reference configurations and power consumption model for 6G UE, considering but not restricted to the following: - TR 38.840 (UEPS), TR38.875 (RedCap), TR38.865 (eRedCap), and TR38.869 (LP-WUS/WUR) for reference configurations - TR 38.840 (UEPS), TR38.875 (RedCap), and TR38.869 (LP-WUS/WUR) for power consumption models Agreement - Study baseline BS setting(s) for evaluating 6G BS EE improvement/impact, considering NR features and 6G BS reference configuration(s) - Study baseline UE setting(s) for evaluating 6G UE EE improvement/impact, considering NR features and 6G UE reference configuration(s)

During the recent RAN1#122 meeting, RAN1 initiated 3GPP’s first 6G Study. Extensive discussions ensued, resulting in key agreements on energy-saving technologies for both UEs and network infrastructure in the 6G era, as outlined below:

Clearly RAN1 discussed to advance energy-saving technologies for 6G UEs and base stations, including updates to reference configurations (TR 38.864), power consumption models, and metrics for evaluating UE/BS energy efficiency, along with defining baseline of 6G performance assessment.

Building on RAN1’s previous work, RAN2 will study protocol-layer innovations and system-level optimizations to realize holistic power savings in 6G networks. Our research will address both UE-centric and BS-centric efficiency enhancements while prioritizing cross-layer synergies and backward compatibility with NR.

2.3 Network Power Saving in 6GR

In 3GPP 5G NR, Cell DTX (Discontinuous Transmission) and DRX (Discontinuous Reception) were introduced to enhance network energy efficiency, specifically in the RRC Connected state. These techniques allow base stations to minimize power consumption by dynamically suspending transmission and reception during inactive periods. However, there exists significant limitation that these mechanisms were not extended to the Idle and Inactive Modes, leaving untapped potential for energy savings in scenarios with low or no user activity.

Observation 3: In 5G NR, Cell DTX/DRX were introduced to enhance network energy efficiency in the RRC Connected state but were not extended to Idle and Inactive Modes, leaving untapped potential for energy savings.

Furthermore, while Release 19 explored on-demand SSB (Synchronization Signal Block) and SIB1 (System Information Block 1) transmission, the implementation of on-demand SSB was confined to SCell operations. This restriction limited their broader applicability in PCell (Primary Cell) and standalone deployments, preventing more widespread energy efficiency improvements.

To address these shortcomings in the 6G era, a more comprehensive approach is needed. One key proposal is to extend Cell DTX/DRX functionality to Idle and Inactive Modes, enabling base stations to enter low-power states during inactivity while ensuring rapid reactivation when needed. Another critical advancement would be the implementation of dynamic, on-demand signaling across all cell types, including PCells and standalone cells, rather than restricting it to SCells.

Proposal 1: RAN2 should study a unified Cell DTX/DRX mechanism that works in RRC Connected, Idle and Inactive Modes.

Figure 2 6G Dynamic Cell On/Off and Coordinated DTX/DRX

A more sophisticated approach to enhancing network energy efficiency involves the strategic separation of coverage carriers from capacity carriers, enabling their independent operation, as in Figure 2 (left-side). This architectural innovation allows for dynamic activation and deactivation of capacity carriers based on real-time traffic demands, significantly reducing unnecessary power consumption while maintaining essential network coverage. By implementing this technology, both network and UE can maximize their deep sleep periods, minimizing energy waste during low-traffic intervals. The 6G system should support both network-initiated and UE-assisted strategies for dynamic cell management (e.g., dynamic cell on/off), offering flexibility in operational approaches. However, this separation may require adjustments to UE initial access procedures to ensure seamless connectivity. Given these potential benefits, RAN2 should prioritize studying carrier separation from the earliest stages of 6GR development, focusing on network deployment strategies, UE initial access procedures, dynamic resource allocation mechanisms, and power management optimization to establish it as a foundational element of the 6G network architecture.

Proposal 2: RAN2 should study carrier separation architecture from 6GR's initial phase, focusing on network deployment strategies and UE initial access procedures to optimize both network energy efficiency and UE power consumption.

Another key strategy for optimizing energy efficiency in 6G involves synchronizing active and inactive periods between user equipment (UEs) and 6G NodeBs (NBs), as in Figure 2 (right-side). By building upon existing cell DRX/DTX and UE DRX mechanisms, UE should be enabled to propose optimized cell DTX/DRX patterns to the network. Further, catering to a diversity of UEs and service requirements, 6GR should target a comprehensive energy saving framework offering a multitude of “energy saving modes” and enabling a coordinated approach between UEs and 6G NodeBs for the selection and application of the suitable energy saving mode. This coordination would facilitate simultaneous energy savings across both UEs and network infrastructure, creating a more efficient ecosystem.

Proposal 3: RAN2 should strive for a joint UE-network power saving mechanism in 6GR to coordinate and jointly optimize energy efficiency across the entire network.

2.4 UE Power Saving in 6GR

Low UE power consumption is not just a desirable feature for 6GR networks but a foundational design objective. It plays a critical role in meeting global sustainability goals, enhancing user experience through extended battery life, and supporting diverse applications—from ultra-low-power IoT devices to high-performance XR/AR applications. While NR introduced iterative power-saving enhancements, 6GR must adopt a holistic and unified approach from the outset to avoid fragmentation and inefficiencies.

6GR should treat UE power saving as a foundational technology, integrating existing NR mechanisms cohesively while eliminating redundancy. Key aspects include:

- Unified Framework: Avoid multiple overlapping mechanisms (e.g., LP-WUS, PEI, DCI 2_6) by designing a single, optimized wake-up signal;

- Backward Compatibility-Free Design: Since 6GR does not need to support legacy LTE/NR constraints, it can adopt more efficient signaling structures;

- Cross-Working Group Coordination: Collaborate with RAN WG4 to evaluate system-level impacts (e.g., sync raster sparsity on cell search efficiency).

For the first point, to illustrate the design methodology, we consider the 6GR wake-up signal (WUS) design by leveraging the concept of integrating LP-WUR within a Single Radio framework, which doese not requiring separate Low Power Radio (LR). Unlike the NR approach, which introduced multiple WUS designs (e.g., LP-WUS, DCP, and PEI) across different UE states, 6GR should prioritize a unified WUS design from the outset to avoid redundancies. Specifically, we propose embedding LP-WUR functionality within the Single Radio architecture, where the WUS can be monitored using the same receiver components as the main radio, with selective power-down of non-essential modules during WUS reception. This approach would reduce UE complexity by eliminating the need for a separate low-power radio (LR) while maintaining power-saving benefits. Additionally, the design should account for joint network and UE energy efficiency by minimizing resource overhead and latency. For RRM measurements during WUS monitoring, SSB can serve as a reference with relaxed periodicity for both serving and neighboring cells. To further optimize power consumption, the UE could perform neighboring cell measurements during WUS monitoring, ensuring consistent coverage with other downlink signals like SSB.

Observation 4: 5G NR introduced multiple fragmented wake-up signal (WUS) designs (e.g., LP-WUS, DCP, PEI) across different UE states, resulting in implementation complexity and suboptimal power efficiency due to lack of a unified approach.

So we propose to study the integration of LP-WUR within a Single Radio architecture to enable efficient WUS monitoring without requiring a separate receiver. This design should focus on minimizing resource overhead, joint energy consumption, and UE complexity while maintaining compatibility with existing downlink signals.

Proposal 4: RAN2 should study the integration of WUR/WUS within a Single Radio architecture to enable efficient WUS monitoring without requiring a separate receiver.

While 5G NR introduced various power-saving techniques leveraging Uplink Assistance Information (UAI) to enhance UE energy efficiency, many of these mechanisms have not been adopted in real-world deployments. Despite their theoretical benefits, practical implementation challenges—including network complexity, backward compatibility requirements, and the required UE side information —have hindered their widespread adoption. For instance, features such as UE-assisted BWP switching and early RRC release based on traffic patterns could significantly reduce power consumption but remain underutilized.

To overcome these limitations, the advancement of UAI in 6G must focus on three key aspects: simplicity, interoperability, and intelligent automation. By enabling UEs to dynamically report their operational states and traffic characteristics, UAI can facilitate more adaptive and context-aware power-saving strategies. This includes optimizing sleep cycles, resource allocation, and signaling overhead based on real-time UE behavior while maintaining performance levels.

The table below demonstrates some examples of potential energy savings achievable through UAI-driven optimizations, based on study from 3GPP NR:

Table 1 UAI driven power saving techniques in NR

| Technique | Applicable Traffic Patterns | Expected gain [2] |
| --- | --- | --- |
| Fast RRC release | Bulk download with following idle periods (YouTube, Netflix, web browsing, …) | 95% ~ 99% reduction by transition to RRC_Idle/Inactive |
| BWP adaptation | Low-latency persistent traffic (gaming, live streams, ..) | 16% ~ 45% reduction by bandwidth-reduction |
| SCell release | High-volume data download for sustained periods (FTP, video download, …) | 12% ~ 57.75% reduction by carrier component(CC) reduction |

Observation 5: 5G's UAI-based power-saving features faced low adoption adoption due to implementation complexity and backward compatibility issues.

Proposal 5: RAN2 should study enhanced UAI frameworks that can be practically implemented and effectively utilized by operators, networks, and UEs.

2.5 Access Stratum Security

Security has been a major USP for 3GPP technologies from the beginning. With each generation, 3GPP has introduced considerable enhancements to security mechanisms. For instance, 5G NR (New Radio) Access Stratum (AS) security introduced significant advancements over 4G LTE (Long Term Evolution), including mandatory support of integrity protection for user data, stronger cryptographic algorithms, encryption of the International Mobile Subscriber Identity (IMSI) and usage of Temporary Mobile Subscriber Identity (TMSI), secure initial signalling messages and mutual authentication between the UE and network.

However, the security threats and vulnerabilities also have emerged over the time with the increasing complexity of networks, higher data rates, and emerging threats like quantum computing attacks. It is imperative that 6GR provides solutions to maintain trust, reliability, and resilience in the 6G ecosystem.  Two major threats that need to be addressed in 6GR are related to the protection of lower layer signalling (MAC CE) and the broadcasted system information message.

MAC signalling protection

In NR as well as the older technologies like 4G LTE or 3G UMTS (Universal Mobile Telecommunications System), PDCP (Packet Data Convergence Protocol) layer, which is in the upper part of the user plane protocol stack performs encryption and integrity protection of user data. There is no cryptographic protection defined for the control signalling in the lower layers. This was mainly due to the assumption of low threat exposure for the lower layer information at the time of defining security architecture. However, with the evolution of the system, lower layers are performing a larger number of functions compared to the time when the PDCP based AS security was introduced. For instance, with the introduction of L1/L2 triggered mobility (LTM), the MAC (Medium Access Control) sublayer can be used for performing mobility from one cell to another. Tampering of LTM event triggered measurement reports or the LTM cell switch command can directly impact mobility of the UE. Moreover, with Inter-CU LTM, security related parameters such as NCC are send without protection.

Even attacks on the (MAC Control Elements) MAC CE(s) such as Buffer Status Reports (BSR), Power Headroom Reports (PHR) or Discontinuous reception (DRX) commands directly impact the UE scheduling and power consumption. Plain text transmission of commands used for Timing Advance (TA) control can impact the user privacy. Similarly, an attack on MAC CE(s) used for SCell Activation and Deactivation can lead to a considerable impact on the user performance. We also need to consider that the attackers also have evolved over the past twenty years, from the time the PDCP based ciphering and integrity were introduced. Along with this, a large number of newer use cases which require high reliability, lower latency or both are supported now. Based on all these, it is clear that earlier perception of low threat exposure for lower layer signalling does not hold good anymore, and there is a strong need to protect the lower layer signalling such as MAC CE(s) We also acknowledge that all the MAC CE(s) may not be critical to protect. For instance, the impact of a security attack on a MAC CE like Duplication Avoidance may be minimal. Thus, it may be sufficient to protect a set of selected MAC CE(s).

Observation 6: In 5G NR, MAC CE signalling carrying critical information may be subject to potential security vulnerabilities.

Though the need for protection of some of the MAC CE(s) such as LTM cell switch command, where identified during the 5G NR time frame, introducing such a major change to the security architecture was complex and infeasible during the lifecycle of an existing technology. But with 6G, with stricter requirements on reliability and latency increased focus on energy savings and AI and adaption of diverse use cases, this protection is no longer optional. So we propose that RAN2 needs to study enhancing the protection of MAC CE(s). RAN2 can consider extending the existing 5G security mechanisms of ciphering and integrity protection to MAC CE(s), in collaboration with SA3.

Figure 3: Protection of MAC CE(s)

Proposal 6: RAN2 to study enhanced AS layer security to protect MAC CE signalling.

Broadcasted System Information (SI) protection

Figure 4: Security concerns with false base station in 5G

Present day wireless communication systems are vulnerable to potential threats from false (fake) base stations, where the false base station is a device that impersonates as a cellular base station to intercept and manipulate communications towards mobile devices. The false base station could trick the mobile devices in the vicinity into connecting to it by emitting a stronger signal as the cellular base station or it may completely block the connectivity for these mobile devices to the genuine base station. Specifically, the various security threats include user tracking, data theft or fraud by intercepting unencrypted data or authentication codes, Denial-of–Service (DoS) attacks, downgrade attacks by forcing device to connect to less secure network protoocols and performance degradation (e.g. draining battery by repeated search for connection). The recent incident of a security breach with KT operator in South Korea (Aug 2025) involving 4 fake base stations resulted into micropayments fraud and has proven the worst fears.

More particularly, the broadcast messages such as MIB (Master Information Block) and SIBs (System Information Blocks) which carry vital system parameters that govern the initial access and connectivity to the network are in transmitted in plain text and could be easily tempered with by the fake base station. SA3 has previously studied on these security concerns and also worked out the solutions, which are based on the digital signing the system information using non-Post Quantum Cryptography (PQC) or PQC asymmetric cryptography algorithm [TR 33.809]. Accordingly, mobile devices can verify and validate the authenticity of the cell and take appropriate actions in terms of pursuing the connection or reselecting to another cell. Incidentally, RAN2 and SA3 had a liaison exchange on this topic [R2-2206976/8985] during 5G standardization phase (Release 18 time). But it did not progress much for 5G, primarily due to the concerns about complexity in modifying existing system information design in the middle of a generation.

Observation 7: In 5G NR, system information broadcast is prone to potential threats from False Base Stations.

As the 6G is offering an entirely new and fresh opportunity to consider the security aspects without any conerns of backward compatibility, we understand RAN2 needs to seriously engage in this study in coordination with SA3. The broader issues for RAN2 to enable protection of system information includes a careful evaluation of the solutions and their potential impacts to RRC and related performance and signalling aspects.

Figure 5: Protection of system information

Proposal 7: RAN2 to study enhanced AS layer security to protect broadcast system information message and mitigate threat from false base stations.

3. Conclusion

With lessons learned from NR, the following observations and proposals are derived:

Observation 1: 5G networks face inherent limitations in energy efficiency due to fundamental trade-offs between power savings and QoS requirements, always-on signaling, and lack of harmonization between UE and network power-saving technologies.

Observation 2:5G networks also encountered challenges from increasing network complexity due to mixed deployments and heterogeneous device integration, as well as compatibility gaps arising from the gradual development of new features.

Observation 3: In 5G NR, Cell DTX/DRX were introduced to enhance network energy efficiency in the RRC Connected state but were not extended to Idle and Inactive Modes, leaving untapped potential for energy savings.

Observation 4: 5G NR introduced multiple fragmented wake-up signal (WUS) designs (e.g., LP-WUS, DCP, PEI) across different UE states, resulting in implementation complexity and suboptimal power efficiency due to lack of a unified approach.

Observation 5: 5G's UAI-based power-saving features faced low adoption due to implementation complexity and backward compatibility issues.

Observation 6: In 5G NR, MAC CE signalling carrying critical information may be subject to potential security vulnerabilities.

Observation 7: In 5G NR, system information broadcast is prone to potential threats from False Base Stations.

Proposal 1: RAN2 should study a unified Cell DTX/DRX mechanism that works in RRC Connected, Idle and Inactive Modes.

Proposal 2: RAN2 should study carrier separation architecture from 6GR's initial phase, focusing on network deployment strategies and UE initial access procedures to optimize both network energy efficiency and UE power consumption.

Proposal 3: RAN2 should strive for a joint UE-network power saving mechanism in 6GR to coordinate and jointly optimize energy efficiency across the entire network.

Proposal 4: RAN2 should study the integration of WUR/WUS within a Single Radio architecture to enable efficient WUS monitoring without requiring a separate receiver.

Proposal 5: RAN2 should study enhanced UAI frameworks that can be practically implemented and effectively utilized by operators, networks, and UEs.

Proposal 6: RAN2 to study enhanced AS layer security to protect MAC CE signalling.

Proposal 7: RAN2 to study enhanced AS layer security to protect broadcast system information message and mitigate threat from false base stations.

4. Reference

- RP-251881	New SID: Study on 6G Radio	NTT DOCOMO	RAN#108

- TS 38.840 	Study on User Equipment (UE) power saving in NR. (Release 16) 	3GPP