3GPP TSG RAN WG2 Meeting #131bis	R2-2507219

Prague, Czech Republic, Oct. 13th-17th, 2025

Agenda Item:	8.9.2

Source:	ETRI, Korea University

Title:		Discussion on leftover issues with Store and Forward satellite operation

Document for:	Discussion and Decision

1	Introduction

In this paper, we would like to discuss and present our views on leftover issues with Store and Forward satellite operation: paging optimization in S&F and RIL [C001] sf-OperationModeNeigh.

2	Discussion

2.1	Paging optimization in S&F

In the previous meeting, RAN2 #131, RAN2 discussed paging optimization in S&F as follows [1]:

R2-2506288 Report of [AT131][305][R19 IoT NTN] S&F open issues Sateliot discussion IoT_NTN_Ph3-Core Proposal 1: Given that there is no immediate need for having to increase the size of a satellite ID and define any new satellite ID, RAN2 is respectfully requested not to pursue this. - We don’t increase the size of satellite ID and or define any new satellite ID as part of this Rel-19 WI Proposal 2: Given that it is not clear how P1 of [4] can optimize paging, RAN2 is respectfully requested not to pursue this. - HW thinks paging optimization is essential and easy to support. Nokia agrees. Ericsson see the benefit - QC thinks this is not easy and we cannot even discuss this alone without involvement of other groups. - Samsung is not sure how this would work - Toyota thinks this is useful - ZTE thinks in any case the UE would still have to monitoring system information. HW is not sure - Ericson thinks that SI update would not be trigger - Can come back in the next meeting

The proposal referred to as “P1 of [4]” in the above box is as follows [2]:

Proposal 1: An indication is introduced in the System information to indicate that the UE could skip monitoring paging and initiating RRC connection for MT data reception.

The above proposal was made considering that UEs do not need to monitor the paging or initiate RRC connection when feeder link is not available and the NW does not have any buffered MT data.

In short, there is no need to introduce a new indication in SIB to optimize paging for S&F satellite operation. Looking back at the meeting in May, i.e., in RAN2 #130, the following agreement was made [3]:

Agreements: ... 2. In a S&F network deployment which also exhibits discontinuous coverage, existing mechanisms to handle discontinuous coverage can be leveraged (e.g. satellite assistance information, UE not needing to perform idle mode tasks when the UE determines that is out of coverage, etc.). There is no need to modify existing discontinuous coverage features due to the addition of S&F Satellite operation. FFS if we clarify in discontinuous coverage procedure in idle mode that the UE also takes into account the information about NAS configured S&F monitoring list. ...

The above agreement is the result of discussions on whether to utilize S&F Monitoring List or S&F Wait Timer for relaxation of idle mode tasks. On the other hand, the proposal of “P1 of [4]” in the box at the very top considers the case where NW has an empty buffer for MT data. However, both the agreement and the proposal consider a fundamentally the same issue in that they consider the case where NW has no MT data. Consequently, we understand that RAN2 has already agreed to leverage the existing discontinuous coverage if NW wants UEs to relax idle mode tasks when it has no MT data.

RAN2 have already agreed to leverage the existing discontinuous coverage if S&F NW wants UEs to relax idle mode tasks.

Based on this observation, introduction of an indication in SIB for the paging optimization is to revert the above RAN2 agreement. The introduction can be justified only when the benefit brought by the new indication in SIB or the limitations imposed by leveraging the existing discontinuous coverage are clearly identified.

- RAN2 not to pursue an indication in SIB for the paging optimization in S&F satellite operation.

- The existing mechanism of the discontinuous coverage in SIB32 is leveraged when S&F NW wants UEs to skip monitoring paging and initiating RRC connection for MT data reception.

If the existing mechanism of the discontinuous coverage in SIB32 can be leveraged for relaxation of idle mode tasks, it is necessary to introduce assistance information such as S&F mode for each satellite in SIB32.

Coverage of a satellite operating in S&F mode can be different for UEs whether they support S&F operation or not. For example, UEs supporting S&F operation can determine that it is in-coverage if it is within the footprint of the satellite operating in S&F mode.

However, the current discontinuous coverage does not consider S&F satellite operation. The current SIB32 can have only the discontinuous coverage information of satellites operating in normal mode. Thus, even if UEs supporting S&F operation are within the footprint of a satellite operating in S&F mode and thus they are in-coverage, they have no choice but to determine that it is out-of-coverage.

- S&F mode of each satellite is broadcast for discontinuous coverage in SIB32, where the S&F mode indicates only one of S&F mode and normal mode.

2.2	[C001] sf-OperationModeNeigh

RIL [C001] is as follows:

[WI]: IoTNTN [Class]: 2 [Title]: sf-OperationModeNeigh [Tdoc]: R2-250xxxx [Delegate]: CATT (Da Wang) [Status]: ToDo [Description]: As we comment during the email discussion, we think the intention of agreement is to add S&F mode and nomal mode indication in SIB33, which doesn’t have the intention to include “barred” and “notBarred” in SIB33. Because currently the barred/notBarred information is not provide from neighbour cell to serving cell. [Proposed Change]: NeighSatelliteInfo-v19xy ::= SEQUENCE { sf-OperationModeNeigh-r19 ENUMERATED {s&f, normal} OPTIONAL, -- Need OP t-ModeSwitchingNeigh-r19 TimeOffsetUTC-r17 OPTIONAL -- Need OR } [Rapporteur comments]: This was discussed and companies have different understanding on the intention of the agreement. So we need to stick to the agreement itself for now: - The S&F mode indication (i.e., sf-OperationMode) and the S&F mode transition time (i.e., t-ModeSwitching) of the neighbor satellite are signaled in SIB33 per neighbor satellite. Besides, this is aligned with the NOTE added in 36.304, that a UE may deprioritize or choose not to access a SF cell based on this indication. Companies can submit contributions to this issue.

As the rapporteur comments, companies had split views on [C001] due to different understanding of the agreement. In our view, the agreement is just to introduce S&F mode and time information of the neighbour satellite in SIB33 based on the followings:

Firstly, recalling the discussion related to the agreement in the previous RAN2 #130 meeting, no company mentioned the barring information included in sf-OperationMode when it is used for neighber satellites. Apparently, sf-OperationMode is simply an example as written in the agreed text.

Secondly, if sf-OperationMode with barring information needs to be introduced for neighbour satellites, its purpose and usage must be explicitly described. Otherwise, such information will cause significant spec. ambiguity.

- RAN2 to clarify that the S&F mode signalled in SIB33 per neighbour satellite indicates only one of S&F mode and normal mode without any barring information.

- RAN2 to adopt the proposed change for [C001].

3	Conclusion

In this paper, the following observation is made:

Observation 1	RAN2 have already agreed to leverage the existing discontinuous coverage if S&F NW wants UEs to relax idle mode tasks.

In this paper, we propose the followings:

Proposal 1	RAN2 not to pursue an indication in SIB for the paging optimization in S&F satellite operation.

Proposal 2	The existing mechanism of the discontinuous coverage in SIB32 is leveraged when S&F NW wants UEs to skip monitoring paging and initiating RRC connection for MT data reception.

Proposal 3	S&F mode of each satellite is broadcast for discontinuous coverage in SIB32, where the S&F mode indicates only one of S&F mode and normal mode.

Proposal 4	RAN2 to clarify that the S&F mode signalled in SIB33 per neighbour satellite indicates only one of S&F mode and normal mode without any barring information.

Proposal 5	RAN2 to adopt the proposed change for [C001].

References

R2-2506203, Report from session on NR NTN and IoT NTN, Session chair (ZTE), RAN2 #131, Aug. 2025.

R2-2505437, Further consideration on Store and Forward, Huawei, HiSilicon, China Telecom, RAN2 #131, Aug. 2025.

R2-2504673, Report from session on NR NTN and IoT NTN, Session chair (ZTE), RAN2 #130, May. 2025.