3GPP TSG-RAN WG2 Meeting #131bis	R2-2507256

Prague, Czech Republic, 13 – 17 October 2025

Agenda item:	8.2.2

Source:	Nokia

Title:	Open issues for AIoT

WID/SID:	Ambient_IoT_solutions - Release 19

Document for:	Discussion and Decision

# 1	Introduction

After RAN2#131 the AIoT MAC rapporteur identified the following open issues;

- How to capture security parameter in Paging message (Issue 1-7) - Checking whether there is new case for “no upper layer data available” other than long writing operation which may impact to how to set MDI field (Issue 3-7) - Paging ID length impact from new SA2 LS in S2-2507793 (Issue 1-3)

This Tdoc discuss’ the highlighted topics

# 2	Discussion

## 2.1	Issue 1-3 considerations on paging ID length impact

In RAN2#131 the paging ID length was discussed based on current SA2/CT4 progress and it was agreed that an 8-bit length field would be prepended to the paging ID in order to support any length provided by SA2/CT4. The value of 8 bits were included to support the maximum length of 256 bits for the paging I.

8-bit length field (in unit of bit) is assumed to indicate the paging ID length, based on current SA2/CT4 conclusion.

In Aug meeting, SA2 sent a new LS to RAN2 in S2-2507793 on AIoT Device Permanent ID Length, indicating SA2 identified requirements for longer device ID. In the LS, SA2 asks whether RAN2 can accept the paging ID extension size up to 256 or even 496 bits. Note that the increase does not account for the overhead, so the additional 128 bits of information from the ‘old’ paging ID should still be considered. Additionally, a few additional bits should be considered due to the extended length field i.e. 9 bits in case of 256+128 bits and 10 bits in case if 496+128 bits.

Observation 1: SA2s query considers whether RAN2 can support an increased paging ID of either 256 or 496 bits.

Observation 2: The actual paging ID length may be up to 394 or 634, depending on RAN2 conclusion

The use case for the extended bits is amongst others to support healthcare, and considering the vast number of different equipment and the urgency of correct inventory, we believe that AIoT can provide a meaningful supplement to ensure cost effectiveness and heighten quality of treatments.

In RAN2#131 it was raised during discussion on paging aspects, that the closer towards the maximum length of 1000 bits the messages over AIoT air interface reaches, the less coverage the feature will experience. This is true, but given that RAN2 adopts the length field, it can be up to deployment strategies to cohesion between paging ID length and coverage area of the reader. Thus, in case the final paging message can contain the 634 or 394, it is reasonable for RAN2 to enable support.

Observation 3: The increased device ID brings valid use cases to the AIoT feature.

Observation 4: Supporting longer paging length i.e. to accommodate healthcare inventory may extend the adoption of the AIoT feature.

The consideration on length accounts not only for the paging, but with the current increase in size also for the permanent ID response. Additionally for the D2R reply in msg2, step 5 in 5.2.1-1 from 33.369 we get the following security flow to consider as below;

5.2.2 Authentication procedure The authentication procedure is aligned with inventory procedure and command procedure in 6.2.2 and 6.2.3 of TS 23.369[2]. Figure 5.2.1-1: Authentication procedure << Text omitted >

In the above illustration, the D2R response in the inventory request may either be

- RANDAIoT_d + <device ID> in case of no privacy protection, or

- RANDAIoT_d + RESAIoT in case of privacy protection

For the case a), increasing the device ID size has a direct impact not only on the paging request, but also the D2R reply which will increase to a maximum of 762, bringing the payload for a simple device very close to the maximum value for an otherwise assumed to be robust procedure, being inventory. For case b) however, since the RESAIoT is calculated as a Hash of the full ID, it will always remain with the length of 256, plus the added 128 from the RANDAIoT. This means that enabling privacy mitigates the impact of the increased paging ID size. However, this comes at a cost that the AIoT-F needs to revert the hashing, which may be a processing heavy procedure.

We understand that medical equipment may be desired to include privacy anyway, since it may not be wise that everyone can detect and inquiry the stock of i.e. a hospital, thus we believe that the increased paging ID would in most cases be used together with the privacy feature.

Observation 5: Privacy through hashing mitigates the impact of the increased paging ID length.

Observation 6: Enabling privacy protection affects AIoT-F effort as it needs to reverse the hashing process.

Observation 7: For medical equipment, privacy may be a KPI.

Proposal 1: RAN2 to reply to SA2 that 634 bits can be supported as paging ID.

Proposal 2: RAN2 to discuss whether to add to the LS to SA2 that the increased paging ID may only be supported with enabled privacy, or note it in stage 2 specification.

## 2.2	Issue 3-7 considerations on no upper layer data available

In RAN2#131 the “no upper layer data available case” was discussed i.e. the case where the device has not had enough time to produce data for the read command. In such case, we agreed that the device will simply send the data SDU with length 0, and then the reader may request again at a later point.

- The reader determines no data available case by SDU length 0. As more data indication is mandatory, the device sets this bit to "0". - The reader, in response to 0 SDU in the device’s MAC response, may send a follow-up R2D Upper Layer Data Transfer message at a later time to schedule another D2R Upper Layer Data Transfer message from the device. - The follow-up R2D Upper Layer Data Transfer message includes the Received Data Size field with the Received Data Size field set to value 0, without including the original command. - RAN2 would like to check if there is a case where NAS doesn’t provide a response at all. If this case exists, RAN2 will discuss this issue

However, there is an open issue; whether there is a case where NAS would never provide a response i.e. whether RAN2 need to specify a procedure for the device to signal that data will never be available.

In our view, the various paging IDs are implicitly used to determine different vendors/device type and thus also implies knowledge within AIoT-F whether or not data can be generated by the device. As of this, the use-case where a device will continue to reply with the SDU of length 0 upon reader queries should not occur. In case of the device not replying (error case) the device could be considered broken and/or the reader will simply accept defeat and stop trying. Nevertheless, such case is not up to RAN2 conclusion. Similar solution was implicitly agreed when the energy indication was precluded, since lack of energy is one case where data cannot be generated.

Observation 8: Whether/how to indicate the no data after continuous attempts by the reader should be up to RAN3.

Observation 9: RAN2 to assume that no case exists where the AIoT-F is unaware that the device will generate no data.

Proposal 3: RAN2 to assume no case exists where the NAS provides no response at all while AIoT-F expects it.

# 3	Conclusion

Observation 1: SA2s query considers whether RAN2 can support an increased paging ID of either 256 or 496 bits.

Observation 2: The actual paging ID length may be up to 394 or 634, depending on RAN2 conclusion

Observation 3: The increased device ID brings valid use cases to the AIoT feature.

Observation 4: Supporting longer paging length i.e. to accommodate healthcare inventory may extend the adoption of the AIoT feature.

Observation 5: Privacy through hashing mitigates the impact of the increased paging ID length.

Observation 6: Enabling privacy protection affects AIoT-F effort as it needs to reverse the hashing process.

Observation 7: For medical equipment, privacy may be a KPI.

Proposal 1: RAN2 to reply to SA2 that 634 bits can be supported as paging ID.

Proposal 2: RAN2 to discuss whether to add to the LS to SA2 that the increased paging ID may only be supported with enabled privacy, or note it in stage 2 specification.

Observation 8: Whether/how to indicate the no data after continuous attempts by the reader should be up to RAN3.

Observation 9: RAN2 to assume that no case exists where the AIoT-F is unaware that the device will generate no data.

Proposal 3: RAN2 to assume no case exists where the NAS provides no response at all while AIoT-F expects it.