3GPP TSG-RAN2 Meeting #131bis	R2-2507258

Prague, Czech Republic, Oct. 13th - 17th, 2025

Agenda item:	8.2.2

Source:	Qualcomm Incorporated

Title:	Remaining issues of Ambient IoT

WID/SID:	Ambient_IoT_solutions – Release 19

Document for:	Discussion and Decision

# Introduction

The Rel-19 Ambient IoT WI was approved in [1] and the RAN2 objectives were described as follows:

- RAN2 scope: - Specify the necessary functions and procedures for an Ambient IoT compact protocol stack and lightweight signalling procedure to enable DO-DTT and DT data transmission: - A-IoT Paging, including subsequent paging for the same service. Support the options that a paging message contains one identifier, and that a paging message contains no identifier. Note: RAN2 aims to design a paging message format such that multiple identifiers can be contained in one paging message, for forward compatibility purposes. - A-IoT Random access, including re-access for failure handling. Contention-based and contention-free cases are supported. For the contention-based random access, only Solution 1 (3-step only) is included. - A-IoT data transmission, including data (re-)transmission for failure handling. Segmentation is supported at least in D2R. - Only MAC layer is included

In this paper, we will discuss some remaining issues of Rel-19 Ambient IoT (A-IoT).

# Discussion

In RAN2 #131, SA3 LS [2] regarding the security parameters in A-IoT paging message was discussed, and RAN2 made the following agreements.

Agreements

- RAN2 thinks it is feasible from a signalling perspective to add the 128 bits.  However, from RAN2 perspective the less overhead the better, so SA3 should avoid adding additional parameters if possible.

- Indicate to SA3 that RAN2 tries to minimize number of bits required.   Have a maximum size of 1000bits, and whatever they include has to fit in the 1000bits considering bits from all TSG.

- RAN2 will wait for SA3 conclusions in October on whether the “128bit random number in the paging request message” is always required or not.

- Reply to SA3

- -     RAN2 thinks it is feasible from a signalling perspective to add the 128 bits.  However, from RAN2 perspective the less overhead the better, so SA3 should avoid adding additional parameters if possible.

- -     Indicate to SA3 that RAN2 tries to minimize number of bits required.   Have a maximum size of 1000bits, and whatever they include has to fit in the 1000bits considering bits from all TSG.

-	Indicate space pressure from all the WG

From RAN2 perspective, it is feasible to add 128 bits security information for Rel-19 A-IoT, and RAN2 also pointed out the signaling overhead is one of the important aspects of A-IoT paging message, and RAN2 expects to avoid adding more additional parameters if possible. RAN2 will wait for SA3 conclusion whether the 128 bits security information in A-IoT paging message is always required or not.

In the latest SA3 LS [3], SA3 concluded that SA3 designed security aspects on Ambient IoT service for isolated private network only. The details of security aspect of the Filtering Information and the single AIoT Device Identifier can be found in [4]. In the clause 5.2.2 of SA3 spec. [4], step 3 states that NG-RAN shall include RANDAIOT_n in the paging request message to the AIoT device in addition to other device identification information, and step 2 is that AIOTF shall send inventory request message including RANDAIOT_n to NG-RAN. Since SA3 specification uses ‘shall’ for including the security information, it is clear that the security information RANDAIOT_n should be mandatory in the A-IoT paging message.

Observation 1: SA3 specification indicates the paging message shall include RANDAIOT_n in addition to other device identification information.

SA3 may have considered various security scenarios. For example, consider a non-protected/non-private deployment where authentication would be needed: Even when a legitimate NW may configure to perform the authentication, the attacker/ rouge reader nearby could ‘pretend’ that RANDAIOT_n is ‘not used’ by flipping the bit that indicates optionality, making the devices believe they are in private/protected environment (the device may not know whether it is in private or public setting), thereby violating security protection in case of non-physically-secured/non-private deployment.

Thus, if anything needs to be changed or if there is misunderstanding on the security information, it should be discussed in SA3 first and if SA3 decides to update their specification to make it optional, RAN2 can follow SA3 decision. For RAN2, this should be added as a mandatory field in the paging message, which is currently missing from the A-IoT MAC specification.

Proposal 1: Add a 128-bit mandatory field in A-IoT paging message for both CBRA and CFA to indicate the security information.

Another remaining issue from the last RAN2 meeting is whether there is a case that ‘NAS does not respond at all’. The following agreements were made.

Agreements on no data available due to delay in NAS

- The reader determines no data available case by SDU length 0.   As more data indication is mandatory, the device sets this bit to "0".

- The reader, in response to 0 SDU in the device’s MAC response, may send a follow-up R2D Upper Layer Data Transfer message at a later time to schedule another D2R Upper Layer Data Transfer message from the device.

- The follow-up R2D Upper Layer Data Transfer message includes the Received Data Size field with the Received Data Size field set to value 0, without including the original command.

- RAN2 would like to check if there is a case where NAS doesn’t provide a response at all.   If this case exists, RAN2 will discuss this issue.

There may be a case where the A-IoT device fails to process the received A-IoT NAS command and is not able to generate any A-IoT NAS command response message. Thus, RAN2 should first confirm it is valid that NAS doesn’t provide a response at all.

Observation 2: It is a valid scenario where A-IoT device fails to process the received A-IoT NAS command and is not able to generate any A-IoT NAS command response message.

If the ‘no NAS response’ case can be confirmed to be valid, the 0 SDU based solution is not enough. Reader should be indicated with more information to differentiate whether this is the ‘delayed NAS response’ or ‘no-NAS response’. Otherwise, Reader may continue sending R2D messages to schedule the D2R Layer Data Transfer message from the device even when A-IoT device has no NAS response to send.

Proposal 2: RAN2 confirms it is valid that in some cases A-IoT NAS doesn’t provide a response at all.

Proposal 3: Besides 0 SDU in the D2R Layer Data Transfer message, A-IoT device should indicate to Reader there is ‘no NAS response expected’ in order to differentiate with the ‘delayed NAS response’ case.

# Conclusion

Based on the discussion and observations, we’d recommend RAN2 to discuss and adopt the following proposals:

Observation 1: SA3 specification indicates the paging message shall include RANDAIOT_n in addition to other device identification information.

Proposal 1: Add a 128-bit mandatory field in A-IoT paging message for both CBRA and CFA to indicate the security information.

Observation 2: It is a valid scenario where A-IoT device fails to process the received A-IoT NAS command and is not able to generate any A-IoT NAS command response message.

Proposal 2: RAN2 confirms it is valid that in some cases A-IoT NAS doesn’t provide a response at all.

Proposal 3: Besides 0 SDU in the D2R Layer Data Transfer message, A-IoT device should indicate to Reader there is ‘no NAS response expected’ in order to differentiate with the ‘delayed NAS response’ case.

# References

- RP-250796, Solutions for Ambient IoT (Internet of Things) in NR.

- R2-2505064, LS on security parameter in paging request message, SA3

- R2-2506753, Reply LS on paging ID length, SA3

- TS 33.369 v1.0.0, Security aspects of Ambient IoT service for isolated private networks, 3GPP