3GPP TSG-RAN WG2 Meeting #131bis	R2-2507260

Prague, Czech Republic, Oct. 13th – 17th, 2025

Agenda Item	: 9.7.2

Source	: LG Electronics Inc.

Title	: Discussion on how to support voice call via IoT-NTN

Document for	: Discussion and Decision

1.	Introduction

The work item on Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 4 approved for Rel-20 and the WID is shown in [1] including the following objectives.

The aim of this work item is to specify the enhancement of NB-IoT-NTN to support IMS voice call over GSO with the following objectives: - Down-select between CP and UP solutions for voice support over NB-IoT-NTN until RAN#110 [RAN2] - Coordination with SA is required - The objective can be updated with further details based on the outcome of the above down-selection - Support of semi-persistent scheduling for NB-IoT-NTN for DL and UL data transmission for voice packets [RAN2, RAN1] - Support of necessary modifications to RRC connection setup procedure for NB-IoT-NTN [RAN2] - Support of necessary modifications for emergency call for voice over NB-IoT-NTN [RAN2] - Study and if feasible, specify UE transmit power higher than PC1 (e.g. up to 37dBm) for NB-IoT-NTN [RAN4] - Note: The enhancements to support voice over NB-IoT-NTN via GSO in this work item may be also applicable to NGSO cases without additional specification enhancements - Note: Coordination with SA4 is expected

According to the WID, SA also approved a new SA2-led SID on IMS voice call over GSO via NB-IoT-NTN in SP-250400. As WT#1 of the SID, both CP and UP-based solutions are being studied, and the SI is to be concluded by TSG#110 (Dec. 2025), including the down-selection between CP and UP solutions.

In RAN2, considering the schedule of the SA and the dependency on the SA, down-selection between CP and UP solutions should be discussed mainly at the upcoming meeting. In this document, we investigate which solution is preferred and show our views on the other relevant issues of this topic.

2.	Discussion

2.1 Down-selection between UP and CP solutions

The IMS service is supported in the current TN UP with a QoS management scheme, which can provide a guaranteed bit rate. While IoT-TN does not support a guaranteed bit rate since IoT-TN has been developed to support small sized and/or intermittently generated data transmission. Therefore, IoT-TN uses a maximum of two DRBs for user data and one SRB for control signaling. In order for IoT-NTN to support voice call service, we can think about the way that the current TN QoS management scheme is applied to the IoT-NTN under the restricted number of bearers.

For example, if IoT-NTN supports IMS service via UP, one DRB carries IMS (IP Multimedia Subsystem) signaling traffic (e.g., SIP (Session Initiation protocol) message), while the other DRB can deliver real time packets (e.g., RTP/IP/UDP streams). These two DRBs have different QoS requirements. For the voice call service, the DRB for IMS signaling demands high reliability and moderate latency, whereas the DRB for voice packets requires low latency and jitter, with a certain tolerance for packet loss. The DRBs for the IoT-NTN can be configured considering the voice call service requirement. This approach seems simple to implement because it looks possible by just adopting the simplified QoS management scheme in the current TN.

In contrast, supporting voice service via a CP solution, RAN2 has to introduce a new dedicated SRBs for supporting voice service: One for legacy control messages (e.g., RRC signaling), a second for IMS signaling, and a third for voice packet delivery. While SRBs generally uses AM RLC mode for reliability, voice packet delivery would require UM RLC mode to reduce latency and avoid retransmission delays. Additionally, the QoS management procedure, which is traditionally handled at the DRB level, would need to be newly defined for SRBs. These will increase specification impact and implementation complexity. So, if the benefits of when SRBs are used for supporting IoT-NTN voice service is not clear, it looks better to use DRB in terms of the spec impact and simplicity.

Observation 1: If there are not significant performance differences between CP-based solution and UP-based solution, we prefer the way to reuse the legacy IoT-TN mechanism as much as possible.

Proposal 1: Support UP-based solution which can reuse the legacy management in IoT-TN.

Proposal 2: Whether to use dedicated bearer for voice call service can be discussed after the decision of which solution (e.g., CP-solution or UP-solution) will be applied.

2.2 Other issues for whether to support SPS, whether to use dedicated bearer and a new cause value

The voice call service requires low latency to deliver real-time service with minimal delay and consistent link quality to avoid jitter. In addition, voice traffic typically consists of small packets generated at regular intervals by codecs such as AMR-WB or EVS. These packets should be transmitted consistently without pack loss. The SPS (Semi-persistent Scheduling) can be helpful to support for the voice call. Unlikely dynamic scheduling, the SPS can allocate resources on a per-packet basis. The pre-allocated resource at a fixed intervals can be helpful to support voice call service in terms of maintaining low latency, stable jitter performance, and reducing signalling overhead.

In the current IoT-TN supports SPS only for BSR, and not for the normal data packet. We suspect that the reason is that the dynamic resource allocation is more efficient given traffic pattern of the IoT service, and the available bandwidth is limited. However, for the IoT-NTN, applying SPS to normal data packet could be considered. The long propagation delay inherent to NTN makes pre-allocated resources beneficial for reducing transmission latency.

If it is decided that SPS is applied to the IoT-NTN normal data, there are remaining issues which plane will manage this SPS function and whether the SPS function will be applied sonly for the dedicated bearer for the voice call. Therefore, we recommend discussing SPS support for voice call service after the decision on which plane will handle the voice call service.

Observation 2: For supporting the voice service, a consistent transmission with minimal latency should be guaranteed to maintain the service quality.

Observation 3: In the current spec, IoT-TN supports SPS only for BSR. However, considering the long propagation delay in NTN, applying SPS to IoT-NTN voice packet could be beneficial.

Observation 4: Whether to apply SPS to the voice packets can be related to the decision which plane handle the voice service and whether the dedicated bearer is needed for the voice packets.

Proposal 3: whether to support SPS for IoT voice call service can be discussed after the decision of which solution (e.g., CP-solution or UP-solution) will be applied.

For the emergency call, a fast connection should be supported. When UE sends RRCConnectionRequest message to the eNB, the NW checks the cause value of the message and then handles the connection establishment procedure properly, considering the cause value. If the cause value has high priority, the NW establishes the RRC connection quickly and allocates the resources with high priority. The current cause value of the RRCSetupRequest message already includes cause values like “emergency” or “mo-VoiceCall”. Before introducing a new cause value, we can discuss whether the current cause value, such as “emergency” or “mo-VoiceCall” can be reused.

Proposal 4: We can discuss whether the existing cause values, such as “emergency” or “mo-VoiceCall” can be reused for the emergency call of the IoT-NTN.

3.	Conclusion

Observation 1: If there are not significant performance differences between CP-based solution and UP-based solution, we prefer the way to reuse the legacy IoT-TN mechanism as much as possible.

Proposal 1: Support UP-based solution which can reuse the legacy management in IoT-TN.

Proposal 2: Whether to use dedicated bearer for voice call service can be discussed after the decision of which solution (e.g., CP-solution or UP-solution) will be applied.

Observation 2: For supporting the voice service, a consistent transmission with minimal latency should be guaranteed to maintain the service quality.

Observation 3: In the current spec, IoT-TN supports SPS only for BSR. However, considering the long propagation delay in NTN, applying SPS to IoT-NTN voice packet could be beneficial.

Observation 4: Whether to apply SPS to the voice packets can be related to the decision which plane handle the voice service and whether the dedicated bearer is needed for the voice packets.

Proposal 3: whether to support SPS for IoT voice call service can be discussed after the decision of which solution (e.g., CP-solution or UP-solution) will be applied.

Proposal 4: We can discuss whether the existing cause values, such as “emergency” or “mo-VoiceCall” can be reused for the emergency call of the IoT-NTN.

4.	Reference

- RP-252473, Revised WID: Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 4.