3GPP TSG-RAN WG2 Meeting #131bis	R2-2507265

Prague, Czech Republic, 13 – 17 October 2025

Agenda item:	8.12.2

Source:	Nokia

Title:	MAC Open issues

WID/SID:	NR_MIMO_Ph5-Core - Release 19

Document for:	Discussion and Decision

# 1	Introduction

Following RAN2#131, the NR_MIMO_Ph5 work item was declared complete in RAN2, and all critical points are considered to be addressed. However, in our view some clarifications are still needed on the MAC and  in this contribution we share our views on some of the MAC open issues listed in  [Post131][217][MIMO_Ph5] CR for TS 38.321 (Samsung).

# 2	MAC Open Issues

## 2.1	Issue 1: UEI PUCCH resource release upon sTAG TAT expiry

For a UEI report configuration (CSI-ReportConfig) associated with a SCell, the PUCCH resource used for transmitting the UEI report may be configured either on the SCell itself or on another cell acting as the PUCCH cell. In addition, for a Mode-B type-1 CG associated with a UEI report configuration of an SCell, the configured grant must be on the same SCell.

Currently, when the TAG associated with an SCell expires, the PUCCH resource of a UEI report configuration is not released  if that resource is located on a different cell belonging to another TAG. This results in a situation where the PUCCH resource for the UEI report configuration remains configured, but PUSCH transmission for CSI reporting is no longer available due to the expiry of the TAT. In such a case PHY behaviour regarding whether the UE should still transmit the PUCCH for the UEI report remains unclear.

Therefore, we propose the following clarification:

Proposal 1: For Mode B UEI reporting, upon expiry of the TAG of an SCell, the UE shall release the PUCCH resources configured in the UEI report configuration of that SCell, regardless of whether the PUCCH resource is located on the SCell itself or on a different cell. The TP for proposal 1 is given below:

## 5.2 Maintenance of Uplink Time Alignment … 1> when a timeAlignmentTimer expires: … 2> else: 3> if the timeAlignmentTimer is associated with a TAG for an SCell configured with only this TAG; or 3> if the timeAlignmentTimer is associated with a TAG for an SCell, and if the SCell is configured with two TAGs and the timeAlignmentTimer associated with the other TAG is not running: 4> flush all HARQ buffers for all such SCells; 4> notify RRC to release PUCCH, if configured for all such SCells; 4> notify RRC to release SRS, if configured for all such SCells; 4> clear any configured downlink assignments and configured uplink grants for all such SCells; 4> clear any PUSCH resource for semi-persistent CSI reporting for all such SCells; 4> maintain NTA (defined in TS 38.211 [8]) of this TAG. 5> if there is PUCCH resource configured on a serving cell referred to by pucch-Cell included in CSI-ReportUE-IBR of an associated CSI-ReportConfig that provides these configured uplink grants: 6> notify RRC to release the PUCCH resource for all such serving cells; 4> maintain NTA (defined in TS 38.211 [8]) of this TAG.

## 2.2	Issue 10: DRX Active time in Mode-A UE IBR

As per 38.321, DRX active is defined as:

When DRX is configured, the Active Time for Serving Cells in a DRX group includes the time while: …… - a PDCCH scheduling a UE-initiated CSI reporting has not been received after transmitting UE Initiated Report Indication on PUCCH for mode-A UE-initiated CSI reporting (as specified in TS 38.214 [7]).

During the open issue discussion, it was noted that if the PDCCH for a CSI report is never received - either because the network does not schedule the PDCCH after receiving the UEIRI, or because the network fails to receive the UEIRI on PUCCH - the UE remains in DRX active time indefinitely, since the condition is always fulfilled. This leads to unnecessary power consumption.

For the above scenario, our understanding is aligned with Option 1 described in the offline discussion document for Issue 10 – that is the UE stays in the active state until the next PUCCH resource.

However, in our view, this behavior is already clearly specified in the MAC specification, and therefore no change to the MAC spec is required.

Proposal 2:  If a PDCCH scheduling a mode-A UEI CSI report is not received after transmission of UEIRI, UE stays in the active state until the next PUCCH ressource for transmitting UE Initiated Report Indication. No MAC spec change.

# 3	Conclusion

Proposal 1: For Mode B UEI reporting, upon expiry of the TAG of an SCell, the UE shall release the PUCCH resources configured in the UEI report configuration of that SCell, regardless of whether the PUCCH resource is located on the SCell itself or on a different cell.

Proposal 2:  If a PDCCH scheduling a mode-A UEI CSI report is not received after transmission of UEIRI, UE stays in the active state until the next PUCCH ressource for transmitting UE Initiated Report Indication. No MAC spec change.