3GPP TSG-RAN WG2 Meeting #131bis 	R2-2507270

Prague, Czech Republic, Oct. 13th-17th, 2025

Agenda Item:	10.3.2

Source:	Panasonic

Title:	RRC states and connection management for 6GR

Document for:	Discussion

# Introduction

RAN plenary meeting#108 has approved the 6G radio (6GR) study item (RP-251881). The study represents an opportunity to revisit the foundations of radio system design, free from the constraints of backward compatibility, and thus focusing on a transformative leap in capability, efficiency, and simplicity.

In this paper, we discuss RRC modeling and connection management for 6GR.

# Discussion

## RRC states

In 5G NR, 3 RRC states have been defined: RRC_IDLE, RRC_INACTIVE and RRC_CONNECTED. The following table 1 summarizes the main differences.

Table 1. RRC States in 5G NR

| RRC_IDLE | RRC_INACTIVE | RRC_CONNECTED |
| --- | --- | --- |
| - No RRC connection is established - UE sleeps most of time - UE can wake up to, e.g. monitor paging, perform cell reselction, initiate RRC connection | - Intermediate state between IDLE and CONNECTED - RRC connection has been established, but inactive (suspended) - UE is allowed to sleep in a similar way as in IDLE - From CN perspective, UE is still connected (the same as in CONNECTED) - UE RAN context is kept at both UE and gNB for facilitating the transition from INACTIVE to CONNECTED: | - RRC connection is established - Active data transmission to/from gNB |

However, the current 3-state design has some drawbacks. First, for both RRC_IDLE and RRC_INACTIVE, network needs to track UE in order to decide the area to send paging message. But two different UE tracking mechanisms are used. For IDLE state, UE tracking is managed by CN (e.g. AMF) via Registration Area. For INACTIVE, UE tracking is managed by RAN (e.g. gNB) via RAN Notification Area (RNA). This is because CN is typically not aware the UE transitions between RRC_INACTIVE and RRC_CONNECTED, where in both cases UE remains connected to CN. If RAN decides to transition UE into INACTIVE, it is RAN’s responsibility to track the UE. As a result, both network and UE needs to maintain two levels of tracking in order to cope with transitioning between IDLE and INACTIVE states, increasing complexity at both UE and network.

Second, INACTIVE state does not provide much gain compared to IDLE state, because:

- INACTIVE state has similar power consumption as IDLE state

- INACTIVE state has similar latency as IDLE state in terms of transitioning to CONNECTED state (although UE RAN context is kept in INACTIVE, the dominant factor for latency is random access procedure, which is required for both INACTIVE and IDLE state to transition to CONNECTED)

In view of the above, it is worth questioning whether 6G can simply the RRC state design. For example, if we can unify IDLE and INACTIVE into one single RRC state instead of removing INACTIVE, the state management in both network and UE becomes simpler. Further, duplicated UE tracking mechanism can be removed.

In this design direction, two-state RRC modeling can be introduced: RRC connected state and RRC non-connected state. The RRC non-connected state can include the following cases:

- Case 1: RRC connection has not been established before (e.g. Initial state after power on)

- Case 2: RRC connection has been established before, but now is released

- Case 3: RRC connection has been established before, but now is suspended

Proposal 1. For 6GR RRC state design, consider 2-state RRC modeling by unifying RRC_IDLE and RRC_INACTIVE into a single RRC non-connected state.

For the new 2-state design, when transitioning from connected state into non-connected state, network can decide on whether to store UE RAN context. Then during state transition, network can indicate to UE the decision (whether UE should store RAN context). The stored RAN context can be used when connection is resumed. Specifically, when transitioning from non-connected state to connected, UE and network can exchange information on whether it has stored RAN context and make a decision whether to use the stored RAN context or not. This provides a flexible way to manage UE RAN context. The following figure 1 illustrates the UE context management with state transitioning.

Fig.1. two-state RRC model with UE RAN context management

Proposal 2. In the 2-state RRC model, network controls the following:

- whether UE RAN context should be stored once UE is transitioned into non-connected state,

- whether the stored UE RAN context should be used once UE is transitioned to connected state.

When the network decides to maintain the UE RAN context, it can be stored at the (last serving) base station without CN awareness. Alternatively, it can be stored at CN function entity such as AMF. In the latter case, interaction between RAN and CN is needed. The following section 2.2 discusses such interaction.

## Connection management states

In 5G NR, two CM (Connection Management) states are used to reflect the NAS signaling connection of the UE with AMF. These are CM-CONNECTED and CM-IDLE. In CM-CONNECTED, a NAS signaling connection is established, typically involving an RRC connection between the UE and the gNB, and an N2 connection between the gNB and the AMF. In CM-IDLE, no NAS signaling connection exists. The RRC_IDLE state corresponds to CM-IDLE. When the UE is in CM-CONNECTED, it can be in either RRC_CONNECTED or RRC_INACTIVE. Table 2 summarizes this. In general, CN is not aware of the UE transitions between CM-CONNECTED with RRC_CONNECTED and CM-CONNECTED with RRC _INACTIVE, unless CN is notified by RAN. It is up to RAN (e.g. gNB) to determine the conditions to enter RRC_INACTIVE.

Table 2. RRC states vs. NAS states in 5G NR

| 5G NR RRC States | 5G NR NAS States |
| --- | --- |
| RRC_IDLE | CM-IDLE |
| RRC_INACTIVE | CM-CONNECTED |
| RRC_CONNECTED |  |

For the new 2-state RRC model discussed in section 2.1, if the UE RAN context is maintained at CN, a new NAS state can be introduced to facilitate the UE RAN context management. This new NAS state allows the CN to differentiate whether UE RAN context is kept or not during RRC non-connected state.

For example, The CM-SLEEP state is a newly introduced NAS state for a UE. While the UE is in CM-SLEEP, it is still in RRC non-connected, meaning the connection between the UE and the network node (e.g., base station) is not established. However, the CM-SLEEP state is distinct from CM-IDLE in that it explicitly signifies that UE RAN context is kept by the CN (e.g., AMF).

The mapping between NAS states and compatible RRC states for 2-state RRC model is summarized in Table 3.

Table 3. Mapping between RRC states and NAS states for the proposed model

| NAS state | RRC state |
| --- | --- |
| CM-SLEEP - Connection between CN (e.g.AMF) and last serving BS is suspended (inactive, but the parameters for the connection are kept) - UE RAN context is sent to AMF when last serving BS transitions the UE to RRC State 1. AMF stores the UE RAN context. - UE reachability is managed by Registration Area in AMF | RRC non-connected |
| CM-IDLE - As in 5G NR | RRC non-connected |
| CM-CONNECTED - As in 5G NR | RRC connected |

Proposal 3. For 2-state RRC model, consider to maintain the UE RAN context at CN when UE is in RRC non-connected state.

# Conclusion

In this paper, we have discussed RRC modeling and connection management for 6GR. The proposals are as follows.

Proposal 1. For 6GR RRC state design, consider 2-state RRC modeling by unifying RRC_IDLE and RRC_INACTIVE into a single RRC non-connected state.

Proposal 2. In the 2-state RRC model, network controls the following:

- whether UE RAN context should be stored once UE is transitioned into non-connected state,

- whether the stored UE RAN context should be used once UE is transitioned to connected state.

Proposal 3. For 2-state RRC model, consider to maintain the UE RAN context at CN when UE is in RRC non-connected state.