3GPP TSG-RAN WG2 Meeting #131bis 	R2-2507278

Prague, Czech Republic, Oct. 13th-17th, 2025

Agenda Item:	10.4

Source:	Panasonic

Title:	Considerations for 6G mobility design

Document for:	Discussion

# Introduction

RAN plenary meeting#108 has approved the 6G study item (RP-251881). Among all objectives, objective 4 is for mobility.

(4) Mobility for 6GR (for all RRC states), including related RRM [RAN2, RAN1, RAN4, RAN3]

In this paper, we discuss some initial considerations for 6G mobility design.

# Discussion

## Mobility mechanisms vs. connection states

Currently, mobility is handled differently in different RRC states. For the idle and inactive states, mobility is handled by the device through cell reselection (UE based mobility), while for the connected state, mobility is handled by the RAN based on measurement (network based mobility).

More specifically, in idle and inactive states, UE decides on which cell to camp on as long as the received power is sufficiently high. If necessary, UE contacts the network through random access to update the camped cell (e.g. if the camped cell is outside the current registration area). The purpose is to ensure that the UE can be reachable by the network by paging when needed.

By contrast, in connected state, network is in control of selecting the serving cell for the UE to ensure the connectivity is retained for active communication when UE is moving around. This is achieved by the network to provide measurement objects (e.g. reference signals of neighboring cells) to the UE and request measurement reports from the UE. Based on the reporting, the network makes a decision on whether to handover the device to a new cell. Alternatively, the network can configure a list of candidate cells and corresponding handover triggering conditions to the UE, and then let UE to trigger the handover once the conditions are met (e.g. referred to as conditional handover).

The different mobility mechanisms for different connection states are optimized for different purposes. For UE based mobility in idle and inactive states, UE power saving is more prioritized. On the other hand, for connected state, the network is in tighter control for the UE mobility handling because the radio link quality and load balancing need to be ensured for active communication.

For 6G mobility design, we don’t see reason to deviate from the above principle. Although it can be further discussed on the RRC state design (see our companion paper [1] for this topic), we expect that 6G should have a connected state where RRC connection is established, and a non-connected state where RRC connection is not established. For mobility handling, UE based mobility and network based mobility can be applied to non-connected state and connected state, respectively.

Proposal 1. For 6G mobility design, consider UE based mobility for non-connected state and network based mobility for connected state.

## Intra-cell vs. inter-cell mobility

For 6G, cell concept is useful at least for mobility handling purpose. A cell can be defined by synchronization signal and master information block, with similar functionality to CD(cell-defining)-SSB in 5G, or it can be defined as the unit to have CGI. Other broadcast information such as SIB/RACH/paging can be transmitted over a smaller area than a cell, e.g. per SSB beam of TRP or per BWP, for common configuration across the specific area.

Compared to 5G, the number of TRPs belonging to the same cell might increase dramatically. Consequently, a lot of mobility events can happen within a cell, meaning that the UE serving beam is switched without changing the serving cell. We may refer to this as intra-cell mobility. By contrast, an inter-cell mobility means that UE’s serving cell is changed. A major difference is that inter-cell mobility requires UE to apply a new RRC configuration corresponding to the new serving cell where intra-cell mobility is transparent to RRC layer.

For 6G mobility, both intra-cell mobility and inter-cell mobility need to be considered. How to integrate or differentiate these mobilities needs further discussion.

Proposal 2. For 6G mobility design, consider both intra-cell mobility and inter-cell mobility.

## Modular design for connected state mobility

Mobility schemes in 5G have been evolving since the first release of NR, just to name a few: basic handover supported from Rel-15, conditional handover from Rel-16, Dual Active Protocol Stack (DAPS) handover, Conditional PSCell change (CPC) and Conditional PSCell Addition (CPA) from Rel-16/17, and low-layer trigger mobility (LTM) from Rel-18.

However, in 5G each mobility scheme requires separate configuration and they operate independently from each other. For example, if UE supports both CHO and LTM, the RRC containers for each mobility scheme can only be configured separately, even though the candidate cell configurations for each mobility scheme have almost the same contents. This brings unnecessary signalling overhead and increase in UE storage. In 6G, more efficient and unified design should be targeted.

One design direction could be to use one mobility scheme as a baseline for 6G mobility, where LTM could be a good candidate for baseline mobility scheme. However, the drawback of such design is that not all use cases require very low latency for handover as provided by LTM. Further, complexity of LTM might be too demanding for some low-end devices. For example, to perform early DL sync during LTM, UE needs to track the neighbouring TCI states while at the same time maintaining communication with current serving cell.

Alternatively, instead of having one mobility scheme as baseline, 6G mobility can rely on modular design. For example, the following modules can be considered:

- Module 1: Pre-configured RRC containers for candidate cell configurations

- Module 2: measurement configuration

- Module 3: early synchronization configuration

- Module 4: mobility triggering schemes based on L1 signalling from NW

- Module 5: mobility triggering schemes based on L2 signalling from NW

- Module 6: mobility triggering schemes based on L3 signalling from NW

- Module 7: mobility triggering schemes: UE triggered based on event

- Other possible modules.

A selected set of modules can be configured by NW to UE based on the use cases to accomplish the required mobility function. This provides good flexibility with efficient signalling. Note that in the above module design, the early synchronization (i.e. module 3) is decoupled from LTM to make such mechanism applicable for other mobility scheme, e.g. to be used together with L3 handover (together with module 5). Further, with the above module design, a common set of measurement configurations (i.e. module 2) can be flexibly linked to multiple mobility triggered schemes (modules 4,5,6,7)

Proposal 3. For 6G mobility design, consider modular design to enable flexible association among different modules.

## AI/ML based mobility

In 5G, AI/ML based mobility has been studying RRM measurement prediction, event prediction, and RLF prediction. It remains to be seen how the study results can be integrated into 6G design. Specifically, whether and how a universal AI/ML model can provide outputs of mobility event prediction including RRM data and RLF.

Furthermore, another promising direction is to make use of positioning information for AI/ML model to improve the mobility prediction, which currently has not been studied sufficiently. For example, by having data base collecting the information such as which beam(s) of which cell(s) are available at which location, AI/ML model can suggest the serving cell and serving beam to the UE without UE performing measurement and reporting. Such data base can also be useful, for example, to predict the mobility events of cell B based on the UE measurement result on cell A, especially when cell A and cell B are different frequencies.

Proposal 4. For 6G mobility design, consider integrating AI/ML based mobility.

# Conclusion

In this paper, we have discussed some initial considerations for 6G mobility design. The proposals are as follows.

Proposal 1. For 6G mobility design, consider UE based mobility for non-connected state and network based mobility for connected state.

Proposal 2. For 6G mobility design, consider both intra-cell mobility and inter-cell mobility.

Proposal 3. For 6G mobility design, consider modular design to enable flexible association among different modules.

Proposal 4. For 6G mobility design, consider integrating AI/ML based mobility.

# Reference

[1] R2-2507270, RRC states and connection management for 6G, Panasonic