3GPP TSG-RAN2 Meeting #131bis	R2-2507280

# Prague, Czech Republic, Oct 13th – 17th, 2025

Agenda item:		8.11.2 (NR_duplex_evo)

Source:	LG Electronics Inc.

Title: 	Remaining MAC issues on SBFD

Document for:	Discussion and Decision

1.	Introduction

In MAC running CR discussion [1], following open issues are suggested to discuss based on the contributions:

Issues for further discussion with tdoc contributions in RAN2#131bis: - MAC-2: How to clarify "the preambleReceivedTargetPower used in Msg3 transmission power derivation is that configured for legacy RO, in the case that Msg3 is transmitted in non-SBFD symbols and SBFD RACH config option 2 is configured." - MAC-3: Whether, and (if support) how, to support the compensation for the difference in preamble received target power between SBFD RO and legacy RO (in addition to the compensation for the power ramping difference that is already supported), for ensuring preamble transmit power continuity, during RO type switching.

This document discusses the highlighted open issue on SBFD.

2.	Discussion

[MAC-2] Clarification on preambleReceivedTargetPower when Msg3 is transmitted on non-SBSD symbol

In RAN1#120 and RAN1#120bis meetings [2-3], it is agreed to apply preambleReceivedTargetPower configured for non-SBFD RO when the Msg3 PUSCH is transmitted on non-SBFD symbol, regardless of whether SBFD RO or non-SBFD RO is used for RA preamble transmission.

Agreement (confirmed on Thursday with the following changes in red) For determination of the Msg3 PUSCH transmission power and when separate preambleReceivedTargetPower for additional-ROs is configured for RACH configuration Option 1 or Option 2 - preambleReceivedTargetPower configured for legacy-RO is used if Msg3 PUSCH is transmitted in non-SBFD symbols; - preambleReceivedTargetPower configured for additional-RO is used if Msg3 PUSCH is transmitted in SBFD symbols. - FFS: Option 2 Agreement For determination of the Msg3 PUSCH transmission power for RACH configuration Option 2: - preambleReceivedTargetPower configured for legacy-RO is used if Msg3 PUSCH is transmitted in non-SBFD symbols; - preambleReceivedTargetPower configured for additional-RO is used if Msg3 PUSCH is transmitted in SBFD symbols; - FFS whether/how to support separate msg3-DeltaPreamble for non-SBFD symbols and SBFD-symbols.

On the other hand, in the current running MAC CR[4], it is ambiguous whether preambleReceivedTargetPower configured for non-SBFD RO is indicated when the Msg3 PUSCH is transmitted in non-SBFD symbol, since there is no further clarification on preambleReceivedTargetPower. Specifically, when the SBFD RO is used for preamble transmission, preambleReceivedTargetPower should be different from the initialized value for preamble transmission if the Msg3 PUSCH is transmitted in non-SBFD symbol. Therefore, further clarification is needed in order to avoid any misinterpretation of preambleReceivedTargetPower and to clearly specify that preambleReceivedTargetPower configured for non-SBFD RO is indicated for this case.

5> if the received UL grant indicates that the corresponding PUSCH transmission is in SBFD symbols as specified in clause 11.1 of TS 38.213 [6]: 6> if sbfd-RACH-SingleConfig (see TS 38.331 [5]) is configured for the Random Access procedure: 7> indicate the sbfd-RACH-SingleConfig-preambleReceivedTargetPower if configured, or the preambleReceivedTargetPower otherwise, and the amount of power ramping applied to the latest Random Access Preamble transmission to lower layers (i.e. (PREAMBLE_POWER_RAMPING_COUNTER – 1) × PREAMBLE_POWER_RAMPING_STEP + POWER_OFFSET_2STEP_RA + POWER_OFFSET_RO_TYPE). 6> else if sbfd-RACH-DualConfig (see TS 38.331 [5]) is configured for the Random Access procedure: 7> indicate the preambleReceivedTargetPower included in the sbfd-RACH-DualConfig, and the amount of power ramping applied to the latest Random Access Preamble transmission to lower layers (i.e. (PREAMBLE_POWER_RAMPING_COUNTER – 1) × PREAMBLE_POWER_RAMPING_STEP + POWER_OFFSET_2STEP_RA + POWER_OFFSET_RO_TYPE). 6> else: 7> indicate the preambleReceivedTargetPower and the amount of power ramping applied to the latest Random Access Preamble transmission to lower layers (i.e. (PREAMBLE_POWER_RAMPING_COUNTER – 1) × PREAMBLE_POWER_RAMPING_STEP + POWER_OFFSET_2STEP_RA + POWER_OFFSET_RO_TYPE). 5> else (i.e., the received UL grant indicates that the corresponding PUSCH transmission is in non-SBFD symbols as specified in clause 11.1 of TS 38.213 [6]): 6> indicate the preambleReceivedTargetPower and the amount of power ramping applied to the latest Random Access Preamble transmission to lower layers (i.e. (PREAMBLE_POWER_RAMPING_COUNTER – 1) × PREAMBLE_POWER_RAMPING_STEP + POWER_OFFSET_2STEP_RA + POWER_OFFSET_RO_TYPE).

During the discussion on how to further clarify on preambleReceivedTargetPower in the above text, two different ways are suggested:

- Option 1: Specify in the format of ‘preambleReceivedTargetPower included in xxx,’ in order to easily find the parent IE for simple UE implementation

- Option 2: Specify in the format of ‘preambleReceivedTargetPower not included in yyy,’ in order to clearly specify the intended operation of RAN1 agreement.

For Option 1, it is true that if the parent IE is specified, the UE implementation would be simplified since the UE only needs find the preambleReceivedTargetPower included in the indicated IE. However, it should be noted that it is not easy to specify the exact IE that the UE should refer to, since the parent IE would be different based on how the set of Random Access resources is configured and which feature combination is selected for this Random Access procedure. Specifically:

- If the UE selects the set of Random Access resources which is not associated with any feature, the UE should use preambleReceivedTargetPower in rach-ConfigCommon (without suffix) which is directly included in BWP-UplinkCommon IE.

- If the UE selects the set of Random Access resources associated with feature(s), the UE needs to determine the parent IE based on the Random Access resources configuration:

  - If the FeatureConbinationPreambles IE for the selected feature combination is included in rach-ConfigCommon (without suffix) for non-SBFD RO, the UE should use preambleReceivedTargetPower in rach-ConfigCommon (without suffix) which is directly included in BWP-UplinkCommon IE.

  - If the FeatureConbinationPreambles IE for the selected feature combination is included in rach-ConfigCommon-17 in additoinalRACH-Config-r17 IE for non-SBFD RO, the UE should use preambleReceivedTargetPower in rach-ConfigCommon-17 for the selected feature combination, which is included in additionalRACH-Config-r17 IE.

Therefore, it is slightly preferred to use the format of ‘not in yyy’ (i.e., Option 2), in order to clearly specify the intended IE to clarify for the case when the SBFD RO is used for preamble transmission but the Msg3 PUSCH is transmitted on non-SBFD symbol.

Proposal 1. [MAC-2] Use the format of ‘not in yyy’ to specify that preambleReceivedTargetPower configured for non-SBFD RO is used if Msg3 PUSCH is transmitted in non-SBFD symbols

If proposal 1 is agreeable, TP in Annex A should be adopted as a baseline.

Proposal 2. Adopted TP in Annex A.

3.	Conclusion

In this contribution, it is discussed about MAC issue for SBFD. This document includes following proposals.

Proposal 1. [MAC-2] Use the format of ‘not in yyy’ to specify that preambleReceivedTargetPower configured for non-SBFD RO is used if Msg3 PUSCH is transmitted in non-SBFD symbols

Proposal 2. Adopted TP in Annex A.

4.	Reference

[1] R2-2507158 Summary of Rel-19 SBFD MAC open issue discussions for maintenance

[2] Report of 3GPP TSG RAN WG1 #120

[3] Report of 3GPP TSG RAN WG1 #121

[4] R2-2506606 Introduction of Rel-19 Evolution of NR duplex operation (SBFD) for MAC spec

Annex A.	TP for proposal 1

5.1.4 Random Access Response reception (…omitted) 3> else: 4> apply the following actions for the Serving Cell where the Random Access Preamble was transmitted: 5> process the received Timing Advance Command (see clause 5.2); 5> if the received UL grant indicates that the corresponding PUSCH transmission is in SBFD symbols as specified in clause 11.1 of TS 38.213 [6]: 6> if sbfd-RACH-SingleConfig (see TS 38.331 [5]) is configured for the Random Access procedure: 7> indicate the sbfd-RACH-SingleConfig-preambleReceivedTargetPower if configured, or the preambleReceivedTargetPower otherwise, and the amount of power ramping applied to the latest Random Access Preamble transmission to lower layers (i.e. (PREAMBLE_POWER_RAMPING_COUNTER – 1) × PREAMBLE_POWER_RAMPING_STEP + POWER_OFFSET_2STEP_RA + POWER_OFFSET_RO_TYPE). 6> else if sbfd-RACH-DualConfig (see TS 38.331 [5]) is configured for the Random Access procedure: 7> indicate the preambleReceivedTargetPower included in the sbfd-RACH-DualConfig, and the amount of power ramping applied to the latest Random Access Preamble transmission to lower layers (i.e. (PREAMBLE_POWER_RAMPING_COUNTER – 1) × PREAMBLE_POWER_RAMPING_STEP + POWER_OFFSET_2STEP_RA + POWER_OFFSET_RO_TYPE). 6> else: 7> indicate the preambleReceivedTargetPower and the amount of power ramping applied to the latest Random Access Preamble transmission to lower layers (i.e. (PREAMBLE_POWER_RAMPING_COUNTER – 1) × PREAMBLE_POWER_RAMPING_STEP + POWER_OFFSET_2STEP_RA + POWER_OFFSET_RO_TYPE). 5> else (i.e., the received UL grant indicates that the corresponding PUSCH transmission is in non-SBFD symbols as specified in clause 11.1 of TS 38.213 [6]): 6> indicate the preambleReceivedTargetPower (not included in the sbfd-RACH-DualConfig, if configured) and the amount of power ramping applied to the latest Random Access Preamble transmission to lower layers (i.e. (PREAMBLE_POWER_RAMPING_COUNTER – 1) × PREAMBLE_POWER_RAMPING_STEP + POWER_OFFSET_2STEP_RA + POWER_OFFSET_RO_TYPE).