3GPP TSG-RAN WG2 Meeting #131bis	R2-2507288

Prague, Czechia, 13th – 19th Oct, 2025

Agenda item:	8.8.4

Source:	Samsung

Title:	Support for OCC RACH-less

WID/SID:	NR_NTN_Ph3-Core

Document for:	Discussion and Decision

# Introduction

In RAN#106 the latest NR-part of the WID NR NTN Phase in Rel-19 was agreed [1] where the relevant parts to this contribution are:

2. Uplink Capacity/Cell Throughput Enhancement for FR1-NTN [RAN1, RAN2, RAN4] - Specify Orthogonal Cover Codes (OCC) for DFT-s-OFDM PUSCH at least for multiplexing 2 or 4 UEs when PUSCH repetitions are used - Specify necessary signalling - Update RF requirements accordingly, if needed - Notes for this objective: - The enhancement is not targeting improvements/impacts of MU-MIMO capability - The enhancement is not targeted to PUSCH DMRS - No enhancement for initial access - Enhancements to PRACH are not in scope. - This feature may be applicable for UEs operating in terrestrial networks based on a common design

In this contribution, we provide a discussion on RAN2 aspects of uplink capacity/cell throughout enhancement for FR1-NTN.

# Discussion

## OCC for RACH-less handovers

As is made clear from the WID and the agreements, the enhancement targets PUSCH. One aspect where whether OCC is applicable that has yet to be agreed is whether OCC can apply to PUSCH transmissions during RACH-less handovers. We think that allowing for OCC to be applied during PUSCH transmission for RACH-less can be beneficial, especially in a non-terrestrial network where system-wide there is expected to be a lot of handovers being performed, thus it is important to ensure that the handovers are performed efficiently. In fact this in our view is one of the key argument for introducing RACH-less handovers for NTN Release 18.

In the last meeting, the OCC applicability to RACH-less handovers was discussed. One of the arguments against OCC for RACH-less handovers would be that it may be difficult to ensure that timing advance is lined up between different UEs during the handover procedure. However, we believe that this would likely be an issue for a network even outside of the handover procedure. Also, this would likely be mostly useful in FR1, where there is less sensitivity.

OCC is currently applicable to CG and DG PUSCH. This means that OCC could be applicable to both CG and DG RACH-less.

Proposal 1: Confirm OCC is applicable for RACH-less handovers by suitable network implementation.

In order to indicate this possibility to an implementer, we believe that it can be useful to document it in 38.300. An example of this can be found in Annex A.

Proposal 2: RAN2 to consider documenting OCC being applicable for RACH-less handovers in Stage 2 as seen in Annex A.

# Conclusion

In this contribution we discussed OCC applicability with the following proposals:

Proposal 1: Confirm OCC is applicable for RACH-less handovers by suitable network implementation.

Proposal 2: RAN2 to consider documenting OCC being applicable for RACH-less handovers in Stage 2 as seen in Annex A.

# Reference

RP-243300, Revised WID: Non-Terrestrial Networks (NTN) for NR Phase 3, Thales, CATT, RAN#106, Madrid, Spain, December 2024.

3GPP TS 38.300, NR; NR and NG-RAN Overall Description; Stage 2, V19.0.0, September 2025.

# Annex A – 38.300 TP

-------------- 38.300 V19.0.0 [2] --------------

16.14.11	Support for NR NTN capacity enhancements

To improve uplink capacity in NTN, the following enhancements are supported for PUSCH transmission:

-	Inter-slot orthogonal cover code with length 2 to multiplex up to two UEs;

-	Inter-slot orthogonal cover code with length 4 to multiplex up to four UEs.

Inter-slot orthogonal cover code is supported for RACH-less handovers, as described in clause 9.2.3.6.

-------------- 38.300 V19.0.0 [2] --------------