3GPP TSG-RAN WG2 Meeting #131bis	R2-2507291

Prague, Czech Republic, Oct. 13th-17th, 2025

Agenda item: 		10.3.3

Source:		Lenovo

Title:		Considerations for 6G unified CP and UP designs for TN and NTN

Document for:		Discussion

- Introduction

The latest RAN#109 meeting agreed the following text proposal for TR38.914 regarding 6G NTN:

The RAN design for the 6G Radio Access Technologies shall be designed to fulfil the following requirements: - … - The 6G RAN architecture shall be designed considering both terrestrial network and non-terrestrial network. - …

And as per RAN1#122 agreement, NTN will be consideration in the very beginning of 6G radio design:

RAN1#122 agreement For harmonized 6GR design for TN and NTN, RAN1 studies to identify the technical aspects affected by NTN characteristics, as well as lessons learned from NR/IoT NTN

In this contribution we discuss 6G NTN under the principle of unified TN-NTN designs in 6G radio for both CP and UP.

- Discussion

## 6G TN and NTN scenarios

In 4G/5G, the IoT/NR NTN deployment is considered as an extension of 4G/5G TN with specific protocol enhancements or optimizations, e.g., to cope with the NTN characteristics including large propagation delay and satellite movement. Due to the workload and progress of LTE-A/5G-A standardization, the study and enhancement for IoT/NR NTN have been focusing on the typical reference scenarios with the satellites as NTN payloads, which are the LEO satellites with the maximum payload velocity and Doppler shift, and the GEO satellites with the maximum propagation delay and differential propagation delay. Meanwhile, satellites on other orbits (MEO or HEO) and other NTN payloads (HAPS) are considered extendable from LEO and GEO scenarios, and ATG scenario is implicitly supported simply applying enhancements for NTN.

Under the above principle, the standardization of IoT/NR NTN causes many differences between 4G/5G TN and NTN with non-unified CP signaling, UP handling as well as corresponding UE behaviors. Even within the NTN itself, the characteristics of specific NTN payloads (e.g., HAPS) were not fully considered. For example, the ephemeris contents and signaling are completely designed to serve the needs of satellites. On the other side, with the evolution of services and infrastructure some NTN characteristics including BS/TRP movement and temporary availability are also becoming applicable in TN.

Observation 1: For IoT/NR NTN in 4G/5G, only LEO and GEO deployments are studied as reference scenarios for CP and UP enhancements, while other NTN payloads including MEO, HEO and HAPS are not specifically considered.

Therefore, the following principle shall be considered for 6G RAN toward unified TN-NTN designs of CP and UP solutions.

Proposal 1: The 6G RAN design shall aim for unified CP and UP solutions for TN and NTN covering all possible NTN deployment scenarios (HAPS/LEO/MEO/HEO/GEO).

For the first release of IoT/NR NTN in Rel-17, only the transparent architecture was supported. The cell generated by a GEO satellite is assumed to be Earth-fixed cell (EFC), while the cell generated by an LEO satellite is assumed to be quasi-Earth-fixed cell (quasi-EFC) or an Earth-moving cell (EMC), and mobility enhancements were not considered for the EMC. For the second release of IoT/NR NTN in Rel-18, transparent architecture was also assumed, and the mobility enhancements were extended to EMC. For the third release of IoT/NR NTN in Rel-19, regenerative architecture with full gNB on board was supported.

During this process throughout 3 releases, both the CP and UP solutions have encountered solution adjustment or even duplicated designs to adapt the evolution of cell type and architecture. For example the distance-based events include Event D1 for EFC and quasi-EFC in Rel-17, and Event D2 in Rel-18, while in fact the definitions and conditions are almost the same. To achieve the goal of unified TN-NTN design, these architectures as well as the cell types shall all be considered in the first place in case of divergence in solutions.

Observation 2: For IoT/NR NTN in 4G/5G, there has been gradual process in CP and UP designs to support different architectures (transparent and regenerative) and cell types (EFC, quasi-EFC and EMC) throughout Rel-17 to Rel-19.

Therefore, the following principle shall also be considered for 6G RAN toward unified TN-NTN designs.

Proposal 2: The 6G RAN design shall aim for unified CP and UP solutions for TN and NTN covering all possible cell types (EFC, quasi-EFC and EMC) and architectures (transparent architecture and regenerative architecture at least full 6G BS on board).

## 6G TN and NTN UL synchronization scheme

In IoT/NR NTN, UL synchronization in time domain and frequency domain is mandatory for NTN access, and to obtain and maintain UL synchronization, the UE is required to have valid GNSS position and ephemeris information (also common TA of the feeder link for transparent architecture). For the time domain, before and during connection to an NTN cell, the UE shall compute the RTT between UE and the RP based on the GNSS position, the ephemeris, and the Common TA parameters, and autonomously pre-compensate the NTA for the RTT between the UE and the UL synchronization reference point. For the frequency domain, the UE shall compute the frequency Doppler shift of the service link, and autonomously pre-compensate for it in the uplink transmissions, by considering UE position and the ephemeris.

For the TA derivation in legacy 4G/5G TN, UL frame number for transmission from the UE shall start NTA+NTA,offset⨯Tc before the start of the corresponding DL frame at the UE. A UE can be provided with NTA,offset for serving cell by n-TimingAdvanceOffset , and NTA by Timing Advance Command (TAC) field in MAC RAR and refinement of NTA by TAC MAC CE. To accommodate the propagation delay in NTN, several timing relationships are enhanced by a Common TA on feeder link and UE-specific TA on service link on the basis of TN scheme.

Observation 3: For IoT/NR NTN in 4G/5G, the TA compensation at UE UL synchronization is designed separately as add-on to the TN scheme.

This legacy UL synchronization scheme for IoT/NR NTN has multiple disadvantages, including:

- UE complexity: The capabilities of GNSS positioning, ephemeris acquiring and time/frequency pre-compensation are mandatory to access NTN cells. The UE also needs to guarantee real-time calculation for satellite position, movement and propagation delay.

- Signalling overhead: The size of ephemeris information (ephemerisInfo) of one NTN payload (satellite or HAPS) is 200 bits (orbital parameters) or 168 bits (position & velocity vectors), and the number of ephemeris information is limited to 5 (1 serving cell and 4 neighbour cells). The size of common TA information is 60 bits, making the size of one satellite information approximately 300 bits. Besides, TA reporting is necessary to provide the service link RTT information to the gNB, and PDD reporting is necessary to provide the service link delay difference information to gNB.

- Service interruption: UE is required to have a valid serving cell ephemeris and acquire its update before expiry, or the UE will trigger UL synchronization loss that suspends all UL transmission and flush HARQ buffer until it is obtained. For normal HO and CHO without RACH-less HO configuration, UE needs to calculate and pre-compensate the TA to the target cell before initiating RACH. For RACH-less HO, the UE still needs to calculate and pre-compensate the TA to the target cell for UL transmission.

- Security risk: The ephemeris information, either in the form of orbital parameters or position & velocity vectors, reveals the current and future positions of the NTN payload (satellite or HAPS), making it vulnerable to planned security attacks. In particular for the regenerative architecture wherein the gNB is on the NTN payload, these information exposes the current and future positions of gNBs, which is a significant safety risk considered for TN.

- Disunity of value units (slot, symbol, ms and Tc) and ranges from CP signalling perspective.

- Duplicated calculations e.g., for service link propagation delay and distance between UE and reference location.

Observation 4: For IoT/NR NTN in 4G/5G, the UL synchronization relies on UE position and ephemeris information as mandatory requirements, which leads to additional UE complexity, signaling overhead, service interruption and security risk.

This process was carried out based on the fact that the IoT/NR NTN deployment is considered as an extension of 4G/5G TN with specific protocol enhancements or optimizations. However, it also causes many differences between 4G/5G TN and NTN with non-unified signaling and UE behaviors for UL synchronization. In 6G to achieve the goal of harmonized TN-NTN design, a unified UL synchronization scheme (e.g., determination of TA to be compensated by UE) is expected, which may have the chance to reduce the UE complexity, signaling overhead, service interruption and security risk.

Proposal 3: The 6G RAN design shall aim for a unified scheme of UL synchronization for both TN and NTN, which may be decoupled from UE position information and/or ephemeris information.

To facilitate the unified scheme of UL synchronization for both TN and NTN, unified CP signaling solution for UL synchronization is expected as well to minimize UE complexity, signaling overhead and service interruption. For example, common signaling for the common part of TA that needs to be compensated by UEs within a cell ranging from typical TN to typical NTN propagation delays, and dedicated signaling for individual adjustment to correct the differential TA at a specific UE. Based on the unified CP signaling, the corresponding UP reporting (TA reporting) and timer handling (timer extension or start offsetting with UE-BS RTT) can be reconsidered as well.

Proposal 4: The 6G RAN design shall aim for unified CP signaling and UP handling to cover the differences between TN and NTN, with minimized UE complexity and signaling overhead.

## 6G TN and NTN data collection and utilization

The data collection and utilization have been considered beneficial for network deployment, management, performance and service provisioning. For the data collection, in addition to the legacy schemes (e.g., SON and MDT), sensing is also a promising direction that enables use cases of detection and/or tracking of passive objects (UAVs, human, vehicles and AGVs). To achieve a better balance between supporting the legacy user data and transferring the collected data for various purposes, there are also expected enhancements to implement special handling for the data collection and transfer (e.g., the concept of “data plane”). The data transfer function and protocol design shall be able to cope with different propagation delays (e.g., as other user plane layers do in IoT/NR NTN).

Proposal 5: The 6G RAN design shall aim for a unified scheme of data transfer (e.g., data plane) for both TN and NTN.

For the sensing services, the current studies on ISAC have been mostly focusing on the scenarios, requirements and general procedures of providing sensing services alongside with communication leveraging TN infrastructures (e.g., TRPs and gNBs on the ground), while the NTN payloads (satellites and HAPS) also have the advantages in providing sensing services in both horizontal and vertical directions with their large coverage and high altitude deployment. The NTN payloads embarking sensing modules can serve as the sensing nodes (i.e., TRPs) to implement ISAC, while the NTN characteristics shall also be considered in sensing procedures, e.g. sensing capability exchange that can be related to satellite hardware limits, satellite beam management and ISL, and sensing service availability that can be related to satellite movement and three-dimensional coverage.

Proposal 6: The 6G RAN design shall aim for a unified scheme of sensing for both TN and NTN.

For the data utilization, AI/ML models and algorithms have been considered, and the work has been started since 5G-A. A unified AI/ML framework for TN and NTN can also help in 6G TN-NTN integrated networks. For example, automatic network type selection and mobility prediction with overlapping coverage of TN and NTN, satellite-ground channel estimation and beam management e.g., based on CSI feedback, and dynamic resource scheduling and allocation among multi-orbit satellites and TN infrastructures.

Proposal 7: The 6G RAN design shall aim for a unified AI/ML framework for both TN and NTN.

- Conclusion

In this contribution we discuss 6G NTN under the principle of unified TN-NTN design in 6G radio. It is observed that:

Observation 1: For IoT/NR NTN in 4G/5G, only LEO and GEO deployments are considered as reference scenarios, without specific considerations for other orbits of satellites or other types of NTN payloads including HAPS.

Observation 2: For IoT/NR NTN in 4G/5G, there has been gradual process to support different architectures (transparent and regenerative) and cell types (EFC, quasi-EFC and EMC) throughout Rel-17 to Rel-19.

Observation 3: For IoT/NR NTN in 4G/5G, the TA compensation at UE UL synchronization is designed separately as add-on to the TN scheme.

Observation 4: For IoT/NR NTN in 4G/5G, the UL synchronization relies on UE position and ephemeris information as mandatory requirements, which leads to additional UE complexity, signaling overhead, service interruption and security risk.

And the following proposals are given:

Proposal 1: The 6G RAN design shall aim for unified CP and UP solutions for TN and NTN covering all possible NTN deployment scenarios (HAPS/LEO/MEO/HEO/GEO).

Proposal 2: The 6G RAN design shall aim for unified CP and UP solutions for TN and NTN covering all possible cell types (EFC, quasi-EFC and EMC) and architectures (transparent architecture and regenerative architecture at least full 6G BS on board).

Proposal 3: The 6G RAN design shall aim for a unified scheme of UL synchronization for both TN and NTN, which may be decoupled from UE position information and/or ephemeris information.

Proposal 4: The 6G RAN design shall aim for unified CP signaling and UP handling to cover the differences between TN and NTN, with minimized UE complexity and signaling overhead.

Proposal 5: The 6G RAN design shall aim for a unified scheme of data transfer (e.g., data plane) for both TN and NTN.

Proposal 6: The 6G RAN design shall aim for a unified scheme of sensing for both TN and NTN.

Proposal 7: The 6G RAN design shall aim for a unified AI/ML framework for both TN and NTN.