3GPP TSG RAN WG2 #131bis    	                            R2-2507292

Prague, Czech Republic, Oct. 13th-17th, 2025

Agenda Item:	10.4

Source:	     KDDI Corporation

Title:	Views on Mobility for 6GR

Tdoc Type:	Discussion

Document for: 	Discussion and decision

- Introduction

At the RAN#108 meeting, new SID on Rel-20 study of 6G Radio was approved [1], and at the RAN#109 meeting, the SID was revised [2]. Mobility for 6GR is one of the objectives as follows:

(4) Mobility for 6GR (for all RRC states), including related RRM [RAN2, RAN1, RAN4, RAN3]

In this contribution, we provide our initial views on mobility for 6GR in association with the guidance provided by RAN2 chair lady as follows [3]:

10.4	 Mobility

General mobility principles, types (e.g. L3, CHO, LTM, RLM/RLF, cell reselection), and measurements. Including Inter-RAT and intra-RAT mobility.

- Discussion

2.1 General mobility principles

In 5G NR, the principal mobility schemes—legacy L3 handover (Rel‑15), Conditional Handover (CHO, Rel‑16), and L1L2 Triggered Mobility including Conditional LTM (LTM/CLTM, Rel‑18/19/20)—were introduced in different releases and work items, each with its own signaling model and target/candidate configuration structures. When two or more schemes are configured for the same UE, the network typically repeats preparation for the same target/candidate cells per scheme (e.g., over Xn/NG), which inflates both the size and frequency of RRC messages and inter‑node signaling. In parallel, the coexistence of L3 RRM and L1 CSI/beam measurement frameworks leads to duplicated reference signal configurations and separate reporting paths; with the “known cell” coupling, UEs may effectively perform measurement work twice before L1 neighbour measurements are valid. These factors collectively enlarge RRC PDUs, and processing demands, increasing the risk of HO command decode failures/retransmissions and extending interruption time in challenging radio conditions. They also multiply UE/NW code paths and consistency checks, which raises complexity, cost, and defect risk and complicates field operations.

Observation 1: In 5G, configuration messages and candidate/target configurations grew and duplication across mobility, especially when multiple schemes co-exist for the same UE, inflating UE/NW complexity and signalling overhead.

A unified design for 6G mobility that introduces a scheme‑agnostic RRC “mobility container” and common signaling would directly remove the above inefficiencies. By preparing and distributing one reusable target/candidate configuration (with stable CandidateConfigID and delta updates) that can be referenced by mobility triggers alike, the network avoids duplicate preparation and repeated transmission of largely identical information. Defining shared procedure modules—such as early UL/DL synchronization with potential RACH‑less execution, early CSI acquisition, and fast recovery/rollback—once and making them callable from any cell‑switch type prevents re‑specification of similar features and smooths runtime fallback/upgrade across mobility. The result is smaller and less frequent control messages, reduced UE/NW implementation complexity, improved robustness in poor radio (fewer HO command failures), and simpler testing and operations.

Proposal 1: Study a unified design for 6G mobility (e.g., common RRC containers and signalling).

In Multi‑TRP and FR2 multi‑beam operation, the dominant mobility events are frequent beam/TRP transitions within or across cells. Mobility performance is therefore largely determined by the latency of TRP/antenna switching, including early synchronization and beam activation, rather than by classical cell‑level reconfiguration. When mobility relies solely on L3‑triggered handover, the procedure introduces added RRC preparation/command processing and RACH delays, which are ill‑suited to the cadence of beam‑level dynamics in FR2 and ultra‑dense deployments. This mismatch increases interruption time, failure probability, and signaling load, particularly under rapid fading and hotspot scenarios where repeated small‑area transitions are common. Consequently, mechanisms optimized for low‑latency TRP/beam switching are required as the primary means to sustain continuity and cell‑edge throughput in 6G.

Observation 2 : With multi‑TRP and FR2 beam management which produce frequent beam/TRP switches, mobility performance is necessary by low-latency TRP/antenna switching. L3‑only HO incurs RRC/latency penalties.

The 6G mobility specifications should be Multi‑TRP‑capable from Day‑1, defining procedures, information elements, and capability signaling to exploit multiple coordinated TRPs for mobility and continuity. The standard should enable by Multi‑TRP anchoring to minimize interruption and resource overhead. Establishing Multi‑TRP readiness at Day‑1 mitigates coverage‑shrink risks, reduces ping‑pong at cell level, and provides a scalable foundation for ultra‑dense, beam‑centric deployments.

Proposal 2: Specify mobility standards that can utilize multi-TRP from 6G Day 1.

2.2 Mobility types

Field experience indicates that only a limited subset of 5G mobility features achieved commercial deployment, while option proliferation increased configuration complexity and reduced implement ability. LTM, although introduced late in 5G, demonstrated strong potential for low‑latency mobility within a node, whereas L3 HO remains the universally applicable and robust baseline across nodes and RATs. For 6G Day‑1, it is therefore prudent to prioritize commercially feasible mechanisms and avoid expanding the option space beyond what deployments can absorb. In particular, frequent beam/TRP transitions in FR2 and distributed deployments argue for LTM (including Subsequent LTM) as the natural fit for intra‑DU and intra‑CU inter‑DU scenarios. Conversely, extending LTM into legacy RAT domains is disruptive, so L3 HO should remain the inter‑RAT baseline.

Observation 3: Only a small subset of 5G mobility features saw real deployment. Day‑1 for 6G mobility needs to prioritize commercially feasible solutions (e.g., L3 HO and LTM) and avoid proliferating options.

RAN2 study adopting LTM as the Day‑1 baseline for intra‑6G mobility, while retaining L3 HO as the baseline for inter‑RAT and inter‑node scenarios, by uplifting and consolidating the corresponding 5G specifications into 6G with minimal deltas. The study should codify Subsequent LTM for sustained high‑frequency switching, define capability signaling and policy selection to choose between LTM and L3 HO per scenario, and include standardized rollback/failure recovery. This approach targets interruption time and signaling load reductions for intra‑node mobility while preserving robustness and interoperability for inter‑RAT mobility. The work should quantify gains via KPIs (e.g., interruption time, HO failure, signaling volume) and ensure backward/migration compatibility. Establishing this focused baseline at Day‑1 limits option proliferation and accelerates practical deployment.

Proposal 3: Study whether LTM can be the Day‑1 baseline for 6G mobility(i.e., apply 5G standards for LTM as 6G standards).

2.3 Measurements

In 5G, the coexistence of L3 RRM and L1 CSI/beam measurement frameworks results in duplicated reference‑signal configurations and separate reporting paths. Such duplication is ill‑suited to 6G’s denser topologies and FR2 multi‑beam dynamics where timely beam/TRP evidence is essential. Moreover, older single‑TRP assumptions risk coverage shrink and exacerbate ping‑pong at cell level under rapid spatial fading. Day‑1 6G mobility therefore requires Multi‑TRP readiness and a beam‑centric decision basis so that most transitions are resolved below the cell boundary with minimal RRC involvement.

Observation 4: Dual measurement stacks (L3 RRM and L1 CSI/beam) cause duplicated RS configurations, separate reporting paths—unsuitable for 6G’s denser or high-speed environments.

Base beam‑specific measurements and mobility decision‑making on a Multi‑TRP environment from Day‑1. Common RS configuration and a single reporting pipeline shall be specified so “measure once, use many” applies to both beam‑ and cell‑level needs. The framework should include stability‑aware triggers to suppress cell‑level ping‑pong while leveraging TRP‑level agility for continuity. This approach maximizes coverage reliability and minimizes RRC load in beam‑centric deployments.

Proposal 4: Beam measurements and decision-making for mobility should be based for 6GR mobility.

2.4 Pain Point Scenarios

Fixed measurement‑gap periods in 5G have been shown to either under‑sample the channel, increasing RLF/HO failures, or over‑sample it, increasing interruption and energy consumption. As 6G targets broader bands, higher velocities, and denser beams/TRPs, a uniform gap setting cannot satisfy the competing needs for freshness, QoS continuity, and power efficiency. Multi‑TRP anchoring and dual‑RX capabilities can often avoid gaps altogether while maintaining traffic. Absent gapless capability, scenario‑aware adaptation of the gap cadence tied to velocity, band, and beam stability is required. Aligning gap timing with SSB/CSI‑RS/DRX boundaries further reduces collisions and hidden costs.

Observation 5: Fixed measurement‑gap periods lead to either under‑measurement (RLF/HO failures) or over‑measurement (energy/interruption). Adaptive, scenario‑aware gaps are needed.

RAN2 should study an Adaptive Measurement Gap Period (AGP) in coordination with RAN4, whereby the network signals a nominal period and allowed scaling, and the UE adapts the effective period based on velocity, band, beam stability, and service constraints. Gapless operation should be preferred whenever Multi‑TRP/dual‑RX can maintain user‑plane anchoring; otherwise, temporary burst‑shortening of the gap may be used around mobility‑critical events. The adaptation should align with reference‑signal periodicities and DRX cycles to minimize disruption. As part of this study, the feasibility and bounds for shortening the gap period shall be evaluated per band and device capability. KPIs shall cover interruption contribution, energy impact, and mobility success rates.

Proposal 5: Study whether the measurement gap period can be shortened in coordination with RAN4

Current operational practice and specifications predominantly rely on downlink quality indicators (e.g., RSRP/RSRQ/SINR) to drive mobility, leaving post‑switch uplink quality largely unobserved at the decision point. Emerging 6G use cases, including uplink‑heavy sensing, live content creation, and industrial control, demand visibility into uplink quality immediately after a switch. The absence of early uplink feedback can delay detection of unsuitable targets, leading to prolonged outage, reattempt cycles, or excessive conservatism in target selection. A lightweight, time‑bounded post‑switch UL verification is therefore required to close this visibility gap. Such verification also enables faster recovery/rollback when the uplink path is impaired.

Observation 6: In 5G NR, mobility decisions and reporting are predominantly downlink‑driven (e.g., RSRP/RSRQ/SINR), with no standardized, immediate visibility of uplink quality after mobility; as 6G introduces uplink‑centric services, this post‑switch UL “blind spot” necessitates standardized, lightweight UL quality verification.

RAN2 should study and define a lightweight post‑switch uplink quality reporting mechanism, triggered immediately after mobility completion, to validate target suitability and enable fast recovery. The mechanism should integrate with unified recovery/rollback modules and avoid duplicative signaling across mobility types. This addition improves user experience and reduces prolonged failure in uplink‑sensitive services.

Proposal 6: Study whether the UL quality after mobility can be used.

Experience from 5G networks shows that high‑speed mobile environments require extensive operator tuning of mobility parameters (e.g., TTT, hysteresis, measurement periodicity, reporting thresholds), which is operationally burdensome and error‑prone. Static parameterization performs poorly across wide velocity ranges and differing service requirements, causing either late/early handovers or ping‑pong. In 6G, the diversity of terminal speeds and service needs is expected to increase further. Without velocity‑aware adaptation, interruption time and failure rates will remain elevated in high‑speed corridors. A standardized, Day‑1 mechanism for speed‑aware mobility behavior is therefore necessary.

Observation 7: Operational experience shows that high‑speed scenarios in 5G require heavy manual tuning of mobility parameters.

RAN2 should introduce Day‑1 velocity‑aware mobility operation that adapts key parameters—such as TTT, hysteresis, measurement/report periodicity, and event thresholds—based on UE speed class and service policy. The framework should allow network‑signaled profiles and UE‑assisted estimation, with safeguards to prevent oscillation and excessive signaling. This standardized adaptation reduces tuning burden while improving mobility robustness in high‑speed scenarios.

Proposal 7: Study whether to introduce the feature to adapt to high-speed mobile environments (e.g., changes the values of mobility-related parameters) from 6G Day 1.

- Conclusion

Based on the discussion in the previous sections we propose the following:

Observation 1: In 5G, configuration messages and candidate/target configurations grew and duplication across mobility, especially when multiple schemes co-exist for the same UE, inflating UE/NW complexity and signalling overhead.

Observation 2 : With multi‑TRP and FR2 beam management which produce frequent beam/TRP switches, mobility performance is necessary by low-latency TRP/antenna switching. L3‑only HO incurs RRC/latency penalties.

Observation 3: Only a small subset of 5G mobility features saw real deployment. Day‑1 for 6G mobility needs to prioritize commercially feasible solutions (e.g., L3 HO and LTM) and avoid proliferating options.

Observation 4: Dual measurement stacks (L3 RRM and L1 CSI/beam) cause duplicated RS configurations, separate reporting paths—unsuitable for 6G’s denser or high-speed environments.

Observation 5: Fixed measurement‑gap periods lead to either under‑measurement (RLF/HO failures) or over‑measurement (energy/interruption). Adaptive, scenario‑aware gaps are needed.

Observation 6: In 5G NR, mobility decisions and reporting are predominantly downlink‑driven (e.g., RSRP/RSRQ/SINR), with no standardized, immediate visibility of uplink quality after mobility; as 6G introduces uplink‑centric services, this post‑switch UL “blind spot” necessitates standardized, lightweight UL quality verification.

Observation 7: Operational experience shows that high‑speed scenarios in 5G require heavy manual tuning of mobility parameters.

Proposal 1: Study a unified design for 6G mobility (e.g., common RRC containers and signalling).

Proposal 2: Specify mobility standards that can utilize multi-TRP from 6G Day 1.Proposal 3: Study whether LTM can be the Day‑1 baseline for 6G mobility(e.g., apply 5G standards for LTM and L3 HO as 6G standards).

Proposal 3: Study whether LTM can be the Day‑1 baseline for 6G mobility(i.e., apply 5G standards for LTM as 6G standards).

Proposal 4: Beam measurements and decision-making for mobility should be based for 6GR mobility.

Proposal 5: Study whether the measurement gap period can be shortened in coordination with RAN4

Proposal 6: Study whether the UL quality after mobility can be used.

Proposal 7: Study whether to introduce the feature to adapt to high-speed mobile environments (e.g., changes the values of mobility-related parameters) from 6G Day 1.

- References

[1] RP-251881, “New SID: Study on 6G Radio”, NTT DOCOMO (Moderator), RAN#108, June 2025.

[2] RP-252912, “Revised SID: Study on 6G Radio”, NTT DOCOMO (Moderator), RAN#109, September 2025.

[3] R2-2506701, “Agenda for RAN2#131bis”, (Chair: InterDigital)