3GPP TSG-RAN WG2 Meeting #131bis	R2-2507293

Prague, Czech Republic, 13th – 17th October 2025

Agenda item:	  10.3.3

Source:	  Lenovo

Title:                            6GR Common User Plane and Control Plane aspects

Document for:	  Discussion and Decision

# Introduction

In this paper, we discuss common aspects related to User Plane and Control Plane, targeted for the Study on 6G Radio [1].

# Discussion

## Access stratum security aspects

Security architecture

The 5G NR Access Stratum (AS) security architecture protects communication between the UE and gNB. It is built on mutual authentication and key management established in the Non-Access Stratum (NAS), after which AS security keys are derived. Two main functions are applied: integrity protection to ensure authentic and unaltered reception and ciphering to protect confidentiality of both signaling and user plane data. Security contexts are refreshed to mitigate replay and key compromise risks. The security is performed in PDCP layer, which sits below RRC and user plane SDAP layer.

Figure 1: 5G gNB and UE Protocol Stack

Protection of MAC layer control information has been an important discussion in 5G, especially around the work for L1/ L2 mobility, where some MAC CEs have been designed and specified carrying information centred around UE mobility. Also, various papers have been published both inside and outside of 3GPP, showing vulnerability due to unprotected L1 and L2 information transmitted over the air [3 -11].

Observation 1: Various concerns have been raised about vulnerability due to unprotected L1 and L2 information transmitted over the air in 5G NR.

Our simple analysis shows that an attack is limited to DoS if a UE is successfully fooled into transmitting a handover complete message to a rogue base station, as the UE will eventually realize that the base station does not have ciphering or integrity protection keys for the UE. On the other hand, it might be possible to position and trace the UE using its transmitted measurement reports while using a MAC CE. This could be serious depending on if the attacker can map the C-RNTI to a more specific User Identification – which itself should not be trivial. So, while protection of lower layer information is desirable, it is not seen as critical. In 5G it was consciously decided to not introduce security at MAC layer for this purpose. 6G offers a clean slate and this is now certainly one possibility.

Observation 2: Vulnerability due to unprotected L1 and L2 information transmitted over the air in 5G NR can’t easily be exploited by an attacker to manifest a serious threat.

Observation 3: In 5G it was consciously decided to not introduce security at MAC layer. 6G offers a clean slate and this is now certainly one possibility.

The above question (whether to have security @ MAC) is tightly coupled with CU-DU split, which has been useful deployment option in 5G at least for some operators. As shown in Figure 1, the standardized split is between PDCP and RLC sublayers. This allows security in central units and requires no security changes when moving between DUs controlled by the same CU box. This makes the UE mobility easier by not requiring PDCP Re-establishment. But if the user plane protocol should be considered for a broader change,  be more application aware, and get rid of inefficiencies related to two SNs for even 1-1 mapped PDCP/ RLC PDU, it might be worth considering if the security could be added to a different sublayer, e.g., to MAC (like in UMTS). We discuss in a separate paper the L2 aspects in more detail. But as mentioned previously, bringing security in DU could make mobility even inside the CU box bit more cumbersome. As one option the security in MAC may only be performed for information originating in MAC and is limited to the certain scenario – in implementation using a functional call, security implementation being modular may not be a big deal but then introducing new MAC SN overhead (to obtain a COUNT as security freshness parameter) can be costly. This is important and needs therefore further discussion.

RAN2 should start discussion about need for protecting L2 control information, specifically the MAC CEs, as part of a broader discussion on placement of security function in the AS protocol sublayers.

## AI/ML framework considerations

To enhance the quality of experience, it will be more important than ever in 6G to improve the utilization of resources and enable faster QoS adaptation. AI/ML is one of the key features that must be native to the RAN protocol. Beyond use case-specific AI/ML functionality, 6G-RAN will be required to facilitate a flexible protocol design with AI as an integral component (AI-native) to cater to different device types, such as smartphones, wearables, sensors, IoT devices, AR/VR devices, holographic devices, etc. Given that these devices differ in their individual device capabilities, in essence, their ability to support AI/ML features may largely vary. For example, IoT devices typically have limited processing and storage capacity, while smartphones are equipped with higher computational capabilities. The RAN protocol design from 6G Day-1 should consider flexibility to handle these devices, ranging from no AI capability to full-featured AI devices. For designing an AI-native protocol, the core L2/L3 functions, e.g., MAC functions, such as link adaptation, resource allocation, UE reports, etc, should be studied as adaptive AI functions which can bring benefits for stringent services in terms of their demanding QoS requirements.

RAN2 is suggested to target a futureproof protocol design to cater to diverse device types supporting different levels of AI/ML capabilities.

An essential aspect to consider would be to optimize the core MAC functions, such as scheduling procedures. As the scheduler and its resource allocation process are at the heart of the protocol stack, it is of prime importance to address the challenges in resource management due to the rise in QoS demands for services like immersive communication, XR, URLLC, etc. For example, knowledge about the upcoming data traffic and its statistics in advance, such as time of occurrence, volume, and QoS of data, can be beneficial in scheduling decisions and support services that require low-latency transmissions. It can positively affect the signaling procedures for resource allocation, such as scheduling requests (SR), buffer status report (BSR), etc.

We present a simple example for one of the potential study directions to facilitate AI-native behavior in MAC functions, focusing on an intelligent user plane design to improve uplink throughput and reduce latency. We illustrate an intelligent scheduling mechanism approach in Figures 2 and 3 below. Predictions at the network-side using some assistance (UE status information) from the UE can be applicable for a less volatile scenario. However, the performance is likely to be limited by the uplink data knowledge available at the network-side. This may lead to signaling of additional information from the UE to the NW (compared to the legacy uplink reporting, such as SR/BSR) to accomplish prediction outcomes with high accuracy. In contrast (Figure 3), necessary measures for uplink scheduling decisions can be performed closer to the edge, i.e., at the UE-side, which can be reported to the network well in advance by exploiting the knowledge and experience of the device.

With the growing demand of sporadic, bursty, and user-initiated traffic, for example, immersive communication and XR/VR applications, UE has a better knowledge of the data traffic and thus it is in a better position to fine-tune certain L2 settings (e.g., RLC t-Reassembly, LCH priority, etc.). Therefore, with the help of AI/ML models, UE can select certain network parameter values autonomously within some given limits (as configured by the network). Enabling the UE to select a suitable parameter set to incorporate real-time adaptation will support faster QoS adaptation.

Figure 2: AI-Native MAC Function (network-sided)

Figure 3: AI-Native MAC Function (UE-sided)

RAN2 will study user plane enhancements, considering support of AI/ML-enabled features from an overall AI/ML framework perspective and not limiting it to specific use-cases.

In 5G, AI/ML-based mobility has been studied. The study has focused on evaluating the benefits of using AIML in mobility use cases, namely RRM measurement prediction and L3 measurement event prediction. RAN2 has also studied the potential specification impact of enabling RRM measurement prediction, L3 measurement event prediction, and relevant mobility procedure in RRC_CONNECTED state within the NR system. The simulation results for RRM measurement prediction show that the AI algorithms outperform non-AI in terms of prediction accuracy. The simulation results indicate a reduction of the HOF rate in most cases when the handover is executed based on a predicted measurement event in advance. Therefore, we propose that AI/ML-based RRM prediction and L3 event prediction can be supported in 6GR.

Another use case, i.e., RLF prediction, was deprioritized in 5G and studied only in a limited way without evaluation via simulations. In 6G, we can continue working on RLF prediction. In Rel-18 and Rel-19 5G LTM, L1 measurement report and L1 event are introduced for LTM mobility. The AI-based mechanism can be extended to L1 measurement result prediction and L1 event prediction for robustness purposes.

RAN2 will study the support of AI/ML-based mobility, e.g., L1/L3 measurement prediction, L1 event/L3 event prediction, and RLF prediction.

For the application of AI/ML-based features, an essential aspect of the applicability of the AI/ML functionality has been introduced during 5G AI/ML work. The applicability status of an AI/ML functionality or configuration dictates whether the functionality can be applied for inference based on model availability, UE internal conditions, and network-side conditions if available. An applicability reporting framework in Release-19 enables information exchange related to the AI/ML functionality/configuration applicability between the UE and the network. The reporting of initial applicability is performed using RRCReconfigurationComplete based on the received RRC configuration, and the subsequent reporting of change in applicability/inapplicability status of a functionality/configuration is signaled using the existing framework via UE Assistance Information (UAI). Since UAI was originally designed for UE preference related to parameters such as preferred RRC state, DRX preference, overheating, etc., which allows UE to optionally provide its preference to the network, if enabled. Given that the applicability/inapplicability of AI/ML-related configuration is crucial for meaningful inference outcomes, applicability reporting and consecutive network behavior should not be restricted to UE preference. In particular, due to the dynamic nature of AI/ML applicability based on UE internal conditions and/or network-side conditions, UE may encounter a situation where a previously applicable inference configuration becomes inapplicable, which can lead to an inaccurate/garbage inference outcome if not reconfigured/deconfigured by the network. Therefore, it’s important for RAN2 to study the requirements for AI/ML-related signaling and consider enhancements to ensure consistency in the application of AI/ML functionality at the UE and the network side. An AI/ML-specific framework (including applicability reporting framework) should be analysed to consider the benefits and impacts of UE autonomous behavior on the applicability/inapplicability of AI/ML functionality.

RAN2 is suggested to study enhancements for an AI/ML-specific applicability framework (e.g., applicability reporting and updates) to ensure consistency at the UE and the network side, e.g., timely deactivation of inapplicable AIML configuration.

## Transfer of various types of data

In legacy systems, the UE can be configured by the network to report measurement results and related information for various purposes, as summarized in the table (Table 1) below.

First, we would like to clarify that the collection and transfer of data discussed here focuses on data with low priority compared to legacy control information, such as L1/L3 measurement reports. Unlike legacy reporting of measurements, these data types do not demand stringent latency requirements. The L1/L3 measurement reports — which do require low latency — are typically transferred via UCI/RRC, as in legacy systems. This approach is straightforward and well-established. For features that require data with a different level of urgency and priority than legacy data, such as data with non-stringent latency requirements for AIML, QoE monitoring, MDT, and SON — separate data collection and transfer frameworks were adopted during the 5G discussions, as each feature was addressed independently. These frameworks are summarized in the table below. However, this fragmented approach has resulted in non-unified UE handling, complicating implementation and maintenance.

Table 1: Legacy UE reports and corresponding framework

| Report type | Signaling Framework | Latency requirement |
| --- | --- | --- |
| L1 measurement report | CSI report via UCI | Stringent |
| L3 measurement report | RRC measurement report via SRB1 | Stringent |
| QoE | RRC MeasurementReportAppLayer via SRB4/SRB5 Can be paused by NW | Relaxed |
| ImmediateMDT | RRC measurement report via SRB1 | Moderate |
| LoggedMDT | UE log, Logged MDT report available indication, UEInformationRequest via SRB1 UEInformationResponse via SRB2 | Relaxed |
| SON | UE log, SON report available indication, UEInformationRequest via SRB1 UEInformationResponse via SRB1 | Moderate |
| NW side data collection (AIML) | UE log, Logged Data available indication, UEInformationRequest via SRB1 UEInformationResponse via SRB6 | Relaxed |

Observation 4: In legacy 5G, the data collection/transfer for each feature (e.g., MDT, QoE, SON, AIML) was discussed separately, which leads to an ununified framework design.

In addition, legacy data collection/transfer design relies significantly on the control plane, often resulting in inefficiencies.  For example, in 5G-A Release 19, several alternatives were brought up for data collection for the purpose of training AI/ML models. For network-side models, the data collection/transfer is mainly performed using the existing RRC signaling framework. Whereas, for UE-side models, RAN2 has not made significant progress, and a potential solution via user plane transfer is under study in SA2. We would like to emphasize that although the RRC-based data transfer for network-side model demands less specification impact, the current design is inefficient for such AI/ML-related data transfer. In particular, considering the legacy InformationRequest and InformationResponse framework, in which every transfer is triggered in response to an individual request message and is restricted by the size limitation of the InformationResponse message. The general data transfer framework for AI/ML should be extended, considering the requirement of large data volume to be logged and transferred with respect to different use cases and inference configurations.

Observation 5: The legacy data collection/transfer via control plane signalling is sometimes inefficient.

Therefore, for 6G, a more unified and efficient data collection/transfer framework design should be discussed since Day-1. Overall, the data collection/transfer design can consider two cases due to different data consumers. As shown in Figure 4, the data transfer and data storage can be performed in some cases at the RAN node or at an entity in the CN/OAM. For example, in Case-I, the RAN node collects and utilizes the data for its own usage, e.g., training RAN side AI/ML models or sensing data to support the communication. Whereas in Case-II, some other network entities in CN/OAM, e.g., sensing server, UE server, etc, may collect and process the data with or without awareness of RAN. Certain data transfer requirements can be driven by each of these data type individually (e.g., AI/ML, MDT, QoE, sensing, etc.); for example, a low latency requirement for sensing-related data and a non-real-time latency requirement for AI/ML training-related data. At the same time, it is necessary to take into account the data transmission path and the data management entity (responsible for storing and managing the data) for these various types of data. Consequently, an overall consistent and unified framework must be designed in 6G for the management of the data collection-related processes and the data itself.

Figure 4: Cases for the transfer of various types of data

The following aspects need to be considered when designing the framework for data collection and transfer:

- Large volume of data

- Data termination entity

- Common data management entity

- Signaling overhead

- User privacy and consent

- Burden on the UE for data collection/transfer

RAN2 will study a common framework to facilitate the collection of diverse types of non-stringent latency data (e.g., AIML, sensing, MDT, QoE, SON) from the UE, considering different data consumers (RAN/CN/OAM/external servers) and vendors.

## Sensing

According to the SID on 6GR [1], the support of sensing should be studied. From RAN2 perspective, the objectives for sensing include the data transfer design, the higher layer procedures and protocol aspects.

(3) Radio interface protocol architecture and procedures for 6GR [RAN2, RAN1, RAN4, RAN3] - User Plane architecture and protocol design [RAN2] - Control Plane architecture (including RRC states) and protocol design [RAN2, RAN3] - Access stratum security aspects, in alignment with requirements from SA3 [RAN2] - Radio signalling framework for UE capabilities, aiming at improvements and simplification compared to 5G NR [RAN2, RAN1, RAN4] - Data transfer design to support various types of data (e.g. AI/ML, Sensing, etc) [RAN2, RAN3] (6) Radio Access Network architecture, interface protocols and procedures considering support of various services and functionalities (e.g. AI/ML, sensing, etc). [RAN3, RAN2] - Overall RAN architecture aspects [RAN3, RAN2] - RAN-CN functional split, interface, protocol stack and procedures [RAN3] - RAN internal functional split, interfaces, protocol stacks and procedures, [RAN3, RAN2] (9) Sensing – Studies to be based on use cases and associated requirements, as defined in [TR38.914] - PHY functions and procedures for sensing technology (e.g., waveform. reference signals, measurement feedback, etc…) [RAN1, RAN4] - Evaluate sensing performance and if necessary, extend channel modelling, for the selected use cases [RAN1] - Aspects of integration with communication services [RAN1] - higher layer procedures and protocol aspects [RAN2] - RAN4 aspects of sensing including RF, coexistence, and testability in coordination with other WGs [RAN4] Note: RAN1 identify detailed requirements, if triggered by TSG RAN

With regards to the higher layer procedures and protocol aspects, RAN2 should start work on sensing based on prioritized sensing modes and use cases due to the following reasons:

Sensing modes

As shown in Figure 5, there are 6 candidate sensing modes which can be considered for 6GR. Among the candidate sensing modes, the TRP-based mono-static sensing for UAV use case is already being considered as part of the Rel-20 5G-A study on ISAC (led by RAN1 and RAN3). However, due to time constraints and to avoid complexity, it may not be possible to support all of them in Day-1 6GR. From our perspective, the following sensing modes should be given higher priority and considered in the study:

- TRP-based sensing modes (TRP mono-static, TRP bi-static), and

- Bi-static sensing modes involving UEs (TRP-UE, UE-TRP). According to our simulations performed for proximity sensing/collision avoidance for UAV use case, the UE-assisted sensing (TRP-UE bi-static) provides some promising benefits in terms of the achievable sensing SNR performance.

As a consequence, the UE-based mono-static/bi-static sensing modes may need to be deprioritized in Day-1 6GR.

Furthermore, among the prioritized sensing modes, only the bi-static sensing modes involving UEs (TRP-UE, UE-TRP) impact AS L2/L3 procedures and protocols with regards to:

- Capability information definition and exchange

- Sensing entity (UE, RAN/TRP) discovery/selection/switching

- Sensing service continuity (sensing target/UE mobility among cells or within a given sensing area)

- Configuration and reporting of sensing data/measurement results

Figure 6 illustrates an exemplary signaling procedure for 6G sensing involving UE and NW entities. Figure 4 is applicable for the bi-static sensing modes TRP-UE and UE-TRP. The sensing function (SF) may be part of RAN or CN. In any case, we understand that the involvement of the sensing function (SF) and CN entities in the sensing procedure is in the remit of SA2, so coordination with them is needed.

Other aspects which could be considered as well in the study include, e.g., multi-mode sensing, i.e., combination with non-3GPP sensing data, and integration of sensing procedures with 6G positioning framework.

Figure 5: Candidate sensing modes

Use cases

Referring to the SA1 6G TR 22.870 [2], numerous use cases (UCs) for sensing have been captured in the TR. They can be categorized as follows:

- Object detection and tracking (e.g., UAV, vehicles, AGV, pedestrians)

- Sensing-assisted positioning (e.g., to achieve higher positioning accuracies)

- Sensing-assisted communication (e.g., to improve beam management)

- Environment monitoring

- Environment digitalization/reconstruction

- Motion monitoring

Also, due to time constraints and to avoid complexity, it may not be possible to support all of them in Day-1 6GR. Therefore, it needs to be discussed and decided which use case(s) to be prioritized for the study. In this respect, the UCs of object detection and tracking for the selected set of target types have been studied in Rel-19 5G-A by RAN1 from the perspective of channel modelling and deployment assumptions, which may serve as an introduction to the 6GR studies. Moreover, use cases for enhancing RAN operations, e.g., sensing-assisted positioning and communication, can be considered with a higher priority in Day-1 6GR.

Figure 6: Exemplary signaling procedure for 6G sensing involving UE and NW entities

RAN2 is suggested to study AS L2/L3 protocol enhancements and procedures based on prioritized sensing modes (e.g., TRP-based sensing modes and Bi-static sensing modes involving UEs) and use cases (e.g., use cases of object detection and tracking, and enhancing RAN operations for sensing-assisted positioning and communication), and coordinate with SA2 on the network architecture for sensing.

# Conclusion

Based on the discussion above, we propose:

Observation 1: Various concerns have been raised about vulnerability due to unprotected L1 and L2 information transmitted over the air in 5G NR.

Observation 2: Vulnerability due to unprotected L1 and L2 information transmitted over the air in 5G NR can’t easily be exploited by an attacker to manifest a serious threat.

Observation 3: In 5G it was consciously decided to not introduce security at MAC layer. 6G offers a clean slate and this is now certainly one possibility.

Observation 4: In legacy 5G, the data collection/transfer for each feature (e.g., MDT, QoE, SON, AIML) was discussed separately, which leads to an ununified framework design.

Observation 5: The legacy data collection/transfer via control plane signalling is sometimes inefficient.

- RAN2 should start discussion about need for protecting L2 control information, specifically the MAC CEs, as part of a broader discussion on placement of security function in the AS protocol sublayers.

- RAN2 is suggested to target a futureproof protocol design to cater to diverse device types supporting different levels of AI/ML capabilities.

RAN2 will study user plane enhancements, considering support of AI/ML-enabled features from an overall AI/ML framework perspective and not limiting it to specific use-cases.

RAN2 will study the support of AI/ML-based mobility, e.g., L1/L3 measurement prediction, L1 event/L3 event prediction, and RLF prediction.

RAN2 is suggested to study enhancements for an AI/ML-specific applicability framework (e.g., applicability reporting and updates) to ensure consistency at the UE and the network side, e.g., timely deactivation of inapplicable AIML configuration.

RAN2 will study a common framework to facilitate the collection of diverse types of non-stringent latency data (e.g., AIML, sensing, MDT, QoE, SON) from the UE, considering different data consumers (RAN/CN/OAM/external servers) and vendors.

RAN2 is suggested to study AS L2/L3 protocol enhancements and procedures based on prioritized sensing modes (e.g., TRP-based sensing modes and Bi-static sensing modes involving UEs) and use cases (e.g., use cases of object detection and tracking, and enhancing RAN operations for sensing-assisted positioning and communication), and coordinate with SA2 on the network architecture for sensing.

# References

- [1] 	  RP-252912, Revised SID: Study on 6G Radio.

- [2]	  3GPP TR 22.870 V0.4.0 (2025-09) Study on 6G Use Cases and Service Requirements.

[3] 	https://arxiv.org/html/2506.09502v2#S3

[4]	3GPP S3-252053 “Living document on NR mobility enhancement”, Samsung.

[5] 	3GPP S3-251938 “Update conclusion for KI#1”, ZTE Corporation.

[6] 	3GPP S3-252103 “LTM -  Conclusions” , Apple.

[7] 	3GPP S3-252188 “Conclusion update for Key issue #1”, LG Electronics.

[8] 	3GPP S3-251958 “New SID on MAC layer security”, ZTE Corporation.

[9] 	3GPP S3-252190 “Security consideration for MAC CE and lower layers”, OPPO.

[10] 	3GPP S3-252192 “Study on Security Aspects of Lower Layers”, OPPO.

[11] 	3GPP TS 38.321 “NR; Medium Access Control (MAC) protocol specification”.