3GPP TSG-RAN WG2 Meeting #131BIS						                                 R2-2507296

Prague, Czech Republic, Oct 13th - 17th 2025

Title: 	On RIL Z010, Z011, Z004/J009, Z005/H009, Z007, J008, H007 and RAN centric NW side data collection

Source: 	ZTE Corporation

Agenda item:	8.1.3

Document for:	Discussion/Decision

- Introduction

This tdoc is to share our views on the left issues and RILs on NW side data collection in the current version of RRC specification.

- Discussion

Z010

In RAN2#131bis meeting, it has been agreed to introduce the logging periodicity for logged NW side data collection, but RAN2 never discuss what is the logging behaviour if the logging periodicity is configured.

In our understanding, the basic intention of logging periodicity is to prevent UE from the logging interruption due to collision between the measurement resources for NW side data collection and measurement resources for other purposes, measurement gap, etc. Based on such intention, NW can provide a number of measurement resources in one logging period, and UE is able to select one of them that is not collided with any other measurements for logging..

Observation 1: The basic intention of logging periodicity is to prevent UE from the logging interruption due to the collision between measurement resources for NW side data collection and other measurement resources for different purposes, measurement gap, etc.

However, in the current TS 38.331, the logging periodicity mechanism has been defined as below：

loggingPeriodicity The periodicity that the UE shall use for the logging of the CSI measurements. The loggingPeriodicity is given as a multiple of the periodicity of the resources indicated by csi-LoggedResourceConfig. If loggingPeriodicity is included and set to 'n2', the UE performs the logging of CSI measurements for every 2nd occasion of the resources, if it is set to 'n3', the UE performs logging of CSI measurements for every 3rd occasion of the resources, and so on. If loggingPeriodicity is not included, the UE performs the logging of CSI measurements according to the periodicity of the resources indicated by csi-LoggedResourceConfig, i.e. for every occasion of the resources.

Based on above description, the UE logging behavior ,if logging periodicity is configured, is shown in below:

It can be seen that, the loggingPeriodicity is just a configurable parameter that directly extends the periodicity of the NZP CSI resources, UE is still required to perform the measurement on one specific resource for logging which deviates the basic intention of introducing the loggingPeriodicty in observation 1. In other words, NW can adjust the CSI resource periodicity directly instead of introducing an additional parameter (i.e. loggingPeriodicity) to do that, we do not see any values of the current loggingPeriodicity.

Observation 2: The current loggingPeriodicity is just to extend the periodicity of NZP CSI Resource by an integer multiple, and UE still needs to perform measurement on the certain measurement occasion which seems not helpful for the measurement resource collision case.

Therefore, we propose to change the UE measurement behavior when loggingPeridicity is configured, that is, to allow UE to select any one of measurement resources for logging those are contained in a logging period.

Proposal 1: If loggingPeriodicity is included and set to 'n2', the UE performs the logging of CSI measurements for every 2 occasion of the resources, if it is set to 'n3', the UE performs logging of CSI measurements for every 3 occasion of the resources, and so on.

Z011

In the current version of specification, the time gap indication agreed to have for the BM case 2, please see below agreements

- For temporal domain, the network is made aware whether there is a gap between two consecutive samples.   FFS amount of gap and whether this is implicit or explicit

In this agreement, the time gap indication is just needed for the BM case 2 as the continuous data collection is critical for training the NW side model of BM case 2. However, in the current text procedure for data logging, the time gap indication is mandatory no matter for which use case:

#### 5.5x.3.2 Initiation The UE shall: 1> for each CSI logged measurement configuration associated with a refCSI-LoggedMeasurementConfigId in csi-LogMeasInfoList in VarCSI-LogMeasReport, perform the logging of measurements for the serving cell associated with cellId, in accordance with the corresponding CSI logged measurement configuration within csi-LoggedMeasurementConfigToAddModList: 2> if the csi-LoggedMeasurementEventTriggerConfig is not included and the buffer[for network-side data collection is not full: 3> perform the logging at regular time intervals,[ according to loggingPeriodicity (if present) or according to the periodicity of the resources indicated by csi-LoggedResourceConfig in the corresponding CSI logged measurement configuration within csi-LoggedMeasurementConfigToAddModList, if loggingPeriodicity is not present; 2> if the csi-LoggedMeasurementEventTriggerConfig is included and the buffer for network-side data collection is not full: 3> if threshold within csi-LoggedMeasurementEventTriggerConfig is set to aboveThreshold and the entering condition, as specified in 5.5.4.2, is fulfilled for the serving cell associated with cellId for all measurements taken during timeToTrigger; or 3> if threshold within csi-LoggedMeasurementEventTriggerConfig is set to belowThreshold and the entering condition, as specified in 5.5.4.3, is fulfilled for the serving cell associated with cellId for all measurements taken during timeToTrigger: 4> perform the logging at regular time intervals, according to loggingPeriodicity (if present) or according to the periodicity of the resources indicated by csi-LoggedResourceConfig in the corresponding CSI logged measurement configuration within csi-LoggedMeasurementConfigToAddModList, if loggingPeriodicity is not present; 3> if threshold within csi-LoggedMeasurementEventTriggerConfig is set to aboveThreshold and the leaving condition, as specified in 5.5.4.2, is fulfilled for the serving cell associated with cellId for all measurements taken during timeToTrigger; or 3> if threshold within csi-LoggedMeasurementEventTriggerConfig is set to belowThreshold and the leaving condition, as specified in 5.5.4.3, is fulfilled for the serving cell associated with cellId for all measurements taken during timeToTrigger: 4> stop performing the logging for the corresponding CSI logged measurement configuration within csi-LoggedMeasurementConfigToAddModList; 2> when performing the logging: 3> for each CSI logged measurement configuration associated to refCSI-LoggedMeasurementConfigId in csi-LogMeasInfoList in VarCSI-LogMeasReport: 4> set the csi-RS-MeasResultList and SSB-MeasResultList to include the quantities the UE is logging measurements for, upon receiving the quantities from the lower layers; 4> if the time between the measurements that are logged and included in this instance of csi-LogMeasInfoList and the measurements for the previous instance of csi-LogMeasInfoList with the same refCSI-LoggedMeasurementConfigId, for the same serving cell, is longer than the logging periodicity (if configured) or the periodicity of the measurement resources (if the logging periodicity is not configured): 5> set the timeGap to true; 2> when the memory reserved for the logged measurement information for data collection becomes full, stop logging; 2> when the memory reserved for the logged measurement information for data collection is no longer full, resume logging.

Observation 3: The time gap indication in logged data is just required for the BM case 2 (i.e. Temporal domain), there is no need to have such indication in the logged measurements for the BM case 1 (i.e. spatial domain).

In this sense, the rational way is to introduce an enable flag in logging configuration dedicatedly for BM case 2, and if such enable flag is configured, UE shall log the time gap for each non-continuous records.

Proposal 2: Introduce an enable flag in the CSI-LoggedMeasurementConfig-r19 to enable logging of the time gap indication for the logged measurements:

- If the enable flag is present and the current logged measurement is logged in a longer logging period or resource period from the previous logged measurements:

  - the timeGap shall be present and set to True.

- Otherwise

  - The timeGap shall be absent.

Z004/J009

In the current logging data related text procedure, the first entry of the logged data for one serving cell and/or configuration Id is established only when the corresponding configuration is received, please see below:

#### 5.5x.1.3 Reception of CSI-LoggedMeasurementConfig by the UE Upon receiving csi-LoggedMeasurementConfigToAddModList in the csi-MeasConfig of a serving cell, the UE shall: 1> for each CSI logged measurement configuration included in csi-LoggedMeasurementConfigToAddModList: 2> if the current UE configuation for the serving cell includes the CSI logged measurement configuration associated with the given csi-LoggedMeasurementConfigId: 3> modify the CSI logged measurement configuration according to the configuration received in csi-LoggedMeasurementConfigToAddModList; 2> else: 3> add the received CSI logged measurement configuration to the UE configuration; 2> if the cell identity of the serving cell for which the measurements shall be logged, i.e. the serving cell associated with the serving cell configuration in which csi-LoggedMeasurementConfigToAddModList is received, is not included in an entry in csi-LogMeasInfoCellList in VarCSI-LogMeasReport: 3> include an entry in csi-LogMeasInfoCellList in VarCSI-LogMeasReport; 3> set cellId to the CGI of the serving cell associated with the serving cell configuration in which csi-LoggedMeasurementConfigToAddModList is received, if available. If the CGI is not available for that cell, set cellId to the ARFCNand PCI of the serving cell; 2> if not already present, include an entry in csi-LogMeasInfoList in VarCSI-LogMeasReport and set refCSI-LoggedMeasurementConfigId to the csi-LoggedMeasurementConfigId associated to the CSI logged measurement configuration included in csi-LoggedMeasurementConfigToAddModList; 2> perform measurements logging as specified in 5.5x.3.2.

However, establishing the entry when receiving the configuration may cause some error from UE perspective, for example, in the case of AS buffer has been full, NW reconfigure the UE to add a new NW side data collection configuration, UE cannot establish any entry for this NW side data collection due to the following agreement in RAN2:

=> When UE reaches its buffer limitation the UE stops measurement for data collection purposes and logging.

Observation 4: In the case of AS buffer is full, UE cannot establish any entry for the newly received configuration for a serving cell even though the current text procedure regarding the reception of configuration to say so.

To avoid the issue observed in the above observation, instead of establishing the new entry when receiving the configuration, UE can do that when preforming the data logging, that is UE to create the new entry for a serving cell and/or a configuration ID when logging the first piece of the data corresponding to them.

Proposal 3: The entry for the serving cell and/or the configuration Id in csi-LogMeasInfoCellList in VarCSI-LogMeasReport only can be created when performing the measurement logging.

Besides, the issue mentioned in J009 is somehow related to Z004:

********************************** J009 ***************************************************************************************

| RIL Id | WI | Class | Title | - Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| J009 | AIML | 1 | Discard entries in csi-LogMeasInfoCellList after sending UEInformationResponse | R2-25xxxx | Sharp (LIU Lei) |  | V009 | ToDo |

[Description]: After sending UEInformationResponse message, the logged measurement entries included in the csi-LogMeasInfoList from VarCSI-LogMeasReport are discard. If csi-LogMeasInfoList is empty, the entries included in csi-LogMeasInfoCellList should be discard.

********************************** J009 ***************************************************************************************

If above proposal is agreeable, that is, UE only create the entry when logging the first logged measurement result, so in our understanding, the issue mentioned in J009 can be avoided, and the RIL can be closed without any spec change.

Proposal 4: The issue raised by J009 is not exited if UE creates the entry only when at least one measurement result for this entry is logged.

Z005/H009

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Z005 |  | Class 1 | Start/stop performing L1 measurement in Logged NW side data collection | R2-25xxxx | Fei |  | V015 | ToDo |

In RlL Z005, it points out that, in the current text procedure for logging data, the interaction between RRC layer and PHY layer for starting L1 measurement when performing the logging, and stopping L1 measurement when logging is stopped

Observation 5: The interaction between RRC and PHY layer is missing in the current text procedure regarding measurement logging, as a result, PHY layer still performs the L1 measurement when measurement logging has been stopped in RRC layer which may cause the unnecessary power consumption, PHY layer does not perform the corresponding L1 measurement in time when RRC layer start/resume data logging which may result in nothing can be logged by RRC.

So it is propose that:

Proposal 5: Add the interlayer interaction between RRC layer and PHY layer in the text procedure regarding L1 measurement logging in subclause 5.5x.3.2:

- Indicate to lower layer to start the corresponding L1 measurement when performing the logging

- Indicate to lower layer to stop the corresponding L1 measurement when stopping performing the logging.

Z007

According to RAN2 agreements achieved in RAN2#131bis meeting

- Multiplexing of legacy SON/MDT report and AIML logged data is not supported in the same UE information response message.  Up to the network to ensure that data is not requested at the same time

However, this agreement is only reflected by the following text procedure according to the comments from rapporteur:

/omit for short/ 1> if the logMeasReportReq is present and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport, or if the current registered SNPN identity is included in snpn-ConfigID-List stored in VarLogMeasReport: 2> if VarLogMeasReport includes one or more logged measurement entries, set the contents of the logMeasReport in the UEInformationResponse message as follows: 3> include the absoluteTimeStamp and set it to the value of absoluteTimeInfo in the VarLogMeasReport; 3> include the traceReference and set it to the value of traceReference in the VarLogMeasReport; 3> include the traceRecordingSessionRef and set it to the value of traceRecordingSessionRef in the VarLogMeasReport; 3> include the tce-Id and set it to the value of tce-Id in the VarLogMeasReport; 3> include the logMeasInfoList and set it to include one or more entries from the VarLogMeasReport starting from the entries logged first, and for each entry of the logMeasInfoList that is included, include all information stored in the corresponding logMeasInfoList entry in VarLogMeasReport; 3> if the VarLogMeasReport includes one or more additional logged measurement entries that are not included in the logMeasInfoList within the UEInformationResponse message: 4> include the logMeasAvailable; 4> if bt-LocationInfo is included in locationInfo of one or more of the additional logged measurement entries in VarLogMeasReport that are not included in the logMeasInfoList within the UEInformationResponse message: 5> include the logMeasAvailableBT; 4> if wlan-LocationInfo is included in locationInfo of one or more of the additional logged measurement entries in VarLogMeasReport that are not included in the logMeasInfoList within the UEInformationResponse message: 5> include the logMeasAvailableWLAN; /omit for short/ 1> if the csi-LogMeasReportReq is present: 2> if VarCSI-LogMeasReport includes one or more logged measurement entries, set the contents of the csi-LogMeasReport in the UEInformationResponse message as follows: 3> include the csi-LogMeasInfoCellList and set it to include one or more entries from the VarCSI-LogMeasReport starting from the entries logged first; 3> if the VarCSI-LogMeasReport includes one or more additional logged measurement entries that are not included within the UEInformationResponse message: 4> include the csi-MoreLogMeasAvailable; /omit for short/ 1> if the logMeasReport is included in the UEInformationResponse: 2> submit the UEInformationResponse message to lower layers for transmission via SRB2; 2> discard the logged measurement entries included in the logMeasInfoList from VarLogMeasReport upon successful delivery of the UEInformationResponse message confirmed by lower layers; 1> else if csi-LogMeasReport is included in the UEInformationResponse: 2> submit the UEInformationResponse message to lower layers for transmission via SRBX; 2> discard the logged measurement entries included in the csi-LogMeasInfoList from VarCSI-LogMeasReport upon successful delivery of the UEInformationResponse message confirmed by lower layers; 1> else: 2> submit the UEInformationResponse message to lower layers for transmission via SRB1.

But with above procedure, it still allows NW to provide both logMeasReportReq and csi-LogMeasReportReq in the same message, and if NW doing this, UE shall include the logged data from both MDT and NW side data collection in one UEInformationResponse message, and transfer the logged data from via SRB 2 (i.e. go to the first branch which is highlighted with green, and the branch with yellow highlighted will be skipped), this is obviously not expected.

In this sense, it is suggested to have an explicit description about the restriction in the field description of csi-LogMeasReport, that is, NW cannot present both csi-LogMeasReportReq and logMeasReportReq in one UEInformationRequest message.

Proposal 6: Add the following sentence in the field description of csi-LogMeasReportReq to prevent NW from simultaneously retrieving the logged data from MDT and NW side data collection:

‘ The csi-LogMeasReportReq shall be absent if the logMeasReportReq is present in this message’

J008

Regarding J008,

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| J008 | AIML | 1 | Setting csi-LogMeasInfoList in UEInformationResponse | R2-25xxxx | Sharp (LIU Lei) |  | V009 | ToDo |

We can understanding the intention of J008, the sprit of this RIL is how to deal with the case where the remaining size of one RRC message (i.e.UEInformationResponse) cannot accommodate one entry (i.e. CSI-LogMeasInfoCell) in csi-LogMeasInfoCellList which is earlier to be logged, this case is not clear based on the current text procedure

.

1> if the csi-LogMeasReportReq is present: 2> if VarCSI-LogMeasReport includes one or more logged measurement entries, set the contents of the csi-LogMeasReport in the UEInformationResponse message as follows: 3> include the csi-LogMeasInfoCellList and set it to include one or more entries from the VarCSI-LogMeasReport starting from the entries logged first; 3> if the VarCSI-LogMeasReport includes one or more additional logged measurement entries that are not included within the UEInformationResponse message: 4> include the csi-MoreLogMeasAvailable;

In our understanding, it may have two ways forward:

- Option 1: Do not include this entry (i.e. CSI-LogMeasInfoCell) of csi-LogMeasInfoCellList in the RRC message (i.e. UEInformationResponse).

- Option 2: Include the entries of csi-LogMeasInfoList-r19 contained in CSI-LogMeasInfoCell until the upper limitation of the RRC message is reached.

In option 1, option 1 is the easiest way to go, but it is not workable in some extreme case, for example, the size of the entry is larger than 9KB which is over the maximum size of a RRC message, in this case, all the logged data will be stuck in the AS buffer which cannot be transferred by using UEInformationResponse, as a result, the whole NW side data collection collapses.

In option 2, it can improve the UL grant utilization efficiency and provide more logged data as much as possible in one transmission opportunity, but the option 2 may cause more spec impact.

Compare between option 1 and option 2, option 1 may have less spec impact, but it would cause the procedure error as analyzed above, option 2 is more reasonable in the real deployment although it may cause more spec impact than option 1. In this sense, the option 2 is preferred over the option 1 by us.

Besides, there is a RIL Z008 which is marked as PropAgree which proposes to introduce a finer granularity entry csi-LogMeasInfoMeasConfigList-r19 for accumulating the logged measurements from the same logging configuration Id together in order to reduce the reporting signaling overhead. Please see below:

CSI-LogMeasInfoCell-r19 ::=          SEQUENCE {

cellId-r19                              CHOICE {

cellGlobalId-r19                        CGI-Info-Logging-r16,

pci-arfcn-r19                           PCI-ARFCN-NR-r16

},

csi-LogMeasInfoMeasConfigList-r19                    SEQUENCE (SIZE (1..maxNrofLoggedMeasurementConfigurations-r19)) OF CSI-LogMeasInfoMeasConfig-r19,

...

}

CSI-LogMeasInfoMeasConfig-r19 ::=              SEQUENCE {

refCSI-LoggedMeasurementConfigId-r19    CSI-LoggedMeasurementConfigId-r19,

csi-LogMeasInfoList-r19                    SEQUENCE (SIZE (1..maxLogCSI-MeasReport-r19)) OF CSI-LogMeasInfo-r19,

...

}

CSI-LogMeasInfo-r19 ::=              SEQUENCE {

csi-RS-MeasResultList-r19               SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-Resources)) OF CSI-RS-MeasResult-r19    OPTIONAL,

ssb-MeasResultList-r19                  SEQUENCE (SIZE (1..maxNrofSSBs-r16)) OF SSB-MeasResult-r19                       OPTIONAL,

timeGap-r19                          ENUMERATED {true}                                       OPTIONAL,    ...

}

Observation 6: RIL Z008 which is marked as PropAgree which proposes to introduce a finer granularity entry within csi-LogMeasInfoMeasConfigList-r19 for accumulating the logged measurements from the same logging configuration Id together in order to reduce the reporting signaling overhead.

By taking Z008 into account, we propose that for option 2:

Proposal 7: When UE includes the logged measurements in the UEInformationResponse, the UE can accommodate the logged measurements as in following:

3>	include the csi-LogMeasInfoCellList and set it to include one or more entries from the VarCSI-LogMeasReport starting from the entries logged first;

4> for each entry in csi-LogMeasInfoCellList, set the field cellId in this entry to the corresponding cellId from VarCSI-LogMeasReport, and include the csi-LogMeasInfoMeasConfigList and set it to include one or more corresponding entries from VarCSI-LogMeasReport starting from the entries logged first;

5> for each entry in csi-LogMeasInfoMeasConfigList, set the field refCSI-LoggedMeasurementConfigId in this entry to the corresponding refCSI-LoggedMeasurementConfigId from VarCSI-LogMeasReport, include csi-LogMeasInfoList and set it to include one or more entries from VarCSI-LogMeasReport starting from the entries logged first;

In addition, assuming above proposal is agreeable, the discarding behavior shall be updated as well, that is, UE shall discard the entry as following:

1: If an entry of csi-LogMeasInfoCellList from VarCSI-LogMeasReport has been included in the UEInformationResponse，UE shall discard this entry from VarCSI-LogMeasReport upon the successful transmission of UEInformationReponse.

2: If an entry of csi-LogMeasInfoMeasConfigList from VarCSI-LogMeasReport has been included in the UEInformationResponse，UE shall discard this entry from VarCSI-LogMeasReport upon the successful transmission of UEInformationReponse.

3: If an entry of csi-LogMeasInfoList from VarCSI-LogMeasReport has been included in the UEInformationResponse，UE shall discard this entry from VarCSI-LogMeasReport upon the successful transmission of UEInformationReponse.

Proposal 8: UE shall discard the logged measurement level by level upon the successful transmission of UEInformationResponse:

- If an entry of csi-LogMeasInfoCellList from VarCSI-LogMeasReport has been included in the UEInformationResponse，UE shall discard this entry from VarCSI-LogMeasReport upon the successful transmission of UEInformationReponse.

- If an entry of csi-LogMeasInfoMeasConfigList from VarCSI-LogMeasReport has been included in the UEInformationResponse，UE shall discard this entry from VarCSI-LogMeasReport upon the successful transmission of UEInformationReponse.

- If an entry of csi-LogMeasInfoList from VarCSI-LogMeasReport has been included in the UEInformationResponse，UE shall discard this entry from VarCSI-LogMeasReport upon the successful transmission of UEInformationReponse.

H007

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| H007 | AIML | 1 | Logged measurement configuration modification and release | R2-25xxxx | Dawid |  | vnnn | ToDo |

[Description]:

Currently it is possible for the network to provide an updated logged measurement configuration (i.e. reusing the same csi-LoggedMeasurementConfigId as already included in the UE conifguration). This may lead to ambiguity in the collected data, i.e. once it is reported it will be unclear to which configuration this data referred to.

Similarly, the UE currently does not discard the collected data for a certain logging conifguration even when this conifguraiton is relased. If the UE is subsequently configured with a new clogging configuration later on, reusing the ID of the previously released configuration, similar ambiguity exists. Such situation should be avoided.

[Proposed Change]:

It is proposed to clarify that when the UE receives a modified logging conifguration or releases a logging configuration, the discards the logged data related to the modified/released logging configuration.

******************************************************************************************************************************

The issue mentioned in RIL is indeed existed somehow, but the proposed change will cause the collected data loss which is not expected and a nice solution.

Observation 6: Discarding the collected data when associated logging configuration is released or modified will cause the collected data loss which is waste the UE effort to collect the data.

Regarding the modification case:

there are 3 information elements contained in the csi-LoggedMeasurementConfig in addition to the csi-LoggedMeasurement ConfigId, that is csi-ResourceConfigId, loggingPeriodicity, csi-LoggedMeasurementEventTriggerConfig

CSI-LoggedMeasurementConfig-r19 ::=          SEQUENCE {

csi-LoggedMeasurementConfigId-r19         CSI-LoggedMeasurementConfigId-r19,

csi-LoggedResourceConfig-r19              CSI-ResourceConfigId,

loggingPeriodicity-r19                    ENUMERATED {n2, n3, n4, n5, spare4, spare3, spare2, spare1}            OPTIONAL,  -- Need M[RIL]: B203, AIML

csi-LoggedMeasurementEventTriggerConfig-r19         CSI-LoggedMeasurementEventTriggerConfig-r19                OPTIONAL,  -- Need R

...

}

In our understanding, except for the csi-ResourceConfigId, the change of loggingPeriodicity, csi-LoggedMeasurementEventTriggerConfig does not cause any ambiguity of the collected data as all the collected data from the same measurement resource configuration for training one same NW side model.

For the change of csi-ResoureConfigId, we do not think NW will implement this change in the real deployment as this change is to collect the data for training a different NW side model, in this sense, the smart NW implementation is to add a new csi-LoggedMeasurementConfig with a new Id value rather than reconfiguring the csi-LoggedResourceConfig in the currently existed csi-LoggedMeasurementConfig.

Observation 7: Not all the modification cases for the csi-LoggedMeasurementConfig need UE discard the logged measurements for this logging configuration, such as, modify the loggingPeriodicity, cs-LoggedMeasurementEventTriggerConfig parameter in csi-LoggedMeasurementConfig.

Regarding the release case:

As same as above, if the corresponding logging configuration is released, that means, the data collection for training a NW side model is terminated, if NW requires a new data collection for training another NW side model, smart NW implementation is to retrieve the logged measurement before adding the csi-LoggedMeasurementConfig with the same Id as the csi-LoggedMeasurementConfig is previously released.

Observation 8: NW can be aware of situation before releasing the CSI-LoggedMeasurementConfig, so that the NW is able to do the right things when releasing the old csi-LoggedMeasurementConfig and adding a new csi-LoggedMeasurementConfig with the same configuration Id.

According to the observation 6 and observation 7, it is better to rely on the rational NW implementation to guarantee that there is no any ambiguous data will be retrieved. Instead of simply discarding the collected data, we can capture a simple note in the subclause 5.5x.1.3.

Proposal 9: To capture the following note in the subclause 5.5x.1.3:

‘ It is up to NW ensure that UE will not log the measurements for a csi-LoggedMeasurementConfigId is from different csi-LoggedResourceConfig in the VarCSI-LogMeasReport’

NW side data collection in NR-DC

In the previous meeting, it is confirmed that:

RAN2 confirms that previous agreement excluded SN being configured for OAM centric data collection

However, the NW side data collection has two types, one is OAM centric NW side data collection, the other one is RAN centric NW side data collection. So far as now, there is no any clarification in RAN2 for RAN centric NW side data collection for the scenario of NR-DC.

In our understanding, there are three options of the way forward:

- Option 1: Support NR-DC, but RAN centric NW side data collection only can be configured for MCG (i.e. as same as OAM centric data collection)

- Option 2: Support NR-DC, the RAN centric NW side data collection can be configured for both MCG and SCG

For option 1, it is as similar as the OAM centric NW side data collection. However, considering the real deployment where the NR-DC is normally FR1-FR2 DC mode, which means, FR1 cells are belonging to the MCG while FR2 cells are belonging to the SCG, it is quite odd to only configure NW side data collection for MCG since there is no any requirement of beam management for the FR1 cells in MCG, let alone the AI/ML based beam management.

For option 2, for supporting NW side data collection being configured for both MCG and SCG, there are a lot of issues need to be discussed, for example, the inter-node interaction for configuring the NW side data collection configuration, the data path and corresponding method for reporting logged data to MN and SN, etc. All those issues may have RAN3 impact.

Considering RAN3 has no time to resolve the issue raised by above option 2, it is suggested:

Proposal 10: RAN2 confirms that, for RAN centric data collection in NR-DC scenario, the NW side data collection only can be configured for MCG.

As both OAM centirc and RAN centric NW side data collection only can be configured for MCG in NR-DC scenario, so that this conclusion needs to be captures in TS 37.340 :

Proposal 11: Capture the following information in TS 37.340：

For NW side data collection with data logging in NR-DC scenario, UE only can be configured with data logging for MCG, and correspondingly, the logged data for NW side data collection only can be reported to the MN (as specified in TS 38.331[4]).

- Conclusion

In this paper, we provide our views on the RIL and left issues of NW side-side data collection, We have the following observations, and proposals:

Z010

Observation 1: The basic intention of logging periodicity is to prevent UE from the logging interruption due to the collision between measurement resources for NW side data collection and other measurement resources for different purposes, measurement gap, etc.

Observation 2: The current loggingPeriodicity is just to extend the periodicity of NZP CSI Resource by an integer multiple, and UE still needs to perform measurement on the certain measurement occasion which seems not helpful for the measurement resource collision case.

Proposal 1: If loggingPeriodicity is included and set to 'n2', the UE performs the logging of CSI measurements for every 2 occasion of the resources, if it is set to 'n3', the UE performs logging of CSI measurements for every 3 occasion of the resources, and so on.

Z011

Observation 3: The time gap indication in logged measurements is just required for the BM case 2 (i.e. Temporal domain), there is no need to have such indication in the logged measurements for the BM case 1 (i.e. spatial domain).

Proposal 2: Introduce an enable flag in the CSI-LoggedMeasurementConfig-r19 to enable logging of the time gap indication for the logged measurements:

- If the enable flag is present and the current logged measurement is logged in a longer logging period or resource period from the previous logged measurements:

  - the timeGap shall be present and set to True.

- Otherwise

  - The timeGap shall be absent.

Z004/J009

Observation 4: In the case of AS buffer is full, UE cannot establish any entry for the newly received configuration for a serving cell even though the current text procedure regarding the reception of configuration to say so.

Proposal 3: The logged measurements entry for the serving cell and/or the configuration Id in csi-LogMeasInfoCellList in VarCSI-LogMeasReport only can be created when performing the measurement logging.

Proposal 4: The concern from J009 is no longer exited if UE creates the entry only when at least one measurement result for this entry is logged.

Z005/H009

Observation 5: The interaction between RRC and PHY layer is missing in the current text procedure regarding measurement logging, as a result, PHY layer still performs the L1 measurement when measurement logging has been stopped in RRC layer which may cause the unnecessary power consumption, PHY layer does not perform the corresponding L1 measurement in time when RRC layer start/resume data logging which may result in nothing can be logged by RRC.

Proposal 5: Add the interlayer interaction between RRC layer and PHY layer in the text procedure regarding L1 measurement logging in subclause 5.5x.3.2:

- Indicate to lower layer to start the corresponding L1 measurement when performing the logging

- Indicate to lower layer to stop the corresponding L1 measurement when stopping performing the logging.

Z007

Proposal 6: Add the following sentence in the field description of csi-LogMeasReportReq to prevent NW from simultaneously retrieving the logged data from MDT and NW side data collection:

‘ The csi-LogMeasReportReq shall be absent if the logMeasReportReq is present in this message’

J008

Observation 6: RIL Z008 which is marked as PropAgree which proposes to introduce a finer granularity entry within csi-LogMeasInfoMeasConfigList-r19 for accumulating the logged measurements from the same logging configuration Id together in order to reduce the reporting signaling overhead.

Proposal 7: When UE includes the logged measurements in the UEInformationResponse, the UE can accommodate the logged measurements as in following:

3>	include the csi-LogMeasInfoCellList and set it to include one or more entries from the VarCSI-LogMeasReport starting from the entries logged first;

4> for each entry in csi-LogMeasInfoCellList, set the field cellId in this entry to the corresponding cellId from VarCSI-LogMeasReport, and include the csi-LogMeasInfoMeasConfigList and set it to include one or more corresponding entries from VarCSI-LogMeasReport starting from the entries logged first;

5> for each entry in csi-LogMeasInfoMeasConfigList, set the field refCSI-LoggedMeasurementConfigId in this entry to the corresponding refCSI-LoggedMeasurementConfigId from VarCSI-LogMeasReport, include csi-LogMeasInfoList and set it to include one or more entries from VarCSI-LogMeasReport starting from the entries logged first;

Proposal 8: UE shall discard the logged measurement level by level upon the successful transmission of UEInformationResponse:

- If an entry of csi-LogMeasInfoCellList from VarCSI-LogMeasReport has been included in the UEInformationResponse，UE shall discard this entry from VarCSI-LogMeasReport upon the successful transmission of UEInformationReponse.

- If an entry of csi-LogMeasInfoMeasConfigList from VarCSI-LogMeasReport has been included in the UEInformationResponse，UE shall discard this entry from VarCSI-LogMeasReport upon the successful transmission of UEInformationReponse.

- If an entry of csi-LogMeasInfoList from VarCSI-LogMeasReport has been included in the UEInformationResponse，UE shall discard this entry from VarCSI-LogMeasReport upon the successful transmission of UEInformationReponse.

H007

Observation 7: Discarding the collected data when associated logging configuration is released or modified will cause the collected data loss which is waste the UE effort to collect the data.

Observation 8: Not all the modification cases for the csi-LoggedMeasurementConfig need UE discard the logged measurements for this logging configuration, such as, modify the loggingPeriodicity, cs-LoggedMeasurementEventTriggerConfig parameter in csi-LoggedMeasurementConfig.

Observation 9: NW can be aware of situation before releasing the CSI-LoggedMeasurementConfig, so that the NW is able to do the right things when releasing the old csi-LoggedMeasurementConfig and adding a new csi-LoggedMeasurementConfig with the same configuration Id.

Proposal 9: To capture the following note in the subclause 5.5x.1.3:

‘ It is up to NW implementation to guarantee that UE will not log the measurements for a csi-LoggedMeasurementConfigId is from different csi-LoggedResourceConfig in the VarCSI-LogMeasReport’

RAN Centric NW side data collection

Proposal 10: RAN2 confirms that, for RAN centric data collection in NR-DC scenario, the NW side data collection only can be configured for MCG.

Proposal 11: Capture the following information in TS 37.340：

For NW side data collection with data logging in NR-DC scenario, UE only can be configured with data logging for MCG, and correspondingly, the logged data for NW side data collection only can be reported to the MN (as specified in TS 38.331[4]).

- Annex

TP for Z010

LoggingPeriodicity

The periodicity that the UE shall use for the logging of the CSI measurements. The loggingPeriodicity is given as a multiple of the periodicity of the resources indicated by csi-LoggedResourceConfig. If loggingPeriodicity is included and set to 'n2', the UE performs the logging of CSI measurements for every two occasion of the resources, if it is set to 'n3', the UE performs logging of CSI measurements for every three occasion of the resources, and so on. If loggingPeriodicity is not included, the UE performs the logging of CSI measurements according to the periodicity of the resources indicated by csi-LoggedResourceConfig, i.e. for every occasion of the resources.

TP for Z011

- –	CSI-LoggedMeasurementConfig

The IE CSI-LoggedMeasurementConfig is used to configure a CSI logged measurement configuration. It defines a group of one or more CSI resources for which the UE logs the associated L1 radio measurements.

CSI-LoggedMeasurementConfig information element

-- ASN1START

-- TAG-CSI-LOGGEDMEASUREMENTCONFIG-START

CSI-LoggedMeasurementConfig-r19 ::=          SEQUENCE {

csi-LoggedMeasurementConfigId-r19         CSI-LoggedMeasurementConfigId-r19,

csi-LoggedResourceConfig-r19              CSI-ResourceConfigId,

loggingPeriodicity-r19                    ENUMERATED {n2, n3, n4, n5, spare4, spare3, spare2, spare1}            OPTIONAL,  -- Need M[RIL]: B203, AIML

csi-LoggedMeasurementEventTriggerConfig-r19         CSI-LoggedMeasurementEventTriggerConfig-r19                OPTIONAL,  -- Need R

enableTimeGap-r19                               ENUMERATED {True}                                                OPTIONAL,  -- Need R

...

}

CSI-LoggedMeasurementEventTriggerConfig-r19 ::=          SEQUENCE {

threshold-r19                     CHOICE {

aboveThreshold-r19               MeasTriggerQuantity,

belowThreshold-r19               MeasTriggerQuantity

},

hysteresis                        Hysteresis,

timeToTrigger                     TimeToTrigger,

...

}

-- TAG-CSI-LOGGEDMEASUREMENTCONFIG-STOP

-- ASN1STOP

CSI-LoggedMeasurementConfig field descriptions

csi-LoggedMeasurementConfigId This field indicates the instance of CSI-LoggedMeasurementConfig.

csi-LoggedResourceConfig Resources in which the UE performs channel measurement whose associated measurement results are logged by the UE. The csi-LoggedResourceConfig indicated here contains only NZP-CSI-RS resources and/or SSB resources.

csi-LoggedMeasurementEventTriggerConfig This field is used to configure the UE with event-triggered measurement logging. If this field is included and threshold is set to aboveThreshold, the UE starts performing logging of measurements when the entering condition as specified in 5.5.4.2 is met and stops logging when the corresponding leaving condition as specified in 5.5.4.2 is met. If this field is included and threshold is set to belowThreshold, the UE starts performing logging of measurements when the entering condition as specified in 5.5.4.3 is met and stops logging when the corresponding leaving condition as specified in 5.5.4.3 is met. If this field is not included, the UE starts the measurement logging according to csi-LoggedResourceConfig upon reception.

enableTimeGap-r19 This field is used to enable UE to insert the timeGap indication for the logged measurement result.

loggingPeriodicity The periodicity that the UE shall use for the logging of the CSI measurements. The loggingPeriodicity is given as a multiple of the periodicity of the resources indicated by csi-LoggedResourceConfig. If loggingPeriodicity is included and set to 'n2', the UE performs the logging of CSI measurements for every 2nd occasion of the resources, if it is set to 'n3', the UE performs logging of CSI measurements for every 3rd occasion of the resources, and so on. If loggingPeriodicity is not included, the UE performs the logging of CSI measurements according to the periodicity of the resources indicated by csi-LoggedResourceConfig, i.e. for every occasion of the resources.

#### 5.5x.3.2	Initiation

The UE shall:

1>	for each CSI logged measurement configuration associated with a refCSI-LoggedMeasurementConfigId in csi-LogMeasInfoList in VarCSI-LogMeasReport, perform the logging of measurements for the serving cell associated with cellId, in accordance with the corresponding CSI logged measurement configuration within csi-LoggedMeasurementConfigToAddModList:

2>	if the csi-LoggedMeasurementEventTriggerConfig is not included and the buffer[for network-side data collection is not full:

3>	perform the logging at regular time intervals,[ according to loggingPeriodicity (if present) or according to the periodicity of the resources indicated by csi-LoggedResourceConfig in the corresponding CSI logged measurement configuration within csi-LoggedMeasurementConfigToAddModList, if loggingPeriodicity is not present;

2>	if the csi-LoggedMeasurementEventTriggerConfig is included and the buffer for network-side data collection is not full:

3>	if threshold within csi-LoggedMeasurementEventTriggerConfig is set to aboveThreshold and the entering condition, as specified in 5.5.4.2, is fulfilled for the serving cell associated with cellId for all measurements taken during timeToTrigger; or

3>	if threshold within csi-LoggedMeasurementEventTriggerConfig is set to belowThreshold and the entering condition, as specified in 5.5.4.3, is fulfilled for the serving cell associated with cellId for all measurements taken during timeToTrigger:

4>	perform the logging at regular time intervals, according to loggingPeriodicity (if present) or according to the periodicity of the resources indicated by csi-LoggedResourceConfig in the corresponding CSI logged measurement configuration within csi-LoggedMeasurementConfigToAddModList, if loggingPeriodicity is not present;

3>	if threshold within csi-LoggedMeasurementEventTriggerConfig is set to aboveThreshold and the leaving condition, as specified in 5.5.4.2, is fulfilled for the serving cell associated with cellId for all measurements taken during timeToTrigger; or

3>	if threshold within csi-LoggedMeasurementEventTriggerConfig is set to belowThreshold and the leaving condition, as specified in 5.5.4.3, is fulfilled for the serving cell associated with cellId for all measurements taken during timeToTrigger:

4>	stop performing the logging for the corresponding CSI logged measurement configuration within csi-LoggedMeasurementConfigToAddModList;

2>	when performing the logging:

3>	for each CSI logged measurement configuration associated to refCSI-LoggedMeasurementConfigId in csi-LogMeasInfoList in VarCSI-LogMeasReport:

4>	set the csi-RS-MeasResultList and SSB-MeasResultList to include the quantities the UE is logging measurements for, upon receiving the quantities from the lower layers;

4>	if the enableTimeGap is set to true in the associated csi-LoggedMeasurementConfig and the time between the measurements that are logged and included in this instance of csi-LogMeasInfoList and the measurements for the previous instance of csi-LogMeasInfoList with the same refCSI-LoggedMeasurementConfigId, for the same serving cell, is longer than the logging periodicity (if configured) or the periodicity of the measurement resources (if the logging periodicity is not configured):

5>	set the timeGap to true;

2>	when the memory reserved for the logged measurement information for data collection becomes full, stop logging;

2>	when the memory reserved for the logged measurement information for data collection is no longer full, resume logging.

TP for Z004/J009

#### 5.5x.1.3	Reception of CSI-LoggedMeasurementConfig by the UE

Upon receiving csi-LoggedMeasurementConfigToAddModList in the csi-MeasConfig of a serving cell, the UE shall:

1>	for each CSI logged measurement configuration included in csi-LoggedMeasurementConfigToAddModList:

2>	if the current UE configuation for the serving cell includes the CSI logged measurement configuration associated with the given csi-LoggedMeasurementConfigId:

3>	modify the CSI logged measurement configuration according to the configuration received in csi-LoggedMeasurementConfigToAddModList;

2>	else:

3>	add the received CSI logged measurement configuration to the UE configuration;

2>	perform measurements logging as specified in 5.5x.3.2.

### 5.5x.2	Release of Network-Side Logged Measurement Configuration

#### 5.5x.2.1	General

The purpose of this procedure is to release the logged measurement configuration for network-side data collection.

#### 5.5x.2.2	Initiation

Upon receiving csi-LoggedMeasurementConfigToReleaseList, the UE shall:

1>	for each CSI logged measurement configuration ID included in csi-LoggedMeasurementConfigToReleaseList associated with a serving cell:

2>	if the current UE configuration for the associated serving cell includes a CSI logged measurement configuration with the associated CSI logged measurement configuration ID:

3>	release the CSI logged measurement configuration.

### 5.5x.3	Measurements logging

#### 5.5x.3.1	General

This procedure specifies the logging of available measurements by a UE in RRC_CONNECTED that has a logged measurement configuration for network-side data collection.

#### 5.5x.3.2	Initiation

The UE shall:

1>	for each CSI logged measurement configuration associated with a refCSI-LoggedMeasurementConfigId in csi-LogMeasInfoList in VarCSI-LogMeasReport, perform the logging of measurements for the serving cell associated with cellId, in accordance with the corresponding CSI logged measurement configuration within csi-LoggedMeasurementConfigToAddModList:

2>	if the csi-LoggedMeasurementEventTriggerConfig is not included and the buffer for network-side data collection is not full:

3>	perform the logging at regular time intervals, according to loggingPeriodicity (if present) or according to the periodicity of the resources indicated by csi-LoggedResourceConfig in the corresponding CSI logged measurement configuration within csi-LoggedMeasurementConfigToAddModList, if loggingPeriodicity is not present;

2>	if the csi-LoggedMeasurementEventTriggerConfig is included and the buffer for network-side data collection is not full:

3>	if threshold within csi-LoggedMeasurementEventTriggerConfig is set to aboveThreshold and the entering condition, as specified in 5.5.4.2, is fulfilled for the serving cell associated with cellId for all measurements taken during timeToTrigger; or

3>	if threshold within csi-LoggedMeasurementEventTriggerConfig is set to belowThreshold and the entering condition, as specified in 5.5.4.3, is fulfilled for the serving cell associated with cellId for all measurements taken during timeToTrigger:

4>	perform the logging at regular time intervals, according to loggingPeriodicity (if present) or according to the periodicity of the resources indicated by csi-LoggedResourceConfig in the corresponding CSI logged measurement configuration within csi-LoggedMeasurementConfigToAddModList, if loggingPeriodicity is not present;

3>	if threshold within csi-LoggedMeasurementEventTriggerConfig is set to aboveThreshold and the leaving condition, as specified in 5.5.4.2, is fulfilled for the serving cell associated with cellId for all measurements taken during timeToTrigger; or

3>	if threshold within csi-LoggedMeasurementEventTriggerConfig is set to belowThreshold and the leaving condition, as specified in 5.5.4.3, is fulfilled for the serving cell associated with cellId for all measurements taken during timeToTrigger:

4>	stop performing the logging for the corresponding CSI logged measurement configuration within csi-LoggedMeasurementConfigToAddModList;

2>	when performing the logging:

3>	if the cell identity of the serving cell for which the measurements shall be logged, i.e. the serving cell associated with the serving cell configuration in which csi-LoggedMeasurementConfigToAddModList is received, is not included in an entry in csi-LogMeasInfoCellList in VarCSI-LogMeasReport:

4>	include an entry in csi-LogMeasInfoCellList in VarCSI-LogMeasReport;

4>	set cellId to the CGI of the serving cell associated with the serving cell configuration in which csi-LoggedMeasurementConfigToAddModList is received, if available. If the CGI is not available for that cell, set cellId to the ARFCN and PCI of the serving cell;

3>	if not already present, include an entry in csi-LogMeasInfoList in VarCSI-LogMeasReport and set refCSI-LoggedMeasurementConfigId to the csi-	LoggedMeasurementConfigId associated to the CSI logged measurement configuration included in csi-LoggedMeasurementConfigToAddModList;

3>	for each CSI logged measurement configuration associated to refCSI-LoggedMeasurementConfigId in csi-LogMeasInfoList in VarCSI-LogMeasReport:

4>	set the csi-RS-MeasResultList and SSB-MeasResultList to include the quantities the UE is logging measurements for, upon receiving the quantities from the lower layers;

4>	if the time between the measurements that are logged and included in this instance of csi-LogMeasInfoList and the measurements for the previous instance of csi-LogMeasInfoList with the same refCSI-LoggedMeasurementConfigId, for the same serving cell, is longer than the logging periodicity (if configured) or the periodicity of the measurement resources (if the logging periodicity is not configured):

5>	set the timeGap to true;

2>	when the memory reserved for the logged measurement information for data collection becomes full, stop logging;

2>	when the memory reserved for the logged measurement information for data collection is no longer full, resume logging.

TP for Z005/H009

#### 5.5x.3.2	Initiation

The UE shall:

1>	for each CSI logged measurement configuration associated with a refCSI-LoggedMeasurementConfigId in csi-LogMeasInfoList in VarCSI-LogMeasReport, perform the logging of measurements for the serving cell associated with cellId, in accordance with the corresponding CSI logged measurement configuration within csi-LoggedMeasurementConfigToAddModList:

2>	if the csi-LoggedMeasurementEventTriggerConfig is not included and the buffer, AIML for network-side data collection is not full:

3> instruct lower layer to start the L1 measurement in accordance with the corresponding CSI logged measurement configuration as specified in TS 38.214 [19];

3>	perform the logging at regular time intervals, according to loggingPeriodicity (if present) or according to the periodicity of the resources indicated by csi-LoggedResourceConfig in the corresponding CSI logged measurement configuration within csi-LoggedMeasurementConfigToAddModList, if loggingPeriodicity is not present;

2>	if the csi-LoggedMeasurementEventTriggerConfig is included and the buffer for network-side data collection is not full:

3>	if threshold within csi-LoggedMeasurementEventTriggerConfig is set to aboveThreshold and the entering condition, as specified in 5.5.4.2, is fulfilled for the serving cell associated with cellId for all measurements taken during timeToTrigger; or

3>	if threshold within csi-LoggedMeasurementEventTriggerConfig is set to belowThreshold and the entering condition, as specified in 5.5.4.3, is fulfilled for the serving cell associated with cellId for all measurements taken during timeToTrigger:

4> instruct lower layer to start the L1 measurement in accordance with the corresponding CSI logged measurement configuration as specified in TS 38.214 [19];

4>	perform the logging at regular time intervals, according to loggingPeriodicity (if present) or according to the periodicity of the resources indicated by csi-LoggedResourceConfig in the corresponding CSI logged measurement configuration within csi-LoggedMeasurementConfigToAddModList, if loggingPeriodicity is not present;

3>	if threshold within csi-LoggedMeasurementEventTriggerConfig is set to aboveThreshold and the leaving condition, as specified in 5.5.4.2, is fulfilled for the serving cell associated with cellId for all measurements taken during timeToTrigger; or

3>	if threshold within csi-LoggedMeasurementEventTriggerConfig is set to belowThreshold and the leaving condition, as specified in 5.5.4.3, is fulfilled for the serving cell associated with cellId for all measurements taken during timeToTrigger:

4> instruct lower layer to stop the L1 measurement in accordance with the corresponding CSI logged measurement configuration as specified in TS 38.214 [19];

4>	stop performing the logging for the corresponding CSI logged measurement configuration within csi-LoggedMeasurementConfigToAddModList;

2>	when performing the logging:

3>	for each CSI logged measurement configuration associated to refCSI-LoggedMeasurementConfigId in csi-LogMeasInfoList in VarCSI-LogMeasReport:

4>	set the csi-RS-MeasResultList and SSB-MeasResultList to include the quantities the UE is logging measurements for, upon receiving the quantities from the lower layers;

4>	if the time between the measurements that are logged and included in this instance of csi-LogMeasInfoList and the measurements for the previous instance of csi-LogMeasInfoList with the same refCSI-LoggedMeasurementConfigId, for the same serving cell, is longer than the logging periodicity (if configured) or the periodicity of the measurement resources (if the logging periodicity is not configured):

5>	set the timeGap to true;

2>	when the memory reserved for the logged measurement information for data collection becomes full, instruct lower layer to stop the L1 measurement as specified in TS 38.214 [19],stop logging;

2>	when the memory reserved for the logged measurement information for data collection is no longer full, resume logging.

TP for Z007

csi-LogMeasReportReq

This field is used to indicate whether the UE shall report information about CSI radio measurements logged in RRC_CONNECTED for network-side data collection. This information element shall be absent if the logMeasReportReq is present in this message.

TP for J008

- 5.7.10.3	Reception of the UEInformationRequest message

Upon receiving the UEInformationRequest message, the UE shall, only after successful security activation:

1>	if the idleModeMeasurementReq is included in the UEInformationRequest and the UE has stored VarMeasIdleReport that contains measurement information concerning cells other than the PCell:

2>	if validatedMeasurementsReq is included in the UEInformationRequest and measIdleValidityDuration is included in VarEnhMeasIdleConfig;

3>	set the measResultIdleEUTRA in the UEInformationResponse message to the value of measReportIdleEUTRA in the VarMeasIdleReport for any valid measurement results, if available, and set validityStatus to the value of measIdleValidityDuration in VarEnhMeasIdleConfig for each reported measurement;

3>	set the measResultIdleNR in the UEInformationResponse message to the value of measReportIdleNR in the VarMeasIdleReport for any valid measurement results, if available, and set validityStatus to the value of measIdleValidityDuration in VarEnhMeasIdleConfig for each reported measurement;

3>	discard the VarMeasIdleReport upon successful delivery of the UEInformationResponse message confirmed by lower layers;

3>	remove the measIdleValidityDuration in VarEnhMeasIdleConfig;

2>	else:

3>	set the measResultIdleEUTRA in the UEInformationResponse message to the value of measReportIdleEUTRA in the VarMeasIdleReport, if available;

3>	set the measResultIdleNR in the UEInformationResponse message to the value of measReportIdleNR in the VarMeasIdleReport, if available;

3>	discard the VarMeasIdleReport upon successful delivery of the UEInformationResponse message confirmed by lower layers;

3>	remove the measIdleValidityDuration in VarEnhMeasIdleConfig, if stored;

1>	if the reselectionMeasurementReq is included in the UEInformationRequest:

2>	if validatedMeasurementsReq is included in the UEInformationRequest and measReselectionValidityDuration is included in VarMeasReselectionConfig;

3>	if measReselectionCarrierListNR is present in VarMeasReselectionConfig:

4>	set the measResultReselectionNR in the UEInformationResponse message the valid NR measurement results, if available for any frequency listed in measReselectionCarrierListNR in VarMeasReselectionConfig and set validityStatus to the value of measIdleValidityDuration in VarMeasReselectionConfig for each reported measurement;

3> else:

4>	set the measResultReselectionNR in the UEInformationResponse message to any valid NR measurement results, if available, and set validityStatus to the value of measIdleValidityDuration in VarMeasReselectionConfig;

2>	else:

3>	if measReselectionCarrierListNR is present in VarMeasReselectionConfig:

4>	set the measResultReselectionNR in the UEInformationResponse message the NR measurement results, if available for any frequency listed in measReselectionCarrierListNR in VarMeasReselectionConfig;

3>	else:

4>	set the measResultReselectionNR in the UEInformationResponse message to any NR measurement results, if available;

1>	if the logMeasReportReq is present and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport, or if the current registered SNPN identity is included in snpn-ConfigID-List stored in VarLogMeasReport:

2>	if VarLogMeasReport includes one or more logged measurement entries, set the contents of the logMeasReport in the UEInformationResponse message as follows:

3>	include the absoluteTimeStamp and set it to the value of absoluteTimeInfo in the VarLogMeasReport;

3>	include the traceReference and set it to the value of traceReference in the VarLogMeasReport;

3>	include the traceRecordingSessionRef and set it to the value of traceRecordingSessionRef in the VarLogMeasReport;

3>	include the tce-Id and set it to the value of tce-Id in the VarLogMeasReport;

3>	include the logMeasInfoList and set it to include one or more entries from the VarLogMeasReport starting from the entries logged first, and for each entry of the logMeasInfoList that is included, include all information stored in the corresponding logMeasInfoList entry in VarLogMeasReport;

3>	if the VarLogMeasReport includes one or more additional logged measurement entries that are not included in the logMeasInfoList within the UEInformationResponse message:

4>	include the logMeasAvailable;

4>	if bt-LocationInfo is included in locationInfo of one or more of the additional logged measurement entries in VarLogMeasReport that are not included in the logMeasInfoList within the UEInformationResponse message:

5>	include the logMeasAvailableBT;

4>	if wlan-LocationInfo is included in locationInfo of one or more of the additional logged measurement entries in VarLogMeasReport that are not included in the logMeasInfoList within the UEInformationResponse message:

5>	include the logMeasAvailableWLAN;

1>	if ra-ReportReq is set to true and the UE has random access related information available in VarRA-Report and if the RPLMN is included in plmn-IdentityList stored in VarRA-Report; or

1>	if ra-ReportReq is set to true and the UE has random access related information available in VarRA-Report and if the registered SNPN identity is included in snpn-IdentityList stored in VarRA-Report:

2>	set the ra-ReportList in the UEInformationResponse message to the value of ra-ReportList in VarRA-Report;

2>	discard the ra-ReportList from VarRA-Report upon successful delivery of the UEInformationResponse message confirmed by lower layers;

1>	if rlf-ReportReq is set to true:

2>	if the UE has radio link failure information or handover failure information available in VarRLF-Report and if the RPLMN is included in plmn-IdentityList stored in VarRLF-Report; or

2>	if the UE has radio link failure information or handover failure information available in VarRLF-Report and if the current registered SNPN identity is included in snpn-IdentityList stored in VarRLF-Report:

3>	set timeSinceFailure in VarRLF-Report to the time that elapsed since the last radio link failure or handover failure in NR;

3>	set the rlf-Report in the UEInformationResponse message to the value of rlf-Report in VarRLF-Report;

3>	discard the rlf-Report from VarRLF-Report upon successful delivery of the UEInformationResponse message confirmed by lower layers;

2>	else if the UE is capable of cross-RAT RLF reporting as defined in TS 38.306 [26] and has radio link failure information or handover failure information available in VarRLF-Report of TS 36.331 [10] and if the RPLMN is included in plmn-IdentityList stored in VarRLF-Report of TS 36.331 [10]:

3>	set timeSinceFailure in VarRLF-Report of TS 36.331 [10] to the time that elapsed since the last radio link failure or handover failure in EUTRA;

3>	set failedPCellId-EUTRA in the rlf-Report in the UEInformationResponse message to indicate the PCell in which RLF was detected or the source PCell of the failed handover in the VarRLF-Report of TS 36.331 [10];

3>	set the measResult-RLF-Report-EUTRA in the rlf-Report in the UEInformationResponse message to the value of rlf-Report in VarRLF-Report of TS 36.331 [10];

3>	discard the rlf-Report from VarRLF-Report of TS 36.331 [10] upon successful delivery of the UEInformationResponse message confirmed by lower layers;

1>	if connEstFailReportReq is set to true and the UE has connection establishment failure or connection resume failure information in VarConnEstFailReport or VarConnEstFailReportList and if the RPLMN is equal to plmn-Identity stored in VarConnEstFailReport or in at least one of the entries of VarConnEstFailReportList:

1>	if connEstFailReportReq is set to true and if the UE has connection establishment failure information or connection resume failure information available in VarConnEstFailReport or VarConnEstFailReportList and if the registered SNPN identity is equal to snpn-Identity in networkIdentity stored in VarConnEstFailReport or any entry of VarConnEstFailReportList:

2>	set timeSinceFailure in VarConnEstFailReport to the time that elapsed since the last connection establishment failure or connection resume failure in NR;

2>	set the connEstFailReport in the UEInformationResponse message to the value of connEstFailReport in VarConnEstFailReport;

2>	if the UE supports multiple CEF report:

3>	for each connEstFailReport in the connEstFailReportList in VarConnEstFailReportList:

4>	set timeSinceFailure to the time that elapsed since the associated connection establishment failure or connection resume failure in NR;

2>	for each connEstFailReport in the connEstFailReportList in the UEInformationResponse message, set the value to the value of connEstFailReport in VarConnEstFailReport in VarConnEstFailReportList;

2>	discard the connEstFailReport from VarConnEstFailReport and VarConnEstFailReportList upon successful delivery of the UEInformationResponse message confirmed by lower layers;

1>	if the mobilityHistoryReportReq is set to true:

2>	include the mobilityHistoryReport and set it to include visitedCellInfoList from VarMobilityHistoryReport;

2>	include in the mobilityHistoryReport an entry for the current PCell, possibly after removing the oldest entry if required, and set its fields as follows:

3>	set visitedCellId to the global cell identity or the physical cell identity and carrier frequency of the current PCell:

3>	set field timeSpent to the time spent in the current PCell;

3>	if the UE supports PSCell mobility history information and if visitedPSCellInfoList is present in VarMobilityHistoryReport:

4>	for the newest entry of the PCell in the mobilityHistoryReport, include visitedPSCellInfoList from VarMobilityHistoryReport;

4>	if the UE is configured with a PSCell:

5>	for the newest entry of the PCell in the mobilityHistoryReport, include the current PSCell information in the visitedPSCellInfoListReport, possibly after removing the oldest PSCell entry of a PCell in the mobilityHistoryReport, if required, and set its fields as follows:

6>	set visitedCellId to the global cell identity or the physical cell identity and carrier frequency of the current PSCell:

6>	set field timeSpent to the time spent in the current PSCell while being connected to the current PCell;

4>	else:

5>	for the newest entry of the PCell in the mobilityHistoryReport, include a new entry in the visitedPSCellInfoListReport, possibly after removing the oldest PSCell entry of a PCell in the mobilityHistoryReport, if required, and set its fields as follows:

6>	set field timeSpent to the time spent without PSCell in the current PCell since last PSCell release since connected to the current PCell in RRC_CONNECTED;

3>	else if the UE supports PSCell mobility history information:

4>	if the UE is configured with a PSCell:

5>	for the newest entry of the PCell in the mobilityHistoryReport, include the current PSCell information in the visitedPSCellInfoListReport, possibly after removing the oldest PSCell entry of a PCell in the mobilityHistoryReport, if required, and set its fields as follows:

6>	set visitedCellId to the global cell identity or the physical cell identity and carrier frequency of the current PSCell:

6>	set field timeSpent to the time spent in the current PSCell while being connected to the current PCell;

4>	else:

5>	for the newest entry of the PCell in the mobilityHistoryReport, include a new entry in the visitedPSCellInfoListReport, possibly after removing the oldest PSCell entry of a PCell in the mobilityHistoryReport, if required, and set its fields as follows:

6>	set field timeSpent to the time spent without PSCell in the current PCell since connected to the current PCell in RRC_CONNECTED;

1>	if the successHO-ReportReq is set to true and if the UE has successful handover related information available in VarSuccessHO-Report and if the RPLMN is included in the plmn-IdentityList stored in VarSuccessHO-Report; or

1>	if the successHO-ReportReq is set to true and if the UE has successful handover related information available in VarSuccessHO-Report and if the current registered SNPN identity is included in snpn-IdentityList if stored in the VarSuccessHO-Report:

2>	if the successHO-Report in the VarSuccessHO-Report concerns a DAPS handover and if a PDCP PDU has been received from the source cell of the concerned HO and a non-duplicated PDCP PDU has been received from the target cell of the concerned HO:

3>	set upInterruptionTimeAtHO in VarSuccessHO-Report to include the time elapsed between the time of arrival of the last PDCP PDU received from the source cell of the concerned handover and the time of arrival of the first non-duplicate PDCP PDU received from the target cell of the concerned handover, as measured at the time of arrival of the first non-duplicate PDCP PDU received from the target cell;

2>	if the successHO-Report in the VarSuccessHO-Report concerns a mobilityFromNRCommand:

3>	set timeSinceSHR in VarSuccessHO-Report to the time that elapsed since the execution of the MobilityFromNRCommand associated to the successful handover report in the VarSuccessHO-Report;

2>	set the successHO-Report in the UEInformationResponse message to the value of successHO-Report in the VarSuccessHO-Report, if available;

2>	discard the VarSuccessHO-Report upon successful delivery of the UEInformationResponse message confirmed by lower layers;

1>	if the successPSCell-ReportReq is set to true and if the UE has successful PSCell change or addition information available in VarSuccessPSCell-Report and if the RPLMN is included in plmn-IdentityList stored in VarSuccessPSCell-Report; or

1>	if the successPSCell-ReportReq is set to true and if the UE has successful PSCell change or addition information available in VarSuccessPSCell-Report and if the current registered SNPN identity is included in snpn-IdentityList if stored in the VarSuccessPSCell-Report:

2>	set the successPSCell-Report in the UEInformationResponse message to the value of successPSCell-Report in the VarSuccessPSCell-Report;

2>	discard the VarSuccessPSCell-Report upon successful delivery of the UEInformationResponse message confirmed by lower layers;

1>	if the coarseLocationRequest is set to true:

2>	include coarseLocationInfo, if available;

1>	if the flightPathInfoReq is included in the UEInformationRequest and the UE has (updated) flight path information available, set the flightPathInfoReport in the UEInformationResponse message as follows:

2>	include the list of up to maxWayPointNumber waypoints, if any, along the flight path;

2>	if the includeTimeStamp is set to true, for each included waypoint:

3>	if available, set the field timestamp to the time when UE intends to arrive at the waypoint;

1>	if the csi-LogMeasReportReq is present:

2>	if VarCSI-LogMeasReport includes one or more logged measurements ,set the contents of the csi-LogMeasReport in the UEInformationResponse message as follows:

3>	include the csi-LogMeasInfoCellList and set it to include one or more entries from the VarCSI-LogMeasReport starting from the entries logged first;

4> for each entry in csi-LogMeasInfoCellList, set the field cellId in this entry to the corresponding cellId from VarCSI-LogMeasReport, and include the csi-LogMeasInfoMeasConfigList and set it to include one or more corresponding entries from VarCSI-LogMeasReport starting from the entries logged first;

5> for each entry in csi-LogMeasInfoConfigList, set the field refCSI-LoggedMeasurementConfigId in this entry to the corresponding refCSI-LoggedMeasurementConfigId from VarCSI-LogMeasReport, include csi-LogMeasInfoList and set it to include one or more entries from VarCSI-LogMeasReport starting from the entries logged first;

3>	if the VarCSI-LogMeasReport includes one or more additional logged measurement entries that are not included within the UEInformationResponse message:

4>	include the csi-MoreLogMeasAvailable;

1>	if the logMeasReport is included in the UEInformationResponse:

2>	submit the UEInformationResponse message to lower layers for transmission via SRB2;

2>	discard the logged measurement entries included in the logMeasInfoList from VarLogMeasReport [RIL]: J009 AIML upon successful delivery of the UEInformationResponse message confirmed by lower layers [RIL]: V105 AIML;

1>	else if csi-LogMeasReport is included in the UEInformationResponse:

2>	submit the UEInformationResponse message to lower layers for transmission via SRBX;

2> if the entries in csi-LogMeasInfoCellList from VarCSI-LogMeasReport has been included in the UEInformationResponse and the successful delivery of the UEInformationResponse message confirmed by lower layer:

3> discard the entries in csi-LogMeasInfoCellList from VarCSI-LogMeasReport.

2> if the entries in csi-LogMeasInfoMeasConfigList from VarCSI-LogMeasReport has been included in the UEInformationResponse and the successful delivery of the UEInformationResponse message confirmed by lower layer:

3> discard the entries in csi-LogMeasInfoMeasConfigList from VarCSI-LogMeasReport.

2> if the entries in csi-LogMeasInfoList from VarCSI-LogMeasReport has been included in the UEInformationResponse and the successful delivery of the UEInformationResponse message confirmed by lower layer:

3> discard the entries in csi-LogMeasInfoList from VarCSI-LogMeasReport.

1>	else:

2>	submit the UEInformationResponse message to lower layers for transmission via SRB1.

NOTE: It is up to the network to ensure that logged data based on logMeasReportReq and csi-LogMeasReportReq are not requested in the same message.

TP for H007

- 5.5x.1.3	Reception of CSI-LoggedMeasurementConfig by the UE

Upon receiving csi-LoggedMeasurementConfigToAddModList in the csi-MeasConfig of a serving cell, the UE shall: [RIL]: H007 AIML

1>	for each CSI logged measurement configuration included in csi-LoggedMeasurementConfigToAddModList:

2>	if the current UE configuation for the serving cell includes the CSI logged measurement configuration associated with the given csi-LoggedMeasurementConfigId:

3>	modify the CSI logged measurement configuration according to the configuration received in csi-LoggedMeasurementConfigToAddModList;

2>	else:

3>	add the received CSI logged measurement configuration to the UE configuration;

2>	if the cell identity of the serving cell for which the measurements shall be logged, i.e. the serving cell associated with the serving cell configuration in which csi-LoggedMeasurementConfigToAddModList is received, is not included in an entry in csi-LogMeasInfoCellList in VarCSI-LogMeasReport: [RIL]: Z004, AIML

3>	include an entry in csi-LogMeasInfoCellList in VarCSI-LogMeasReport;

3>	set cellId to the CGI of the serving cell associated with the serving cell configuration in which csi-LoggedMeasurementConfigToAddModList is received, if available. If the CGI is not available for that cell, set cellId to the ARFCN[RIL]: C075, AIML and PCI of the serving cell;

2>	if not already present, include an entry in csi-LogMeasInfoList in VarCSI-LogMeasReport and set refCSI-LoggedMeasurementConfigId to the csi-LoggedMeasurementConfigId associated to the CSI logged measurement configuration included in csi-LoggedMeasurementConfigToAddModList;

2>	perform measurements logging as specified in 5.5x.3.2.

NOTE: It is up to NW to ensure that UE will not log the measurements for a csi-LoggedMeasurementConfigId that is from different csi-LoggedResourceConfig in the VarCSI-LogMeasReport.