3GPP TSG-RAN WG2 Meeting #131-bis																			  R2-2507303

Prague, Czech Republic, Oct 13 - 17, 2025

Source: 	      ZTE Corporation, Sanechips

Title:                 Design principles for 6G

Agenda item:   10.2

Document for: Discussion and Decision

# Introduction

This contribution highlights high-level principles, guidelines specific to RAN2 and discusses specific aspects of these for various deployment scenarios and architectures for 6G. The contribution also highlights aspects where there are dependencies with the work going on in other WGs.

# General design principles and usage scenarios

RAN plenary has endorsed the working principle for 6G (see RP-250766) which stipulates the following guidelines:

- 6G should be enabled using a lean and streamline standard

- Functionalities should be dimensioned appropriately whilst minimizing multiple options for the same functionality and avoiding excessive configurations

RAN2 is in charge of the Radio Interface architecture and protocols, the specification of the Radio Resource Control protocol and the Radio Resource Management procedures.  This generally means that RAN2 needs to first identify the essential functionality for these and then design the overall protocol architecture and procedures for 6GR.

The first step to design the protocol and architecture is to study the required functionality. Once the overall functionality to meet the requirements is clear, then RAN2 can discuss how to arrange this functionality in various protocol layers of the 6G specifications. Thus, the general principle for the study is to first discuss functionality and then study the protocol layer architecture, in that order.

Furthermore, in 5G, for the same or similar purpose, various features have been introduced, which dramatically increases the spec complexity and leads to market fragmentation. For example, for mobility enhancement, several mobility methods have been specified in 5G, e.g. L3 HO, CHO, (S)CPAC, LTM, CLTM, etc. Both CHO and CLTM are introduced for robustness improvement. Similarly, LTM was introduced for latency reduction, however, in late release, RACH-less mechanism is also supported for L3 HO for latency reduction. Another example is about multi-carrier handling (e.g. SUL and CA), Initially, SUL was introduced to enhance uplink coverage, while CA primarily aimed to improve resource utilization and throughput. Both technologies are based on the assumption of a common MAC scheduler for multi-carrier operations. However, as technology has evolved, the boundaries between CA and SUL have become increasingly blurred, with both technologies seeking enhancements in coverage and throughput. This overlap increases the risk of standard and market fragmentation, leading to unnecessary complexity. Hence, to achieve a streamlined design and minimize complexity in both standardization and implementation, in 6G, RAN2 should firstly study the different requirements, and for the same or similar requirements, a unified solution and framework should be pursued.

Proposal 1:	RAN2 study should first focus on the required functionality in 6GR, and only once the functionality is stable, the overall protocol layer architecture and the distribution of functionality across the different protocol layers should be studied with an aim to avoid inter-layer redundancy/overhead for similar functionality

Proposal 2:	Unique functionality should be developed for satisfying a given set of similar 6GR requirements by avoiding defining multiple solutions to satisfy requirements which are similar (e.g., mobility and multi-carrier operation).

For 6G the overall protocol structure will strongly depend on the migration decision in plenary (including the decision on inter-RAT and 6G-6G DC) and the decision on CU/DU split architecture in RAN3. The timeline for this discussion in plenary is as follows.

Figure 1: 6G study timeline for RAN2

The check point for CU/DU split and the check point for DC will have significant implications on overall design decisions in RAN2. These checkpoints only come towards the later part of the study (see Figure 1). In the meanwhile, RAN2 protocol design should proceed and we have to design a system that is optimal based on the current assumptions on the table. RAN2 agreements in the meanwhile should ensure an optimal protocol design for standalone operation whilst not precluding optimal operation in either CU/DU split architecture or DC (including MR-DC) as there will be limited time left (i.e. only 3 quarters) in case we have to do complete redesign of the system for optimal support of these features. In other words, the RAN2 design should enable an optimal protocol stack for operation without DC and without CU/DU split, whilst ensuring forward compatibility with both CU/DU split and DC.

Observation 1: Optimal support of CU/DU split and DC (including inter-RAT DC for migration) will have substantial impact on RAN2 protocol design

Proposal 3:	Interim RAN2 decisions on overall protocol design (before the check points for DC and CU/DU split) should ensure an optimized operation for non-DC and non-CU/DU split architecture whilst ensuring forward compatibility with optimal support for both DC (MR-DC / 6G-DC) and CU/DU split.

When 5G study was initiated, the focus was mainly on identifying the key components of the RAN protocol architecture that are needed to support the requirements for the key usage scenarios including eMBB, URLLC and mMTC. With 6G, in addition to the extension of the above scenarios, there are 3 new day1 scenarios that need to be considered: Ubiquitous connectivity (TN/NTN integration), AI and Sensing. This is depicted in Figure 1 below.

Figure 2: 6G usage scenarios

Then, with respect to the functionality, focus of RAN2 study should be aligned with the usage scenarios highlighted in Figure 2. Specifically, where the functionality is currently there in 5G (especially from Rel-15), the aim of the study should be to reuse the functionality whilst making it streamlined, eliminating multiple options and simplifying the implementations. On the other hand, new functionality will be needed to support new day 1 services such as those which are needed for supporting the Ubiquitous Connectivity, AI and Sensing.

Going into the details, the high-level usage scenarios above may be mapped into various sub features which need to be studied in RAN2 as day 1 features.

- NTN: Aim to integrate in a harmonized way from day1 into the protocol stack without creating exceptions for NTN

- HRLLC: Including support for TSN (largely reusing the 5G framework), i.e. no need to spend too much time during the study item on this.

- SON/MDT: Support should be possible from day1 and mechanisms from 5G can be largely reused (i.e. not much to study during the study item)

- Slicing: Should be supported from day1 but we should wait for the framework from SA2 to be clarified first to understand the detailed design in RAN2

- Relays/repeaters and RIS: Reuse the framework developed in 5G and study any specific enhancements needed to support RIS

- Specific use cases for high mobility: UAV, HST.

- AI/ML related use cases

- Sensing scenarios: New study is required but wait for the initial framework from RAN1 to be finalised first.

Proposal 4:	For 6GR, RAN2 study shall cover the use cases including NTN, HRLLC, SON/MDT, Slicing, Relays (including repeaters and RIS), UAV, HST, AI/ML and Sensing scenarios

Proposal 5: 	For the existing use cases in 5G, RAN2 study should identify at an early stage, functionality that can be reused from 5G as a baseline to minimize the additional work in these areas (e.g. for SON/MDT, TSN, Slicing – depending on SA2 output etc) and focus on functionality that needs to be studied from scratch for 6G (e.g. Sensing)

# Protocol architecture

The protocol architecture for 6GR should support RAN functions to support CP and UP data transfer. In addition, RAN functionality should also support NAS data transfer. The overall protocol architecture will depend on the following:

- Overall system architecture (SA2):

  - Overall QoS framework for 6G will have significant impact on the RAN protocol architecture. In particular, we need to understand whether the QoS-flow based architecture will be largely reused or not.

  - Functional split between CN and RAN, specifically the anchor points for AS and NAS transfer and session management framework

  - General framework for AS/NAS interactions

- RAN architecture (RAN3):

  - The main dependency on RAN3 is whether CU/DU split exists in 6G. This will have significant impact on our protocol design both for UP and CP

- Security architecture (SA3):

  - RAN Security requirements both CP and UP will be needed

  - Any new security requirements for 6GR, e.g. security for system information, security for lower layer signalling messages etc need to be clarified

Regardless of the above dependencies, as a minimum, RAN2 protocol design shall still support CP/UP data transfer and shall also support efficient mechanisms to carry NAS signalling messages. Furthermore, the protocol architecture should also support transfer of a wide variety of data including user plane data (IP traffic), control signalling (that terminates in RAN and in CN – NAS) and other data including data for sensing and AI which either terminates in RAN or in CN.

Proposal 6:	The RAN2 protocol architecture for 6GR should support separation of CP/UP data transfer and functionality to transport signalling (Radio signalling and NAS signalling) and data (UP data, data for sensing/AI). The design should allow termination points for both data and signalling to be either in RAN or in CN.

# Deployment scenarios

In the 5G early discussion, to support 4G-5G migration, the discussion was split into “early phase (EN-DC)” and “late drop (Standalone and other MR-DC cases)”. But for 6G study, the common understanding that 6GR standalone will be supported from Day 1.

For 5G-6G migration, there are two main migration paths:

- 5G-6G spectrum sharing (MRSS);

- 5G-6G Dual connectivity (MR-DC).

Figure 2: 6G deployment scenarios

Details of MRSS, are now under RAN1 discussion, as it impacts the design of 6GR numerology and frame structure, such as, whether the common/compatible SCS and/or aligned TDD switch points should be ensured for 5G-6G MRSS. Moreover, due to the TDD/FDD multiplexing of 5G/6G resources, RAN1 also needs to study the efficient rate matching mechanism in 6GR. Technically, these features do not impact RAN2 function design.

However, it is still unclear in RAN1 whether the shared SSB will be supported for 5G-6G MRSS. If this is not supported, which means 5G and 6G SSB can be configured separately, then rate matching mechanism can be applied. But, if shared SSB is supported, it will impact the design of system information (e.g. MIB, SIB1). For other shared DL RSs (e.g. CSI-RS), no RAN2 impact is foreseen. Therefore, the RAN2 study on the impact of MRSS should wait for RAN1 progress, specifically until RAN1 confirms the support of shared SSB for MRSS.

Proposal 7: 	The support of shared SSB in MRSS will impact the design in RAN2 (e.g. for system information and initial access). RAN2 should postpone the discussion on MRSS-specific enhancements (e.g. shared reference signal or common signaling) pending input from RAN1.

# Conclusion

Proposal 1:	RAN2 study should first focus on the required functionality in 6GR, and only once the functionality is stable, the overall protocol layer architecture and the distribution of functionality across the different protocol layers should be studied with an aim to avoid inter-layer redundancy/overhead for similar functionality

Proposal 2:	Unique functionality should be developed for satisfying a given set of similar 6GR requirements by avoiding defining multiple solutions to satisfy requirements which are similar (e.g., mobility and multi-carrier operation).

Observation 1: Optimal support of CU/DU split and DC (including inter-RAT DC for migration) will have substantial impact on RAN2 protocol design

Proposal 3:	Interim RAN2 decisions on overall protocol design (before the check points for DC and CU/DU split) should ensure an optimized operation for non-DC and non-CU/DU split architecture whilst ensuring forward compatibility with optimal support for both DC (MR-DC / 6G-DC) and CU/DU split.

Proposal 4:	For 6GR, RAN2 study shall cover the use cases including NTN, HRLLC, SON/MDT, Slicing, Relays (including repeaters and RIS), UAV, HST, AI/ML and Sensing scenarios

Proposal 5: 	For the existing use cases in 5G, RAN2 study should identify at an early stage, functionality that can be reused from 5G as a baseline to minimize the additional work in these areas (e.g. for SON/MDT, TSN, Slicing – depending on SA2 output etc) and focus on functionality that needs to be studied from scratch for 6G (e.g. Sensing)

Proposal 6:	The RAN2 protocol architecture for 6GR should support separation of CP/UP data transfer and functionality to transport signalling (Radio signalling and NAS signalling) and data (UP data, data for sensing/AI). The design should allow termination points for both data and signalling to be either in RAN or in CN.

Proposal 7: 	The support of shared SSB in MRSS will impact the design in RAN2 (e.g. for system information and initial access). RAN2 should postpone the discussion on MRSS-specific enhancements (e.g. shared reference signal or common signaling) pending input from RAN1.

- References