3GPP TSG-RAN WG2 Meeting #131-Bis	R2-2507308

Prague, Czech, 13 – 17 October, 2025

Agenda item:	8.4.3

Source:	InterDigital

Title:	LP-WUS options description

WID/SID:	NR_LPWUS-Core - Release 19

Document for:	Discussion and Decision

# 1	Introduction

This TDoc discusses whether the LP-WUS options (namely Option 1-1 and 1-2) needs to be used in the TS 38.300 and TS 38.321.

# 2	Discussion

In the RAN2#131 post-meeting email discussion on LP-WUS open issues, a question was raised if the LP-WUS options (namely Option 1-1 and Option 1-2) should be used in the normative text of TS 38.300 or TS 38.321. The differentiation between these two options in TS 38.321 is based on whether the lpwus-PDCCH-MonitoringTimer is configured or not corresponding to Option 1-2 and Option 1-1, respectively. In TS 38.300 the following description is provided differentiating these options:

# 11 UE Power Saving (…) A UE configured with DRX in RRC_CONNECTED can be configured with LP-WUS. LP-WUS is monitored outside active-time. If LP-WUS is detected, the UE shall start the on-duration timer or LP-WUS PDCCH monitoring timer to start PDCCH monitoring and enter active-time: - If the UE is configured to start on-duration timer after LP-WUS reception, the UE monitors LP-WUS at occasion(s) at a configured offset before the on-duration, and the UE does not monitor LP-WUS when short DRX cycle is used. If the UE is unable to monitor the LP-WUS occasion, it shall start the on-duration timer. - If the UE is configured to start LP-WUS PDCCH monitoring timer after LP-WUS reception, the UE monitors LP-WUS at occasion(s) according to the configured periodicity and offset which can be same or different from the periodicity and offset configured for C-DRX cycle, and the UE monitors LP-WUS regardless of which DRX cycle is used. It the UE is unable to monitor the LP-WUS occasion(s), the LP-WUS PDCCH monitoring timer is not started.

Generally, it is not optimal to use the working names of different options in the eventual technical specifications as those names have context only for people who worked on specifying these options (ie., 3GPP delegates). Since TS 38.213 used these options naming convention in their specification, it seem ok the TS 38.331 may use such naming convention since it directly refers to TS 38.213 sections. However, in terms of TS 38.321, the usage of lpwus-PDCCH-MonitoringTimer unambiguously differentiates these options without any cross-referencing to different TSes. Furthermore, Stage-2 (TS 38.300) description is meant to be a general description of the defined feature and we don’t see any issues with the current description in differentiating the different LP-WUS options.

Proposal: The usage of LP-WUS Option 1-1 and Option 1-2 naming convention is not applied for TS 38.300 nor TS 38.321.

# 3	Conclusion

This document has made the following observations and proposals:

Proposal: The usage of LP-WUS Option 1-1 and Option 1-2 naming convention is not applied for TS 38.300 nor TS 38.321.